Pactor/Amtor/RTTY Multimode HF software

0. License

----------

This software is copyrighted 1993-94 by Tom Sailer, HB9JNX, Weinbergstrasse 76,

8408 Winterthur, Switzerland. You may not use this program for purposes other

than amateur radio. You may not modify this program. You may share this

program, provided you do not charge any fee for use, copying or distributing

and you do not remove or modify any file. Commercial use or distribution is

strictly prohibited without the   w r i t t e n   p e r m i s s i o n   of

the author.

The author hereby disclaims all warranties relating to this software, whether

express or implied, including without limitation any implied warranties of

merchantability or fitness for a particular purpose. The author will not be

liable for any special, incidental, consequential, indirect or similar damages

due to loss of data or any other reason. The person using the software bears

all risk as to the quality and performance of the software.

Files belonging to the DJ6HP version

TERMD    EXE        ALLMODE  INI        LICENSE  TXT        MANUAL   DOC

Files belonging to the AN-93 version

TERMAN93 EXE        ALLMODE  INI        LICENSE  TXT        MANUAL   DOC

Files belonging to the DSP version

CONV170  DSP        CONV200  DSP        CONV425  DSP        CONV850  DSP

CONVM    DSP        CONVM200 DSP        FFTSAMPL DSP        TERM     EXE

ALLMODE  INI        LICENSE  TXT        MANUAL   DOC

Files belonging to the DG1SCR version

FFT_SCR  DSP        FFT_SCRS DSP        CONVMSCR DSP        ALLMODE  INI

LICENSE  TXT        TERMS    EXE        MANUAL   DOC

1. General overview

-------------------

The HB9JNX HF RTTY/Amtor/Pactor software allows you to operate in these

modes using cheap hardware. All the protocols are handled by the computer.

Therefore you need at least a 286 computer, 386 should be better.

There are currently two versions of this software: One for use with a

DJ6HP-like analog converter, the other one for use with the Texas Instruments

DSP Starters Kit with the TMS320C26 (Of course, you can use any hardware

with the TMS320C26 and the TLC32040 whit 40MHz clock frequency). The

DSP version is the preferred one. Its memory ARQ (in Pactor) performs much

superior to that in the DJ6HP version, since the demodulator data is

transferred to the computer with a high precision instead of just "0" or "1".

Additionally, it allows you to use different mark/space separations (useful

to copy commercial RTTY stations) and allows you to set the mark and space

frequencies anywhere in the audio passband of your transceiver. Another

plus is the spectrum display. The demodulator software is what is called

"matched filters", which are the ideal noncoherent FSK demodulator.

This document assumes that you are already familiar with RTTY, Amtor and

Pactor (Pactor is not very different from Amtor from the operators point of

view, except that you are encouraged to type in lower case, since lower case

letters are compressed better). It assumes too that you are familiar with

expressions such as mark frequency, space frequency, how to calculate

the transceiver frequency to receive a particular mark frequency, baud

rates, etc. If not, get help from a nearby ham who's experienced with these

things.

1.1. Connection of the hardware

-------------------------------

1.1.1. DSP Version

------------------

Connect the DSP Starters Kit to the computer as you would for use with the

Texas Instruments debugger. If you've built the hardware yourself, you must

wire the XF line to the RS232 RXD, the BIO line to the RS232 TXD and the

RS line to the RS232 DTR. Since the DSP is a 5V device and RS232 uses +-12V,

you have to USE APPROPRIATE LINE DRIVERS (such as 1488/1489). You should

invert RXD and TXD once, and DTR twice. (If you inverted DTR only once,

change the ComDTRInvert entry in the allmode.ini file, see also Appendix A)

Connect the Transceiver fixed level audio output (if not available, the try

the phones output, but this may be too slow) to the AIC input or auxin, and

the AIC output to the transceiver input. You may adjust the modem output

level in the allmode.ini file, but it is recommended to use an external

resistor divider network. The transmitter is keyed via the RS232 RTS line.

ATTENTION: +-12V. I recommend the use of transformers for the audio input and

output and an optocoupler for RTS, in order to separate the transceiver

and the computer/modem galvanically.

1.1.2. DJ6HP filter converter version

-------------------------------------

In order to operate Pactor, you need first to make the filter bandwidths

larger. Consult the local Packet BBS. Then connect the converter to

a parallel printer port as follows:

- Converter (digital) output to the BUSY line (Pin 11) (low is mark)

- (A)FSK generator to DATA6 (Pin 8) (high is mark)

- PTT to DATA7 (Pin 9). (high is transmit)

- Ground is Pin 18-26.

1.1.3. DG1SCR DSP version

-------------------------

Connect the DG1SCR board to the serial interface. In addition, THE HOST

MUST BE ABLE TO RESET THE DSP USING THE DTR LINE! A simple circuit could look

like this: connect the emitter of a general purpose PNP transistor (such as

BC 560, or 2N2907 I think) to ground, the collector to pin 15 of the MAX691

and the base over a resistor of, say, 10kOhms to the DTR line.

The DG1SCR version uses the same settings as the DSK DSP version, but the

converter files (*.DSP) are different!

1.1.4. AN93 modem version

-------------------------

The AN-93 modem is a state of the art analog modem, designed by Johan

Forrer, KC7WW, and published in QEX issue ??. PTT is controlled via

the DTR line, FSK keying via either TxD or DTR (both are driven), and

receive data (digital) is read from the DCD line. The modem has a built

in 5 Bit A/D Converter, which is optionally connected to a LPT port.

1.1.5. Configuring the software

-------------------------------

The configuration is stored in the ALLMODE.INI file. This is an ASCII text

file. See Appendix A for the details on the entrys in this file.

2. Operation

------------

2.1. Transceiver filters

------------------------

Do not use too narrow receiver IF filters, especially with the DSP version.

These filters usually exhibit a very large phase distortion at the edge

of their passband. Do not use filters narrower than 500Hz. Even 500Hz

may be too narrow, so you probably should try it without the filter.

(Remember: the bandwith taken by a 200 Baud Pactor signal is 400Hz).

Adjust the center frequency of your converter to the center frequency

of the receiver filter.

2.2. Organisation of the user interface

---------------------------------------

The user interface is menu driven. The screen is split into 4 areas:

The Rx window is the largest area. It shows you in white what you received

from the QSO partner, and in blue, what you typed and is already transmitted.

The Tx window is only a few lines and shows what you're currently typing.

Note that the word currently typed is not sent until you type a non-letter

character, in order to allow you to correct mistypings in AMTOR and RTTY.

(In Pactor, you may correct as much as you want, since a BACKSPACE signal

can be sent over the air). The Monitor window shows you what is currently

going on on the air. You need to know the protocol basics in order to

understand what this means. The same holds for the link state window.

The menu is invoked via the F2 key. You may then select every item with

the arrow keys and the ENTER key which is light white. The gray white

items are disabled. Submenus are indicated by an arrow pointing to the right.

I won't explain every menu item, since operation should be quite straight

forward. One note though: On Startup, nothing happens at first. If you want

to listen to a QSO in Amtor or Pactor, you will have to press F4, or if

you want to print RTTY, you will have to press F7. The same can be

accomplished via the menu.

2.3. Tuning

-----------

Tuning in the DJ6HP version should be done with an external indicator, such

as an X-Y-scope. Tuning in the DSP version can be accomplished in two ways:

The spectrum display can be used to tune and to determine the spacing between

mark and space. (Note: RTTY and Amtor usually have 170Hz, Pactor has 200Hz,

but Amtor and RTTY with the PTC have also 200Hz, in short 170Hz and 200Hz

are mixed randomly in the amateur community). During a QSO or for fast tuning

you may use the tuning indicator at the top right of the screen.

2.4. Pactor quirks

------------------

In the Pactor section, there a few things are implemented uncommonly.

There is no automatic algorithm for speed change. That is, you have to

switch manually to 200 baud. Speed down is requested if a packet cannot

be correctly received during 4 cycles. Note that you can only request a

speed up if you are the information receiving station (IRS). The reason

for this is that observations on the band have shown that the original

PTC looses a lot of time trying to speed up without success if the condx

are quite bad. Given the information in the link state window, it should

be quite easy (after you've made some experiences) to decide if 200 bauds

could work. If you try to speed up and fail, the software automatically

falls back to 100 baud. I think this limitation is not too severe since

in any case this terminal software cannot be used for unattended operation.

Pactor disconnect may fail, i.e. if you want to disconnect the other end

you and you miss the first acknowledge, your end tries to disconnect until

the retry counter reaches zero. This is due to (yet another) bug in the

original PTC. Hopefully this will be fixed in the PTCplus design.

(N.B. this does not work correctly with two original PTCs too). You may

stop transmitting QRT packets by manually pressing F4 (for Listen mode)

Appendix

--------

A. ALLMODE.INI File format

--------------------------

A.1. General Format

-------------------

This file is an ordinary ASCII Text file, without any control sequences. It

is organised in sections. The section names are enclosed in square brackets.

[section]

key=value

...

If value is a boolean, then it may be either 0,F,1 or T. Integer numbers may

be either decimal or hexadecimal, terminated with a "H".

A.2. Section [DSP_DSK] (only available in the DSP TI DSK, TERM.EXE, version)

----------------------

AICLevel=0

This parameter specifies the adc input range.

0 = -1.5 .. 1.5 Volts

1 = -3 .. 3 Volts

2 = -6 .. 6 Volts

AICAuxIn=T

This parameter specifies whether the normal input (F) or the auxilary

input (T) should be used.

ConvTXAttenuation=15

This parameter specifies the attenuation of the audio output level (in

decibels, not negative and integer).

ConvAdjust=9500H

In the DSP version, the hole timing is controlled by the DSP. This

parameter may be used to adjust the clock of the DSP board. Increase it

if the clock is too slow. An indication for this is the "dt" value in the

status display, if it is mostly negative, WHEN YOU HAVE BEEN CONNECTED, I.E.

AMTOR SLAVE OR PACTOR SLAVE MODE, then try to increase this value

(and vice versa)

Converters=*.DSP

This is the path and file spec of the DSP software files.

ComRTSInvert=F

The RTS line is used as the PTT. Specifies whether you're using

negative or positive logic.

ComDTRInvert=F

The DTR line is used to reset the DSP. Specifies whether you're using

negative or positive logic. (Use F for the standard TI DSK board)

ComInt=4

Interrupt line for the com port. Only Ints from 3 to 7 are supported.

4 for COM1 and COM3, 3 for COM2 and COM4

ComAddr=3F8H

Address of the com port. 3F8H for COM1, 3E8H for COM2, 2F8H for COM3,

and 2E8H for COM4

A.3. Section [DSP_DG1SCR] (only available in the DSP DG1SCR, TERMS.EXE, version)

-------------------------

ConvAdjust=8000H

In the DSP version, the hole timing is controlled by the DSP. This

parameter may be used to adjust the clock of the DSP board. Increase it

if the clock is too slow. An indication for this is the "dt" value in the

status display, if it is mostly negative, WHEN YOU HAVE BEEN CONNECTED, I.E.

AMTOR SLAVE OR PACTOR SLAVE MODE, then try to increase this value

(and vice versa)

Converters=*.DSP

This is the path and file spec of the DSP software files.

ComRTSInvert=F

The RTS line is used as the PTT. Specifies whether you're using

negative or positive logic.

ComDTRInvert=F

The DTR line is used to reset the DSP. Specifies whether you're using

negative or positive logic. (Use F for the standard TI DSK board)

ComInt=4

Interrupt line for the com port. Only Ints from 3 to 7 are supported.

4 for COM1 and COM3, 3 for COM2 and COM4

ComAddr=3F8H

Address of the com port. 3F8H for COM1, 3E8H for COM2, 2F8H for COM3,

and 2E8H for COM4

A.4. Section [DJ6HP] (only available in the DJ6HP, TERMD.EXE, version)

--------------------

ConvAdjust=23

In the DJ6HP version, the hole timing is controlled by the computer clock.

This parameter may be used to adjust the computer clock. Decrease it

if the clock is too slow. An indication for this is the "dt" value in the

status display, if it is mostly negative, WHEN YOU HAVE BEEN CONNECTED, I.E.

AMTOR SLAVE OR PACTOR SLAVE MODE, then try to decrease this value

(and vice versa) (ATTN: It's exactly the other way round as with the

parameter of the same name in the DSP section!!)

LptAddr=3BCH

This is the address of the LPT port. LPT1 is 378H, LPT2 is 278H, and

the LPT port on the hercules graphics card is 3BCH. Connect the

Converter (digital) output to the BUSY line (Pin 11), the (A)FSK

generator to DATA6 (Pin 8) and PTT to DATA7 (Pin 9). Ground is

Pin 18-26.

A.5. Section [AN93] (only available in the AN-93, TERMAN93.EXE, version)

--------------------

ConvAdjust=23

In the DJ6HP version, the hole timing is controlled by the computer clock.

This parameter may be used to adjust the computer clock. Decrease it

if the clock is too slow. An indication for this is the "dt" value in the

status display, if it is mostly negative, WHEN YOU HAVE BEEN CONNECTED, I.E.

AMTOR SLAVE OR PACTOR SLAVE MODE, then try to decrease this value

(and vice versa) (ATTN: It's exactly the other way round as with the

parameter of the same name in the DSP section!!)

ComRTSInv=F

RTS serves as the PTT line. If PTT logic is inverted, you can correct this

with ComRTSInv.

ComDTRInv=F

DTR and TxD serve as FSK Keying line. If the generated tones are inverted,

then you may correct this with ComDTRInv.

ComDCDInv=F

If analog input with LptIO is disabled, data is input via the DCD line.

If DCD logic is reverse, you may correct this with ComDCDInv.

ComAddr=3F8H

Address of the com port. 3F8H for COM1, 3E8H for COM2, 2F8H for COM3,

and 2E8H for COM4

LptAddr=0H

This is the address of the LPT port. LPT1 is 378H, LPT2 is 278H, and

the LPT port on the hercules graphics card is 3BCH. This is the optional

A/D Input port. Address 0 disables analog input.

A.6. Section [FFT] (only available in the DSP versions)

------------------

DefaultFreq=1275

Specifies the default frequency for the cursors in the FFT display.

DefaultMark=F

Specifies whether the above frequency is the mark frequency or the

space frequency. The other frequency is calculated from this frequency

and the shift set in the FFT screen.

Mono=F

If T, forces the use of a monochrome display mode.

ColSpace=10

ColMark=14

ColGrid=4

ColTrace=15

ColText=7

These are different color settings. Range is from 0 to 15, except for

the monochrome modes, where it is from 0 to 1

A.7. Section [RTTY]

-------------------

Alphabet=0

Specifies which alphabet should be used. Currently these alphabets are

defined:

0 = American national flavour of the ITA 2 alphabet

1 = Great Britain national flavour of the ITA 2 alphabet

2 = German national flavour of the ITA 2 alphabet

AutoRYRY=T

If set, when you start RTTY transmitting, repeated RY is transmitted until

you typed the first word.

Bits=5

The number of data bits. Valid are 5,7 and 8.

CRLF=F

Specifies if a LINEFEED should be sent after CARRIAGE RETURN.

Baud=45

The Baudrate, from 45 to 100.

DataReverse=F

Whether the data bits should be inverted.

Reverse=F

Whether all should be inverted.

UnshiftOnSpace=F

Whether the Unshift on space option should be used. If true, space

characters reset to shift state to letter.

A.8. Section [AMTOR]

--------------------

WRU=DE MY0CALL+?

If not empty, this enables the WRU mechanism. This can be useful for some

mailboxes which support the WRU mechanism. After connecting, the mailbox

sends you it's bannertext, followed by the wru? character. Then this

software gets the key and sends the text you've specified. It should

end with +? to give the key back to the mailbox. Using this mechanism

can save you the manual login procedure.

RxAfterTx=20

This value determines how many milliseconds the software should wait before

starting to send in the SLAVE mode. For DX, set it to TxDelay. If you want

to do a short range QSO, set it to about 50.

TxDelay=20

This is the number of milliseconds your transceiver needs to key up

SelCall=NONE

This is your selcall. It consists of exactly 4 letters. It is usually

the concatenation of the first and the last three letters in your callsign.

if you have only three letters in the callsign, you may take the first one

twice. Eg. HB9JNX => HJNX, HB9ZZ => HBZZ, ZS5S => ZZSS

CrLf=T

Whether a LINE FEED should be sent after a CARRIAGE RETURN

CapsOnly=F

Whether lowercase should be enabled.

Alphabet=0

Specifies which Alphabet should be used. Currently these Alphabets are

implemented:

0 = American national flavour of the ITA 2 alphabet

1 = Great Britain national flavour of the ITA 2 alphabet

2 = German national flavour of the ITA 2 alphabet

TxReverse=F

Whether the TX should be inverted.

Reverse=F

Whether mark and space should be exchanged.

A.9. Section [PACTOR]

---------------------

RxAfterTx=50

This value determines how many milliseconds the software should wait before

starting to send in the SLAVE mode. For DX, set it to TxDelay. If you want

to do a short range QSO, set it to about 50. You can leave it quite high,

since Pactor has a LongPath mode for DX operation.

TxDelay=50

This is the number of milliseconds your transceiver needs to key up

MyCall=MY0CALL

Your Callsign (up to 8 characters)

A.10. Section [LISTEN]

----------------------

CheapPactor=F

If you have a slow computer and it crashed when entering the listen mode

or "LOST INTS" is displayed, you may try to set this parameter to T

A.11. Section [PACTOR_CALLS]

----------------------------

Call##=CA0LL

You may specify a list of the calls you use often. The calls entered here

appear in the Pactor Call menu, so you don't have to type them.

## are consecutive numbers starting with 00.

A.12. Section [AMTOR_CALLS]

---------------------------

Call##=CALL

You may specify a list of the amtor selcalls you use often.

The selcalls entered here appear in the Amtor Call menu,

so you don't have to type them.

## are consecutive numbers starting with 00.

A.13. Section [FILES]

---------------------

Desc##=Title

Name##=Filename

This is a list of the text files you need often. These files appear in

the send file menu. ## are consecutive numbers starting with 00.

A.14. Section [CWID]

--------------------

Speed=180

This is the speed (paris) of the CW id. Set it to a high number

(eg. 180 to 240) to keep the interruption of the data transfer

as short as possible.

Interval=600

This is the interval in seconds of the CW id during an ARQ QSO. Set it

to the highest value which is allowed (set it to 0 to disable CW ID-ing

during an ARQ QSO if you needn't do CW id-ing)

Call=MY0CALL

The text which should be sent as CW id. Omit it if you do not want any

CW id.

A.15. Section [Windows]

-----------------------

NoStatus=F

Lets you hide the status and the monitor window