PEROM programming utility for the DSP4

--------------------------------------

By: Rob Janssen, PE1CHL

The DSP card 4 by Alef Null provides for the use of an ATMEL 29c256

Programmable and Erasable Read Only Memory (PEROM) for use as bootstrap

rom and program storage.  Instead, a standard EPROM can also be used.

The PEROM has the advantage that it can be re-programmed in the circuit,

without requiring an external programmer.  However, there was no software

support for this yet.  Therefore I have written a small program for

the purpose of programming the PEROM in the DSP4.

The DSP4 monitor LEONID provides for downloadable programs.  As I had

a relatively old version of LEONID already in a PEROM, and I do not

have a suitable programmer for EPROM/PEROM myself, it seemed to be

best to write the programming routine as a downloadable program.  That

way I could update to newer versions of LEONID by reprogramming the

PEROM, and there would be no lengthy debugging cycle for my program

which would consist of sending my software to someone who programs it

in a PEROM for me, and then trying it myself.

As an added benefit, I think it is good that the programming software

does not consume space in the DSP4 memory when the unit is in normal

operation, and there is also no risk of accidentally invoking the

programming routines because of software crashes etc.

The operation of my programming procedure is as follows:

1. the programming routine PEROM.LOD is downloaded into the DSP4 and

started

2. a full 32K image to be programmed is downloaded into the DSP4 RAM

(this image can be the BTxxxxxx.BIN file as found on jeeves, or

an image you prepare yourself using DLIB)

3. the CRC of the transferred image is checked, to make sure the download

was okay, and the image is valid

4. the programming routine compares the contents of the transferred image

with the current contents of the PEROM.  For each 64-byte page where

the contents of the PEROM is different from the transferred image,

the programming procedure as specified by ATMEL for the PEROM is

performed

5. the programmed pages are compared byte-for-byte with the RAM image.

the programming procedure is repeated when a mismatch occurs

6. finally, the CRC of the programmed PEROM is once again checked, and the

whole procedure is repeated when an error has occurred

Only programming of a full 32K image is supported.  This is because we must

guarantee that the PEROM allways contains a valid image (correct CRC),

or else the selfcheck feature at boot time would make the DSP4 display

a selftest error.

For the same reason, extensive checks are performed during the programming

procedure, to have as little chance as possible that we end up with an

invalid image in the PEROM.  Of course, the programming procedure must

always be completed without interruption.  When (during the programming

procedure) the red LED is ON, the DSP4 should not be reset or turned off!

To program a PEROM, the following software is required:

1. modified version of DL, which supports programming

2. PEROM.LOD downloadable image (generated from PEROM.ASM source)

When you want to upgrade LEONID using this software, please note the

following difficulty:

Each version of LEONID (at least the versions up to now) defines a different

base address of program code and data for the downloadable programs.  These

addresses are contained in the LEONID.ASM file, symbols 'user_code' and

'user_data'.  Unfortunately, the code downloader has no way of checking

the validity of the addresses of the downloaded program, and when there

is a mismatch between the downloaded .LOD file and the LEONID resident in

PEROM, the DSP4 will just crash instead of execute the program.

PEROM.ASM sends a character over the RS232 just after starting, and the

programming additions to DL will check this, so when the DSP4 crashes

during the download of PEROM.LOD there will usually be an error indication

and nothing is damaged.  However, it is certainly best to make sure that

this mismatch does not occur.

Therefore, you must make sure the PEROM.ASM file, when assembled, will

use the LEONID.ASM include file which matches with the LEONID in the PEROM

BEFORE THE PROGRAMMING OPERATION.

When you have successfully upgraded the version of LEONID in your PEROM,

you will have to re-assemble PEROM.ASM to create a new PEROM.LOD which

can be used for subsequent programming of the PEROM.  During this re-assembly

you will use the LEONID.ASM which corresponds with the newly programmed

LEONID image.

To create PEROM.LOD, proceed as follows:

C> asm56000 -a -b -ic:\dsp4\leonid -ic:\dsp4\tools\include -l perom.asm

^^^^^^^^^^^^^^   ^^^^^^^^^^^^^^^^^^^^^

directory where       directory where

your LEONID.ASM       IOEQULC.ASM is

is located            located

The result is a PEROM.LST and PEROM.LOD file.

To program the PEROM, you must CD to the directory where you have

stored PEROM.LOD

Then issue the command:

C> dl -p2 -f bt940401.bin

DSP CARD 4 program downloader (Apr 10 1994)

Downloading 'perom.lod'

Downloading image................................

Image was received OK

Programming....................................................................

...............................................................................

...............................................................................

...............................................................................

...............................................................................

...............................................................................

.................................................

Programming completed

C>

The output of the DL program shows the progress of the programming

operation: first 'perom.lod' is downloaded, then the specified image

file is downloaded and validated.  Finally the PEROM is programmed.

The dots after 'Programming' each represent a 64-byte page.  The above

output (dots) indicates the pages in the downloaded image and the PEROM

are already equal, i.e. the above example represents programming of an

image which was already in the PEROM.

When the pages are different, a + is shown instead of a dot when the

programming was successful, a - is shown when a verify mismatch occurs.

As long as errors occur, the programming operation is re-tried.  When this

happens your PEROM has probably been exhausted and needs to be replaced.

When the errors cannot be recovered, your DSP4 will no longer boot after

this experiment.  You will need a fresh PEROM, programmed with LEONID

on an external programmer.  (you cannot program a new PEROM in the DSP4)

Good luck with PEROM programming!

Rob