Note by N1OWU:  This is Johan's original documentation for PCTOR,

the ancestor of PSATOR, which is the ancestor of AMTOR56.  Most

of the command descriptions are still correct, but most of the

information about the modem interface and the related configuration

parameters is obsolete.  Sorry, I couldn't find any documentation for

PC-PACTOR...

PCTOR V3.07

PCTOR (c) Johan Forrer KC7WW 1991-1994

Author: Johan B. Forrer KC7WW 26553 Priceview Drive Monroe, OR

97456 United States of America

Shareware notice

This program is your registered copy of PCTOR and is not for

distribution in any form. You may make as many copies for your

own personal use. A Shareware version of the program is available

from the author. PCTOR may not be sold or distributed with

another product without the express written permission of the

author. The author, Johan Forrer, KC7WW will only support

unmodified copies of this software.

If you are not satisfied with the program after registering it,

you have 30 days from your registration date to return it for a

full refund of your money, no questions asked.

Previously registered users may obtain a free upgrade to this

version - please send a formatted disk and SASE (or stamped

mailer or 2 IRC's).

This program may not be used by business or government

institutions without proper licensing. Commercial users please

contact Johan Forrer directly for modifications and/or details of

site licensing.

Acknowledgements

The work and ideas of Peter Martinez, G3PLX, and Paul Newland,

AD7I, has been the source and inspiration for doing the "nuts and

bolts" in this project.

Victor Poor's, W5SMM, helpful suggestions and ideas on

compatibility with MBBIOS, PAMS/APlink, and the implementation of

the extended ASCII set was invaluable.

The helpful suggestions of Frank Gorichar, W7JUF, who beta tested

early versions of this software is greatly appreciated. A special

word of appreciation to beta tester Peter Ferrand, WB2QLL/1, with

helping sort out various bugs. Malcolm, WA9BVS's beta testing has

helped locate numerous obscure bugs.

Thanks to Frank Wyatt, N6FW for forwarding technical

documentation to me.

Many individuals have sent in "wish lists", some of which are now

part of the latest version. As always - your support, criticism,

and suggestions are appreciated.

Please support the efforts of shareware developers.

Table of contents

0.0 Quick summary for this release.

1.0 PCTOR what does it do ?

1.1 Minimum requirements to run PCTOR

1.2 CAUTION

1.3 Planned enhancements

2.0 Introduction

3.0 Hardware Interface

4.0 Installation

4.1 Software

4.2 Customizing the configuration file

5.0 Operating PCTOR

5.1 Function keys

5.2 Command menu

6.0 Appendix

6.1 If problems persist..... 6.2 What if it still does not

work ..... 6.3 A few notes about HF modems. 6.4 Soft error

detection and correction. 6.5 Frequencies of FEC stations

for test purposes. 6.6 Signal analysis module. 6.7 DSP

modem. 6.8 PCTOR update history. 6.9 1992 Newsletter 6.10

1993 Newsletter 7.0 Disclaimer

0.0 Quick summary for version 3.07

------------------------------------------------------- THE

FOLLOWING ANNOUNCEMENT IS IMPORTANT TO SOME USERS.

-------------------------------------------------------

In order to support an new DSP modem on the same COM port that

PCTOR uses, versions higher than 3.05 now only supports

transmitted data output though TxD, (pin 2 on a DB25 or DB9).

This is due to the re-allocation of DTR for other purposes (the

DSP uses it). It is essential to establish whether your serial

port UART is suitable at higher baud rates - most modern serial

ports will work without any problems, some older styles may will

not work too well.

Input and output mask options have been removed. These options

only made sense when PCTOR was an AMTOR-only program. Ever since

PCTOR included support for RTTY and now PacTOR, these mask

settings often contradicted "normal" usage of RS232 signalling

conventions. A general rule that holds for all modes now are:

Mark (binary 1) = -12V Space (binary 0) = +12V

PTT keyed = +12V

This holds for RTTY/ASCII/AMTOR and HF Packet. Although PacTOR

does not commit to any specifically. In order to be compatible

with this convention, please check your modem and transceiver

interfacing.

If the TI DSK-based DSP modem is to be used with PCTOR, please

make sure that it is connected to the serial port, powered up and

ready for use. Also make sure that the "TOR.CNF" file contains

the lines:

dskmdm:2 .... this requests DSP modem support dskbaud:1 ....

1=115200 baud for DSK comms 2=(115200/2) baud for DSK

3=(115200/3) baud for DSK A value of 1 should work for faster

machines, however, if you experience problems with loading the

DSK, use a lower baudrate. dskrtty:1 .... dskascii:1 ....

(Allocates which modem to use, see below) dskamtor:2 ....

dskpactor:3 .... dskpacket:4 ..... 1=170 Hz shift 50 baud

2125/2295 FSK 2=170 Hz shift 100 baud 2125/2295 FSK 3=200 Hz

shift 200 baud 2125/2310 FSK 4=200 Hz shift 300 baud 1600/1800

FSK 10=170 Hz shift 50 baud 1275/1445 FSK 11=425 Hz shift 50 baud

1275/1700 FSK 12=850 Hz shift 50 baud 1275/2295 FSK 20=170 Hz

shift 50 baud 2125/2295 AFSK 21=170 Hz shift 100 baud 2125/2295

AFSK

Further information on the DSK-based DSP modems are available in

a separate document.

Known Bugs:

1. Dos versions below 4.0 will not load hot key buffer. The

reason is that in order to provide a sorted list of buffers,

PCTOR uses the dir /o:n command. This option is not available in

older DOS versions.

New features:

1. For AN-93 users, the parallel port is chosen by the LPT

parameter:

lpt:x where x=1, 2 for LPT 1 or LPT 2. The default is LPT 1.

2. Any COM port (1 - 4) may be specified in the configuration

file. Please make sure that your configuration file contain an

entry:

com:x where x=1, 2, 4, or 4

3. If a TI 320C26 DSK is used as a DSP modem, appropriate modem

code will be loaded automatically when operating modes are

changed.

Bug fixes since version 3.03:

1. Some users reported that they experienced a system lockup

after the initial screen. This was reported in cases where a

sound card was installed in the system, and also with DOS 6. The

problem was traced to a Compiler bug and required some minor

rework of code.

1.0 PCTOR what does it do ?

Using only a low cost external HF modem, PCTOR makes it possible

to run AMTOR on PC compatible hardware without the need of

dedicated multi-mode terminal node controller (TNC) hardware. All

decoding is performed in software in real time using only PC

hardware. The software has been tested to work even on a lowly

4.77 Mhz PC compatible.

PCTOR will run under Microsoft Windows as a DOS application in

full screen mode (please see the supplied .PIF file), however

this kind of operation is presently not recommended. Future

releases of PCTOR will be specific for operation under the

Microsoft Windows platform.

The main features of PCTOR could be summarized as:

1 - Split screen operation: Data received through the AMTOR

translator is displayed on the upper window. The operator's

keyboard entry is displayed on the lower window. A status window

shows real-time operational status of the digital translator.

This includes, timing, path propagation delay, link status,

selcals, and more.

2 - All AMTOR modes are supported: ARQ, FEQ, and "Listen".

Ascii/Baudot at any baudrate (practical maximum is 1200 baud).

3 - The user may define several text buffers, i.e. brag tape, CQ

messages, etc. that may be loaded for transmission with with one

key stroke.

4 - Text files my be transmitted. Typically the user will prepare

these off-line.

5 - Unshift-on-space (UOS) option.

6 - "PLX"-method of upper/lower case with extensions as developed

by Peter Martinez (G3PLX), and Victor Poor (W5SMM).

7 - A "high" reliability option based on multiple bit sampling

and majority voting. Memory ARQ for AMTOR is also supported.

8 - Receive data can be optionally be captured in a file.

Automatic time stamps, as well as user-initiated time stamps may

be entered into this capture file. The capture file can be read

using an ordinary text editor.

9 - PCTOR can open a Dos shell (viable only for 386/486) with the

low level AMTOR decoder running. The low level decoder has

buffered I/O queues to accept traffic even when PCTOR has opened

a DOS shell.

10 - The user can customize the working environment through use

of a configuration file that is read when PCTOR starts up. Usage

of a DOS environment variable "PCTOR" allows easy accommodation

for a variety of operating environments.

11 - Keyboard entry is either in word-edit or character mode.

When in word-edit mode, keyboard data is only entered into the

type-ahead buffer when a space is entered (or carriage return, or

the special symbol sequence "+?"). This way typing mistakes may

be corrected before releasing them over the air.

12 - Advanced signal processing is available when using a special

modem. This type of modem allows PCTOR to analyze internal

circuit voltages in order to perform "soft" error detection and

correction. Please see section 6.4 for further details.

13 - PCTOR's time may be set either for UTC or the user's local

time.

1.1 Minimum requirements to run PCTOR

PCTOR will run on an IBM PC or close compatible computer. DOS 3.1

or higher is required. A monochrome or EGA/VGA display adaptor

will work. Although the software will run on a floppy-based

system, a hard disk is recommended. The definition of "true IBM

compatibility" is fuzzy and a great deal of flexibility has been

built into the programs to handle a wide range of PC hardware.

Please refer to the appendix where some further notes on dealing

with incompatibilities are included.

Some of the DSP options are only feasible on more powerful

machines, i.e. 386 with co-processor or a 486.

On the radio side, an HF modem, i.e. ST-6 or equivalent is

required to decode received audio to +/- 12V RS232 logic signals.

For transmission of AMTOR over the air, the user also has to

provide a PTT interface and either AFSK or FSK. These interfaces

are often included in an HF modem. If the software is used for

monitoring only, only the HF modem is required. The HF modem is

required to convert audio tones to RS232 compatible digital

levels required by PCTOR (please see 6.3 for further notes on HF

modems).

PCTOR uses some of the signal lines of the standard COM1 or COM2

serial ports as a digital interface (the user specifies which COM

port - see "Installation - software").

1.2 Caution

This software may not be suitable for all working environments.

The user should therefore proceed with the usual caution and make

sure critical software is backed up.

Every effort has been made to ensure that the software is "well

behaved", however, the user is reminded that this software relies

on critical timing. Normal system functionality is retained, i.e.

time-of-day and floppy disk timeout activities. The user must

thus be careful not to run other software that relies on similar

"tricks" that PCTOR uses as the consequences are indeterminable.

Normal program usage and program termination will undo the

actions of PCTOR and restore normal system operation. Should a

program failure occur, the only way to restore normal system

operation is to reboot DOS.

1.3 Planned enhancements

Several enhancements are planned for release in the near future.

As a registered user you will be notified of these when it

becomes available. These include:

* PACTOR (a pre-release version is already available). * SAA/CUA

standard interface (Windows Application).

2.0 Introduction

PCTOR is an implementation of CCIR specification 476-2 on an IBM

personal computer or compatible computer. As such, it replaces

the need for dedicated TNC hardware and is an attractive

alternative for the casual user evaluating a new operating mode,

as well as the serious developer that needs to embed low level

I/O functionality into a system.

PCTOR contain a user-interface program that works in conjunction

with several low-level routines that enables a user to operate

and monitor TOR traffic with a minimum of fuss. Its

multiple-window user interface allows simultaneous monitoring of

decoded traffic, decoding status, and user keyboard history.

Pop-up windows is used for setting operational parameters,

file-I/O operations, and canned buffer transmissions.

Function keys are used for quick, easy, and efficient control of

most operations.

PCTOR was developed using Borland C++ Version 3.1. The low-level

interface was developed using Borland Turbo Assembler version

3.0.

3.0 Hardware interface

PCTOR interfaces through signals of the COM1 or COM2 RS232 port.

The following allocation of RS232 signals have been made: Other

signal pins may be connected, however the will play no part in

the operation of PCTOR.

Note that for a minimal interface, DTR, RTS, DCD, and Ground, is

required. This will allow operation on AMTOR. For ascii/baudot,

RD and TD must be utilized. If further software support for the

tuning display or memory ARQ is required, a parallel port

interface is required. Please contact the author for further

details.

For a 25-pin serial connector:

pin 2 (TD) - output data bits (mark -12V, space +12V) pin 4 (RTS)

- PTT (off -12V, on +12V) pin 8 (DCD) - input data bits (mark

-12V, space +12V) pin 7 (Ground)

The following link is required:

pin 3 (RD) connected to pin 8 (DCD).

For a 9-pin serial connector:

pin 3 (TD) - output data bits (mark -12V, space +12V) pin 7 (RTS)

- PTT (off -12V, on +12V) pin 1 (DCD) - input data bits (mark

-12V, space +12V) pin 5 (Ground)

The following link is required:

pin 2 (RD) connected to pin 1 (DCD).

When PCTOR is used in conjunction with the AN-93 analog modem,

the parallel port interface is as follows:

For a 25-pin parallel printer connector:

pin 15 - AD0 (bits 0 - 4 from the A/D convertor) pin 13 - AD1 pin

12 - AD2 pin 11 - AD3 pin 10 - AD4

pin 1 - Strobe (strobe for A/D convertor)

pin 18-25 - Ground

4.0 Installation

4.1 Software

It is assumed that a hard disk is available, though the program

can be run off a floppy disk as well. In fact it is suggested

that you first run off a floppy disk to see if this software is

compatible with other software that you may have on your system.

1. Create a new subdirectory and change directory to it, i.e.

mkdir tor cd tor

2. Copy the contents of the floppy to the new subdirectory.

copy a:*.*

The following files will be copied:

tor.exe -- The split screen AMTOR program. tor.cnf -- Setup

file to custom configure your program. docs -- This

documentation. brag.tor -- An example test file to load.

cq.tor -- Another example file to load. test.tor -- A buffer

file for demonstration of control characters AMTOR.BAT -- A

batch file to run TOR with a parameter. Check to see which

communications port COM 1 or 2 you desire.

3. Customize tor.cnf (see 4.2 "Customizing the configuration").

4. Create, or edit the files with ".tor" extensions. These files

will be read when the program starts and will become the user's

personalized buffers. These are transmitted by using the "hot"

keys ALT-1 through ALT-9. The two included examples are typical.

5. Edit the the batch file "AMTOR.BAT" to suit your screen

display color requirements (Please see section 9).

6. Run the batch file AMTOR, i.e. type AMTOR <enter>, or

alternatively run it directly from the command line.

7. Make sure that you use the "ANSI.SYS" device driver in your

DOS config.sys (please see about ANSI.SYS in your DOS manual).

8. There are three DOS environment variables that is worth

knowing about. If you are not familiar with environment

variables, type "SET" with no additional parameters at the DOS

command line for the present list of environment variables on

your machine. Some programs actually scans this list and may

configure themselves accordingly. As far as PCTOR is concerned,

your time zone, i.e. how many hours your are way from UTC time,

and what abbreviation you would like to use for logging and

operating purposes, is important. This environmental variable is

called "TZ" and you typically include it in your autoexec.bat:

set TZ=PST8 (I use this for setting Pacific Standard Time,

that is 8 hours after UTC. Please consult your DOS manual).

When you need to run PCTOR with say a bunch of different

configuration files, or different ".tor" files, the "PCTOR"

environment variable may be used to distinguish amongst them.

Otherwise PCTOR will use the configuration file and ".tor" files

in the local subdirectory from where you executed PCTOR from. If

for example you need to use a different subdirectory for your

configuration file than where your execute tor.exe, then the

following command need to be typed at the DOS command line, or

included your autoexec.bat (lets assume the configuration file is

in c:\COMM):

set PCTOR=c:\COMM

The third environment variable that is of concern is COMSPEC.

This variable tells PCTOR where to locate "command.com" when a

dosshell is invoked (the F8 key). Usually the root directory is

used, but there are instances where users specifically wish to

locate their command.com elsewhere. DO NOT CHANGE the COMSPEC

variable unless you are very familiar with DOS.

9. Several options are configurable from PCTOR's command line.

This includes the type of display adaptor, the number of lines in

each of the split screen displays, and the colors of displayed

characters, messages, and annunciators. If you are using an

EGA/VGA/SVGA display adaptor, the default settings will probably

be adequate. In that case, leave out any further parameters on

the command line.

The general format of the command line format is:

TOR [/B | /C | /M] [/Wxx] [/R] [[/SCxx] [/SExx] [/SBxx] [/SMxx]

[/SAxx]]

A command line parameters require the leading "/" followed by a

directive such as B,C,M,W,R, or S. These parameters may be

submitted in any order, each occurring only once. Note that for

the display adaptor group, only one parameter, i.e. /B, or /C, or

/M must be present. If for instance you have a monitor that has

both color and black and white capabilities, and you desire to

use a black and white display, use the /B. Using monochrome in

this case will probably lock up your display.

The significance of the command line parameters are given below.

U 3 Parameter Description 3

IIIIIIIIIIIIIIIIIIIIIIIIIIIIIIIIIIIIIIIIIIIIIIIIIIIIIII 3 Display

type (ONLY ONE OF THESE ALLOWED) 3 A' 3 B Black and white on a

color display 3 3 C Color 3 3 M Monochrome 3

IIIIIIIIIIIIIIIIIIIIIIIIIIIIIIIIIIIIIIIIIIIIIIIIIIIIIII 3 Wxx

Size of receive display (lines) 3

IIIIIIIIIIIIIIIIIIIIIIIIIIIIIIIIIIIIIIIIIIIIIIIIIIIIIII 3 R

Receive only - disable transmit facilities 3

IIIIIIIIIIIIIIIIIIIIIIIIIIIIIIIIIIIIIIIIIIIIIIIIIIIIIII 3 Display

color attributes 3 A' 3 SCcc Received traffic character color 3 3

SEcc Echoed traffic character color 3 3 SBcc Background color 3 3

SMcc Message character color 3 3 SAcc Annunciator character color

3 AU * The field xx indicates an integer number of lines. The

field cc indicates a color code as given in the color table

below.

Certain restrictions apply to assignment of foreground and

background colors. Example. The default settings are:

TOR /C /W20 /SC7 /SE15 /SB1 /SM3 /SA14

This translates to: Color display (CGA/EGA/VGA), 22 line receive

display size, SC=Lightgray, SE=White, SB=Blue, SM=Cyan,

SA=Yellow.

Experience have shown that there are various types of monochrome

display adapters around. Even those that are advertized as

"Hercules" compatible often behave differently. Some of these

monochrome display adapters could be used as being "color" as

they will correctly map colors to the appropriate black/white

attributes. Other monochrome adapters, however require the /M

parameter and must not contain any color attributes. Some

experimentation is necessary. Be warned that in some instances an

inappropriate display selection may lock up your machine and

require a reboot to recover.

The receive-only command line option, /R, will disable all

transmit facilities such as F1, F2, as well as functions that

enter data into transmit buffers. It is also recommended in this

instance to remove the keyboard-buffer window altogether using

/W25.

Color constants for setting PCTOR display attributes.

U 3 Color Constant Value Foreground or Background 3

IIIIIIIIIIIIIIIIIIIIIIIIIIIIIIIIIIIIIIIIIIIIIIIIIIIII 3BLACK 0

Both 3 3BLUE 1 Both 3 3GREEN 2 Both 3 3CYAN 3 Both 3 3RED 4 Both

3 3MAGENTA 5 Both 3 3BROWN 6 Both 3 3LIGHTGRAY 7 Both 3 3DARKGRAY

8 Foreground only 3 3LIGHTBLUE 9 Foreground only 3 3LIGHTGREEN 10

Foreground only 3 3LIGHTCYAN 11 Foreground only 3 3LIGHTRED 12

Foreground only 3 3LIGHTMAGENTA 13 Foreground only 3 3YELLOW 14

Foreground only 3 3WHITE 15 Foreground only 3 AU

4.2 Customizing the configuration

To enable a user to streamline the setup of the program to his

requirements, a configuration file, "tor.cnf" is read during

startup. Read and edit the contents of the provided file

"tor.cnf" to suit your own preferences. This file is optional,

but when not found, the system will use defaults. These entries

are all optional, and may appear in any order, however, please do

not enter any spaces between symbols,

i.e.

vid : 1 is NOT the same as vid:1

(The last one is correct).

The following entries may be customized:

id:KCWW/KC7WW ----- set your identity selcal/callsign

selcal:WDRZ/WA8DRZ/6 ----- set the startup selcal/callsign.

wru:QRA KC7WW KCWW ----- your WRU text. com:1 ----- COM port to

use 1= COM 1 2= COM 2 3= COM 3 4= COM 4 lpt:1 ----- LPT port to

use for AN-93 modems 1= LPT 1. 2= LPT 2 opmode:0 ----- defines

start-up mode 0 = AMTOR 1 = BAUDOT 2 = ASCII baud:45 -----

initialize BAUDOT/ASCII baudrate txdelay:30 ----- delay (ms)

after PTT on till data out.This gives your transmitter time to

changeover to transmit. cddelay:3 ----- delay (ms) after a data

block is received before acknowledgement is sent. This gives the

other station's receiver time to recover. timing:25 ----- system

clock timing adjustment uos:0 ----- unshift-on-space OFF diddle:0

----- RTTY diddle is OFF case:0 ----- upper/lower case OFF log:1

----- receive data capture (logging) logfile:tor.log ----- data

capture filename scrollfile:scroll.tmp ----- name of scroll file

- if a ramdisk is available, use the full drive name here

logprog:logger.exe ----- name of user's personal log program

packetprog:pac.bat ----- name of user's packet program rel:0

----- reliability OFF vid:1 ----- DIRECT video I/O enabled (see

appendix on hardware compatibility) fcrlf:1 ----- Files, i.e.

buffers and files to be sent through the ALT-F command should use

the AUTOMATIC CR/LF mode. This means that all CR's in the input

text files will be ignored, and LF will be interpreted as CR/LF.

refr:15 ----- Status update rate. The number of system ticks

between updates, i.e. 15*55mS=0.825 seconds. To fast a rate will

flicker or perhaps may introduce timing problems, too slow will

affect animation of timing bar, clock, etc. time:0 ----- Use

local time in displays and logging. If UTC time is required, use

time:1 (also refer to the TZ environment variable). tuning:0

----- Disable software support for tuning display. To utilize

this feature, use tuning:1. A special modem is required. marq:0

----- Disable memory ARQ. Different levels of memory ARQ are

recognized. Presently only level 1, "hard bits" is recognized. To

utilize level 2 memory ARQ, i.e "soft bits", a special modem is

required. word:1 ----- Turns word-edit mode on. stats:0 -----

Turns statistics off. When stats:1, displays the number of blocks

received, number of blocks in error, and when memory ARQ is

enabled, the number of blocks successfully reconstructed. These

statistics are also inserted in the log file for later retrieval.

dskmdm:0 -----DSP modem 0 = Use external modem 1 = Use DSK.

Note that the syntax isn't tested very rigorously. Please check

your input carefully against the supplied example.

Both the AMTOR mode of PCTOR and RTTY/ASCII uses "common RS232

convention" where:

Mark (binary 1) = -12V Space (binary 0) =

+12V

PTT not asserted (open) = -12V PTT

asserted (keyed) = +12V

Until you have sorted out the required logic interfacing logic,

prepare your transceiver for low power output into a dummy load

(you may find your PTT unexpectedly comes on upon executing the

program).

Now run TOR. If your PTT comes on right away, the sense of RTS

must be inverted before further testing can be done. Normally,

TOR should put you in standby mode (PTT OFF).

Next, on-the air tests will be necessary. Try to decode some FEC.

FEC will be decoded while in standby mode. To make sure that you

are in standby mode, hit F10. Tune in on an FEC transmission and

wait for it to sync. If it fails to decode anything, first try

changing the other sideband setting of your transceiver (if you

normally use LSB, try using USB). This will show whether it's the

interface logic or whether there is some other problem. If TOR

receives FEC in the other sideband setting, the sense of DCD

needs to be changed.

The final test is to determine the logic level required for the

transmitted data. Try an ARQ call. If it fails, you need to

invert the sense of TxD.

There is an additional timing-related entry, "timing:" that need

to be set up. This parameter allows external adjustment of the

system clock pulses to cater for systems with inaccurate clocks.

Very few PC's have accurate clocks and although the default value

(25) will work reasonable well in most cases, adjustment of this

parameter is very desirable. For determining the value of this

timing parameter, see 5.2 - Command menu: usage of the 'T'

option.

5.0 Operating PCTOR

Upon startup, the shareware banner may be displayed if you are

suing the shareware version. In this case, press the ESC key to

proceed to the PCTOR screen.

The contents and layout of the status display line depends on

which options the user has selected. The following description

refers to the case when all the options are enabled. This gives

the reader a description of all possibilities.

PCTOR displays several windows: two status displays, the received

traffic window, and the keyboard display.

The smallest display is at the top of the screen, is the status

window that is updated in real time and some machines may show

some flickering. This is normal.

Link status information, i.e. ARQ/BAUDOT/ASCII, IDLE, TFC, ERR,

RQ, is displayed on the left hand side of the status line.

Adjacent to the link status is the selcal/callsign of the remote

station. This data is used in establishing the link and also for

printing out identification data (see usage of the "Ins" key).

Next, the programmed delays are shown: TD is the time (in

milliseconds) that you allow, after the PTT signal has been

activated, before data output will start. It is one way to

compensate for slower on-switching equipment on your end. It

allows for things to settle, before actual data is being sent.

The default value (30 ms) is a reasonable compromise. CD is the

time (in milliseconds) that allows the remote station's receiver

to recover after it sent a block and is getting ready to receive

your acknowledgment. The default of 3ms is reasonable. Actually,

the total time before the user will be able to sample the

acknowledgement is CD+TD. When you are sending blocks, however,

then TD only plays a role. This split timing scheme allows for

compensation in situations where the remote station experiences

slower receiver recovery that transmit turn-on time. This is

typical for local contacts.

Once some traffic starts flowing, and you were the call

originator on ARQ (i.e. the "Master"), the block receive time

will be displayed. This is an internal timing parameter that

cannot be changed. It includes the delay-parameters at both send

and receive stations as well as the time delay due to the signal

propagation through the atmosphere. If you notice considerable

variation, then multipath propagation is most probably the

reason, however, under high noise conditions, the simple types of

modems may also pass too much pulse noise for the software to

achieve good bit phasing lock.

If the unshift-on-space (UOS) feature has been selected, this

will be displayed as a m symbol. This feature will shift letter

shift when a space character is printed. Under noisy conditions,

this will prevent the printing of data in the wrong case.

If the user has selected the receive screen capture

(autologging), the a symbol will be displayed.

When the reliability option is selected a symbol will be

displayed. Reliability is obtained by sampling each bit multiple

times and using a majority voting.

If the user selected upper/lower case operation, there will be

two arrows, the first one is associated with the case of the

receive text, while the second one is associated with the

transmit case. An "up" arrow means upper case, while a "down"

arrow indicates lower case.

A 24 hour-format clock is also shown on the display. This clock

is used for time stamps that are written to the log file, if

enabled. The clock may be set for local time or wazzu UTC time

(also see section 4.1 for details).

An eleven bit analog indicator on the right hand side shows how

bit phasing is progressing. During FEQ or ARQ, the indicator

should show slow motion from right to left. The speed of the

motion depends on how much your clock and the other station's are

differing relative to each other.

When the special TOR modem is used, and provided that the

"tuning:1" option is set in the configuration file, a tuning

indicator is displayed immediately below the bit phasing display.

This tuning display consists of ten gradations each for mark and

space simulating two bar-graph displays. The position of the

indicator within the bar-graph display is proportional to energy

passing through the mark and space filters in the modem's

discriminator. When there is sufficient energy present, the color

of the displayed indicator will change from a grayed outline to a

solid color (red for mark and yellow for space on color

displays). A well-tuned signal shows as two solid indicators that

rests on the extreme end of the display. The display is quite

steady and sharp.

When using the "marq:1" option is used, the level 1 memory ARQ

procedure is performed during ARQ reception. This involves block

reconstruction from multiple bad blocks using "hard bits". Level

2 memory ARQ uses "soft bits" in addition to "hard bits",

however, the special TOR modem is required. When memory ARQ has

been successful in reconstructing a block, this is shown as "ERR"

with green background instead of the usual red background.

The second status line displays the name of the user-supplied log

file, the name of the user-supplied executable (usually a logging

program), statistics, and the size of the type-ahead buffers. 5.1

Function keys

The function keys have the following functions:

F1 - Initiate an FEC (Ascii/Baudot) call.

F2 - Initiate an ARQ call.

F3 - Force a change-over (works only in ARQ).

F4 - Initiate QRT sequence (ends ARQ/FEC, Ascii/Baudot

transmissions).

F5 - If receive text capture is enabled, will insert a timestamp

into the log file.

F6 - Initiate ARQ listen mode.

F7 - Clear keyboard type ahead and special buffers (used for

WRU), clears keyboard display.

SHFT F7 - Clears receive display.

F8 - Open a DOS shell (use with care - 386/486 machines).

F9 - Not used.

F10 - Abort any operation in progress and revert to standby mode.

End - Enter the +? sequence into the keyboard buffer.

Ins - Enter an identification string into the keyboard buffer.

Del - Force LTR case during receive (FEC/ARQ or Baudot). In

addition will it reset the transmit and received cases to UPPER.

Ctrl-Home - toggles "raw" data mode. For experimenters only -

displays untranslated ITA2 (Baudot) code in Hex. This data is

also written to the log for later analysis.

ESC - Calls up the command menu when in user mode.

Pgup/PgDn/-/_ - Scrollback, any other key will restore screen

ALT-A Executes a signal analysis program. The modulation signal

as recovered after the low pass filter of the modem is sampled

and displayed in a graphics window. A second graphics window

displays the frequency domain equivalent. Only EGA, VGA, and

Hercules-compatibility is supported (also see Appendix for more

on the signal analysis software).

ALT-B - Enter the buffers menu. This allows a data buffer to be

loaded into the keyboard buffer. The user prepares these buffers

ahead of time using a text editor. These files have a "TOR"

extension and will be loaded when TOR is initialized. Also note

the usage of the ALT-1 through ALT-9 keys as "hot" keys to load

any of these buffers with one keystroke.

ALT-C - Prompt user to enter the selcal/callsign of the remote

station AND initiate an ARQ call immediately.

ALT-D - Toggle RTTY diddle on/off.

ALT-F - File transmission menu. Allows the transmission of text

files while in FEC or ARQ. The transmitted text will be displayed

as it is transmitted. There will be no echo to the receive

window.

ALT-H - Displays a help screen, showing the usage of the various

function keys.

ALT-M - "Hot keys" to the DSP modem control program.

ALT-L - Activate the program named in the configuration file

option "logprog". PCTOR remains running in the background. Upon

termination of the user's program, the PCTOR display is restored.

ALT-P - "Hot keys" to PC-PACTOR. Note that PC-PACTOR need to be

available in the same directory than PCTOR.

ALT-O - Toggles output to printer on/off.

ALT-R - Prompt user for the selcal/callsign of the remote station

WITHOUT initiating an ARQ call.

ALT-T - Enter UTC time into the keyboard buffer for transmission

in FEC or ARQ.

ALT-W - Sends the "WRU" command to the remote station. This is

the "figs-D" character in the ITA2 (Baudot) alphabet.

ALT-X - terminates PCTOR in an orderly fashion. Equivalent to the

"Q" option in the command menu.

ALT-Y - "Hot key" to the packet program. Note that this typically

is a batch file. The user must set this up the "packetprog"

parameter in the configuration file. Please see section 4.2.

5.2 Command Menu

The command menu (called up when the ESC key is pressed when in

the main display) has the following one letter commands:

C - Enable or disable upper/lower case operation. D - Set the cd

and tx delays. CD is the time after reception of a data block

until your acknowledgement is forthcoming. TD sets the time delay

after the PTT is activated until data is sent. For local contacts

CD=10, TD=30 will be suitable. For DX contacts CD=1, TD=10 is

suitable. The default settings of CD=3, TD-30 is adequate for

most modern radios.

H - Help with the function keys. I - Set your own ARQ identity. K

- Toggle between word-edit or character keyboard modes. L -

Enable or disable receive capture (logging). O - Change operating

mode AMTOR->BAUDOT->ASCII->AMTOR->... P - Change baudrate (300

max). Q - Terminate PCTOR. T - Set clock timing. This parameter

allows fine adjustments of the master clock. See Appendix A for

further details. U - Sets UOS (unshift on space) on or off. V -

Toggles video mode (DIRECT/BIOS). W - Set up your WRU text. X -

Exit the command menu to main display. The ESC key may also be

used for this purpose. Y - Sends out a 500 HZ pulse train from

pin 20 for diagnostic purposes.

6.0 APPENDIX.

6.1 If problems persists.......

The most common feedback from users appears to be setting up

PCTOR's master clock timing. Once this is done, the user enters

the "magic" number into the configuration file that is used

automatically each time the program is started. Only when

installing the program on another computer will it be necessary

to reset this timing parameter.

Please note that this parameter has noting to do with the speed

of your processor, or whether you use an 8088 or 486. Its about

how to program a certain chip (8253/4) on the PC's motherboard.

Typical symptoms will be that either nothing happens when trying

to decode a FEC signal, or, the FEC sync is obtained, some text

is printed, however, synch is lost and only a few words of

garbage is printed before it starts re-sync. If you have a scope,

or frequency counter available, try the following (its not

serious if you do not have access to test equipment - you will

just have to be a little patient and do some common-sense trial

and error. It will take a little longer, but you will obtain the

same end result. Skip the next paragraph if you do not have

access to test equipment):

Enter the command menu (Hit ESC) and select the "Y" option. This

command should toggle the "Td" data pin at 500 Hz. Use the "+"

and "-" keys to toggle the speed up or down, however, timing

values above 30 and less than 18 are indicative of possible

incompatiblities. Most systems work fine with the default of

timing value of 25. When you have completed the test, return to

the main display and select STANDBY (F10).

Tune in FEC mode to a strong FEC broadcast station such as KMI

WOO, or WLO (see attached list 6.5). Observe the timing indicator

on the right hand corner of the status line. Notice that it may

slide slowly. The objective is to have it as stationary or

indecisive as possible. However, do not be concerned if you

cannot get a perfect steady display, that is quite normal.

Vary the timing value, one step at a time using the "+" or "-"

keys. After changing the clock setting, the previous value as

well as the new value will be displayed. You MUST then exit the

command menu for the new settings to become effective. Once back

to the main display, select STANDBY (F10) and wait for sync to be

detected. Notice that when you return from the command menu to

the main menu, you may see some received text being written to

the receive display. This is text that have been accumulated by

the low-level decoder and was stored in a system buffer while you

were using the command menu. This text should be ignored as the

changed clock setting was not yet in effect.

Once the optimal setting has been found, modify the configuration

file TOR.CNF to include the parameter for your system. Values

ranging from 0 to 36 is valid. This procedure set the system

clock close to +/-50ppm for stable AMTOR operation.

6.2 What if it still does not work ......

At this point there remains one question: how "IBM compatible"

your computer is. Perhaps your computer is quite compatible but

there may be an interface adapter card that is not. It has been

found that some video adapters does special emulation, i.e. runs

one video mode, but allows some other type of equipment to be

attached, i.e. ATI allows you to run VGA mode, yet allows you to

use a CGA display. This kind of hardware introduces either wait

states, or disables the interrupt system at a very low level.

Since PCTOR needs a stable timing source, there will be a

conflict for sure.

PCTOR allows you to minimize these conflicts that are related to

the video display by allowing the user to disable writing

directly to the video memory. By selecting the "V" option in the

command menu thus toggling BIOS on, all video access will be done

through BIOS calls. The status line is also simplified to reduce

the overhead. In some cases, this will do the trick. It is also

possible to reduce the amount of screen-writing activity by

slowing down the rate that the status line is updated (see 4.2

"refr:15" ).

Another problem that has been experienced with older PC

compatibles, i.e. 1984 or so, is support chips on the motherboard

that does not behave like later versions. Although these machines

may be perfectly capable of running most software, they will not

be able to keep accurate timing and may not work well with PCTOR.

If possible, update the 8254 and 8259 chips. Also check the 8250

UART for the same reason.

Although there has not yet been reported problems on the use of

serial/parallel interface cards using ASIC (Application-Specific

Integrated Circuit) chips, some laptops and recent desktop

machines are now using them. These ASIC's have varying levels of

compatibility with the 8250-style chips and some are known to

present real problems. The author uses both styles without

problems, however, it is possible that the software may not be

able to work in all situations. If your problems persist, as a

last resort, check whether perhaps your system uses one of these

ASIC's. Feedback on such problems will be appreciated - perhaps a

solution may be forthcoming.

6.3 A few notes on HF modems

Experience have shown that some modems perform better than

others. We may divide the class of popular modems roughly into

three categories (there are a few other types as well):

(I) The phase locked loop (PLL),

(II) filter type,

(III) digital signal processing (DSP) type.

It is the author's opinion that the PLL type of modems are not

suitable for average HF operations. It is best to avoid this

approach even with extensive front-end filtering.

The filter-type modem have been used with much greater success on

HF. However, there are various kinds of approaches in filter-type

modems:

Vintage designs like the ST-5 and ST-6 employed L/C filters and

are known to work reasonably well, given that some changes are

made to their low-pass filters (mainly to pass the 100 Hz AMTOR

modulation rate).

Active filter designs such as the DJ6HP will also work quite well

under fair conditions, however, do not expect too much under

adverse conditions. Another popular active filter modem is the

BARTG design. This small modem will work reasonably well too,

however it lacks front-end bandpass filtering, and has poor

post-discriminator signal processing. Both these designs are good

for newcomers.

Modern good performance filter-type modems use sophisticated

front end filters in conjunction with well-designed

discriminators and extensive post discriminator filtering

circuitry. They generally have excellent dynamic range to handle

strong signals as well as being able to "dig into the noise" to

handle weaker signals. They will tolerate some degree of

off-frequency operation. It also has become popular to monitor

analog levels of signals within the modem in order to implement

"soft" error correction techniques. Although this does not

necessarily mean an increase the level of hardware

sophistication, additional interfacing and software support is

required.

The DSP approach is still new to HF operation, yet holds a lot of

promise. There are presently some commercial units available.

If you are interested in a small, yet sophisticated filter-type

modem, with "soft" error correction capabilities, please contact

the author for further details.

6.4 Soft error detection and correction

With the introduction of PACTOR, soft error detection and

correction entered the domain of HF communications for amateurs.

The idea of soft error detection and correction has been

described extensively in the literature, mostly in conjunction

with high-speed satellite data links.

A short introduction to soft error detection and correction, also

termed "memory ARQ" follows:

Most HF modems presently in use, does extensive signal processing

to recover of the transmitted modulation. The output of the modem

then is a stream of logical bits (zeroes and ones). These bits

are then further processed by digital processing methods to form

messages.

It is, however, unfortunate that a great deal of useful

information is lost when logical bits are produced by the modem's

hardware. One can think of this thresholding as a one-bit

analog-to-digital (A/D) conversion process (with resulting values

zero and one). If this A/D conversion could have produced more

resolution, i.e. eight, 16, or even 256 distinct levels, then it

would have been possible not only have access to the resultant

thresholded value, but also the magnitude of the signal at the

time that it was thresholded.

One application where knowledge of this magnitude is useful, is

AMTOR ARQ mode. During AMTOR ARQ, blocks of three characters are

transmitted on each chirp. The receiving station analyzes each of

the three received characters for a possible error, and when an

error is detected, it chirps back an RQ reply to the transmitting

station to repeat the last three-character block. It is possible,

and even probable when conditions are poor, that the

retransmitted block may again contain an error. This may result

in several repeated blocks. It is possible to improve on this

"blind RQ" method by using memory ARQ.

If the receiving station uses memory ARQ technique, it would have

stored the A/D values associated with each and every bit of each

of the three received characters. When an error is detected in a

retransmitted block, i.e. after an RQ, an error-correction

algorithm is then used to compute a "confidence" value for each

character of the bad blocks (over two or more blocks). Subsequent

block reconstruction then follows where a reconstructed block is

made up from highest confidence elements. Such a reconstructed

block is then error-tested again. More often than not, this

results in fewer RQ's. PCTOR will show the actual statistics of

how its doing. For HF conditions a success rate of 10% of

received blocks in error, is achievable.

6.5 Some frequencies for FEC stations for test purposes

Logged on: Fri Dec 11 1992 22:25:19 on 8431.5 KHz using PCTOR

V2.02

AT+T HIGH SEAS RADIOTELEX BROADCAST FREQUENCIES ARE AS FOLLOWS:

+++++++++++++++++++++++++++++++++++++++++++++++++ + ITU CHAN

COAST-TX AT+T STATION + + -------- -------- ------------ + + 405

4212.5 WOO + + 412 4215.5 WOM + + 416 4217.5 KMI + + + + 626

6326.5 KMI + + 628 6327.5 WOM + + 629 6328.0 WOO + + + + 831

8431.5 KMI + + 833 8432.5 WOM + + 834 8433.0 WOO + + + + 1303

12630.0 KMI + + 1305 12631.0 WOM + + 1307 12632.0 WOO + + + +

1728 16870.0 KMI + + + + 1818 19689.5 KMI + + + + 2299 22425.5

WOM + +++++++++++++++++++++++++++++++++++++++++++++++++

WX AND HIGH SEAS INFORMATION SCHEDULED BROADCASTS ARE AS FOLLOWS:

STATION WX INFORMATION ------- ------------ -------------- KMI

ODD UTC HR+20 EVEN UTC HR+20 WOM EVEN UTC HR+40 ODD UTC HR+40 WOO

EVEN UTC HR+20 ODD UTC HR+20

6.6 Signal analysis module

Besides the benefits gained by memory ARQ, useful time domain

signal analysis is possible especially when coming across an

unknown signal. Time and frequency domain analysis software are

provided as an external executable program that is accessible via

a hot key from PCTOR. Acquisition of signal data is then

initiated and interfacing with the signal processing program is

relatively smooth and unintrusive.

Two graphical displays are shown. One panel shows the actual

analog signal with its software-thresholded counterpart, the

other panel shows the frequency domain (RMS power spectrum). A

software controlled cursor (using the left/right arrow keys),

allows inspection of power spectral peaks, while different levels

of magnification of the analog signal is user selectable (using

the up arrow key). Continuous conversions are performed while the

ALT key is held down. When the ALT key is up, the present state

of the graphs are frozen.Unfortunately this type of work is only

feasible on faster class machines. A 386/25 with math

co-processor or a 486 processor is a pleasure to use. The signal

processing software, however, has been tested on a 8 Mhz XT

without math co-processor where it took several minutes to run to

completion. Presently only Hercules, EGA, and VGA display

adapters are supported.

6.8 DSP modem

The advantages of DSP technology was already realized in the

PCTOR's early days. At that time both hardware and the

mathematics for applying DSP algorithms was not available.

Affordable DSP hardware and the development of DSP modem software

now makes the application of this technology very attractive.

Besides being more cost-effective than its earlier analog

counterparts, very high performance is possible. Being software

upgradable, future developments are made relatively simple.

The DSP modem firmware available for use with PCTOR is for usage

on RTTY/ASCII, AMTOR, PacTOR, and HF Packet. The modem hardware

is based on a TI 320C26 40 MHz and 14-bit codec. Communication

with the DSP is though the same RS232 port that PCTOR uses. All

firmware is downloaded to the DSP from the PC host.

6.8 PCTOR upgrade history (since 2.03)

Version 2.03

1) Supports baudot and ascii receive/transmit (baudot has the

"diddle" feature). This includes the "O" and "P" options in the

main menu to change modes of operation and transmission speed

respectively. The PC's UART is used for this purpose and thus

requires signals be routed to and from the uart appropriately.

2) UTC or local time may be optionally be selected.

3) Either word-edit or character keyboard modes may be selected

(previously word-edit was standard). See the "K" option in the

main menu..

4) F4 now also terminates mode-L

5) Some minor enhancements and bugs were fixed, i.e. - Reading

the receive mask incorrectly from the .CNF file, - Cleanup of the

CR/LF sequence associated with F5 usage, - Cleanup of the display

when using a DOSSHELL (F8), - Better validation of setup

parameters from the .CNF, - Allow the usage of "?" for help in

the main menu.

6) Optional software support for a companion modem. This includes

an analog mark/space tuning display on the PCTOR status line. It

also allows future development of "soft" error detection and

correction using "memory ARQ". This development is needed to

fully support PACTOR.

7) If you are not interested in ascii/baudot operation, no

changes are needed in your RS232 hardware interface to use

version 2.03. If you are interested in using ascii/baudot,

however, please see 3.0 for additional interfacing requirements.

If you are further interested in using the software tuning

indicator display feature, the companion modem is required as

well as a parallel port interface.

8) Older versions of PCTOR (pre-1.17) used a terminate stay

resident (TSR) program, called TORBIOS. PCTOR now incorporates

all low-level routines previously performed by the TSR.

9) ALT-A "spawns" a special module for modulation analysis. This

involves taking 1024 data samples from the modem's low pass

filter stage, performing an FFT and displaying both analog and

frequency domain waveforms. This allows future development of

analysis of received audio. IMPORTANT NOTICE: Do not attempt this

function on a 8088 or 80286 type processor - it works but takes

several minutes to perform calculations. Only a 386/25 with

co-processor or a 486/33 can do justice to this analysis. At this

time, only EGA, VGA, and Hercules compatible displays are

supported.

10) ALT-L "spawns" any program the user wishes to execute when

this key is pressed. Typically this would be a logging program.

11) The log filename and name of the ALT-L user program is

displayed on the status line.

12) There is an option to enable memory ARQ (MARQ) for AMTOR.

When active, "ERR" is displayed with a green background instead

of the usual red background when processing error blocks. The

MARQ algorithm is similar to that used for PACTOR. This first

level of memory ARQ is based on "hard bits" and will work with

any modem. It does not require the special modem. Future levels

of memory ARQ will also make use of "soft bits".

13). The help screen "ALT-H" is expanded to include an

explanation of the different flags associated with various

options.

Version 2.04

14) ALT-X may now be used for quick exit (tnx SV1BDS).

15) Command.com (the dosshell) is now located through the COMSPEC

environment variable (tnx SV1BDS).

16) All files, i.e. TOR.CNF or *.TOR files are now located

through the environment variable "PCTOR" (please see 5.0 for

application notes) (tnx SV1BDS).

17) Baudot and ascii operations now echo transmitted text to the

receive screen.

18) Any baudrate not greater than 300 bps is allowed on

baudot/ascii, however, only integer values must be entered. The

amateur standard of 45.45 baud is a special case where a baudrate

of 45 will be interpreted as meant to be 45.45 bps (tnx SV1BDS).

19) The delay parameter called TX-DELAY have now been split into

two parts, TD (transmitter turn-on delay), and CD (control delay)

(for further details please see section 4.2 on customizing the

configuration file) (acknowledgement to Bill Henry, K9GWT, QST

June 1992, p.34-45).

20) PCTOR now dynamically locates a free user-interrupt slot in

the range 60h-67h. Previously 60h was hooked, however this may

have caused a problem is some instances. If there is none

available, the user will be notified and prompted, otherwise

installation will proceed as usual without any intervention.

21) The special option indicator symbols on the upper command

line started crowding the time clock digits, so they were moved

down a line to the right of the PCTOR logo.

22) The machine class (8088/86, v20/30, 80186, 80386/80486) is

detected at startup. This may be used in future versions of the

software to customize the time-critical software and available

options to be run on the hardware.

Version 2.07

23) Bug fixes in the BAUDOT/ASCII typeahead as well as changes to

the operation of the typeahead buffer in AMTOR. Please note that

all the keyboard data, the time key (ALT-T), and message buffers

(ALT-1 thru 9) will now be collected in the typeahead buffer in

the order that it was entered. This should work out better as you

will be able to mix text for transmission either from buffers,

and/or data from the keyboard without first waiting for the other

to buffer to empty out.

24) Printer log feature toggled on/off using ALT-O.

25) All traffic sent using the "send files" (ALT-F) will now also

be logged to disk.

Version 2.08/9

26) Bug fixes associated with typeahead when ALT-B was used.

27) Bug fix in exception handler associated with printer OFFLINE.

28) F4, when used in ASCII/BAUDOT will terminate transmission

only when the typeahead buffer is empty.

Version 2.11

29) Does not require that DTR be connected any more. All data

output is now routed through TD.

30) Special embedded control characters J and J controls the PTT.

When using these, a submitted buffer will take control of the

PTT. For an example, please see "test.tor".

Version 2.12

31) When a user backspaced over more than one word in the

typeahead buffer, "WORD MODE" got confused. This bug was fixed as

well as cleaning up backspacing back to previous lines.

Version 2.13

32) Fix "getting one behind" during buffer load. Don't backspace

when buffer is empty.

Version 3.00

33) Introduce color user-definable color scheme. 34) Scroll back

receive data area via disk file 35) Output signal via BOTH TxD

and DTR; DTR with usual OUTMASK option.

Version 3.01/2

36) F8 should not use closeall, but close logfile only. 37) After

ALT-P, restore operating modewazzu 38) It appears that INMASK

settings that inverts DTR also inhibits, BAUDOT/ASCII. Reset DTR

setting for Baudot/ASCII

Version 3.03

39) ALT_M to access DSP modem control 40) Sort *.tor buffers

alphabetically 41) ALT_S sends a file continuously 42) ALT_Y

shells out to Baycom 43) ALT_D to toggle RTTY diddle 44) Two-page

help screen 45) Remove INMASK/OUTMASK options

Version 3.03c

46) Restore the operating after returning from PC-PACTOR, i.e.

after usage of ALT-P. 47) Message buffers now sorted

alphabetically. 48) RTTY diddle control. Selectable in the

configuration file, toggled using the ALT-D key. 49) ALT-P hot

keys to PC-PACTOR. 50) ALT-Y hot keys to Baycom (note: Baycom is

a copyrighted program by DL8MBT and DG3RBU - it is not supplied

as part of the PCTOR package). 51) For users of the DSP modem,

ALT-M brings up the DSP modem submenu. The user can select a

modem of RTTY, AMTOR, PacTOR, or HF Packet. 52) ALT-S sends a

file continuously. 53) Receive-only operation. This is selected

from the command line by inluding /R.

6.9 Newsletter December 1992

With 1992 drawing to a conclusion, it may be worthwhile to

reflect on some things accomplished with PCTOR and contemplate

future milestones.

This newsletter will concentrate on the origins of PCTOR its

present status, and some thoughts on where things will be going

in the near future.

For those not too familiar with the software - PCTOR is a

complete communications package for the PC or close compatibles.

Its main purpose was to encode/decode AMTOR (SITOR) using the

PC's on-board hardware in conjunction with a simple external HF

modem, however rtty modes are now also supported. The interface

with the HF modem is through a COM port. A hardware TNC is thus

not required as all decoding is performed in software right on

the PC. PCTOR is available from the library 6 area of the HamNet

forum on CompuServe.

PCTOR has grown and undergone several revisions over the past

year. Presently version 2.03 is the latest. The user base has

also grown over the past year and support has come from all over

the world. I am extremely grateful for your interest and support.

As to all those suggestions and bug reports from users - your

contributions have been valuable. Some of your ideas have since

been implemented, others are in the queue.

By time that you receive this newsletter, a pre-release of my

PACTOR program for the PC will be available. This version of

PACTOR is for decoding and reading of PACTOR traffic and will

automatically decode 100/200 baud ascii or Huffman text on-the-

fly. This includes PACTOR "ARQ" and "UNPROTO" modes. Please check

CompuServe HamNet library area 6 or send me a formatted disk and

SASE mailer for a free Shareware copy.

6.8.1 PCTOR - some history.

During 1978 I met Alan G3RSP/MM, who was maritime mobile on a

tanker en route to the Persian gulf. I was then involved in

operating an rtty SELCAL station at that time with the call

ZS1HF. We had many interesting QSO's on every occasion that his

ship sailed around the Cape of Good Hope. As a result of these

contacts, I was introduced to Peter Martinez, G3PLX and his early

AMTOR developments. At that time, there was only G3PLX and G3YYD

on HF, with Alan using the ship's radio to communicate with Peter

on AMTOR.

Fortunately for me, I was using the same type of microprocessor

chip that Peter was using - the Motorola M6800. Peter supplied

the source code for AMTOR and with some further work, I was able

to decode AMTOR on my homebrew 6800 computer. Everything needed

to be made yourself - the modem, the decoder, and of course the

modifications to the HF transceiver to improve changeover time.

By the time that I had my first AMTOR QSO, I was ranked amateur

number 15 or so on HF AMTOR - just an indication how fast things

were happening.

My own experimental efforts amounted to a single board system,

complete with filter-type modem and tuning display. This system

could handle baudot, ascii (at various rates), CW with automatic

speed tracking, and AMTOR mode A, B and L. I called it a

universal communications processor (UCP). Details were published

for an amateur club project, however few systems were actually

built. This system served my needs very well for several years

and was a test bed for exploring many new ideas.

During 1983 I came to Oregon as a visiting scientist, wrote the

FCC examination, got my ticket and in due course as KC7WW, was

listening for AMTOR. I subsequently met Bill, K4PA, who was

experimenting with AMTOR as well, imagine - on a KIM

microprocessor system! Bill supplied all the specifics on getting

the necessary FCC permission to operate AMTOR in the USA. At that

time this involved obtaining special temporary authority (STA).

It was about 1984 that I learned of Paul, AD7I's Z-AMTOR, and

shortly after that commercial developments from AEA and others.

The publication of several technical articles followed. The

introduction of the multi-mode TNC as a black box appliance

introduced many new enthusiasts to the world of digital

communications.

During 1990 I started translating the M6800 code to 8088 Assembly

language. The old hardware M6800 was just too cumbersome and

inflexible to compete with the ubiquitous PC. Making AMTOR work

on the PC's hardware was a challenge to say the least. With

modern software tools, however, things became easier to debug

than development work on the old embedded system.

The first release of PCTOR was version 1.03, which was uploaded

to CompuServe Hamnet library 6 during October 1991. It has been

updated several times since then.

It is with fond memories that I think of the early days. I am

indebted to those that helped me along the way - in appreciation

to the memory of Irv Hoff from whose work I learned about HF

modems and much appreciation to Peter Martinez who shared the

secrets of making AMTOR possible on a microprocessor.

6.8.2 The present status of PCTOR

PCTOR consists of two major components: a low-level interface

that handles critical timing at the hardware level, and a user

interface. Previously the low-level interface and user interface

was implemented as two separate modules, combining them made a

lot of sense and simplified things for new users.

The user interface follows the split screen format of operation

that has become standard for this kind of communications

operations. Data received through the software decoder is

displayed in the upper window while the operator's keyboard

entries are displayed in the lower window. In AMTOR mode, the

display of characters on the upper display is actually an echo of

what is being received at the remote station. This feedback helps

an operator judge the state of a link - how well the flow is

progressing, what has been sent already and what else is still

waiting to be sent. This is one of the built-in characteristics

of AMTOR.

Two status lines keeps the user informed of real-time operational

status of the digital translator. The main status line is on the

very top of the display. It shows, besides the link status, other

useful parameters such as path propagation delay, current

callsign and selcal, and the current status of some

user-controlled flags such as unshift-on-space (UOS), upper/lower

case operation, and high reliability. The latest symbol that has

been added is "d", that is used to indicate whether "memory ARQ"

is active or not. More about this feature later.

The real-time clock on the upper status line may now be

programmed in either local time or UTC.

Part of the upper status line and a small portion of the upper

split screen display is used for an on screen tuning indicator.

This option is available only when a special modem is used with

PCTOR. Details on this modem follow in the text below.

The lower status line is located between the upper text display

area and the lower text display area. It contains the name of the

user's log file, logging program's name, and shows the current

size of the user's keyboard buffer, i.e. the number of characters

in the type-ahead buffer. This logging program is a new feature

that was added to allow for the execution of a user-specified

program when ALT-L is pressed. This allows ready access to a

logging program via a hot key from PCTOR while the decoder

remains running in the background. The user, however, has to

supply his own logging program.

A general "help" function is available through the ALT-H key

where function key assignments, as well an explanation of the

indicator symbols are given.

In addition to AMTOR modes ARQ, FEQ, and "listen", ascii and

baudot is now also supported at 45.45, 50, 75, and 110 baud.

Please note that in order to use ascii and baudot, additional

interface wiring is required.

To further simplify operations, a pop-up menu system allows the

user to change some functional parameters on-line. This pop-up

menu system is complete with its own help function where all

available features that may be accessed through the pop-up menu

are listed. The latest additions to this menu are "O" and "P"

options that allows the toggling between AMTOR, baudot, and ascii

modes, and alters the baud rate for baudot or ascii modes of

operation.

In order to customize the state of options and other special

items, a user-prepared configuration file is read when PCTOR

starts up. The most recent keywords that have been added are

LOGPROG, REFR, TUNING, WORD, and MARQ. LOGPROG allows the

specification of the user's logging program. REFR sets the

refresh rate of the status displays. TUNING activates the

on-display bargraph tuning displays. WORD enables word-edit

keyboard entry mode. MARQ activates the memory ARQ algorithm.

Some of these features have been requested by users. Please note

that TUNING and MARQ assumes that special hardware is present in

the user's modem. More about this later in this newsletter.

Further general features are available. Off-the-air received

traffic may be captured in a text file. Automatic and manual

identification and time-stamping facilities are provided. Sending

of text files over the air is possible in all modes, recalling of

canned buffers, i.e. CQ call, or brag tape, for transmission, at

a keystroke.

PCTOR supports the APlink protocol for full upper and lower case

ascii. This is based on the "PLX"-method of upper/lower case with

extensions as developed by Victor Poor (W5SMM).

A "high" reliability option is included to improve received data

integrity beyond the 3/4 (zeroes/ones) ratio test used in AMTOR.

The Unshift-on-space (UOS) option have been available in earlier

versions of PCTOR. It is particularly useful for baudot

operation. This option switches the baudot case shift to upper

case at the reception of a space character. Baudot is

particularly prone to noise and often ends up in the wrong

letter/figs case shift. UOS, resets the case to letter shift each

time a space character is received. Based on the general

assumption that a space character is mostly used to separate

words, letter case is assumed. This method more often than not

will allow better print under adverse conditions. Do not be

concerned with all this case shifting if it does not make

sense.It really comes from an earlier era of mechanical

teleprinters. Most of these issues are now handled by the

software anyhow.

Baudot operators have all sorts of tricks to "help the print

along", especially when conditions are marginal. The baudot

"diddle" is one of them. This amounts to sending "idle"

characters when the keyboard buffer is empty. These idle

characters are carefully chosen to be either the ITA2 characters

letter, figures, or in some cases "blank". PCTOR uses diddle in

baudot mode.

As mentioned above, a new modem with special features has been

under development for usage with PCTOR. This development was

essential to support new modes such as PACTOR. At the same time,

however PCTOR's performance was enhanced through the use of

sophisticated digital signal analysis. This modem is not a

digital signal processing unit, however it is quite sophisticated

and a good performer. The hardware lineup consists of a

highpass/lowpass front end, bi-quad discriminator, active

rectification, and low pass filter optimized for 200 Hz

modulation rate. An 8-bit A/D convertor is included that is used

by the software for analyzing recovered modulation signal in the

memory ARQ algorithm. A ten element bargraph display is provided

as a tuning indicator. Further details of the modem, and its

availability in kit form, will be published shortly.

Memory ARQ is an algorithm for intelligent error correction

during ARQ (PCTOR has an option that allows AMTOR to make use of

it as well). Briefly, during reception of AMTOR data blocks, the

algorithm saves not only logical states of bits from the modem's

slicer stage, but also saves the actual magnitude of the

recovered modulation signal levels associated with each bit. This

additional information thus indicates a confidence level

associated with each logic level. When two or more blocks are in

error, the algorithm proceeds to examine the confidence levels of

individual bits for possible "reconstruction" of a good block

from erroneous blocks. The actual details of the algorithm is

somewhat more complex than this brief introduction.

Besides the benefits gained by memory ARQ, useful time domain

signal analysis is possible especially when coming across an

unknown signal. Time and frequency domain analysis software are

provided as an external executable program that is accessible via

a hot key from PCTOR. Acquisition of signal data is then

initiated and interfacing with the signal processing program is

relatively smooth and unintrusive.

Two graphical displays are shown. One panel shows the actual

analog signal with its thresholded counterpart, the other panel

shows the frequency domain (RMS power spectrum). A software

controlled cursor (using the left/right arrow keys), allows

inspection of power spectral peaks, while different levels of

magnification of the analog signal is user selectable (using the

up arrow key). Continuous conversions are performed while the ALT

key is held down. When the ALT key is up, the present state of

the graphs are frozen.

Unfortunately this type of work is only feasible on faster class

machines. A 386/25 with math co-processor or a 486 processor is a

pleasure to use. The signal processing software, however, has

been tested on a 8 Mhz XT without math co-processor where it took

several minutes to run to completion. Presently only Hercules,

EGA, and VGA display adapters are supported.

6.8.3 Future developments

The modem development is in final stages of printed circuit

layout. The details will be appearing shortly in one of the

amateur or electronic hobbyist journals. A kit of parts will be

also offered.

There has been several requests for PACTOR capability. This will

be the next major upgrade. Although I have been experimenting and

testing PACTOR, the special modem was required to implement true

memory ARQ. Without the A/D convertor addition, true memory ARQ

is not possible, regardless of what you may have been told by

others. If you have wondered of what PACTOR is like - my

experience has been that it is really fun to operate when

conditions are good. If you have ever tried HF packet, PACTOR is

a great improvement. However, when conditions are marginal,

performance degrades rapidly, delays become uncomfortably long,

and not even memory ARQ seem to be of much use. This is where you

wish that it could have reverted to AMTOR. I am of the opinion

that it may become popular with some higher powered

traffic-handling stations. This is, however, my own opinion.

Several requests were for the inclusion of embedded control

functions in text files. For example: a user wants to send an

automatic CQ while contesting: A text file is prepared that

contains the body of the CQ message including some special

control functions. The text file is submitted for transmission in

a similar same way than a buffer message is sent. The following

pseudocode shows the procedure.

Script submission for automatic transmission

1) START: 2) <initiate FEC transmission> 3) CQ CONTEST DE KC7WW

KC7WW (KCWW) 4) CQ CONTEST DE KC7WW KC7WW (KCWW) 5) AR PSE ARQ KN

6) <terminate FEC transmission> 7) <wait for an ARQ link

(timeout=30 seconds)> 8) <if it timed out> go back to START 9)

<cancel automatic mode and enter normal keyboard mode>

Note: F7 flushes the text buffer, F10 forces standby mode.

This simple example shows several kinds of control symbols, i.e.

LABELS, start/stop FEC transmission, and wait with timeout - a

branch address for timeout is supplied. A small language set to

support some of PCTOR's built-in features will be developed. The

script will be parsed for syntax and semantics before actual

submission occurs.

There have been several requests for running PCTOR in a

multitasking environment. This is of course possible, given that

PCTOR has guaranteed one millisecond interrupt rate for low level

services. Although this timing requirement hardly impacts

performance on a 386 or 486 type machine, it is unfortunate that

some of the popular multitaskers require the same hardware that

PCTOR uses for their pre-emptive scheduling. In the end someone

usually looses out. Windows 3.1 on the other hand, being a

co-operative multitasker, does tolerate this kind of intervention

but is not considered "well behaved". It is uncertain what future

versions of Windows will allow. It will be logical to develop a

DLL that contains the low level services as an embedded device

driver that will guarantee such high priority interrupts. The

PCTOR user interface will then be using the familiar Windows CUA

format.

I apologize for the inadequacies of the documentation, especially

for newcomers to the digital modes. It is my impression that most

new users either had some earlier experience with AMTOR, or was

very knowledgeable and capable of putting together all the

required bits and pieces. There appears to be a lack of the

basics for AMTOR, but also what these signals sound like. So,

this is perhaps one area that hopefully could be improved. It

also appears that a complete bibliography for reference purposes

would be helpful.

The inclusion of more digital signal processing is a possibility.

Not only could we benefit from noise reduction algorithms but

also from DSP modems that has been tailored for optimum

performance. This approach uses software to adapt filter

parameters and usually results in really sharp filters. An

external dedicated DSP approach, or perhaps a low-cost sound

board could be used. This kind of development is very

challenging, yet holds promise for the future.

The baudot and ascii mode presently lacks "autostart" and "anti-

space" facilities. If anyone is interested in these features,

digital equivalents are possible that works quite well.

Work on PCTOR has always been a challenge and very rewarding. The

order that features are implemented is dictated mostly by what

available time I have at hand, the hardware requirements, and

often doing the fun things first. A lot of time is also spent on

improving things, often fine tuning the time-critical parts, or

fixing irritating bugs.

Should there perhaps be something that you consider needing

attention, please let me know and we can see what could be done

about it.

To contact me, you may leave an e-mail message for me on

CompuServe (70730,3472), or leave a message at WA8DRZ via packet

or Aplink.

KC7WW @ WA8DRZ.#NOCAL.CA.USA

If all else fails - write to me at the address below.

6.10 1993 Newsletter

This newsletter is to update registered users and those

interested in PCTOR and PC-PACTOR on developments during 1993.

Some background will be discussed and future trends also

presented.

I thank those from whom I have heard over the past year. Your

contributions and support are always greatly appreciated.

This issue of the newsletter is dedicated in memory of

Pim Woest, PA3AGG. Pim was one of the first to

experiment with PCTOR - he became a silent key on

January 31st, 1993. We will miss him.

Newsletter Summary:

1. PCTOR 2. PC-PACTOR 3. HF Modem Developments 4. Future

Enhancements

1. PCTOR

What is it? For those not too familiar with the software: PCTOR

is a complete amateur radio communications package for the PC or

close compa-tibles for operating AMTOR (SITOR), RTTY, and ASCII.

An external HF modem is required to convert received audio to COM

port signals (RS232 levels). As decoding is performed in software

right on the PC, there is no need for a hardware TNC.

The 1992 newsletter covered software up to version 2.03 (included

in PCTOR documentation). PCTOR has grown and undergone several

revisions over the past year. Versions 2.04 - 2.08 included

several bug fixes and improvements for a cleaner interface with

DOS.

When the HF analog modem, AN-93, became available, memory ARQ for

AMTOR and an on-screen tuning display, was added (please see

section 3 on: HF modems).

Version 3.0 introduced a scrollable receive display and

user-definable screen colors.

Version 3.03 is currently under development. It includes support

for a new DSP modem and allows "hot keys" to operate PC-PACTOR or

HF packet from PCTOR. Additional features for customization of

the startup file includes selection of the operating mode and

baud rate. For SWL listeners, transmit features may now be

disabled and the receive screen extended to fully use the

additional display lines.

There has been considerable confusion over which RS232 signals

are used to transmit data. This came about in an effort to

simplify the hardware interface: It was found that AMTOR could

use the "break" mechanism to send data through the "Td" line.

This simplified the interface to four lines, i.e. Rd/DCD - for

input signals, and Td/RTS for output signals. Unfortunately, this

did not work with some older serial ports. For compatibility

sake, both Td and DTR was then made available for data output.

When the DSP project came along, DTR was needed for DSP control.

That is why version 3.03 now only supports serial ports that can

switch data fast enough through Td.

2. PC-PACTOR

What is it? PC-PACTOR is a program for the PC to decode, read and

transmit PACTOR traffic. It will automatically decode 100/200

baud ASCII or Huffman-compressed text.

Version 0.03 was released in December 1992 for monitoring PacTOR

traffic only. I have since had a lot of requests for a full

transmit version of the program. The current version of PC-PACTOR

is 0.16 and includes the ASCII FEC transmit capability, traffic

logging and file transmission. Version 0.20 is presently under

development for full ARQ capability.

I do ask your patience as there are some major implementation

issues to be solved. First is to achieve the 15ppm timing

requirement. The PC clock was never designed for this kind of

accuracy.

PacTOR is based on a proprietary standard developed by SCS,

Germany. In order to receive support from SCS, a license fee has

to be paid. Presently, this is why all other amateur radio

implementations of PacTOR are licensed. SCS, however, gave

permission to develop a Shareware version but indicated that no

support would be available.

Things went fairly smooth during development until it became

clear that there are a number of undocumented exceptions that

needed clarification. To date I have not been able to obtain all

the necessary details from SCS to resolve these outstanding

issues. I will persist, however, in pursuing these on my own,

though it may take a little longer.

Please note that version numbers below 1.00 indicates

"experimental" status and should be considered that.

3. HF Modem Developments

Due to numerous requests for a simple yet good performing HF

modem for use with PCTOR and PC-PACTOR, the AN-93 was developed.

The design is a conservative active filter approach using 170 Hz

shift with 2125/2295 Hz tones for data rates to 200 baud. The

unit is intended for direct FSK operation. The computer interface

is through a serial port that is compatible with PCTOR.

A 10-segment bargraph display is provided for tuning purposes. In

addition, an A/D convertor is provided that samples recovered

modulation for implementation of soft memory ARQ by the PC. In

addition, the A/D signal is used for an on-screen tuning display.

The A/D convertor interfaces to the PC through a parallel port

interface.

To those interested, there still remains a few AN-93 kits still

at $135 plus postage.

A considerable amount of effort was spent on studying DSP math,

algorithms, and hardware. This led to the development of a

prototype DSP unit for HF digital. A small production of these

units followed. This was known as the DSP-100 project. Although

an excellent performer, the unit proved to be overly complex and

expensive. After all, it appears that the software TNC approach

is attractive because of its low cost.

Further DSP developments using a $99 evaluation module from Texas

Instruments, called the "DSK" (DSP Starter's Kit) proved very

promising. The DSK is based on a 40 MHz 320C26 DSP chip and

includes a 44kHz, 14 bit A/D convertor.

DSP applications developed for PCTOR now includes high

performance HF modems for RTTY, AMTOR, PacTOR, and HF packet. The

DSK shares the serial port with PCTOR. User-selected modems are

downloaded from the PC to the DSK's on-chip memory at a 115200

baud.

Some valid concern has been expressed about the "delay"

introduced by such a DSP and subsequent timing issues. This delay

time is known as "group delay" and is the due to the time taken

for digital data to ripple through the DSP's shift registers. I

have measured this delay to be approximately 7mS longer than the

AN-93. This has not yet proved to be a problem for me, but if it

does, there are ways to reduce the group delay.

Which is better - the AN-93 analog or the DSP unit? A unit such

as the AN-93 is very good at what it is designed for, however,

when attempting to make it work at too wide a range of baud

rates, some compromise is necessary. In such a case, versatility

is often traded for ultimate performance. The DSP algorithms on

the other hand, are customized for the application at hand and is

expected to deliver uncompromized performance.

The implementation of very efficient filters is a further

advantage of the DSP approach. The modems used in this

application uses 80th order FIR filters, has outstanding

selectivity, and good dynamic range.

It is expected that in time, further digital modem code will

become available in conjunction with yet newer modes. All that

would be required is new DSP code and decoding software on the

PC.

4. Future enhancements

The most-requested enhancement appears to be for a full PacTOR

implementation. This will receive very high priority.

The script processor for PCTOR remains to be further developed.

This feature will allow the user to embed control characters in

submitted text. These control characters will allow execution of

"test and branch" dependant on internal status variables and

allow "looping" constructs. This would allow special operations,

such as needed for contest mode.

Both hardware and software developments using DSP will continue.

The DSK code may eventually be ported to a PC Soundcard as

DSP-based Soundcard technology reaches maturity. There already

are innovative applications for CW, SSTV, and WEFAX for

Soundcards. The prohibitive cost of development software remains

a problem. The DSK will be used in the meantime.

There are plans for development of a new mode based on ideas from

MIL-STD-188-110. The new mode is expected to fill the need for

more robustness than PacTOR with data between 75 and 300 bps

suitable for HF networking. DSP modems will be used to implement

m-ary modulation at low baud rate, higher bit rates are achieved

by transmission of multiple bits per baud. This new mode will

rely on error-correction techniques to achieve coding gain and

robustness.

A draft of specifications will be available in the near future

and will be submitted to various peer forums for input. Those

interested in participating on this project, please contact me.

7.0 Disclaimer

The author, Johan Forrer KC7WW is not responsible for any damage,

injury, loss of profit or gain associated with the use,

installation, or application of this software.

November 1993 J.B.Forrer KC7WW 26553 Priceview Drive Monroe, OR

97456 United States of America

e-mail: forrerj@FRL.orst.edu CompuServe: 70730,3472