DOC FILE for LOADTEST.ASM

Bob Stricklin, N5BRG

August, 7, 1994

This  program  is used to test your computer connection  with the

DSP-93 and to check the DSP-93 ability to load a large  file cor-

rectly.

Use DSPLOAD to load the LOADTEST.OBJ file provided. LED 211  will

go out after the load is complete. The DSP93 is in a loop waiting

for you to run your terminal program and hit a character. You may

want to start a capture buffer before you hit a character.  After

hitting a character the program will check memory locations which

were loaded with a defined fill value. If an error is found  then

the  program  report the (1) memory location,  (2)  the  expected

value, and (3) the actual value. After all locations are  checked

the program will indicate END OF TEST.

You  may want to change the fill value used to test other  memory

patterns. To do this change the value after the -f switch in  the

LOADTEST.BAT file. Also change the constant  VALUE  near  the top

of the LOADTEST.ASM file. The original values were 55 and 5555.

To force errors put one value in the LOADTEST.BAT file and a dif-

ferent value in the program VALUE constant.

The  TASM  assembler cannot fill  all 64K of memory due  to  some

problem  it has in version TASM 3.0. I think it is  over  running

some int values in the C code. I will be reporting the problem to

the  author. Also DSPLOAD.EXE has a problem loading files  larger

than  about 25K. I think it is the same problem. I  will  address

this myself.

After making any changes in the program use TASM 3.0 to recompile

with the proper include files in the same directory.

If  you  are new to developing code for the  DSP-93 this  program

contains many examples you will be able to learn from. I tried to

comment the code so you can follow the program. The unusual thing

done  here  is to read in locations in program memory  and  check

them.  You will normally be working with data memory not  program

memory.

You  can also see one method for printing strings of  information

on the serial port.

I took advantage of the Header files provided by N4IRR.  Finally,

I  called on monitor subroutines described in the document  MONI-

TOR.ASC. Using the monitor routines can be risky however, because

I did not do a good job of maintaining memory and register integ-

rity  in the monitor. An effort will be made to improve  on  this

with later monitor releases.

By the way after the program runs you can use the monitor and

memory dump to see the areas you have put the test patern in. You

may want to fill the memory before starting with a known value.

Don't forget to hit the G and move into the high speed mode and

upper RAM before you look at the program space. Alternately you

could look at it a DATA memory after hitting the reset. Moving

back and forth like this can help you debug problems or under-

stand how the DSP-93 works better.

Have Fun!