DSP-93 FIRMWARE - MONITOR OPERATION

and

SOFTWARE DEVELOPMENT INFORMATION

Copyright 1994 Tucson Amateur Packet Radio, Corp.

Last Revised 12/09/94

This document is intended to cover the action and interface of the DSP-93

monitor firmware. It also details information to develop your own code

using the DSP-93. The casual user of this unit should only be concerned with

the first section of this document.

It is assumed you have completed assembly of the kit according to the

assembly manual. You have the DSP-93 connected to a computer using a

terminal interface program, the D93WE windows program or the DSP-93Control

application for the Macintosh. You should have your computer set for 19200

Baud with 8 data bits,  1 stop bit, and no parity. If this is all in place

you will see the asterisk (*) when you power up or hit reset.

DSP-93 FIRMWARE - MONITOR OPERATION

When  the  DSP-93 is powered up this firmware  monitor  takes control of the

unit. The monitor conditions the TMS320C25 and then it  begins polling  the

serial data link looking for single  character  instructions  to  execute.

The action taken by the  monitor  during each operation will be explained

in more detail  here.

After  initialization  is complete the monitor enters  a  polling loop

checking the serial port for an input character. Acceptable  input

characters are; ?, A, D, F, G, J, H, L, M, P, R, S,  and T. The monitor is

not case sensitive so the lower case of all the above also applies.

An  entry of `?' will bring a listing of the monitor version  and the

commands as shown here;

*?

DSP-93 TAPR/AMSAT REV 2.15

Copyright <C> 1994 by Tucson Amateur packet Radio Corporation.

A-AR REGISTERS

D-DUMP MEMORY

F-FILL MEMORY

G-FLIP & RUN PROGRAM @ 1000h

J-JUMP TO XXXX & RUN

H-INTEL LOADER HIGH BITS

L-INTEL LOADER  LOW BITS

P-PROGRAMS

M-MODIFY WORD

R-RESET

S-SHOW WORD

T-TEST

Lets  review the operation of each of the commands in  order.  An example

will be shown when applicable for the command. Also  note the results of

the command will vary depending on the state of the DSP-93 or previous

commands issued.

A-AR REGISTERS

This  command  displays the value contained in the eight  16  bit registers

on the TMS320C25.  The first register is used  by  the monitor  and so the

value in register zero will normally be  0400 hex. This is what you should

see when you enter `A' after power up.

*a  0400  0ECD  FFFF  FFFF  FFFF  FFFF  FFFF  0AFF

D-DUMP MEMORY

The  dump memory command displays the specified block of  program or data

memory. The TMS320C25 uses a Harvard Architecture  Memory format  which

means it has separate 16 bit  memory  for  program space and data space.

Therefore when you are working with  memory you must specify whether you

want to use program or data  memory. Monitor commands use a `P' for program

and a `D' for data memory. After  entering a `D' for DUMP and a `P' or a

`D' you  enter  the hex  starting address and the ending address of the

memory  block you  want to see. The monitor is not very forgiving so  you

must enter  exactly  four  hex characters a space and  four  more  hex

characters.  The memory contents will then be presented in  block form with

one 16 bit data address and eight 16 bit data values on each line. This

will continue throughout the range specified. The last block may run beyond

the ending value you specified to round out a block of eight values.

When  you  are operating the DSP-93 in the low speed  mode  memory dumps

for program memory will be the contents of the  EPROM  and the  scratch pad

on the TMS320C25 and the data memory  will  come from  the first bank of

static ram, U104 through U107.  When  you switch  to  the HIGH speed mode

memory dumps will come  from  the static  rams U104 through U107 for

program and from U108  through U111 for data.

*d  [P]ROGRAM or [D]ATA

p

XXXX XXXX

1000 1010

1000  FF80 0476 FF80 0476 FF80 0476 FF80 0476

1008  C808 CA40 6060 E760 FF80 0530 FF80 1008

1010  FF80 10AF FF80 10E8 FF80 1008 CE01 C808

*d  [P]ROGRAM or [D]ATA

d

XXXX XXXX

1000 1050

1000  FF80 0476 FF80 0476 FF80 0476 FF80 0476

1008  C808 CA40 6060 E760 FF80 0530 FF80 FFFF

1010  FFFF FFFF FFFF FFFF FFFF FFFF FFFF FFFF

1018  0101 0202 4800 2000 0102 0220 002A 020A

1020  0800 0041 0040 8025 0000 0000 0100 0004

1028  FFFF FFFF FFFF FFFF FFFF FFFF FFFF FFFF

1030  FFDF FFF7 FFFF FFFF FFFF FFFF FFFF FFFF

1038  035C 0040 0000 0000 8000 4000 0003 0001

1040  0001 8000 0041 4940 6200 0200 0003 0100

1048  FFFF FFFF FFFF FFFF FFFF FFFF FFFF FFFF

1050  FFFF FFFF FFFF FFFF FFFF FFFF FFFF FFFF

F-FILL MEMORY

The  Fill memory command works like the data dump command  except in  this

case you are filling memory with a 16 bit hex value.  If you write over any

program areas you will kill the monitor.   The reset switch will restore

order.

*f  [P]ROGRAM or [D]ATA

p

XXXX XXXX XXXX

2000 2200 fe50

*f  [P]ROGRAM or [D]ATA

d

XXXX XXXX XXXX

3000 3300 fe60

*

G-FLIP & RUN PROGRAM @ 1008h

The `G' command is  used after you have loaded a  program and  you are

ready to run it. The details of this command are covered in the Software

Development section of this document. For now you need to know the `G'

command will cause the DSP-93 to began executing the program loaded in

memory. The Flip referred to here means that the monitor is flipping from

the EPROM over into the static ram.

H-INTEL LOADER HIGH BITS

This is an Intel style hex loader for placing bits in data  memory. To

execute properly the DSP-93 must be in the LOW speed state. I  called  it

Intel style because Intel  loaders  normally  only handle  8  bit code. The

same concepts are  used  however.  After receiving  the  H the DSP-93 is

expecting data  in  the  following sequence;  count (16 bit), address (16

bit), Intel hex  code  (16 bits), some quantity of data (8 bits) and

finally checksum.  Here are a few typical lines of a loadable file;

:08000000FF04FF04FF04FF04EC

:0E002000FF04FF0AFF0AFF04C8CA60E7FF04DE

:20042000FF04C8CA60FE05CA60FE05CA60FE05FE05FF04CECECEC8CA60C855CE55D004D024

If no checksum errors are found the data is placed into memory in the

address locations specified. If an error occurs  a  checksum error message

is transmitted via the serial port. When the loader is finished control is

returned to the monitor.

J-JUMP TO XXXX & RUN

JUMP and run executes in the same manner as the `G' command except

execution begins at the specified address. The interrupt  vectors must

still be in place if you are going to allow  interrupts  to occur.

L-INTEL LOADER  LOW BITS

The  `L'  command works exactly like the `H'  command  except  it deals

with the lower 8 bits of the 16 bit hex data.

M-MODIFY WORD

This  command is used to change the contents of a 16  bit  memory location

in program or data memory. The command can be  used  to force the D/A

output of the TLC3204X to a particular value.  This can  be  done by

modifying location 0001. D/A changes  will  only occur if the AIO chip is

active.

*m  [P]ROGRAM or [D]ATA

d

XXXX

2000 FE50

fddd

P-PROGRAM

This command is used to launch one of the firmware programs located in the

DSP-93 EPROM. To use the command hit reset then enter a `P' and a `?' to see

the list of programs available. Then enter your selection. The code for the

application will be loaded into the DSP-93 and execution of the code will

began.  If you do not want to run a program after entering the `P' use the

escape key or reset.

R-RESET

Entering an `R' will cause the DSP-93 to go through a soft reset. The

results of this should be equivalent to a hardware reset. If you  have

entered the `G' command and you are working in static ram the DSP-93 will

bounce back to the EPROMs just if you hit the reset button. Some of the

areas in data ram are initialized during a reset cycle. For example some

code to keep the DSP-93 monitor alive is placed from 1000h to 100Ch. This

is mentioned in case you want to use the dump command to look at a program

you just loaded after entering a reset.

S-SHOW WORD

This  command  is used to display the contents  of  a  particular memory

location. It can be used to check the current state of the A/D output for

example by checking location 0000. Once again, the AIO chip must be active

for direct A/D access.

*s  [P]ROGRAM or [D]ATA

d

XXXX

2000 FDDD

*

T  MEMORY TEST

This command test the data memory for errors.  The is a test of all

existing data memory. The routine starts a 0FFFh. If it gets an error there

it assumes you do not have the upper 32K of memory installed and the

routine jumps to location 7FFFh and continues testing from there. The

memory is tested  by writing 0000, 5555, and FFFF into every location and

reading it  back.  If  no errors result the test will  loop  through  all

locations and then just give you a `*' prompt. If an error occurs the

location of the error will be reported along with the  value written  and

the value returned. Hopefully this error report  will be useful in

diagnosing a problem.

*t  MEMORY TEST

ERROR LOCATIONS:

FFFF  5555  D004

FFFE  5555  D004

FFFD  5555  D004

etc.....

*

SOFTWARE DEVELOPMENT INFORMATION

Lets begin with a reset and reviewing the status of the DSP-93.

Initialization of the DSP-93 requires several steps. The XF bit, a single

digital output of the TMS320C25, is set  to a high state which is the power

on state. The XF  bit  is critical because it is used to  control the

memory bank selection and the  clock  speed selection in the DSP-93. When

the XF bit is in the high state the EPROMs are mapped into active program

memory and static rams U104 through U107 become the data memory. When XF is

high the clock feeding the TMS320C25 is set at 20 MHz or 10 MHz depending

on the jumper located at J100. J100 has a default strap setting the clock

to 20 MHz. This would only need to be changed if you must use slower EPROMs

than those supplied with the kits.

All  interrupts are disabled initially. Execution with sign  extension  and

overflow saturation is enabled. The TMS320C25 block B0 is configured as RAM

memory. Interrupt vector locations are stored in  the respective registers.

The  16C450 or 16C550 UART is configured for 19200 Baud with 8 data bits,

1 stop bit, and no parity.

If  the CTS line is cleared or low an asterisk (*) is transmitted via  the

UART.  If the CTS line is not cleared the monitor  loops  until CTS is

cleared.

At this point you are ready to load your program into the DSP-93 for

execution.

The `G' command is normally used after you have loaded a  program you want

to execute. This command is entered by DSPLOAD.EXE or D93WE if you are

developing or testing code with these tools. Your program will be loaded

starting at 1000 hex  in the data static ram. The DSP-93 will be in the  LOW

speed mode at this point so static rams U105 through U107 will be used.

When the program is loaded it should always began with vectors to the

various interrupt addresses. These vector address should  be in place in

case an unmasked interrupt occurs. Here is an example of the vector address

code from an assembly listing;

1000 FF 80 10 D6          B       TINT

1002 FF 80 10 C1          B       RINT

1004 FF 80 10 C9          B       XINT

1006 FF 80 10 D7          B       TRAP

1008 FF 80 10 0C          B       GO

The monitor places branch commands in the lower address space for each  of

the above interrupt vectors. They branch to the  respective address shown

in the table above.

As  you  can see program execution will began at  location  1008. When  the

DSP-93 recovers from a reset the monitor code is  placed in  the data ram

and prepared to execute there. If you enter  the `G'  command without

loading a program the DSP-93 will  switch  to high speed operation and

continue running the monitor from the static RAM.

When  you  load a program the  monitor  hands  off execution  control of

the DSP-93 by jumping to 1008 when the  last bit  of code has loaded. When

the final leap is made to 1008  the status of the TMS320C25 registers is;

The XF bit is set low and must remain low.

LARP is set to AR0.

CNFP has been executed which places  one of the TMS320C25 scratch pads

in program memory mode. A CNFD instruction may be executed in users code

to recover this area while in High speed operation.

The memory register locations have been set to cause the following

branches if interrupts are enabled:

TINT to 1000 Hex

RINT to 1002 Hex

XINT to 1004 Hex TRAP to 1006 Hex

DEBUG DOCUMENTATION:

The monitor ROM includes a debug routine developed by Tom McDermott, N5EG.

This routine can be called as a development aid when you are generating new

dsp code are if you are just studying existing code. To use the routine

include the MONITOR.INC file at the beginning of a program you are

assembling with the following statement:

#include  "MONITOR.INC"

The MONITOR.INC file has three key statements which are used by a call to

the debug routine.  The statements are:

DEBUG    .EQU    0400h      ; Start of N5EG Debug Routine

This establishes the label DEBUG in your program. With this in place you can

issue a CALL DEBUG in  your program to use the DEBUG routines.

DEBUGPC     .EQU 007Eh      ; Pass Counter used by CALL DEBUG

This statement establishes a memory pointer.  DEBUGPC should be referred to

as the passcount/mode value. DEBUG assums this memory pointer will be

located in page eight (8).  You must store a value at this memory location

before calling debug.  The value tells debug how you want it to work. Here

are the possible values to store:

= 0      : causes the debug to trap always.

= n      : value gets decremented by one each call, it stops

decrementing at zero and causes debug trap.

= 0FFFFh : causes the debug to trap always, then waits for

keyboard input (any key) to continue.

DEBUGML     .EQU 007Fh      ; Starting data memory location used by CALL

DEBUG

This statement also establishes a memory pointer. DEBUG assums this memory

pointer will be located in page eight (8) also. The pointer is for the first

of eight memory locations which will be sent to the serial port. This will

be handy if you want to watch some of your memory variables change as your

program executes.

Debug uses the AR7 register as holding location for the current memory stack

pointer. The monitor places a value of 0AFFh in this register for the

beginning of the stack location. The stack will be utilized by moving down

towards 0h.  The debug routine on uses a few of the memory stack locations,

about 25,  when it is called. Since AR7 is used to hold the location of

stack you can not use it in your program if you intend to call debug.

You can see an example illustrating  the assembly  code used to call debug

by reviewing the MON_TST.ASM routine.

Here is an example of the output from that routine:

PC=1018    A=00000060  AR=  2000  0EFC  FFFF  101A  FFFF  FFFF  FFFF  0AE8

P=00000000  T=0000  ST0=EE08  ST1=17E0

0060:    0000  0000  0000  0000  0000  33E0  0E08  37E0

On the first line you have the program counter, accumulator, and eight AR

registers. On the second line you see the P, T, ST0 and ST1 registers. On

the third line you see the starting target memory location then the values

sorted in eight memory locations starting with the target and including a

total of eight locations. All of this will be lined up a little better in a

later version of the monitor.

Memory Map

The following memory map exist after you have entered a `G' command from

the monitor. The `G' command is executed after a program download using

DSPLOAD.EXE or the windows program D93WE. The unit is operating entirely in

static ram and in the high speed at this point.

PROGRAM     DATA

PROGRAM                  DATA

0000 Interrupts          0000 TMS320C25

and                      MEMORY MAPPED

Reserved                 REGISTERS

001F ----------          0005 ------------

Reserved

0060 ------------

On DSP Block B2

007F ------------

Reserved

For Monitor         01FF ------------

0200 ------------

On DSP Block B0

02FF ------------

0300 ------------

On DSP Block B1

03FF ------------

0400 ------------

Page 8 Data Memory

0FFF ------------

1000 TINT

1002 RINT

1004 XINT

1006 TRAP

1008 ------------

User Program             User Data

Memory Space             Memory Space

F000 ------------

Sin Table

FFFF ------------        FFFF -------------

Monitor Routines

A collection of monitor routines which should help speed code development

are documented here. Refer to the file MONITOR.INC included with the

release disks for the exact location of these routines. This file should be

included with your program in the following manner;

#INCLUDE "MONITOR.INC"

An effort was made to preserve the state of the TMS320C25 when one of these

routines is called. The register may change if they are involved in the

routine but in most cases the processor will return with pointers and

registers unchanged.

Here is a little more detail on each of the monitor routines.

GET4HEX and GETVAL16

These two routines are the same. Collects a 16 bit hex value from the

serial port. The value should be presented as four ASCII  characters 0

through F  digits. The four hex digits are converted to  a sixteen bit

HEX word and stored in a single memory location for later use. Look at

MONITOR.INC for the storage location and a suggested label. This memory

storage location is for the current page of memory established by the

LDPK instruction.. The LDPK is normally set to page 8 with the

instruction LDPK 8.

HEXOUT

This routine assumes a single HEX digit is stored in the

accumulator. This digit is converted to ASCII and sent to the serial port.

INBIT

Retrieves incoming data from the serial port. Data is placed in the

indirect address location pointed at by AR(ARP).

OUTBIT

Sends the lower 8 bits of AR(ARP) to the serial port. This routine  uses

memory location 0060h of the current page for temporary storage.

Location 0060h is used to store the UART register information while the

routine is sending the data. The register information from the UART is

used to determine if CTS is set or cleared and if the transmit data

buffer is full.

OUTBIT2

Sends the lower 8 bits of location 0062h of the current page to the

serial port. This routine  uses memory location 0060h of the current

page for temporary storage. Location 0060h is used to store the UART

register information while the routine is sending the data. The register

information from the UART is used to determine if CTS is set or cleared

and if the transmit data buffer is full.

PRVAL16

The 16 bits n a  defined memory location  of the current page are

converted to ASCII and sent to the serial port. MSB is sent first.  Look

at MONITOR.INC for the proper memory location and label to use for this

routine.

This routine uses HEXOUT.

RESET

Jump to this location to restart the monitor.

SD_CRLF

Sends a single carriage return and line feed to the serial port.

SD_STR

Call  this  routine with a 16 bit address stored in  the  current AR(ARP)

register  will  cause  a string to be sent to  the  serial port.  The

string  should  begin at the AR(ARP) memory pointer  and  end with 00. Only

the lower 8 bits of memory bytes will be  sent.

The string data must be in data memory space to print.

Here is an example of a string and the calling routine.

STR1    .string " D S P - 9 3   T A P R / A M S A T   R E V   2 . 1 5"

.word   00

S_END

LRLK    AR0,2000h  ; String will be placed here in data memory

RPTK    S_END-STR1 ;Number of characters to move

BLKP    STR1,*+    ; Moves string from program to data memory

LRLK    AR0,2000h  ; point to memory location

CALL    SD_STR

SP1  through SP10

These routines will send the indicated number of spaces (20 Hex).

MWAIT_1 and MWAITB

The  WAIT routines are shown here. You can enter the  routine  at WAIT_1 or

WAIT_B with your own values set for AR1 and AR2. With a 40 MHz clock the

wait from WAIT1 should be about 32 milliseconds.

WAIT1

SST1    MON6       ; Saves  SST1

SST     MON7       ; Saves SST

LRLK    AR2,020h

CALL    WAIT_A

LDPK    0h

LST1    MON6       ; Restores SST1

LST     MON7       ; Restores SST

RET

WAIT_A  LRLK    AR3,02710h ; 02710h = 10,000

LARP    AR3

WAIT_B  BANZ    WAIT_B,*-  ; Loop through count

LARP    AR2

BANZ    WAIT_A,*-  ; Loop through count

RET

The distribution disk should contain a program called MON_TST.ASM. This

routine demonstrates the use of these assembly routines.

HARDWARE PORT IO ASSIGNMENTS

The following hardware port assignments are used by the DSP-93. These

assignments are identified and labeled in a include file called PORTS.INC.

When interfacing with any of the ports you should include this file with

your code and use the labels provided. There is little chance you may

damage the DSP-93 when writing code but if you error in assign port

information and  execute the code you may key your transmitter if it is

connected. This may cause injury to you or  damage to your equipment.

PORT CONNECTIONS AND PROGRAM ASSIGNMENTS

PORT 00h

CODE        CONNECTION       NAME       DESCRIPTION

D15-D0      NOT DEFINED FOR BASIC SYSTEM

PORT 01h     UART WRITE

CODE        CONNECTION       NAME       DESCRIPTION

D7-D0       U212-1-U212-8    SIO_TX     SIO TRANSMIT BYTE (8 BIT)

D8-D15      Not defined

PORT 02h     HIGH SPEED CLK & H_AIO

CODE        CONNECTION       NAME       DESCRIPTION

D15-D0      NOT DEFINED FOR BASIC SYSTEM

PORT 03h     UART CONTROL REGISTER

CODE        CONNECTION       NAME       DESCRIPTION

D0-D7       Not defined

D8-D9-D10   A0-A2 on U212 UART control registers

D11-D15     Not defined

PORT 04h     H_AIO SELECT OUTPUT

CODE        CONNECTION       NAME       DESCRIPTION

D3 D2 D1 D0

X  X  X  1

PORT 05h     UART READ  (READ ONLY)

CODE        CONNECTION       NAME       DESCRIPTION

D7-D0       U212-1-8         SIO_RD     SIO READ BYTE (8 BIT)

D8-D15      Not defined

PORT 06h     TNC OUTPUT

CODE         CONNECTION   NAME       DESCRIPTION

D11 D10 D9 D8

X   X  X  1     TNC-1        *CDI       TNC *CDI   AND LED L215

X   X  1  X     TNC-6        *KEYI      TNC *KEY I AND LED L216

X   1  X  X     TNC-10       *KEYI      TNC *KEY I AND LED L217

1   X  X  X     TNC-9         CTSI      TNC  CTS I

CODE         CONNECTION   NAME       DESCRIPTION

D15 14D 13D 12D

X    X   X   1   TNC-11      XCLKI       TRANSMIT CLOCK

X    X   1   X   TNC-13      RCLKI       RECEIVE CLOCK

X    1   X   X   TNC-17      RDI         RECEIVE DATA

1    X   X   X   TNC-20      TDI         TRANSMIT DATA

PORT 07h  RADIO PORT SELECT AND GAIN & AIO ENABLE

CODE          CONNECTION       NAME       DESCRIPTION

D2 D1 D0

0   0  0    I/O201 PIN 3     AUX IN     AUXILIARY AUDIO IN RADIO 1

0   0  1    I/O202 PIN 3     AUX IN     AUXILIARY AUDIO IN RADIO 2

0   1  0    Jx03   PIN 14    AUX #2     AUDIO BUSS #2

0   1  1    Jx03   PIN 12    AUX #1     AUDIO BUSS #1

1   0  0    I/O202 PIN 5     XMIT 2     TRANSMIT MONITOR RADIO 2

1   0  1    I/O201 PIN 8     REC IN     RECEIVE AUDIO RADIO 1

1   1  0    I/O201 PIN 5     XMIT 1     TRANSMIT MONITOR RADIO 1

1   1  1    I/O202 PIN 8     REC IN     RECEIVE AUDIO RADIO 2

D5 D4 D3

0  0  0     FEEDBACK         GAIN=      Use MEASGAIN to determine your

0  0  1     FDBK R201        GAIN=      units gain.

0  1  0     FDBK R204        GAIN=

0  1  1     FDBK R205        GAIN=

1  0  0     FDBK R206        GAIN=

1  0  1     FDBK R207        GAIN=

1  1  0     FDBK R208        GAIN=

1  1  1     FDBK R203        GAIN=MAX

D7 D6

X  1        U215-10          L212       PANEL LED 212

1  X        U215-11          L211       PANEL LED 211

D14         U210-9           *AIO_EN    ENABLE FOR AIO DR & DX SIO

D15         U208-2           *AIO_RST   Reset for AIO converter (*RESET)

PORT 08h

CODE        CONNECTION       NAME       DESCRIPTION

D15-D0      NOT DEFINED FOR BASIC SYSTEM

PORT 09h

CODE        CONNECTION       NAME       DESCRIPTION

D15-D0      NOT DEFINED FOR BASIC SYSTEM

PORT 0Ah     TNC INPUT  (READ ONLY)

CODE        CONNECTION       NAME       DESCRIPTION

D11 D10 D9 D8

X   X  X  1 TNC-2            CDO       TNC *CD O

X   X  1  X TNC-5            RTSO      TNC *RTS O

X   1  X  X TNC-8                      TNC PIN 8

1   X  X  X TNC-7            CONNO     TNC  CONN 0

D15 D14 D13 D12

X   X   X   1  TNC-14        RCLKO      REC CLOCK

X   X   1   X  TNC-16        XCLKO      TRANSMIT CLOCK

X   1   X   X  TNC-18        RDO        RECEIVE DATA

1   X   X   X  TNC-19        TDO        TRANSMIT DATA

PORT 0Bh  RADIO PORT DIGITAL CONTROL

CODE   CONNECTION     NAME DESCRIPTION

D2 D1 D0

X  X  1     I/O201 PIN 2     *PTT        PUSH TO TALK RADIO 1

X  1  X     I/0201 PIN 7     *FREQ UP    FREQUENCY TUNE UP PULSE

1  X  X     I/0201 PIN 1     *FREQ DN    FREQEUNCY TUNE DOWN & 470 TO PIN 6

1  X  X     I/0201 PIN 4     *ICOM       470 OHM RESISTOR TO PIN 5

D5 D4 D3

X  X  1     I/O202 PIN 2     *PTT        PUSH TO TALK RADIO 2

X  1  X     I/0202 PIN 7     *FREQ UP    FREQUENCY TUNE UP PULSE

1  X  X     I/0202 PIN 1     *FREQ DN    FREQEUNCY TUNE DOWN & 470 TO PIN 6

1  X  X     I/0202 PIN 4     *ICOM       470 OHM RESISTOR TO PIN 5

D7  D6

1   X       U215-13           L214       PANEL LED L214

X   1       U215-12           L213       PANEL LED L213

PORT 0Ch

CODE        CONNECTION       NAME       DESCRIPTION

D15-D0      NOT DEFINED FOR BASIC SYSTEM

PORT 0Dh

CODE        CONNECTION       NAME       DESCRIPTION

D15-D0      NOT DEFINED FOR BASIC SYSTEM

PORT 0Eh

CODE        CONNECTION       NAME       DESCRIPTION

D15-D0      NOT DEFINED FOR BASIC SYSTEM

PORT 0Fh

CODE        CONNECTION       NAME       DESCRIPTION

D15-D0      NOT DEFINED FOR BASIC SYSTEM

SIN TABLE

The DSP-93 kits are supplied with two banks of 32K by 16 bit static RAM

memory. You may double your memory and fill the remaining SRAM sockets.

A  540 degree SIN/COS table is included with the monitor.  The table  is

16 bit values in increments of .25 degrees.  The  table starts  at

program memory location defined in MONITOR.INC. The data is in EPROM

when  the monitor  is in LOW speed and it is moved to program memory

location F000 through F86F when you switch to high speed. Look at the

table using the dump command. Hit the reset button then enter the

following command;

*d  [P]ROGRAM or [D]ATA

p

XXXX XXXX

7751 7fc0

7751  008C 011C 01AC 0238 02C8 0358 03E8 0474

7759  0504 0594 0620 06B0 0740 07D0 085C 08EC

7761  097C 0A08 0A98 0B24 0BB4 0C44 0CD0 0D60

7769  0DEC 0E7C 0F08 0F98 1024 10B4 1140 11CC

7771  125C 12E8 1378 1404 1490 151C 15AC 1638

7779  16C4 1750 17DC 1868 18F4 1984 1A10 1A9C

7781  1B24 1BB0 1C3C 1CC8 1D54 1DE0 1E68 1EF4

7789  1F80 2008 2094 211C 21A8 2230 22BC 2344

7791  23D0 2458 24E0 2568 25F0 267C 2704 278C

7799  2814 289C 2920 29A8 2A30 2AB8 2B3C 2BC4

77A1  2C4C 2CD0 2D54 2DDC 2E60 2EE8 2F6C 2FF0

...

*

This table is provided in ROM to reduce the time required for download of

your program. It was not located in the standard memory configuration

because of the space limitations. Most of the applications coded to date

use very little program memory space. If you do fill the additional memory

and develop a very large program that extends above program memory 0F00h

the monitor will write over your code with the sin table. Future releases

of the monitor and documentation will address this issue.

**********************************End of File **************************

#############################################################################