************************************************************************

*                                                                      *

* Copyright (c) David L. Mills 1995                                    *

*                                                                      *

* Permission to use, copy, modify, and distribute this software and    *

* its documentation for any purpose and without fee is hereby granted, *

* provided that the above copyright notice appears in all copies and   *

* that both the copyright notice and this permission notice appear in  *

* supporting documentation, and that the name University of Delaware   *

* not be used in advertising or publicity pertaining to distribution   *

* of the software without specific, written prior permission. The      *

* University of Delaware makes no representations about the            *

* suitability this software for any purpose. It is provided "as is"    *

* without express or implied warranty.                                 *

*                                                                      *

************************************************************************

FSK modem/TNC for HF RTTY

This program implements a FSK modem/TNC for HF asynchronous Baudot (ITA-

2) and synchronous SITOR/AMTOR (CCIR 476 Mode B) signalling. It operates

at speeds to 100 baud, either as a TNC with ASCII input/outut via the

onboard UART, or as a regenerator with clean AFSK output reconstructed

from a possibly noisy input. The algorithms use Butterworth and matched

filters, soft-decision detection, maximum a-priory (MAP) estimation and

Viterbi decoding. In case of continuous asynchronous signalling, the

program can operate in a synchronous mode using a phase-lock loop to

flywheel through momentary carrier fades. The predetection design and

many of the operator features were inspired by the Dovetron TU of long

ago, but implemented in DSP, rather than analog form. The postdetection

design, including MAP estimation, Viterbi and matched-filter decoding

and synchronous mode was inspired by a LSI-11 program also of long ago.

If this thing can't pull RTTY and SITOR signals from the muck, nothing

can.

The program operates as both a TNC and a regenerator. When operating as

a TNC, the modem is connected to a HF transceiver operating in LSB mode,

so that the space tone is transmitted at the lower radio frequency. In

receive mode, received Baudot characters are converted to ASCII and sent

to the terminal program at whatever speed and character format selected

by the downline loader. In transmit mode, ASCII characters received from

the terminal program are converted to Baudot and used to modulate the

AFSK output. Since the program can operate with both radio ports, this

feature might be useful in a crossband repeater.

The transmit/receive protocol operates as in the PK-232 TNC. The x

command switches to transmit mode and keys the PTT line, following which

text is transmitted as received from the terminal program. The ASCII ACK

(ctrl-F) character switches to receive mode when the transmit buffer is

empty; the ASCII EOT (ctrl-D) character purges the buffer and switches

to receive mode immediately. The p{1-2} command selects the radio port

to use for the transmit, receive and tune functions. Separate first-in

first-out (FIFO) buffers of 1000 characters each are used to save

transmitted and received text. Flow control for the transmit and receive

buffers is not yet implemented.

The frequencies of the mark and space tones are designed to match the

AEA PK-232 TNC in either narrow shift (WIDE OFF) or wide shift (WIDE ON)

mode. In narrow shift modes, the input and output mark tone is 2125 Hz

and the input and output space tone is 2295 Hz. In wide shift modes, the

input mark and space tones vary to match the shift, with a center

frequency of 1700 Hz. In wide shift modes, the output mark tone is 1275

Hz and the output space tone is 2125 Hz. The o command can be used to

switch between narrow and wide output shifts. The baud rate of the

transmit signal is equal to that of the receive signal.

The character translation tables use the US versions of the ITA-2

(Baudot) alphabet and the ITA-5 (ASCII) alphabet. Operating speeds used

by both the receiver and transmitter are selectable from the set 100,

75, 50, 45.45, 25 and 10 baud. Speeds below 45.45 baud are useful in

marginal conditions where the FSK signal can barely be heard by ear on a

SSB voice receiver. The predetection matched filter at 45.45 baud

provides 6dB of processing gain over the typical 100-Hz postdetection

lowpass filter used in many TNCs. In addition, the 10-baud rate can

provide an additional 6dB gain.

Half/Full-Duplex Modes

The program can operate in four modes: half-duplex regenerator, analog

loopback echo and digital loopback echo modes, and full duplex mode. The

e{1-1} command is used to switch between these modes. The configuration

of the modulator and demodulator units and their associated buffers in

each mode is described the following paragraphs.

Receive regenerator mode (e1, e2) is used during receive for the half-

duplex analog loopback echo (e1) and digital loopback echo (e2) modes.

The configuration in this mode is shown in the following diagram.

+---------------+

|               |    +------+

AFSK -->+  Demodulator  +-+->| FIFO |--> Receive

|               | |  +------+

+---------------+ |

+------------+

+---------------+              |

|               |    +------+  |

AFSK <--+   Modulator   +<---+ FIFO |<-+

|               |    +------+

+---------------+

When operating as a regenerator, an ordinary TNC is connected to the

AFSK output and used to demodulate and decode the regenerated signal.

The regenerator demodulates the input signal, converts to ASCII, then

converts back to Baudot. The conversion removes extraneous nonprinting

characters and regenerates LTRS and FIGS control characters, including a

LTRS character following CR/LF. The regenerated tones can also be used

to drive a transmitter or crossband repeater.

In transmit analog loopback echo mode (e1), characters are echoed as

actually sent. The configuration in this mode is shown in the following

diagram.

+---------------+

|               |    +-------+

+->+  Demodulator  +--->| FIFO  |--> Echo

|  |               |    +-------+

|  +---------------+

|

|  +---------------+

|  |               |    +-------+

AFSK --+--+   Modulator   +<---+ FIFO  |<-- Transmit

|               |    +-------+

+---------------+

In transmit digital loopback echo mode (e2), characters are echoed as

received from the terminal program. The configuration in this mode is

shown in the following diagram. Note that the demodulator is not used,

other than to drive the various monitor LEDs.

+---------------+

|               |    +-------+

+->+  Demodulator  | +->| FIFO  |--> Echo

|  |               | |  +-------+

|  +---------------+ |

|                    +-------------+

|  +---------------+               |

|  |               |    +-------+  |

AFSK --+--+   Modulator   +<---+ FIFO  |<-+-- Transmit

|               |    +-------+

+---------------+

In full-duplex mode (e3), the modulator and demodulator, together with

their buffers, are functionally separate. No provisions for echo are

made. The configuration in this mode is shown in the following diagram.

+---------------+

|               |    +-------+

AFSK -->+  Demodulator  +--->| FIFO  |--> Receive

|               |    +-------+

+---------------+

+---------------+

|               |    +-------+

AFSK <--+   Modulator   +<---+ FIFO  |<-- Transmit

|               |    +-------+

+---------------+

Tuning Instructions

First, make sure LED 2 is on to indicate the program is running. Set the

baud rate, mark/space shift and filter type using the b{1-6} and m{1-8}

commands listed below. Tune in a RTTY signal and adjust the signal level

so that LED 1 is on most of the time, but not solidly lit. LED 3 should

be on at this time; if not, use the c{+-} command to adjust the signal

gate threshold. Adjust the receiver frequency to maximize the intensity

of LED 5 (mark). The t{+-} command can be used to tune the receiver, if

the receiver and cable supports the DSP-93 signals. Use the m{+-}

command to tune the space frequency and maximize the intensity of LED 6

(space). LED 7 shows the mark signal of the regenerated signal. It

flashes to indicate a valid character is being received. Adjust the

carrier threshold with the c{+-} command for solid copy while avoiding

garbles during deep signal fades.

A speaker can be connected to the speaker jack, along with a AFSK

transmitter and/or external TNC. The d{0-9} command can be used to

select various signals generated in the software, as detailed below. In

addition, a DC-coupled scope can be connected directly to the AIO analog

output. The d5 command selects a synthesized signal that sounds vaguely

like a Teletype Model 12 at the speaker jack. With practice, signals can

be tuned in accurately by watching the LEDs and listening to the various

signals on the speaker. A word of caution, however; this thing is really

very selective, especially when the predetection matched filter is in

use, where it needs to be tuned to within 10 Hz for optimum results.

Character-Synchronous Operation

The program can operate in two modes, asynchronous and synchronous,

depending on signal quality and operator preference (s command). In

asynchronous mode, each character is decoded separately. In synchronous

mode, the program measures the intercharacter spacing and continuously

adjusts the sample phase for optimum decoding without requiring a start-

bit check. This improves performance, especially under multipath

conditions resulting in deep fades of either the mark or space channels.

In such cases when valid character framing is lost, the decoder simply

scraps the garble without losing valid framing for subsequent

characters.

The signal quality is determined from the Viterbi distance, called the

autostart signal, which is integrated using a relatively long time

constant. When this signal exceeds a threshold set by the a{+-} command,

LED 4 is turned on. If LED 4 flashes with LED 7, the program is

operating in asynchronous mode; when it is on continuously, the program

is operating in synchronous mode. When synchronous mode is selected, the

transmitter sends an idle character (LTRS) if the transmit buffer is

empty and neither a ACK or EOT has been received from the terminal

program. This helps the remote station maintain synchronization during

pauses when no text is being transmitted.

Filtering and Limiting Functions

There are two input bandpass FIR filters implemented in this program,

one for narrow 170 Hz shift, the other for wider shifts to 850 Hz. There

are two sets of channel filters, one implemented as a synchronous

lowpass matched filter, the other as a synchronous lowpass IIR filter.

Separate filters are implemented for each of the mark and space signals.

These are selected using the m{1-8} command for various shifts and

filter selections.

The Butterworth IIR lowpass filters (m{1-4} command) are useful under

most operating conditions with RTTY signals. They are narrow enough for

good performance, but wide enough for easy tuning. In this case,

postdetection matched filters are used in both the mark and space

channels, both to optimize the transient response and as an

interpolation filter. Alternatively, synchronous predetection matched

filters (m{5-8} command) can be used for greater selectivity with RTTY

signals. In this case postdetection filters are not necessary. All

matched filters are matched to the baud rate and have bandwidths

approximately equal to the reciprocal of the baud rate. In the case of

the synchronous predetection matched filter, this makes tuning very

sensitive, especially at the lower baud rates, and requires, for

example, an adjustment for TNCs with compromise 200-Hz shift.

A transient clipper is located at the input to the program before the

input bandpass filter. Its intent is to kill noise spikes that might

overload later stages in the program, but has no significant affect at

ordinary signal levels. A variable gain limiter is implemented between

the input bandpass filter and the synchronous lowpass filters. The l{0-

5} command can be used to select limiter gain from 0 to 30dB in 6dB

steps The i command can be used to invert the detector signal for

upright/inverted shifts. The c{0-2} command can used to disable either

the mark channel or space channel, if required. Gain factors for all

combinations of filters have been optimized for most conditions.

However, the system gain following the limiter can be adjusted using the

g{+-} command. There should be no need to change the default adjustment

for ordinary operation.

The Autostart Function

The autostart function operates as a sophisticated squelch mechanism to

reduce garbles due to noise and signals other than RTTY. It is intended

for use as a crossband repeater, where reliable signal detection is

required. This function mutes the decoder, unless received signals pass

a suite of sanity checks and signal quality thresholds. There are two

quality estimators which affect the autostart function. The first,

called the mark signal gate, determines carrier and noise estimates as

follows. When the differential signal (mark minus space) is most

positive during the last eight bit times, the corresponding mark signal

is most likely a carrier sample and the space signal a noise sample.

When the differential signal is most negative, the corresponding space

signal is most likely a carrier sample and the mark signal a noise

sample. The carrier estimate is the larger of the carrier samples, while

the noise estimate is the exponential time average of the average of the

noise samples.

If the autostart function is disabled (a0 command) and the difference

between the carrier and noise estimate is greater  than an adjustable

threshold set by the c{+-} command, the decoder is muted. If the mark

signal gate is selected (a1 command), the carrier estimate is derived

only from the mark channel; if this signal exceeds the threshold, the

mark signal status bit is set.

The second quality estimator, called the pulse gate, determines the

decoder performance as follows. The Viterbi distances for received

characters are summed and averaged. If the pulse gate is selected (A2

command), and the average distance exceeds an adjustable threshold set

by the a{+-} command, the pulse gate status bit is set. When the

selected gate status bit is set, a counter is initialized to the

equivalent of 200 characters. This counter decrements as long as the

status bits remain set; if it decrements to zero, the decoder output is

clamped at mark.

Both the mark signal gate and pulse gate utilize a special voting

algorithm which counts the number of times the gate condition is

satisfied in a fixed window of samples. If more than an upper threshold,

the gate is opened allowing received characters to print; if less than a

lower threshold, received characters are blocked. There are separate

algorithms for the signal gate, pulse gate and SITOR synchronization

gate.

While the mark signal gate operates as soon as a character arrives, the

false-alarm rate is rather high. The pulse gate may take several

characters to operate, but has a much lower false-alarm rate. It is

designed for long periods of inactivity, such as when the system is

waiting for a selective call and it doesn't matter if a few characters

are lost at the beginning of transmission. During a period of activity,

the pulse gate should be turned off.

SITOR Mode

The SITOR/AMTOR Mode B (FEC) mode is compatible with the AMTOR mode

often used for HF broadcasts such as the W1AW bulletins. This modem

supports both receiving and transmitting in this mode at all baud rates

and shifts supported by the RTTY mode, but defaults to 100 baud and 170-

Hz shift. Only these defaults are permitted under current FCC

regulations. The y command enables SITOR mode; the r command resets the

modem and returns to RTTY mode. The same transmitter control functions

in RTTY mode are also active in SITOR mode. Although transmission is

continuous, if no characters remain to be transmitted, the CCIR 476

synchronization sequence RQ-ALPHA is transmitted. At intervals of about

ten seconds, five repetitions of this sequence are transmitted in order

to allow resynchronization of receivers and as time fill.

The SITOR decoder is an unusual design based on matched filters and

maximum likelihood principles. In fact, the decoder never makes a

decision based on classifying bits as zeros or ones. Rather, an approach

based on soft decisions is carried all the way through the processing

steps, with the final decision based on a correlation of each received

seven-bit code vector (plus its repetition) with each of the 35 CCIR 476

code vectors. The one with maximum correlation value wins and the value

becomes the signal quality estimate.

A type-I phase-lock loop with a linear phase detector is used to derive

symbol (bit) synchronization, so that the in-phase sample is used for

further processing. A set of 14 accumulators is used to resolve which

bit of the CCIR 476 code vector (two characters) represents the correct

character phase. Each accumulator value is determined by the correlation

of the last received 14 signal samples with the CCIR 476 synchronization

sequence RQ-ALPHA. Successful synchronization requires several sanity

checks and signal thresholds to be passed.

Modem Commands

All commands consist of single characters in either upper or lower case,

some with digit or +- modifiers. The defaults are presently set at 600-

Hz shift (m2), 75 baud (b2), 30dB limiter gain (l5), pulse gate off

(a0), sync enable off (s), display AFSK generator (d0), analog loopback

echo (e1), 850-Hz output shift (o).

Command   Default   Function

--------------------------------------

a{0-2}    a0        select autostart mode

0 disable autostart

1 enable mark signal autostart

2 enable pulse signal autostart

a{+-}     midrange  adjust autostart pulse gate threshold

b{1-6}    b2        select baud rate

1 100

2 75

3 50

4 45.45

5 25

6 10

b{+-}     midrange  adjust baud rate up/down 1 us

c         c0        select channels

0 mark/space

1 mark only

2 space only

c{+-}     midrange  adjust signal gate threshold

d{0-9}    d0        select display/output

0 AFSK generator

1 AIO input signal

2 filtered/limited input signal

3 mark signal

4 space signal

5 detector output

6 autostart signal

7 PLL phase signal

8 noise signal

9 carrier signal

e{1-3}    e1        select analog/digital loopback echo

1 analog loopback echo

2 digital loopback echo

3 full duplex

g{+-}     midrange  adjust filter gain

i         normal    select upright/inverted shift (toggle)

l{0-5}    l5        select limiter gain

0 0dB

1 6dB

2 12dB

3 18dB

4 24dB

5 30dB

m{1-8}    m2        select mode, filter and shift

1 IIR filter, 850-Hz shift

2 IIR filter, 600-Hz shift

3 IIR filter, 400-Hz shift

4 IIR filter, 170-Hz shift

5 MF filter, 850-Hz shift

6 MF filter, 600-Hz shift

7 MF filter, 400-Hz shift

8 MF filter, 170-Hz shift

m{+-}     midrange  adjust space frequency up/down 10 Hz

o{0-2}    c0        select output shift

0 follow input shift (170 Hz narrow, 850 Hz wide)

1 170 Hz shift

2 850 Hz shift

p{1-2}    1         select radio port

1 select radio port 1

2 select radio port 2

r                   master reset

s         off       enable/disable sync mode (toggle)

t{+-}               adjust radio frequency up/down one step

x         receive   transmit

y                   select SITOR mode

Note: For those commands with numeric or +- suffixes, the suffixes can

be used repeatedly for the same function. The function is redetermined

at the next command letter.

Indicators

LED 1     spike limiter threshold

LED 2     program running

LED 3     signal gate indicator

LED 4     pulse gate indicator

LED 5     mark channel output

LED 6     space channel output

LED 7     output mark indicator

LED 8     power indicator

Scope Signals

In transmit mode, the AIO output is always connected to the AFSK

generator. In receive mode, the AIO output can be connected to a DC-

coupled scope to view various internal signals of interest, including

the AFSK generator. The d command can be used to switch among the

signals listed above. Even when a scope is not available, a speaker can

be used as a fairly accurate tuning indicator and operating aid. The d1

monitor point is the AIO input signal supplied from the receiver. The

signal amplitude should be adjusted so that LED 1 flickers occasionally,

but is not on all the time. If the amplitude is too high, the signals

will be distorted, which is readily apparent to the ear. The d2 monitor

point is the input signal after the FIR filter and limiter. If the

limiter gain is reduced to unity (l0 command), the signal can be

centered in the filter passband by ear and checked with LEDs 3 and 4.

The d5 monitor point can be used as an aural activity monitor. When no

channel activity is present, the speaker will be silent. If a carrier is

present and characters are being received, the sounds produced mimic a

clunky old Model 12. That wasn't intentional, but it is satisfying.

Bugs

The transmit mode needs a watchdog timer.

The transmitter needs a right margin check and auto-CR/LF, especially

when using analog loopback echo.

Flow control should be implemented for both the transmit and receive

circular buffers.

Automatic speed select and upright/invert detect function could be

relatively easy to implement. Automatic normal/wideshift selection and

AFC would also be a valuable features, but harder to implement.

An automatic crossband repeater function would be fun. The modem could

listen on each port alternatively looking for an active pulse gate. When

heard the modem would key up the alternate transmitter.

Distribution Contents

readme.txt     this file

modem.asm      TASM source file, copiously commented and adorned

modem.obj      TASM object file, suitable for downline loading

modem.lst      TASM listing file

Include files from the DSP93 Distribution

regs.inc       TMC320 registers

ports.inc      DSP93 ports

serial.inc     16550 UART

Last updated 5 June 1995

David L. Mills, W3HCF

Electrical Engineering Department

University of Delaware

Newark, DE 19716

302 831 8247 office, 302 831 9211 home

mills@udel.edu