To: DSP-93 users...

Don, K4JPJ, has produced the following description of memory usage

in the DSP-93. This will be placed in the "doc" set in future

distributions. Thanks to Don for writing this up.

Now, the rest of you, choose your topic and write it up.

Ron W5RKN

===================================================================

Here are some "words" on the DSP-93 memory organization.  I had

to find the following out the hard-way.  Maybe it will help

others understand the details of the DSP-93.  There seemed to be

a gap between the TI literature and the TAPR material.  The TI

stuff gives everything about the TMS320, including a great deal

that is inapplicable to the specific DSP-93 application.  The

DSP-93 stuff left a big gap in what was going on with how

external memory is used, though there are clues here and there.

I didn't want to put such a tome out on to the dsp-93@tapr.org

folk without a) someone looking it over for errors, and b)

knowing if they might want it.  I figured I run it past you

first.

I banged this out on the keyboard while it was fresh, figuring

that not only would help others (Charlie, W4OQM, told me "take

good notes" for him), but it might serve as a reference a couple

years down the road when I've forgotten the details and need a

refresher.

Charlie, W4OQM,  and I have come up with enough ideas for the

DSP-93 to keep a small army of programmers busy for the next eon,

however lacking minions to command we will probably be satisfied

to accomplish just one.  For this we need to understand the unit

beyond how to hook it up to a TNC for receiving packet.

I've thought about (for years) *THE ULTIMATE* CW decoder, using

DFT or spectral estimation techniques rather than narrow band

filtering (I don't know if theoretically one has the advantage

over the other).  Charlie, has been tinkering with a scheme to

eliminate kerchunking on the local repeater (detect voice) as

well as deal with DTMF and other tones.  We may never get the

ideas finished, but working towards it 90% of the fun.

73's

Don, K4JPJ

------------------

MEMORY MAPPING

The purpose of the following is to provide some detail as to how

the DSP-93 implements memory in conjunction with the TMS320.

Integral to this is scheme for providing a monitor in EPROM and

high/low speed operation.

The Harvard architecture used in the TMW320 is quite different

from the typical microprocessor.  There are two memories utilized

during one instruction--data and program.  Until experience is

gained working with this architecture it is easy to forget this

basic principle.  There are two memories active at the same time.

By having two memories, a single instruction can load a word out

of program memory and do something with a word out of data

memory, all within the same cycle.  It facilitates implementing

filters and other digital signal processing algorithms.  For

example, stepping down a table of constants, such as filter

coefficients and doing a multiply and add to accumulator with a

data array of signal values can be accomplished at full machine

cycle speed with no overhead for instruction fetches, nor double

accessing of a single memory.

Program and data memory can change, and keeping track of "what is

which" is needed.  How this is done is largely a function of how

the DSP-93 design used the TMS320.  Therefore, the TMS literature

will not give the whole story necessary to understand the DSP-93.

Various clues are given in the DSP-93 docs and literature, but

this tries to bring it together into one place.  The following

details show how memory is mapped under various conditions.

The DSP-93 has a number of physical memories.  Some memory is

internal to the TMS320C25 chip, and other is external.  The

tables below outline the physical memories, as well as the

conditions which determine whether they are used by the as

program or data.  Following the tables is more discussion of how

the configuration is organized and used.

Internal memory (TMS320C25):

Label     Can be configured as     Size

B0        Data or program *        0100h

B1        Data only                0100h

B2        Data only                0010h

* - B0 is in data mode after a hardware reset, monitor reset, or

CNFD instruction.  It is program after a monitor "G" command, or

CNFP instruction.

External memory:

How the external memories are configured depends on the state of

the XF bit.  Hardware reset turns the XF bit ON, as will the SXF

instruction.  RXF turns it OFF.  SRAM U104, U105, U110, and U111

come with the basic kit and provide two 32K memories (one program

and one data).  Four more SRAM IC's will raise the amount to 64K.

IC number      XF = ON(1) "Slow speed"    XF = OFF "Fast speed"

SRAM           ----------------------   ------------------------

U104, U105     Data    0000 - 7FFF(2)   Prog      0000 - 7FFF(2)

U106, U107     Data    8000 - FFFF      Prog      8000 - FFFF(3)

U108, U109     N.A.    .... - ....      Data      0000 - 7FFF(2)

U110, U111     N.A.    .... - ....      Data      8000 - FFFF

EPROM

U102, U103     Prog    0000 - ????(2)   N.A.      .... - ....

(1) - XF line/bit is ON after a hardware reset, after a monitor

reset command ("R"), or after a SXF instruction.  RXF turns the

bit off, and it is also turned off after a monitor "G" command.

(2) - Addresses below 0400h access internal memory in the TMS320

and therefore are not available for use in the external memory.

(3) - Addresses FF00h - FFFFh access internal memory, B0, when B0

has been configured as program memory (normally after a monitor

"G" command).

Note that all memory is organized as 16 bit words, and not bytes.

Address 1000h addresses a 16 bit word, and address 1001h

addresses the next 16 bit word.  (In a byte oriented machines

such as the PDP-11, which can address 16 bit words or 8 bit

bytes, the address 1001h would address the second byte of a word,

thus making 16 bit word addresses, 1000h, 1002, 1004h, etc.--such

is NOT the case here).  Along the same lines remember that the

accumulator is 32 bits long so be careful about unintended sign

extension.

Internal memory, B0, is configured to the data mode after a hard

reset.  Also, the monitor in EPROM configures B0 to data when a

"R" (reset) command is executed, and to program when a "G"

command is executed.  Switching B0 modes is done with the

instructions CNFP and CNFD, (se program, and set data,

respectively), or the hardware reset that switches it to data.

External memory is switched via the XF line out of the TMS320.

This line is controled by the XF bit.  A hardware reset sets

this bit high.  The bit can be set/reset by the instructions

SXF/RXF, respectively.

When the XF bit is high, such as after power-up reset, EPROM

which contains the monitor programs is active as program memory.

This makes it possible to get the machine running with something

intelligent.  Since inexpensive EPROM is slow, the XF also

selects a low sped mode (divides the 40 mhz clock by two, or

four depending on jumper J100).  In slow speed the processor is

slow enough to accommodate the EPROM.  XF also makes external

memories U104,5,6,7 switch to data mode and U108,9,10,11 not

accessible.  This arrangement allows the EPROM monitor to load

program into data memory (U104,5,6,7 and B0).  This switching is

necessary since the TMS320 does not execute instructions which

store anything into program memory; the downloaded program is

placed into the DSP-93 as data.

With the monitor program executing out of EPROM, the usual step

is to download a program from the general purpose computer.  The

program being downloaded is stored as data, in data memory, which

is U104,5,6,7 at this time.  Upon completion of the downloading,

the general purpose computer issues a "G" command which causes

the DPS-93 monitor to turn off the XF bit.  This switches

U104,5,6,7 from data memory to program memory.  The monitor also

jumps to location 1008h (of program memory) to start the program

that was just loaded (into what was data memory).

With XF low, program normally runs out of external SRAM,

U104,5,6,7, using U108,9,10,11 for data storage.  B0, B1, and B2

can also be used.  B0 as stated before can be configured either

as data or program, though remember that the monitor sets it to

program after the loading process completes and a "G" command is

given.  Operating out of internal memory is somewhat faster than

external memory and may needed for time critical operations.  The

TMS320C2X User's Guide shows timings for instructions according

the memory combination being used.

Since B0 can be switched between data and program it can be

loaded with a program which can be executed.  During the normal

program downloading process B0 is configured as data, so it can

be loaded with program no differently than U104,5,6,7.  The

addresses where the program loads of course must be correct.  If

the program has code which was preceded by .ORG directive with

the address so that the code assembles in locations 0200h -

02FFh, the loader will put that code into B0.  Some code .ORG'ed

to 1000h is also needed.  When the loading process completes, the

monitor does a "G" command and jumps to 1008h.  The program at

1008h can then jump to B0 and execute the code which was loaded.

When B0 is configured as program it is no longer at locations

0200h - 02FFh, but occupies FF00 - FFFFh.  Therefore, the code

assembled at 0200h - 02FFh must be capable of executing properly

when moved to locations FF00h - FFFFh.  The jump at from the

program in U104,5,6,7 will be to FFxyh, if the beginning of the

code loaded into B0 is 02xyh.

External memory is not accessed for addresses below 0400h as

these are reserved for the TMS320.  Also, FF00h - FFFFh of

program memory is notaccessed when B0 is configured as program.

Note that some addresses are really registers within the TMS320,

such as locations 0 and 1 which are used to load/receive the

serial shift register data to/from the AIO chip.  The TMS manual

covers these in detail.

External memory, (prog, fast), 0400h - 1000h holds the monitor

which is used when in the fast mode (since the monitor in the

EPROM is only available in the slow mode).  If these locations

are blasted, such as with a monitor Fill command, the monitor is

lost and a hard reset is required.

The TMS320 cannot load and store (i.e. move) data from program

memory to program memory.  Data-to-data memory can be

accomplished, as well as program-to-data and data-to-program.

Therefore is not likely that a runaway program will blast the

monitor stored in 0400h - 1000h.

The monitor does not use B0 during the slow mode.  Therefore it

is possible to load the entire 256 words with program, or Fill it

via the monitor.  However, if a reset command is executed by the

monitor, locations 0200h - 0275h are overwritten with "stuff"

from the monitor, wrecking what might have been downloaded into

B0.  As long as a monitor reset does occur between the loading of

B0 and utilization of B0 (either as data or program), then the

full page can be used.  Otherwise, only those locations not used

by the monitor can be used ( 0276h - 02FFh ).

[N5BRG] The monitor does use B0. It uses the "stuff" that is

placed in B0 during the transition from low speed to high speed.

So this will need a few changes.

Sine table--

A 540 deg, 0.25 deg step, sine table is located in EPROM.  The

monitor moves this table into locations (prog, fast) FF00h

F86Fh.  This is just below the B0 addresses of FF00h.  However,

if the additional 32K or memory has not been installed, the sine

table will not be in SRAM.  Furthermore, ROM is not accessible

when executing program in U104,5,6,7.

One alternative is to assemble the table from the source code,

SINCO512.DAT, with an appropriate .ORG and load it into program

memory as part of a program.  It can also be loaded separately,

with a separate loading step to start the program that uses the

table.

Another alternative is a program to move the monitor sine table

into the lower 32K of (prog, fast) SRAM.  This can be done by a

small program that .ORGs into B0 (02C0h for example), plus a

branch from the normal program start, 1008h, to the B0 code after

loading (FFC0h). This small program then turns ON the XF bit

which switches U104,5,6,7 to data and makes EPROM available as

program memory.  The sine table is then moved from the EPROM to

data memory, and the XF bit turned back off.  The program can

then jump down to 100Dh and pick up where it left off, but now

with the sine table moved into the high end of the lower 32K.

Note that the same process of loading B0 then branching to B0

after the loading and switch to high speed can be used to run

code out of internal program memory, so as to achieve maximum

speeds.

Don, K4JPJ