PASSTHRU.EXE

The program PASSTHRU.EXE is intended to toggle the Passthru mode of the

Paccomm TNC for the TAPR/AMSAT DSP-93. This mode allows the DSP-93/Paccomm

TNC combination to be operated using a single serial COM port on your

computer. When the Passthru command is enabled (Passthru = ON), the common

serial port is connected to the DSP-93 to allow for modem loading. When

the Passthru command is disabled (Passthru = OFF), the common serial port

is connected to the Paccomm TNC. This arrangement allows one PC serial port

to be used for both DSP-93 modem loading, and communication with the Paccomm

TNC, significantly reducing the PC serial port resources that must be

dedicated to the DSP-93/Paccomm TNC combination.

The program is used as follows:

PASSTHRU <COMPORT>,<PRINTFLAG>,<MODE>

COMPORT is the COM port you are using with the DSP-93 and

Paccomm TNC. Valid entries are 1 and 2.

PRINTFLAG is a switch that enables diagnostic message printing

to the CRT. Normally left OFF. Valid entries are 0 = OFF, 1 = ON.

MODE is the Passthru state that you wish to set. Valid entries

are 0 = Passthru OFF, 1 = Passthru ON.

Example: PASSTHRU 1,0,0

Uses COM1 to disable Passthru (Passthru = OFF) with no diagnostic

printing.

The serial baud rate for this program is fixed at 19200 baud. Be sure that

you set the Paccomm TNC up for this baud rate prior to using it!

DSPBAUD.EXE

The program DSPBAUD.EXE is intended to toggle the HDLC baud rate of the

Paccomm TNC for the TAPR/AMSAT DSP-93. This program can be run from a batch

file to allow an application like WiSP to automatically configure the Paccomm

TNC HDLC baud as needed for each satellite. For example, it is necessary to

set the TNC HDLC baud rate to 1200 baud for PSK satellites (AO-16, LO-19,

etc.) and to 9600 baud for DFM satellites (UO-22, KO-23, etc.)

The program is used as follows:

DSPBAUD <COMPORT>,<PRINTFLAG>,<BAUD>

COMPORT is the COM port you are using with the DSP-93 and

Paccomm TNC. Valid entries are 1 and 2.

PRINTFLAG is a switch that enables diagnostic message printing

to the CRT. Normally left OFF. Valid entries are 0 = OFF, 1 = ON.

BAUD is the HDLC Baud rate of the Paccomm TNC. Valid entries

are 0 = 1200 baud, 1 = 9600 baud.

Example: DSPBAUD 1,0,1

Uses COM1 to switch the Paccomm TNC HDLC baud rate to 9600 baud

with no diagnostic printing.

The serial baud rate for this program is fixed at 19200 baud. Be sure that

you set the Paccomm TNC up for this baud rate prior to using it!

Putting It All Together

A typical batch file (run prior to a satellite pass using the Schedule

Events option in WiSP, for example ) for a 1200 baud PSK satellite looks

something like this: AO16.BAT

PASSTHRU 1,0,1          <---- enable passthru mode

DSPLOAD psk93r1j.obj    <---- load DSP-93 modem

PASSTHRU 1,0,0          <---- disable passthru mode

DSPBAUD 1,0,0           <---- enable 1200 baud TNC HDLC

A typical batch file (run prior to a satellite pass using the Schedule

Events option in WiSP, for example ) for a 9600 baud DFM satellite looks

something like this: KO23.BAT

PASSTHRU 1,0,1          <---- enable passthru mode

DSPLOAD fsk93k1.obj     <---- load DSP-93 modem

PASSTHRU 1,0,0          <---- disable passthru mode

DSPBAUD 1,0,1           <---- enable 1200 baud TNC HDLC

Of course, all the executables (and modem files) should be located in the

WiSP directory. If not, path information must be included in the batch file

to allow the PC to find the requested files.

These programs work well at my station. That's as far out as I'm willing to

stick my neck. If you find that they work for you, please feel free to use

them. Lastly, if you think of any modifications that might make them more

useful to you, please let me know.

Best 73s  John KA1JMF