Set TNC to 1200 baud!

================================

PKTP1.OBJ - Radio Port 1, gain 4

PKTP2.OBJ - Radio Port 2, gain 4

================================

To assemble, use the command "asm fsk.asm -dRn -dGn"

where Rn is R1 or R2 for radio ports 1 or 2 and

where Gn is one of G0, G1, G2, G3, G4, G5, or G6 to set the gain

Pressing 'r' or 'R' on the terminal or sending an 'r' or 'R' to the modem

on the DSP-93 serial port will terminate the modem program and return

control to the DSP-93 Monitor.

1200 bps full-duplex standard packet modem (Bell 202)

LED213 is DCD

LED214 is PTT

LED215 is ~DCD

LED216 is level indicator

LED217 is duty cycle

LED218 is power

After working 9600 bps FSK, standard packet seems a bit tame. Anyway,

here is a basic modem of standard (Bell 202) 1200 bps packet.

Audio from the receiver is digitized and band-pass filtered with a 48-tap

FIR filter. The band-pass filter output drives a delay line discriminator,

which is then low-pass filtered with a 32 tap FIR and sliced to recover the

received data stream. This stream is output on pin 17 of the DSP-93's

modem disconnect header.

LED 215 is slaved to DCD (low = true). Detection of a 0 bit starts a 50 ms

counter for DCD. LED 216 lights on incoming positive signal peaks =>

75% of the desired signal level (see tuning below).

Incoming TX data is read from pin 19 of the DSP-93's modem disconnect

header. This data stream drives a dual-frequency VCO ramp; a 1 bit runs

the ramp at 2200 cycles/sec, and a 0 bit drives the ramp at 1200

cycles/sec. The ramp output indexes into a sinewave look-up table to

create the AFSK output.

LED 217 is driven by the code's duty cycle. However, you have to look at

U215 pin 1 to see it; the buffer driving LED 217 is too slow to follow the

action. This code runs in less than 40% of a sample period; there is plenty

of time to add bells and whistles...go for it!

The DSP-93 assembly manual shows the interconnection cable I am using

between the DSP-93 and my MFJ1270C TNC. I recommend that you use

a shielded cable, with the shield tied to pin 15 at each header. Note that

several local jumpers are used on the TNC modem disconnect header.

Pin 6 is connected to pin 10 with a PC trace, shown as a dotted line. Only

five connections are made at modem disconnect on the DSP-93. DO NOT

MAKE ANY CONNECTIONS OTHER THAN THOSE SHOWN.

The assembly manual also shows where the pins on the connector that

PLUGS INTO THE DSP-93 RADIO 1 JACK go. I use an 8 pin microphone

connector at the other end of the cable and plug into the radio's mic

connector to run standard packet.

To set the modem up for receive, tune in traffic on your local digipeater

and crank up the receiver's volume control until LED 215 (DCD) turns off

cleanly and LED 216 glows steadily at about 75% of LED 217's

brightness. You can double check this level with your scope by looking at

pin 26 of the AIO. Here you will see a �3 V audio signal just going into

limiting.

The TX output audio level is set by first running R214 down to the bottom,

and slowly cranking it up until the

transmitter output, monitored on another receiver, sounds about a load as

a strong digipeater. In the case of my ICOM 275H, it wants about 50 mV p-p

(unloaded) at the microphone input. NOTE that this level is much lower

than when running FSK into the varactor input!!!

BTW, the DSP-93 generates a lot of 2 meter interference if the top of its

metal case is left off (you do have yours in a metal case, right?!).

This code also attempts to lock out the PTT signal if the TNC is turned off.

However, it may not work with every TNC....beware.

Frank H. Perkins, Jr., WB5IPM

License and Disclaimer

A no cost, non-exclusive license to use the enclosed software is granted

to all users of the TAPR/AMSAT DSP-93 DSP computer, with the restriction

that it be used only for non-commercial amateur radio applications. ALL

other uses are prohibited under this license. Copyright 1991, 1992, 1993

and 1994, Frank H. Perkins, Jr., WB5IPM. All rights reserved.

This software is EXPERIMENTAL. It is offered AS IS, without Warranty or

Guarantee of any kind. NO representation is made that this software is

suitable for any purpose, or that its operation is reliable. Use this software

strictly at your own risk. The author, AMSAT and TAPR accept no liability

for any loss or injury arising directly or indirectly from its use.

Never use this software under circumstances where its failure to operate

or its failure to operate as expected, or the failure of the attendant

hardware could create a dangerous situation, cause loss of data, or could

cause damage to your amateur radio equipment, computer equipment, or

any other equipment or property.

Please contact Greg Jones at TAPR or the author for information on

commercial licenses of this software.

Frank H. Perkins, Jr.