Radio Port 1!

No TNC needed!

==========================

APT_93B.OBJ - Radio port 1

==========================

This code is to collect data from the NOAA Weather satellites.  After

collection of the raw data from the wx birds, the data must be converted

by an image analysis program.  Read on....

***DON'T EXPECT A NORMAL AMATEUR RADIO TO RECEIVE GOOD (IF ANY)

PICTURES.  USE OF THIS CODE REQUIRES A SPECIAL WIDE BAND FM

RECEIVER!!!***

Here are a couple of points on running the code:

Video is sent at the rate of 2400 bytes/sec, using the RS232-C link

between the DSP-93 and your computer. Note that you don't need

the TNC ON or even connected for this modem.

The RS232-C link runs at 38.4 kbps, 8-N-1. You will need a '550

UART or equivalent to keep up with this data rate. I use Procomm to

capture the video data using the RAW ASCII download mode.

To use the modem, load the code in the normal way (at 19.2 kbps

using Bob's loader [dspload]) and start up Procomm on the same COM port

that you loaded code from. Be sure Procomm is set to 38.4 kbps.

Put Procomm in the RAW ASCII download mode using a filename of

your choice.

Tune in an APT signal and adjust the audio level so that LED216

blinks ON at the sync pulses (tick-tocks), with a bit of flickering in

between. Start the APT video download with the lower case "g" key

on your PC keyboard, and stop it with the lower case "s" key. Its

easy to download several megs on a high APT pass. be sure to

close your download file on Procomm after you hit the "s" key

(Procomm uses the ESC key to terminate a RAW ASCII download).

I use NASA's IMDISP program to display the video. You can get the latest

version off of DRIG. Starting IMDISP file settings are 400 to 800 lines,

2400 samples/line, 8 bits/ sample, 0 bytes header. Then tweak to suite.

Frank WB5IPM

Half-Baked Software et al ....

License and Disclaimer

A no cost, non-exclusive license to use the enclosed software is granted

to all users of the TAPR/AMSAT DSP-93 DSP computer, with the restriction

that it be used only for non-commercial amateur radio applications. ALL

other uses are prohibited under this license. Copyright 1991, 1992, 1993

and 1994, Frank H. Perkins, Jr., WB5IPM. All rights reserved.

This software is EXPERIMENTAL. It is offered AS IS, without Warranty or

Guarantee of any kind. NO representation is made that this software is

suitable for any purpose, or that its operation is reliable. Use this software

strictly at your own risk. The author, AMSAT and TAPR accept no liability

for any loss or injury arising directly or indirectly from its use.

Never use this software under circumstances where its failure to operate

or its failure to operate as expected, or the failure of the attendant

hardware could create a dangerous situation, cause loss of data, or could

cause damage to your amateur radio equipment, computer equipment, or

any other equipment or property.

Please contact Greg Jones at TAPR or the author for information on

commercial licenses of this software.

Frank H. Perkins, Jr.