RS-232 Programmer by Jens D. Madsen

Command line arguments

COMn    where n is 1..4 selects COMn

HIGH    TxD high when eeprom programming (com84 compatibility)

REMOVE  Remove driver from memory

BYPASS  Bypass Windows port virtualization

Original Hardware Schematics

See JDM84.GIF.

EEPROM programming by using pin 5-8 and 11-14.

In Circuit Programming

Use /MCLR, RB7, RB6 and Vss (Gnd) for In Circuit Programming.

Automatic 'GO' function is added by using a diode and a

resistor (680 ohm) between Vcc and /MCLR:

1N4148

Vcc ---|>|-----[680E]--- /MCLR