TAC-2 Circuit Description

Tom Clark – W3IWI

29 Sept 96

Introduction: This will serve as a working document for the TAPRized “Totally Accurate Clock” (TAC-2) design. The TAC-2 is my general purpose follow-on to my original TAC design that is discussed at URL:

ftp://aleph.gsfc.nasa.gov/GPS/totally.accurate.clock/

The original TAC had the following salient specifications:

- “Clean” mechanical mounting and “Plug & Play” electrical inter-connections to GPS receiver with a ~3” x ~4” circuit board.

- Provision of (somewhat) isolated electrical power interface to GPS receiver from a +12VDC “primary” power source and isolated RS232 I/O to/from a PC-class host computer.

- Provision of a wide (100 or 200 msec duration) RS232 1PPS signal to host computer on the DCD handshaking line to allow accurate setting of the computer’s clock.

- Buffered and isolated high accuracy 1PPS outputs for the user and to a flashing front-panel LED indicator.

- Optional “or-gate” RS232 input to GPS receiver from RTCM SC-104 DGPS beacon receiver.

- Optional battery backup support for GPS receiver.

- Optional antenna re-biasing circuitry.

- Optional MMIC L-band RF amplifier.

My original TAC was designed to support only the original Motorola “PVT-6” receiver (which Motorola  later renamed the ONCORE BASIC). The new TAC-2 interface circuit board is fully compatible with the original TAC (except that item H, the L-band MMIC amplifier, has been deleted). The new TAC-2 PCB has the following enhancements:

- In addition to the ONCORE BASIC, the TAC-2 offers “Plug & Play” support for the Garmin GPS 20/-25 and Motorola ONCORE VP (with the addition of a few optional components) receivers and on-board jumper options to support many other receivers.

- The RTCM SC-104 “or gate” has added squelch logic to suppress DGPS inputs when higher priority inputs (like configuration data sent from the host computer) are present.

- A new expansion port designed to simplify the interface with an embedded microprocessor board  has been added. This interface can parisitically monitor or control the normal GPS-to-computer communications.

- The user 1PPS outputs have been augmented with an optional fast, low-Z pulse driver.

- A narrow (~5 usec) 1PPS pulse has been added to be used in a future disciplined oscillator module to achieve Cesium-like frequency stability with a low-cost crystal oscillator.

- Because the different receivers have differing power requirements, a number of power options have been added. This includes the ability to have a “mini-UPS” that can buffer the TAC in the event of a brief power failure.

To discuss the design in more detail, the reader should fetch the file

ftp://aleph.gsfc.nasa.gov/GPS/tac2/tac2-dwg.zip

In that file are four PostScript drawings with file names like tac2-sh1.ps thru tac2-sh4.ps; these are the current working schematics and will be called “Sheet 1” thru “Sheet 4” in this discussion. The .zip file also contains 2:1 PostScript drawings for the 4 layers of the PCB (tac2-top, tac2-bot, tac2-in1 and -in2) , the top and bottom solder mask layers (tac2-smt and -smb), silk screen (tac2-sst) and drill master (tac2-drl). And it contains a working-level annotated Bill of Materials in Excel (tac2-bom.xls) and Lotus (tac2-bom.wks) formats.

1:SHEET 1 – RS232 I/O:

1a: RS232 Port #1  The main (computer) RS232 Port#1 is handled by U2, a generic MAX232-like chip, which has two RS232 line drivers (TX1 & TX2) and two RS232 line (RX1 & RX2) receivers. The line drivers and line receivers are all inverting inverting devices. TX1 generates the TXD RS232 data signal sent to the  host computer. TX2 generates a 1PPS RS232 signal sent to the computer on the DCD handshaking line. P1 is a 10-pin connector designed to be routed to a DB9 female connector to interface directly with a PC-AT standard port. The 10th wire is available as a “frame ground” for the metallic shell on the DB9 connector. The RX1 section of the MAX232 receives RS232 data from the PC. RX2 is used as an inverter as described later in section ???. The unused pins on P1 are available on solder pads adjacent to the connector in case they are needed for a custom application.

1b: RS232 Port #2   The 2nd RS232 port is intended for use with either an RTCM SC104 DGPS beacon receiver or as a connection to a second host computer. The connector P2 provides an interface to a standard DB9 RS232 port. The jumper area JP1 permits the P2 connection to be “swapped” without the need for a null modem. Cutting the default traces associated with the DB9’s pins 2 & 3 permits swapping TXD and RXD. In addition, the 1PPS signal can be changed from the default DCD line to RI.

U1A is a Schmidt trigger used as an RS232 line receiver. Diodes D1 & D2 along with R1 prevent the input to U1A from going above +5v or below zero volts (the gate inputs on U1 are internally diode protected, but I decided it was best to provide additional protection). RP1A sets the default when Port#2 is not used to a logic zero.

1c: Port#2 “squelch”  Port#2 is enabled only if its second input (pin 2) is high. If there is any activity on the Port#1 RXD input, diode D3 pulls this pin low, disabling Port#2 RXD. This pin stays low for a time interval determined by the R2C1 time constant (~10 msec for the parts values shown). In addition, this port can be disabled from the expansion port by pulling the RX Holdoff line low.

1d: Combining the RXD signals  U1B serves as an or-gate to combine the RXD signals from Ports #1 and #2. A 3rd input to the or-gate is provided by D4 and RP1D so that an embedded processor connected to the expansion port can send signals to the GPS receiver. U1B re-inverts the signals (zero low, one high) and is sent directly to the GPS receiver’s RS232 input (Sheet 3) as GPS RXD. [Note: Both the Motorola ONCORE BASIC and Garmin receivers have “RS232” inputs. However both work just fine when driven by 0/+5 v CMOS logic without converting it to “real RS232”. Also on Sheet 3 is an optional inverter to handle the Motorola ONCORE VP, which uses inverted TTL logic levels.]

1e: TXD signals from GPS to Ports #1 and #2   U1C functions as an inverter for the GPS receiver’s RS232 TXD output signals. R3 provides a current limiting resistor so that RS232 levels won’t burn out the internal protective diodes in U1C [Note – we need to experiment with this resistor. 2.2k or 4.7k may be better.]. For the ONCORE VP with inverted TTL levels, the RX2 section of U2 is used as an inverter and the jumper JP2 needs to be changed from the default. The second input on U1C is normally pulled high thru RP1B, but the embedded microprocessor can squelch TXD by pulling the TX Holdoff line low. The diode D5 and pullup resistor RP1C allow inverted data from the expansion port tp be sent to the host PC.

2: SHEET 2 -- 1PPS I/O

2a: The “normal” 1PPS Output Path  The GPS receiver is assumed to have a positive-going 1PPS pulse that is “wide” (100 msec for the GPS-20, 200 msec for the Motorola receivers) that has a rising edge coincident with the UTC second (the pulse can be offset by know amounts in the Motorola receiver). This signal is first inverted in U3B (a 74AC04 is chosen for U3 to provide a low propagation delay and high output drive) to make 1PPS INV. The 1PPS INV signal directly drives the U2 TX2 section to make the DCD 1PPS signal. U3C reinverts the signal to drive a front panel LED via R4 and to provide a buffered wide 1PPS signal to the expansion port. U3D+E+F are paralleled via RP2A+B+C to provide a buffered 1PPS signal to the user. [Note – the original TAC had two such 74AC04-derived drivers].

The Schmidt trigger U1D provides a clean, well defined negative-going 1PPS signal called INV PULSE. The pulse goes negative coincident with the GPS UTC second and stays negative for a time interval determined by the C2+RP2G network (plus the negligible propagation delay thru U3B) set at about 5 usec.

All the 1PPS outputs appear on the 10-pin header P3. If IDC cable is used, the cable can be stripped into wire pairs to go to the various external connections. Alternately, several 2-pin headers can be used for the interconnections.

2b: Precision Low-Z Pulse Driver  The user has the option of adding U4, an Elantec 7242C dual CMOS driver chip, as a dual precision 1PPS driver. The 1PPS polarity from each half can be set independently by changing jumpers at JP15/16. The default is a positive-going pulse (pins 2-3 and 4-5 jumpered), but it can be strapped to be negative-going (pins 1-2 and 3-4 jumpered). The supply voltage for U4 can be either the normal “+5” or defaults to the “+6” volt bus (see Sheet 4) with JP13. Two sections of the RP2 resistor pack are paralleled as “snubbers”. With the +6v supply and each RP2 section = 47 ohms, the U4 outputs will drive ~+4v into 50 ohm loads.

2c: Alternative 1PPS Inputs  While the Motorola and Garmin receivers have similar “user friendly” 1PPS signals, not all receivers are so convenient. For example, the Trimble SV-6 has a negative-going 1 usec wide 1PPS signal driven by an open collector transistor. The optional JP14 and U5 circuitry is available to accommodate this case. If the default jumper on JP14 (between pins 1-3) is cut and jumpers 1-2 and 3-4 are added, then RP1F provides the needed pullup for the SV-6’s open collector output. U5 (a CMOS 555 timer) then functions as a one-shot to provide the desired “wide” positive-going 1PPS signal with a width determined by the R5+C4 timing network.

In the event of a narrow positive-going pulse, JP14 has pins 3-5 jumpered to allow U3A to invert the signal. U3A’s output is sent to U5 by jumpering pins 4-6 and U5 functions as described above. If there is a triggering problem with U5, add C3 and cut the default jumper across it in order to insure that U5 is “hit” with a clean, negative going pulse.

3: Sheet 3 – GPS Receiver and Expansion I/O

3a: ONCORE BASIC  To connect an ONCORE BASIC receiver, a 10-pin header is installed at P4 (note that Motorola used a non-standard pin numbering convention for the BASIC. Sheet 3 shows pin numbers on P4 that match Motorola’s documentation.). The BASIC has an on-board DC-DC power converter so its power can be either +5V or +12V. The jumper area JP17 defaults to the +12v connection, but can be changed if desired. The 4 mounting holes for the BASIC match the holes in the corners of the TAC-2 PCB and it was my intention that the BASIC receiver be mounted below the TAC-2 PCB. It is to be connected with a short (~1.5”) 10-pin IDC cable between the receiver and P4.

3b: ONCORE VP  To connect a ONCORE VP, the user should install a 10-pin socket at J1. The VP will then have its mounting holes match 4 of the holes on the TAC-2 PCB (with the VP above the TAC-2 PCB). Since the VP uses inverted TTL levels on its serial port, the user adds the optional inverter consisting of Q1, R6 and R7. The user also must insure that the jumper JP2 (Sheet 1) is changed.

3c: Garmin GPS-20  The Garmin power+I/O connector is less convenient to use than for the Motorola receivers, and the user will have to prepare a short (~2”) 5-wire cable with the Garmin connector on one end. The other end solders to the TAC-2 PCB at JP18. The silk-screen on the TAC-2 PCB identifies the appropriate pin numbers for the Garmin connector. JP18 is also used as an interface area for any other receivers. The mounting holes for the Garmin match holes on the TAC-2 and the Garmin can be mounted either above or below the TAC-2 PCB.

3d: Expansion I/O  The 16-pin plug area at P5 provides connections for other “widgets”. I have in mind a clock display module and a disciplined oscillator module that will use this connector. This connector has 3 1PPS signals (plus-going, minus-going and 5 usec wide pulse as described on Sheet 2),   the TXD+RXD+ holdoff signals (described on Sheet 1) and DC power. P5 has 3 uncommitted pins for expansion.

4. Sheet 4 -- POWER

4a: The “+5” Power supply   The main +5v power for the GPS receiver and the TAC-2 logic is obtained using a standard, lossy TO-220 7805-type regulator at U6. The TAC-2 PCB has a generous amount of metal to act as a heatsink, but there should be room for a finned radiator if needed.

Experience has shown that it is desirable to allow for trimming of the “+5” supply voltage. At least on GPS-20 receiver required about +5.2v to function properly. In addition, the Low-Z pulse driver (EL7242C, U4) wants more like +6v to be able to generate a full 5v pulse into a 50 ohm load. Therefore I allowed for a “+6v” bus which can be dropped to +5 in one or two 1N4001 diodes at D7+D8. In order to “tweak” the 7805 to the desired voltage, the “gnd” of U6 (and the heatsink) is allowed to float above true ground by a resistor (or a diode) at R9, with the voltage tweaked by the R8+R9 network. [Hopefully the “alpha” group will help define just what TBD parts are needed for R8, R9, D7 and D8 in the TAPR kit!]

4b: The 12v Input Circuit   Although the PCB shows it as an 8-pin DIP, D6 is a quad 1N4001 diode array which does several things:

- If the user supplies +12VDC, it can be connected between pins 2 & 3 of P8 (also available as a 2-pin solder area adjacent to P8). The two diodes that are then in the circuit (the right-hand pair on Sheet 4) insure that the user can’t do something dumb and apply the wrong polarity – the lower diode will short to gnd and the upper diode only passes the “good” polarity.

- The other pair of diodes (left-hand pair) then allow the user to install a small battery in the TAC (like a 9v NiCd) to provide a “mini-UPS” capability. The battery only sources current if the +12v supply is below the battery voltage. In this case, R11 (value TBD) supplies charging current to the battery.

- The user can use the entire D6 quad as a power supply rectifier if the user wants to use an AC wall-mount transformer. In this case, AC is supplied to P8 pins 2&4. Since the capacitors on the PCB are small, this will require an external filter capacitor across pins 1&3.

[Note Regarding D6: Greg suggests using discrete 1N4001s because TAPR has a lot of them; the layout is such that the diodes fit side-by-side neatly providing they can have leads bent to fit a 0.3” spacing.

An alternate is one of the circular epoxy-packaged quad diode packages that are about 0.4” in diameter; the standard part has lead spacings of 0.22”,so the leads have to be spread a bit to hit the corners of the DIP footprint. I originally thought there was a standard 8-pin DIP part with the layout I used, but apparently I was wrong.]

4c: Battery Backup Option   This circuit is cloned from the original TAC. Many of the ONCORE receivers don’t come with batteries (which keeps BBRAM alive and keeps a 32 kHz oscillator + clock alive). So we have used either 9v or 5v NiCd cells in our production TACs. D9 + R10 + P6 allows me to keep this as an option. The TAPR Garmins have a battery already, so this is not needed for that application.

4d: Antenna Bias Option   This circuitry lets us supply special antenna bias voltages for some applications.  The circuit is pretty self-explanatory.