Tucson Amateur Packet Radio

8987-309 E. Tanque Verde Rd #337

Tucson, Arizona 85749-9399

Office: (817) 383-0000 Fax: (817) 566-2544

Non-Profit Research and Development Corporation

TAC-2 Assembly Manual

This document was written April 1997. Rev. 0: April DD, 1997.

Copyright 1997 Tucson Amateur Packet Radio Corp.

The information contained in this document has been carefully checked and is believed to be entirely reliable. However, no responsibility is assumed for inaccuracies. Tucson Amateur Packet Radio Corporation (TAPR) reserves the right to make changes in any products to improve reliability, function or design without obligation to purchasers of previous equipment. TAPR does not assume any liability arising out of the application or use of any product or circuit described herein; neither does it convey license under its patent rights or the rights of others. Reproduction or translation of any part of this work beyond that permitted by sections 107 or 108 of the 1976 United States Copyright Act (or its legal successor) without the express written permission of Tucson Amateur Packet Radio Corporation is unlawful except as noted below. Requests for permission to copy or for further information should be addressed to Tucson Amateur Packet Radio Corporation. Except as noted above, permission is hereby granted to any non-profit group or individual to reproduce any portion of this document provided that: the reproduction is not sold for profit; the intent of the reproduction is to further disseminate information on Amateur Packet Radio; the reproduction is not used for advertising or otherwise promoting any specific commercial product; full credit is given to Tucson Amateur Packet Radio Corporation (including address) as the original source of information; and Tucson Amateur Packet Radio Corporation is notified in writing of the reproduction.

Table of Contents

1. INTRODUCTION

2. PARTS LIST

3. CONSTRUCTION NOTES

4. CONSTRUCTION

5. INSTALLATION

- Garmin GPS-20

- Motorola Oncore Basic

- Motorola Oncore VP

- Trimble SK8

6. OPERATION

7. THEORY OF OPERATION

8. BOARD LAYOUT

--------------------------------------------------

1. INTRODUCTION

The "Totally Accurate Clock" TAC-2 printed circuit board (PCB) is intended to serve several purposes:

- It provides a "universal" electrical and mechanical interface for a number of common OEM board-level GPS receivers including specifically:

- Garmin GPS-20

- Motorola Oncore

- Trimble SK8

- It provides interfaces for the 1 pulse-per-second (1PPS) signal generated by these receivers:

- Low-impedance, fast rise-time 1PPS signals for "laboratory" applications.

- RS232 level 1PPS signals for computer applications.

- Specialized 1PPS interfaces for an add-on PCB that will stabilize a low-cost crystal oscillator to an accuracy ~ 1 part-per-billion

- It provides several different power supply options to make use of your GPS receiver easier. The power interface is similar to (and compatible with) those used for computer disk drives:

- A low-cost, (7805-type) regulator.

- A high-efficiency switching power supply.

- Direct 5 volt power.

- It has provision for an Uninterruptible Power Supply (UPS) to buffer the GPS receiver through brief power outages.

- It provides Battery Backup so the GPS receiver can wake up "smart."

- It can provide isolated power for an amplified GPS antenna.

Since the TAC-2 has so much flexibility, there are several options you will have to choose from during construction. The "base" implementation will satisfy the needs of many radio amateurs with support for the Garmin GPS-20 and Motorola ONCORE Basic receivers. A few jumper changes enable support for the Motorola ONCORE VP/UT receiver. Some added parts and some more jumper changes are needed if you plan to use a Trimble SK-8. Your first decision is to determine which GPS receiver you plan on using with your TAC-2 and which options.  Once you have chosen then carefully follow the construction instructions.  In the future if you change GPS receiver or want to add an option, it can be added later.

NOTES:

- Please check the shipment for any possible errata sheet(s) and/or additions/corrections to instructions provided in this manual.

- Please read the Construction Notes prior to starting assembly.

- Determine which GPS receiver and options you plan to install.  Then find and read your GPS receiver installation and options before starting assembly.

Overall, construction and check-out of the TAC-2 will probably take you only an evening or two.  Warm up your soldering iron, take your time, and enjoy!

ACKNOWLEDGEMENTS

The Tucson Amateur Packet Radio (TAPR) TAC-2 Kit was made possible by the pioneering efforts of Tom Clark, W3IWI, and the active participation of a number of Amateur Radio operators and organizations worldwide.  TAPR wishes to acknowledge their contribution to this effort.

Individuals:

...and the builders of the first run kits whose feedback made this version of the manual possible.

Kit Builders:

--------------------------------------------------

2. PARTS LIST

This parts list is organized by quantity and part type.  Please verify that all parts are present, checking the space [_] provided as you locate and verify it against this list.  You may wish to take this opportunity to sort the parts into a compartmented container, such as an egg carton, muffin tin, or other container as you inventory them.  This will aid you in building.

Base Configuration: The following parts are required for the base configuration of the TAC-2 which are independent of the GPS receiver or options you plan to install.

Resistors

1/4 watt, 5% carbon film:

[_] 1	R1		1 Megohm (brown-black-green-gold)

[_] 1	R2		510 ohm (green-brown-brown-gold)

[_] 1	R4		tbd

[_] 1	R5		tbd

[_] 1	R6		180 ohm (brown-gray-brown-gold)

8-pin Single Inline Package (SIP) Resistor Packs:

[_] 1	RP1		4 x 2.2k ohm (222)

[_] 2	RP2,RP3	4 x 10k ohm (103)

[_] 2	RP4,RP5	4 x 47 ohm (470)

Capacitors

Capacitors may be marked in various ways. The typical markings are given but may vary. Find all that match the typical markings given and the remaining ones, if any, should become apparent by elimination.

[_] 14	C1,CB1-4,6-14  .01uF (104) Mylar or Monolithic

[_] 1	C2		470 pF (471) Mylar or Monolithic

[_] 8	CT1-8		1 uF  Electrolytic or Tantalum capacitors

Integrated Circuits

Integrated Circuits come from various manufacturers and may have differing prefixes and/or suffixes. For example, if the part is listed as a 74LS00, it may be marked SN74LS00N or MC74LS00P or DM74LS00N or F74LS00P or some other variation. The key is that the sequence 74LS00 appears in the part number. A four-digit number, such as 8834, indicates the year and week of manufacture and should not be confused with the part number.

NOTE: Do not handle the ICs at this time! Carefully remove the black foam carrier with ICs from the bag and verify the ICs against this list. Then return the foam with the ICs to the bag. Do not touch the ICs!

[_] 1	U1		74HC132 (14 pin DIP)

[_] 1	U2		MAX232, ISL232CPE, TC232CPE (16 pin DIP)

[_] 2	U3,U4		74AC04 (14 pin DIP)

[_] 1	U6		LM7805 3-terminal Regulator

Diodes

[_] 8	D1-7,D10	1N4148 Silicon Signal Diode

[_] 2	D8,D9		1N4002 Silicon Power Diode

[_] 1			LED

Radio-frequency Choke

[_] 1	RFC1		6.8 uh choke

Sockets and Connectors

[_] 3	U1,U3,U4	14-pin DIP IC Sockets

[_] 1	U2		16-pin DIP IC Socket

[_] 1	P7		4 pin polarized MOLEX header

[_] 3	P1-3		2x5 pin Male Header

[_] 1	JP9		1x5 pin Male Header

[_] 1	JP11		1x2 pin Male Header

Miscellaneous

[_] 1			PC Board, TAC-2 Rev. C

[_] 1			Assembly Manual (This Document)

[_] 1	U6		Heatsink

[_] 1	P1		DB9F IDC cable and connector assembly

[_] 4			3/8"  4-40 threaded standoffs for GPS receiver

[_] 4			3/4"  4-40 threaded standoffs for TAC-2 PCB

[_] 17			1/4"  4-40 screws

[_] 1			#4 lockwasher

[_] 1			4-40 nut

[_] 1	P7		Header Receptacle 4-pin 0.1" Housing

[_] 5	P7		Pins for Header Receptacles

Specific Configuration and Options: Depending on the GPS receiver and options you will be installing, you should have the following parts.

Motorola ONCORE Basic receiver:

[_] 1	P4		2x5 pin Male Header

Motorola ONCORE VP/UT receiver:

[_] 1	J1		2x5 pin Female Header

[_] 1	JP3		1x3 pin Male Header

[_] 10			2-pin connectors

Trimble SK-8 receiver:

NOTE: The following parts are supplied as a separate parts option for Trimble SK-8 receiver users.  If you are installing a SK-8 and did not receive these parts notify the TAPR office.

[_] 1	U5		7555 Integrated Circuit (8 pin DIP)

[_] 1	U5		8-pin DIP Socket

[_] 1	Q1		2N3906 PNP transistor

[_] 1	R3		tbd

[_] 1	R9		1 Megohm Resistor (brown-black-green-gold)

[_] 1	C3		.01uF Capacitor (104)

[_] 1	C4		.01uF Capacitor (104)

[_] 1	CB5		.01uF Capacitor (104)

[_] 1	J2		2x5 pin Female Header

[_] 1	JP2		2x5 pin Male Header

[_] 1	JP3		1x3 pin Male Header

[_] 1	JP4		2x4 pin Male Header

Garmin GPS-20 receiver:

[_]	No extra parts are needed

NOTE: Shaded parts are TAC-2 options only.  These parts should be ignored if building without the High-efficiency Switching Power Supply, or External Antenna Preamp Bias options.

High-efficiency Switching Power Supply Option:

[_] 1	U6		Power Trends PT5101 or 78SR Switcher

[_] 1	CE1		220 uF Electrolytic Capacitor

[_] 1	Z1		6.2V Zener Diode

[_] 1	RFC2		6.8 uh choke

External Antenna Preamp Bias Option:

[_] 1	U7		LM78L05 3-terminal voltage regulator

[_] 1	D11		1N4002 Silicon Power Diode

[_] 3	CB15-17	.01uF Capacitor (104)

[_] 1	CT9		1 uF Electrolytic or Tantalum Capacitor

[_] 1	R7		500 ohm potentiometer

[_] 1	R8		510 ohm resistor (green-brown-brown-gold)

[_] 1	JP12		1x2 pin Male Header

[_] 1	Z2		Zener Diode (optional, voltage as required)

--------------------------------------------------

3. CONSTRUCTION NOTES:

You are now ready to begin construction of the TAC-2.  It is recommended that when assembling the TAC-2, that you use a very fine pointed soldering tip on an iron of relatively low wattage (25 watts maximum, 15 watts is ideal).  Due to the proximity of some of the traces on the PC board, solder bridges are a very distinct possibility.  A low wattage soldering iron, fine pointed tip, clean rosin core solder (small size) is the order of the day.  Following these points could eliminate several hours of troubleshooting (or worse).  This is good practice when working on any kit.

In addition to the soldering iron and solder, you will need small flush or semi-flush cutting pliers and small-tipped long nosed pliers. A magnifying glass may prove helpful to identify the values of the small components.

Pay careful attention to the directions that follow. Keep the tip of your soldering iron bright and clean, wiping it frequently on a wet rag or sponge. Make solder joints carefully, but swiftly. Prolonged heat on a PC board pad can be as disastrous as too much heat, and ruining this PC board can be expensive! Two to three seconds should be enough time to apply heat to any joint.

The accompanying component placement drawing shows the placement of all parts on the circuit board.  Pay particularly close attention to:

1) Read this entire document prior to starting construction of your kit.  There are several final configurations for the TAC-2 (depending on which type of GPS receiver you have) and some parts may or may not require installation for your specific configuration.

2) Identifying the pins on headers - Pin 1 is identified by the square pad on silkscreen.

3) Some resistor values may seem unclear as to color and value.  Be very sure of value prior to installation.  Access to an ohm meter will be very helpful.

--------------------------------------------------

4. CONSTRUCTION

Check the PC Board and verify that the exposed, tinned pads are clean and shiny.  If they are not, scrub the board LIGHTLY with a household cleanser (such as "AJAX" or "COMET") and rinse with clean water, then dry with a clean soft towel.

[_] PC Board clean

Refer to the layout diagram in Figure 1 for clarification of parts placement.  Figure 1 is also attached as a full size diagram at the end of the manual.

NOTE: Shaded Parts are contained in the TAC-2 options only.  These parts should be ignored if building without the options.  These are not needed with all kits.

IC Sockets

NOTE: If any socket pins are bent, carefully straighten them with a pair of long-nose pliers before assembly. Some types of IC sockets have crimps in the pins to hold them in place when automatic wave-soldering is performed. These sockets may be tricky to install if you are not familiar with them. If your kit contains these sockets, you may want to straighten the pins before attempting to insert them into the PC board.

When installing IC sockets, double check to ensure that the socket is seated properly against the board with the notch matching the silkscreen. Pin 1 (nearest the socket notch) has a square solder pad. Be sure that all IC socket pins are showing on the solder side of the board. Next, tack-solder two diagonally opposite corners first (such as pins 1 and 8 on a 14-pin socket).

Then solder the remaining pins of that socket before proceeding to the next one. If you find a socket is difficult to install, remove it and double-check for a bent pin.

NOTE: Take care now to avoid solder bridges!

Install the following IC sockets.

[_] U1		14-pin

[_] U2		16-pin

[_] U3		14-pin

[_] U4		14-pin

If you will be using a Trimble SK-8 receiver, then also install:

[_] U5		8-pin

Now check your work. All leads should be soldered. There should be no solder bridges (a blob of solder that shorts two adjacent soldered connections) or cold (gray and/or grainy looking) solder connections. You should have no IC sockets remaining.

[_] OK so far.

Resistors

Resistors lie flat on the PC board and have a lead spacing of 0.400". You may wish to use a lead former to preform the resistor leads for neatest appearance.

Install the following 5% resistors:

[_] R1		1 Megohm (brown-black-green-gold)

[_] R2		510 ohm (green-brown-brown-gold)

[_] R4		tbd

[_] R5		tbd

[_] R6		180 ohm  (brown-gray-brown-gold)

WARNING! Be careful when clipping leads, as they have a tendency to fly towards your eyes! Take appropriate precautions (grasp leads and wear eye protection).

[_] Solder and clip the leads (10 total)

If you will be using a Trimble SK-8 receiver, then also install:

[_] R3		tbd

[_] R9		1 Megohm (brown-black-green-gold)

If you will be using the External Antenna Preamp Bias Option, then also install:

[_] R8		510 ohm resistor (green-brown-brown-gold)

[_] R7		500 ohm potentiometer

Carefully check the board for solder splashes, solder bridges and cold solder joints.

[_] OK so far.

SIP (Single Inline Package) Resistors used in the TAC-2 are symmetric and not polarized.  It is recommended that you match the dot or Pin 1 on the resistor with the pad marked with a square on the silkscreen. All RPx Resistor Packs are 8-lead parts containing 4 identical resistors. The resistors should end up flush with the circuit board. It is easiest to solder the end pins first, then inspect the placement, and the solder the other 6 pins (just like you did by soldering the corner pins on the IC sockets).

[_] RP1	4x2.2k ohm

[_] RP2	4x10k ohm

[_] RP3	4x10k ohm

[_] Solder the leads (24 total)

[_] RP4	4x47 ohm

[_] RP5	4x47 ohm

[_] Solder the leads (16 total)

RF Chokes

[_] RFC1	6.8uh

[_] Solder and clip the leads (2 total)

Carefully check the board for solder splashes, solder bridges and cold solder joints.

[_] OK so far.

Capacitors

NOTE: All capacitors should be mounted as nearly flush to the board surface as practical without stressing the leads. The first capacitors to install are located on the upper right-hand quadrant of the PCB:

[_] C1		.01uF	(104)

[_] C2		470pf	(471)

[_] CB1	.01uF	(104)

[_] CB6	.01uF	(104)

[_] CB7	.01uF	(104)

[_] Solder and clip the leads (12 total)

Now work in the top left-hand quadrant:

[_] CB2	.01uF	(104)

[_] CB11	.01uF	(104)

[_] CB12	.01uF	(104)

[_] CB13	.01uF	(104)

[_] Solder and clip the leads (10 total)

In the bottom half of the board:

[_] CB3	.01uF	(104)

[_] CB4	.01uF	(104)

[_] CB8	.01uF	(104)

[_] CB9	.01uF	(104)

[_] CB10	.01uF	(104)

[_] CB14	.01uF	(104)

[_] Solder and clip the leads (12 total)

Carefully check the board for solder splashes, solder bridges and cold solder joints.

[_] OK so far.

NOTE: The following capacitors are polarized. Electrolytic capacitors (tubular metal cans with the leads coming out of one end) have the negative lead marked with a bold black band; tantalum capacitors (teardrop-shaped epoxy-covered blobs) have the positive lead marked with hard-to-read little plus (+) signs.

[_] CT5	1uF	(105)

[_] CT6	1uF	(105)

[_] CT7	1uF	(105)

[_] Solder and clip the leads (6 total)

The following capacitors are immediately adjacent to U2 (MAX232). All the "+" signs should point away from U2 and toward U5/R9.

[_] CT8	1uf	(105)

[_] CT1	1uf	(105)

[_] CT2	1uf	(105)

[_] CT3	1uf	(105)

[_] CT4	1uf	(105)

[_] Solder and clip the leads (10 total)

If you will be using a Trimble SK-8 receiver, then also install:

[_] CB5	.01uF	(104)

[_] C3		.01uF	(104)

[_] C4		.01uF	(104)

If you will be using the External Antenna Preamp Bias Option, then also install:

[_] CB15	.01uF	(104)

[_] CB16	.01uF	(104)

[_] CB17	.01uF	(104)

NOTE: The following capacitor is polarized.

[_] CT9	1uf	(105)

Now check your work. All leads should be soldered. There should be no solder bridges or cold solder connections. You should have no resistors, diodes or capacitors remaining.

[_] OK so far.

Diodes

Diodes are polarized, with the cathode end being banded. Observe polarity when installing the following diodes. The diode band must match the band shown on the PCB's silk screen. Leads are at a 0.3" spacing. Diodes D1 through D7 are located between RP1 and RP2.

[_] D1		1N4148	NOTE: The cathode band on

[_] D3		1N4148	these 4 diodes all point towards RP2

[_] D7		1N4148

[_] D5		1N4148

[_] Solder and clip the leads (8 total)

[_] D4		1N4148 	NOTE: The cathode band on

[_] D2		1N4148 	these 3 diodes all point towards RP1

[_] D6		1N4148

Depending on the input voltage you choose for your TAC-2, install D10 in either the 5 or 12 position.

[_] D10 	1N4148

[_] Solder and clip the leads (8 total)

NOTE: The 1N4002 diodes have heavy leads. Make sure that they get hot enough for solder to flow properly!

[_] D8		1N4002

[_] D9		1N4002

[_] Solder and clip the leads (2 total).

If you will be using the External Antenna Preamp Bias Option, then also install:

[_] D11	1N4002

Carefully check the board for solder splashes, solder bridges and cold solder joints.

[_] OK so far.

Connectors

The male headers will next be installed.  The plastic body of the part should rest flush with the top surface of the PC board.  The short end if the pins goes into the PC board, the long end sticks up.  Install them using the same procedures you used for the IC sockets. Note that the PCB silkscreen has a square to denote pin #1.

WARNING: Do not hold these parts with your fingers as they quickly get very hot while soldering in place.

[_] P1		2x5 pin Male Header

[_] P2		2x5 pin Male Header

[_] P3		2x5 pin Male Header

[_] JP9		1x5 pin Male Header		UPS Power

[_] JP11	1x2 pin Male Header		Ext Battery

The following connector is polarized. Install it with the plastic polarizing tab away from the edge of the board and towards the diode D8, matching the silkscreen artwork. This connector matches the 4-pin connector used on many disk drives:

[_] P7		4 pin polarized MOLEX connector

If you will be using the External Antenna Preamp Bias Option, then also install:

[_] JP12	1x2 pin Male Header		Ant Bias

If you will be using a Motorola ONCORE BASIC receiver (this is Motorola's original receiver, also called the PVT-6 with a PCB 100mm x 70mm in size), then also install P4 on the bottom of the PCB:

[_] P4		2x5 pin Male Header

If you will be using a Motorola ONCORE VP/UT receiver, then also install:

NOTE: J1 is installed on the bottom of the PCB.

[_] J1		2x5 pin Female Header	ONCORE VP/UT I/O

[_] JP3		1x3 pin Male Header		TXD Polarity Selection

If you will be using a Trimble SK-8 receiver, then also install:

NOTE: J2 is installed on the bottom of the PCB.

[_] J2		2x5 pin Female Header	Trimble SK-8 I/O (only 8 pins used)

[_] JP3		1x3 pin Male Header		TXD Polarity Selection

[_] JP4		2x4 pin Male Header		1PPS CFG

NOTE: The headers at P5 and P6 are intended for future expansion. Don't worry about these at this time.

Carefully check the board for solder splashes, solder bridges and cold solder joints.

[_] OK so far.

Transistor

NOTE: Transistors are polarized components.  Match the body of the transistor with the silkscreened outline and carefully bend the transistor leads to match the hole pattern in the PC board.  The bottom of the body of the transistor should be not more than ¼” above the PC board.

If you will be using a Trimble SK-8 receiver, then also install:

[_] Q1		2N3906 PNP transistor

NOTE: U7 is an integrated circuit shaped like a transistor.  Install U7 in the same manner as stated in the note above for Q1.

If you will be using the External Antenna Preamp Bias Option, then also install:

[_] U7		LM78L05 3-terminal voltage regulator

Power Supply

At this point you need to decide which of three power supply options you plan to use:

- 7805 Voltage Regulator - A low-cost voltage regulator.  This regulator comes supplied with the TAC-2 kit.  Input voltage range is 7.5 to 12 volts.

- PT5101 or 78SR Switcher Voltage Regulator - For high efficiency regulation under a wide variety of input voltages, Power Trends PT5101 or 78SR Switcher voltage regulator may be installed.  Input voltage range is 7 to 26 volts.

- Direct 5 volts power supply - A voltage regulator is not needed.  Either you plan to use your own 5 volt power supply or you plan on installing your TAC-2 inside your computer and using its 5 volt power supply.

NOTE: The Garmin GPS-20 requires +5 volts +/- 5%.

7805 Voltage Regulator

Bend the leads of the 7805 voltage regulator in a 90 degree angle in the direction of the mounting tab so that it lies over the silk-screen outline of the device.  Bend the leads such that the center leg fits into the hole marked Z.

Install the supplied heatsink and the 7805 to the PCB before soldering.  Fasten the 7805 and the heatsink using one ¼” 4-40 screw, lockwasher, and nut.

Before soldering the 7805 to the PCB ensure that the fins of the heatsink do not touch any other components or a short may result!

[_] Install Heatsink

[_] U6		7805 3-terminal voltage regulator

[_] Fasten 7805 and heatsink to the PCB with ¼” 4-40 screw, lockwasher, and nut.

[_] Solder in place

PT5101 or 78SR Switcher Voltage Regulator

Install the following components in the same manner as stated above for capacitors, diodes, and chokes.

NOTE: CE1 and Z1 are polarized components.

[_] CE1	220 uF Electrolytic Capacitor

[_] Z1		6.2V Zener Diode

[_] RFC2	6.8 uh choke

Bend the leads of the Power Trends PT5101 or 78SR Switcher voltage regulator in a 90 degree angle in the direction of the mounting tab so that it lies over the silk-screen outline of the device.  Fasten the voltage regulator using one ¼” 4-40 screw, lockwasher, and nut.

[_] U6		Power Trends PT5101 or 78SR Switcher

Direct 5 volts power supply

Using a cut-off lead from a resistor or capacitor, install a jumper between the “+5PC” hole and the “+” hole of CE1.

[_] Jumper installed

Installation of specific parts

Your TAC-2 is nearly complete.  Every part should be installed except the Integrated Circuits.  Check your work for incorrect soldering and parts installation prior to completing the TAC-2 kit.

[_] Everything looks simply wonderful to me!

Preliminary Voltage Checks

Before installing the Integrated Circuits, it is best to do a preliminary voltage check.  This is a relatively easy check to make and can save you headaches later.  But first you need to construct a power supply cable.

Supplied with your TAC-2 kit is a 4-pin connector and pins.  Follow the directions below depending on the Power Supply option you chose above.

7805 Voltage Regulator or PT5101 or 78SR Switcher Voltage Regulator:

[_] Assemble the 4-pin connector with ground to pin 3 and positive voltage applied to pin 4.

Direct 5 volts power supply:

[_] Assemble the 4-pin connector with ground to pin 2 and positive voltage applied to pin 1.

[_] Apply power to the TAC-2 PCB.

Using a voltage meter measure the following voltages:

[_] +5 volts at pin 14 of U1

[_] +5 volts at pin 16 of U2

[_] +5 volts at pin 14 of U3

[_] +5 volts at pin 14 of U4

If you will be using a Trimble SK-8 receiver:

[_] +5 volts at pin 8 of U5

[_] Remove power to the TAC-2.

[_] All voltage checks satisfactory.

Integrated Circuits

Observing static precautions and polarity, install the following ICs in their sockets.  Pay particular attention to aligning the notch of the IC with the socket:

NOTE:  Make sure all power is removed before installing ICs.

[_] U1 74HC132

[_] U2 MAX232, ISL232CPE, or TC232CPE

[_] U3 74AC04

[_] U4 74AC04

If you will be using a Trimble SK-8 receiver, then also install:

[_] U5 7555

Final Voltage Check

Now that all the ICs are install, here is one final voltage check.

[_] Apply power to the TAC-2 PCB.

Using a voltage meter measure the following voltages:

[_] +10 volts at pin 2 of U2

[_] -10 volts at pin 6 of U2

[_] Remove power to the TAC-2.

[_] All voltage checks satisfactory.

The next section of these instructions deals with the installation of the GPS receiver of your choice.

--------------------------------------------------

5. INSTALLATION

Garmin GPS-20

If the Garmin GPS-20 was purchased from TAPR, you received a power/data cable with connector shell and pins.  Assemble this cable according to the assembly information that came with the GPS-20.  You need to construct the cable with 5 wires:

3 - 1PPS output

4 - Serial Data output

5 - Serial Data input

8 - Ground

10 - +5 VDC (+/- 5% at 180 mA)

These 5 wires will connect to the TAC-2 PCB at JP6.

[_] Solder the cable assembly into JP6 pin-for-pin on the bottom side of the TAC-2 PCB.

[_] Install four 3/8" 4-40 threaded standoffs on the bottom side of the TAC-2 PCB using four ¼” 4-40 screws. [***need a diagram on which holes to use]

[_] Install the Garmin GPS-20 receiver onto the standoffs making sure that the antenna connector is facing away from the TAC-2 PCB.  Use four ¼” 4-40 screws.

[_] Plug in the power/data cable connector into the GPS-20.

[_] Skip to the OPERATION section.

Motorola Oncore Basic

[_] Install four 3/8" 4-40 threaded standoffs on the bottom side of the TAC-2 PCB using four ¼” 4-40 screws. [***need a diagram on which holes to use]

[_] Install the Motorola Oncore Basic receiver onto the standoffs making sure that the antenna connector is facing away from the TAC-2 PCB.  Use four ¼” 4-40 screws.

[_] Plug in the DB9F IDC cable and connector assembly into P1.

[_] Skip to the OPERATION section.

Motorola Oncore VP

[_] Install four 3/8" 4-40 threaded standoffs on the bottom side of the TAC-2 PCB using four ¼” 4-40 screws. [***need a diagram on which holes to use]

[_] Install the Motorola Oncore VP receiver onto the standoffs making sure that the antenna connector is facing away from the TAC-2 PCB.  Use four ¼” 4-40 screws.

[_] Skip to the OPERATION section.

Trimble SK-8

[_] Install four 3/8" 4-40 threaded standoffs on the bottom side of the TAC-2 PCB using four ¼” 4-40 screws. [***need a diagram on which holes to use]

[_] Install the Trimble SK-8 receiver onto the standoffs making sure that the antenna connector is facing away from the TAC-2 PCB.  Use four ¼” 4-40 screws.

[_] Skip to the OPERATION section.

--------------------------------------------------

6. OPERATION

[_] Plug in the DB9F IDC cable and connector assembly into P1.

[_] Connect your GPS receiver to an external antenna.

[_] Connect the TAC-2 to your Personal Computer.

[_] Apply power to the TAC-2.

NOTE:  If this is the first time the GPS receiver has been powered up in this location, it will take 15-20 minutes to acquire a fix.  Until that time, the 1PPS signal will not be available.

[_] Load and configure the program Showtime according to the accompanying instructions - Operation of your TAC-2.

[_] Upon start up of Showtime, once per second beeps should be heard and then time displayed on the screen.

[_] This completes the assembly of your TAC-2!  Congratulations!  At this point you may install the TAC-2 in a suitable enclosure.  Suggested enclosure methods are available on the enclosed flyer.

--------------------------------------------------

7. THEORY OF OPERATION

1:SHEET 1 - RS232 I/O:

1a: RS232 Port #1  The main (computer) RS232 Port#1 is handled by U2, a generic MAX232-like chip, which has two RS232 line drivers (TX1 & TX2) and two RS232 line (RX1 & RX2) receivers. The line drivers and line receivers are all inverting devices. TX1 generates the TXD RS232 data signal sent to the  host computer. TX2 generates a 1PPS RS232 signal sent to the computer on the DCD handshaking line. P1 is a 10-pin connector designed to be routed to a DB9 female connector to interface directly with a PC-AT standard port. The 10th wire is available as a "frame ground" for the metallic shell on the DB9 connector. The RX1 section of the MAX232 receives RS232 data from the PC. RX2 is used as an inverter as described later in section ???. The unused pins on P1 are available on solder pads adjacent to the connector in case they are needed for a custom application.

1b: RS232 Port #2   The 2nd RS232 port is intended for use with either an RTCM SC104 DGPS beacon receiver or as a connection to a second host computer. The connector P2 provides an interface to a standard DB9 RS232 port. The jumper area JP1 permits the P2 connection to be "swapped" without the need for a null modem. Cutting the default traces associated with the DB9's pins 2 & 3 permits swapping TXD and RXD. In addition, the 1PPS signal can be changed from the default DCD line to RI.  U1A is a Schmidt trigger used as an RS232 line receiver.  Diodes D1 & D2 along with R1 prevent the input to U1A from going above +5v or below zero volts (the gate inputs on U1 are internally diode protected, but I decided it was best to provide additional protection).  RP1A sets the default when Port#2 is not used to a logic zero.

1c: Port#2 "squelch"  Port#2 is enabled only if its second input (pin 2) is high. If there is any activity on the Port#1 RXD input, diode D3 pulls this pin low, disabling Port#2 RXD. This pin stays low for a time interval determined by the R2C1 time constant (~10 msec for the parts values shown). In addition, this port can be disabled from the expansion port by pulling the RX Holdoff line low.

1d: Combining the RXD signals  U1B serves as an or-gate to combine the RXD signals from Ports #1 and #2. A 3rd input to the or-gate is provided by D4 and RP1D so that an embedded processor connected to the expansion port can send signals to the GPS receiver. U1B re-inverts the signals (zero low, one high) and is sent directly to the GPS receiver's RS232 input (Sheet 3) as GPS RXD. [Note: Both the Motorola ONCORE BASIC and Garmin receivers have "RS232" inputs. However both work just fine when driven by 0/+5 v CMOS logic without converting it to "real RS232". Also on Sheet 3 is an optional inverter to handle the Motorola ONCORE VP, which uses inverted TTL logic levels.]

1e: TXD signals from GPS to Ports #1 and #2   U1C functions as an inverter for the GPS receiver's RS232 TXD output signals. R3 provides a current limiting resistor so that RS232 levels won't burn out the internal protective diodes in U1C [Note - we need to experiment with this resistor. 2.2k or 4.7k may be better.]. For the ONCORE VP with inverted TTL levels, the RX2 section of U2 is used as an inverter and the jumper JP2 needs to be changed from the default. The second input on U1C is normally pulled high through RP1B, but the embedded microprocessor can squelch TXD by pulling the TX Holdoff line low. The diode D5 and pullup resistor RP1C allow inverted data from the expansion port tp be sent to the host PC.

2: SHEET 2 -- 1PPS I/O

2a: The "normal" 1PPS Output Path  The GPS receiver is assumed to have a positive-going 1PPS pulse that is "wide" (100 msec for the GPS-20, 200 msec for the Motorola receivers) that has a rising edge coincident with the UTC second (the pulse can be offset by know amounts in the Motorola receiver). This signal is first inverted in U3B (a 74AC04 is chosen for U3 to provide a low propagation delay and high output drive) to make 1PPS INV. The 1PPS INV signal directly drives the U2 TX2 section to make the DCD 1PPS signal. U3C reinverts the signal to drive a front panel LED via R4 and to provide a buffered wide 1PPS signal to the expansion port. U3D+E+F are paralleled via RP2A+B+C to provide a buffered 1PPS signal to the user. [Note - the original TAC had two such 74AC04-derived drivers].The Schmidt trigger U1D provides a clean, well defined negative-going 1PPS signal called INV PULSE. The pulse goes negative coincident with the GPS UTC second and stays negative for a time interval determined by the C2+RP2G network (plus the negligible propagation delay through U3B) set at about 5 usec.

All the 1PPS outputs appear on the 10-pin header P3. If IDC cable is used, the cable can be stripped into wire pairs to go to the various external connections. Alternately, several 2-pin headers can be used for the interconnections.

2b: Alternative 1PPS Inputs  While the Motorola and Garmin receivers have similar "user friendly" 1PPS signals, not all receivers are so convenient. For example, the Trimble SV-6 has a negative-going 1 usec wide 1PPS signal driven by an open collector transistor. The optional JP14 and U5 circuitry is available to accommodate this case. If the default jumper on JP14 (between pins 1-3) is cut and jumpers 1-2 and 3-4 are added, then RP1F provides the needed pullup for the SV-6's open collector output. U5 (a CMOS 555 timer) then functions as a one-shot to provide the desired "wide" positive-going 1PPS signal with a width determined by the R5+C4 timing network.  In the event of a narrow positive-going pulse, JP14 has pins 3-5 jumpered to allow U3A to invert the signal. U3A's output is sent to U5 by jumpering pins 4-6 and U5 functions as described above. If there is a triggering problem with U5, add C3 and cut the default jumper across it in order to insure that U5 is "hit" with a clean, negative going pulse.

3: Sheet 3 - GPS Receiver and Expansion I/O

3a: ONCORE BASIC  To connect an ONCORE BASIC receiver, a 10-pin header is installed at P4 (note that Motorola used a non-standard pin numbering convention for the BASIC. Sheet 3 shows pin numbers on P4 that match Motorola's documentation.). The BASIC has an on-board DC-DC power converter so its power can be either +5V or +12V. The jumper area JP17 defaults to the +12v connection, but can be changed if desired. The 4 mounting holes for the BASIC match the holes in the corners of the TAC-2 PCB and it was my intention that the BASIC receiver be mounted below the TAC-2 PCB. It is to be connected with a short (~1.5") 10-pin IDC cable between the receiver and P4.

3b: ONCORE VP  To connect a ONCORE VP, the user should install a 10-pin socket at J1. The VP will then have its mounting holes match 4 of the holes on the TAC-2 PCB (with the VP above the TAC-2 PCB). Since the VP uses inverted TTL levels on its serial port, the user adds the optional inverter consisting of Q1, R6 and R7. The user also must insure that the jumper JP2 (Sheet 1) is changed.

3c: Garmin GPS-20  The Garmin power+I/O connector is less convenient to use than for the Motorola receivers, and the user will have to prepare a short (~2") 5-wire cable with the Garmin connector on one end. The other end solders to the TAC-2 PCB at JP18. The silk-screen on the TAC-2 PCB identifies the appropriate pin numbers for the Garmin connector. JP18 is also used as an interface area for any other receivers. The mounting holes for the Garmin match holes on the TAC-2 and the Garmin can be mounted either above or below the TAC-2 PCB.

3d: Expansion I/O  The 16-pin plug area at P5 provides connections for other "widgets". I have in mind a clock display module and a disciplined oscillator module that will use this connector. This connector has 3 1PPS signals (plus-going, minus-going and 5 usec wide pulse as described on Sheet 2),   the TXD+RXD+ holdoff signals (described on Sheet 1) and DC power. P5 has 3 uncommitted pins for expansion.

4. Sheet 4 -- POWER

4a: The "+5" Power supply   The main +5v power for the GPS receiver and the TAC-2 logic is obtained using a standard TO-220 7805-type regulator at U6. The TAC-2 PCB has a generous amount of metal to act as a heatsink, but there should be room for a finned radiator if needed.

Experience has shown that it is desirable to allow for trimming of the "+5" supply voltage. At least on GPS-20 receiver required about +5.2v to function properly. In addition, the Low-Z pulse driver (EL7242C, U4) wants more like +6v to be able to generate a full 5v pulse into a 50 ohm load. Therefore I allowed for a "+6v" bus which can be dropped to +5 in one or two 1N4001 diodes at D7+D8. In order to "tweak" the 7805 to the desired voltage, the "gnd" of U6 (and the heatsink) is allowed to float above true ground by a resistor (or a diode) at R9, with the voltage tweaked by the R8+R9 network. [Hopefully the "alpha" group will help define just what TBD parts are needed for R8, R9, D7 and D8 in the TAPR kit!]

4b: The 12v Input Circuit   Although the PCB shows it as an 8-pin DIP, D6 is a quad 1N4001 diode array which does several things:  If the user supplies +12VDC, it can be connected between pins 2 & 3 of P8 (also available as a 2-pin solder area adjacent to P8). The two diodes that are then in the circuit (the right-hand pair on Sheet 4) insure that the user can't do something dumb and apply the wrong polarity - the lower diode will short to gnd and the upper diode only passes the "good" polarity.  The other pair of diodes (left-hand pair) then allow the user to install a small battery in the TAC (like a 9v NiCd) to provide a "mini-UPS" capability. The battery only sources current if the +12v supply is below the battery voltage. In this case, R11 (value TBD) supplies charging current to the battery.  The user can use the entire D6 quad as a power supply rectifier if the user wants to use an AC wall-mount transformer. In this case, AC is supplied to P8 pins 2&4. Since the capacitors on the PCB are small, this will require an external filter capacitor across pins 1&3. [Note Regarding D6: Greg suggests using discrete 1N4001s because TAPR has a lot of them; the layout is such that the diodes fit side-by-side neatly providing they can have leads bent to fit a 0.3" spacing. An alternate is one of the circular epoxy-packaged quad diode packages that are about 0.4" in diameter; the standard part has lead spacings of 0.22",so the leads have to be spread a bit to hit the corners of the DIP footprint. I originally thought there was a standard 8-pin DIP part with the layout I used, but apparently I was wrong.]

4c: Battery Backup Option   This circuit is cloned from the original TAC. Many of the ONCORE receivers don't come with batteries (which keeps BBRAM alive and keeps a 32 kHz oscillator + clock alive). So we have used either 9v or 5v NiCd cells in our production TACs. D9 + R10 + P6 allows me to keep this as an option. The TAPR Garmins have a battery already, so this is not needed for that application.

4d: Antenna Bias Option   This circuitry lets us supply special antenna bias voltages for some applications.  The circuit is pretty self-explanatory.

--------------------------------------------------

8. BOARD LAYOUT

Figure 1.  Board Layout