AX.25 Version 2

Multi-channel

TNC  FIRMWARE

(version 2.1)

Copyright 1987, Ronald E. Raikes (WA8DED)

This firmware supports the full AX.25  link-layer  protocol,

version 2.0 as described in the ARRL specification dated  October

1984,  as  well  as   the   pre-existing   version   1.x.    This

implementation supports multiple  simultaneous  link  connections

with either version protocol.  This release has been compiled for

a maximum of four connections, although any reasonable number  of

connections is possible by changing  one  MAXLNK  symbol  in  the

source header file.  The  firmware  is  contained  in  one  26256

EPROM, and is intended to  be  installed  in  a  TAPR  TNC-2  (or

equivalent, such as the MFJ-1270 or AEA  PK-80)  in  socket  U23.

RAM memory is automatically sized, supporting both  16k  and  32k

configurations.

Commands and information are sent to the tnc in the form  of

lines.  Lines may be up to 256  characters  long,  including  the

terminating CARRIAGE RETURN.  If the 256th character  entered  is

not a CARRIAGE RETURN, it will be discarded and a BELL  character

will be output to the terminal.  BACKSPACE and DELETE may be used

to remove single characters from the line.  The entire  line  may

be permanently backspaced out by entering a CONTROL-U or CONTROL-

X.  A CONTROL-R will temporarily backspace out any  partial  line

to allow incoming frames to be  displayed.   A  second  CONTROL-R

will then restore  the  line  to  allow  continuation  of  entry.

During the time a partial line is saved, only  another  CONTROL-R

will be  accepted  from  the  keyboard  (with  the  exception  of

xon/xoff, of course).  BELL characters are echoed to the terminal

when entered or  removed.   Lines  which  begin  with  an  ESCAPE

character (echoed as '* ') are interpreted  as  commands.   If  a

command is issued with no parameter, the current  value  of  that

commands parameter is displayed.  Lines without a leading  ESCAPE

character are sent as information.

The firmware provides the operator  with  five  virtual  tnc

channels, numbered 0 to 4.  The terminal is logically attached to

only one of these  channels  at  a  time,  selected  by  the  'S'

command.  Information sent on channel 0 is always  unproto.   The

unproto path may be set by issuing a 'C' command when  channel  0

is selected.  Channels 1 - 4 are also unproto  if  they  are  not

currently connected.  Outgoing connect requests may be issued  on

any unconnected channel, while incoming connect requests will use

the first available  channel  (provided  the  maximum  number  of

connections set  by  the  'Y'  command  will  not  be  exceeded).

Information received on a connected channel that is not currently

selected will remain queued there until that channel is selected.

The STA led indicates there is queued information,  and  the  'L'

command may be used to  determine  the  channel(s)  where  it  is

located.  Information  for  transmission  is  sent  only  to  the

currently selected channel.  When a connection is ended, received

information will remain queued until it has been displayed.  If a

new digipeater path  is  desired  while  a  connection  is  being

established or is in progress, it is not necessary to  disconnect

first.  Simply re-issuing the 'C' command will  re-establish  the

connection via the new path without any loss of information.

Which protocol version is used to initiate a  connection  is

controlled by the 'V' command, but the version  will  be  changed

automatically, if necessary, to conform to the version of the tnc

responding.  Version 2 protocol is more  effecient  in  terms  of

network  throughput  and   loading,   especially   under   severe

conditions.  Version 2 protocol is the default and should be used

whenever possible.  When version 2 protocol is used, a  watch-dog

timer is started whenever information is not  being  transmitted.

If the tnc remains idle for three minutes, it will poll the other

tnc to determine  if  the  link  is  still  established.   If  no

response is received after the number of tries  set  by  the  'N'

command, a link failure is reported.  This  procedure  will  also

detect the case where someone connects and  then  leaves  without

disconnecting.  Changing the protocol version during a connection

is not permitted.

The 'F', 'I', 'N', 'O', and 'V' commands maintain individual

parameters for each channel.  The value stored in  channel  0  is

used to initialize channels 1 - 4  upon  power  up,  and  to  re-

initialize channels 1 - 4 after a disconnect.   This  allows  the

values to be changed independently on each channel, prior to  and

during a connection, and then automatically revert  back  to  the

standard values when the connection  is  ended.   A  'D'  command

issued on a disconnected channel 1 - 4  will  also  re-initialize

that channel.

Frame monitoring is controlled  by  the  'M'  command.   The

command parameter determines the types of frames  monitored,  and

is a list of desired  frames  chosen  from  the  letters  in  the

following table:

LTR       FRAME

---       -----

N        None

I        I frames

U        UI frames

S        Supervisory frames

C        Monitor while connected

+        Call signs to be included (maximum of 8)

-        Call signs to be excluded (maximum of 8)

The '+' and '-' parameters may not be used together.   If  either

is used, it must be the last parameter (followed by one to  eight

callsigns, if applicable).  If no list of callsigns is  specified

to be included or excluded, all callsigns will be candidates  for

monitoring.  Entering a '+' or '-' with no callsigns  will  empty

the list.

An asterisk displayed after a callsign  in  the  digipeater  list

indicates the frame was transmitted by that station.  The control

field displayed will be one of the following:

NAME   DESCRIPTION

----   -----------

RRa  - Receive Ready

RNRa - Receive Not Ready

REJa - Reject

UI   - Unnumbered Information

DM   - Disconnected Mode

SABM - Connect Request

DISC - Disconnect Request

UA   - Unnumbered Acknowledge

FRMR - Frame Reject

Iab  - Information

?ccH - Unknown

a  = Next expected frame number (0 - 7)

b  = Frame number of this frame (0 - 7)

cc = Hexadecimal value

In addition, one of the following characters will  be  displayed,

reflecting the protocol version, command/response bits,  and  the

poll/final bit:

(blank) = version 1 frame without poll/final bit

! = version 1 frame with poll/final bit

^ = version 2 command frame without poll bit

+ = version 2 command frame with poll bit

- = version 2 response frame with final bit

v = version 2 response frame without final bit

The protocol identifier field is displayed in hexadecimal

An unattended mode, controlled by the 'U' command,  provides

for sending user supplied text to a connecting station, and  then

allows that station to leave a  brief  message.   This  mode  can

operate on all channels simultaneously, but in no way limits  the

operators ability to interact with one of the connected  channels

or  the  ability  to  make  outgoing  connect   requests.    When

unattended mode is enabled, link status messages  are  queued  to

the associated channel and not output to the terminal unless that

channel  is  currently  selected.   Link  status  messages   will

therefore  be  displayed  in   chronological   order   with   the

information from that channel.  In addition, text supplied by the

user with the 'U' command  will  be  sent  to  any  station  that

connects.  If channel 0  is  left  selected,  stations  may  then

connect and leave messages on channels 1 - 4 (limited by the  'Y'

parameter, of course).  The 'L' command may be used to  determine

if messages have been left on any channel.  Selecting  a  channel

containing messages will cause all link  status  and  information

from that channel to be displayed.  If  xon/xoff  handshaking  is

enabled, CONTROL-S and CONTROL-Q may  be  used  to  regulate  the

output to the terminal to allow comfortable reading.

COMMAND SUMMARY

===============

COMMAND        PARAMETER               DESCRIPTION

-------        ---------               -----------

A (1)            0             Auto linefeed disabled

1             Auto linefeed enabled

C        Cs1 [Cs2 ... Cs9]     Connect path (0=unproto path)

D                              Disconnect

E (1)            0             Echo input disabled

1             Echo input enabled

* F (4)           1-15           Frame acknowledge (seconds)

G               [0]            Get information (host mode)

[1]            Get link status (host mode)

* I                Cs            Tnc source callsign

JHOST (0)        0             Terminal mode enabled

1             Host mode enabled

L              [0-4]           Display channel status

M (IU)        NIUSC+-          Monitor mode

* N (10)          0-127          Number of tries (0=forever)

* O (4)           1-7            Number of outstanding I frames

P (64)          0-255          P-persistence value

QRES                           Re-start firmware

R (1)            0             Repeater disabled

1             Repeater enabled

S (0)           0-4            Select channel (0=unproto)

T (30)          0-127          Transmitter delay (10ms)

U (0)   0      [text]          Unattended mode disabled

1      [text]          Unattended mode enabled

* V (2)            1             Version 1 protocol initiated

2             Version 2 protocol initiated

W (10)          0-127          Slot time interval (10ms)

X (1)            0             Transmitter PTT disabled

1             Transmitter PTT enabled

Y (4)           0-4            Maximum connections

Z (3)            0             Flow disabled, xon/off disabled

1             Flow enabled, xon/off disabled

2             Flow disabled, xon/off enabled

3             Flow enabled, xon/off enabled

@   B                          Display number of free buffers

D  (0)       0             Full duplex disabled

1             Full duplex enabled

S                          Display current link state

T2 (100)    0-65535        Timer T2 interval (10ms)

T3 (18000)  0-65535        Timer T3 interval (10ms)

V  (0)       0             Callsign validation disabled

1             Callsign validation enabled

Default values are shown in parenthesis

*  These commands are applicable to each connection channel

(Values set on channel 0 are used upon power up and

disconnect to initialize each connection channel)

COMMAND DESCRIPTION

===================

The 'A' command is used to enable or disable  the  automatic

insertion of LINEFEED characters after CARRIAGE RETURN characters

to the terminal.

The 'C' command is used to initiate a link connection.  Note

that 'v' or 'via' is not required (but is  allowed)  between  the

destination callsign and the digipeater callsigns.  A 'C' command

may be  issued  on  a  channel  already  in  use  to  change  the

digipeater callsigns, but not the destination  callsign.   A  'C'

command issued when channel 0 is selected sets the unproto path.

The 'D' command is used to initiate  a  link  disconnection.

If there is unsent or unacknowledged information  remaining,  the

disconnect request frame will not be sent until  all  information

has been transmitted and acknowledged.  No additional information

will be received after the 'D' command has been issued.  A second

'D' command may be entered  to  force  the  transmission  of  the

disconnect request frame before all information has been sent and

acknowledged.  A 'D' command issued during the establishment of a

link or after a disconnect request  frame  has  been  transmitted

will cause an immediate return to the disconnected state.  A  'D'

command issued on a disconnected channel will  re-initialize  the

connection dependent parameters to the values stored  in  channel

0.

The 'E' command is used to enable or disable the echoing  of

input (commands and information) to the terminal.

The 'F' command is used to  set  the  frame  acknowledgement

interval.  This interval is used to compute the timeout  interval

before a packet is retransmitted, using the formula:

time (seconds) = frame ack * (2 * number of digipeaters + 1)

A separate frame acknowlegement interval value is maintained  for

each connection channel.  The value stored in channel 0  is  used

to  initialize  each  connection  channel  upon   power   up   or

disconnection.

The 'G' command is used to interrogate virtual tnc  channels

when host mode is enabled.  If no  parameter  is  specified,  the

next chronological item (information  or  link  status)  will  be

returned, provided there is one.   This  command  is  invalid  in

terminal  mode.   A  later  section  is  devoted  to  host   mode

operation.

The 'I' command is used to set the tnc source callsign.  The

initial value is all blanks.  Changing the  tnc  source  callsign

while connected is not permitted.  If the tnc source callsign  is

left blank, the tnc will not allow connect  commands  or  unproto

transmissions.  The callsign stored  in  channel  0  is  used  to

initialize   each   connection   channel   upon   power   up   or

disconnection.

The 'JHOST' command is used to select between  terminal  and

host modes.  A later section is devoted to host mode operation.

The 'L' command is used to display the link status of one or

all channels.   Information  displayed  includes  the  connection

path, number of receive frames not yet displayed, number of  send

frames not yet transmitted, number of transmitted frames not  yet

acknowledged, and the  current  retry  count.   A  '+'  character

preceeding the channel number indicates  the  currently  selected

channel.  Operation of this command when host mode is enabled  is

somewhat different, and is described in a later section.

The 'M' command is used to set the  frame  monitoring  mode.

The command parameter determines the types of  frames  monitored,

and is a list of desired frames chosen from the  letters  in  the

following table:

LTR       FRAME

---       -----

N        None

I        I frames

U        UI frames

S        Supervisory frames

C        Monitor while connected

+        Call signs to be included (maximum of 8)

-        Call signs to be excluded (maximum of 8)

The '+' and '-' parameters may not be used together.   If  either

is used, it must be the last parameter (followed by one to  eight

callsigns, if applicable).  If no list of callsigns is  specified

to be included or excluded, all callsigns will be candidates  for

monitoring.  Entering a '+' or '-' with no callsigns  will  empty

the list.

The 'N' command is used to set the maximum number of times a

frame  will  be  transmitted  without  receiving  an  appropriate

acknowledgement, before a link failure is  assumed.   A  separate

maximum number of tries value is maintained for  each  connection

channel.  The value stored in channel 0  is  used  to  initialize

each connection channel upon power up or disconnection.

The 'O' command  is  used  to  set  the  maximum  number  of

unacknowledged I frames that may be outstanding at any one  time.

A separate maximum number of unacknowledged  I  frames  value  is

maintained for each connection  channel.   The  value  stored  in

channel 0 is used to  initialize  each  connection  channel  upon

power up or disconnection.

The 'P' command is used to set the p-persistence value.   P-

persistence is used  during  simplex  operation  as  a  means  of

channel  arbitration.   Whenever   there   are   frames   to   be

transmitted, the tnc first waits for a clear channel.   Once  the

channel is clear, a random number between 0 and 255 is generated.

If the random number is less than or equal to  the  p-persistence

value,  the  PTT  line  is  asserted  and  transmission   begins.

Otherwise, the tnc delays for the slot time interval set  by  the

'W' command and the entire process is repeated.

The  'QRES'  command  is  used  to  restart  the   firmware,

including re-initialization  of  battery-backed  RAM  to  default

parameters.

The 'R' command is used to enable or disable the digipeating

of frames.

The 'S' command  is  used  to  select  the  current  channel

number.

The 'T' command is used to set the transmitter  keyup  delay

interval.  The parameter is specified in 10ms increments.

The 'U' command is used  to  enable  or  disable  unattended

modes.

The 'V' command is used to select whether  version  1  or  2

protocol will be used to initiate a link connection.  A  separate

protocol version value is maintained for each connection channel.

The value  stored  in  channel  0  is  used  to  initialize  each

connection channel upon power up or disconnection.  Interrogating

this parameter during a  connection  will  reflect  the  protocol

version currently being  used  on  that  channel.   Changing  the

protocol version during a connection is not permitted.

The 'W' command is used to set the p-persistence  slot  time

interval.  P-persistence is used during simplex  operation  as  a

means of channel arbitration.  Whenever there are  frames  to  be

transmitted, the tnc first waits for a clear channel.   Once  the

channel is clear, a random number between 0 and 255 is generated.

If the random number is less than or equal to  the  p-persistence

value set by the 'P'  command,  the  PTT  line  is  asserted  and

transmission begins.  Otherwise, the tnc delays for the slot time

interval and the entire process is repeated.   The  parameter  is

specified in 10ms increments.

The 'X' command is used to enable or disable the transmitter

PTT line.

The 'Y' command  is  used  to  set  the  maximum  number  of

connections that may  established  by  incoming  requests.   This

command has no  effect  on  the  operators  ability  to  initiate

outgoing connection requests.

The 'Z' command is used to enable or  disable  flow  control

and xon/xoff handshaking to the terminal.   If  flow  control  is

enabled, output to the terminal will be inhibited while  entering

commands or information.  If flow control is disabled, output  to

the terminal will not be restricted.  Flow control  and  xon/xoff

handshaking should be disabled during periods in which the tnc is

operated without a terminal, to  avoid  suspending  output  which

will consume buffers.  If xon/xoff handshaking  is  enabled,  crt

scrolling may be stopped and started using CONTROL-S and CONTROL-

Q characters.  Flow control  and  xon/xoff  handshaking  are  not

performed when host mode is enabled.

The '@'  command  is  a  software  maintenance  command.   A

parameter of 'B' will display the number of  free  buffers.   The

'D' parameter is used to enable or disable full duplex  operation

of the HDLC port.  A parameter of 'S' will  display  the  current

link state.  The 'T2' parameter is  used  to  set  the  timer  T2

interval, just as the 'T3' parameter is used to set the timer  T3

interval.  The timer intervals are specified in 10ms  increments.

Timer T2 controls  the  amount  of  delay  between  the  time  an

information frame is received and the time the resulting response

frame  is  sent.   This  delay  allows  multiple  frames  to   be

acknowledged with a single response.  Timer T3 is  used  maintain

link integrity.  If there is no activity during the T3  interval,

the tnc  will  poll  to  verify  the  distant  station  is  still

connected.  The 'V'  parameter  is  used  to  enable  or  disable

callsign validation.

HOST MODE OPERATION

===================

Host mode is intended to provide a user  interface  suitable

for operation under control of a host  processor.   Commands  and

information to the tnc, as well as status  and  information  from

the tnc, are clearly identified to allow orderly and  unambiguous

communication.  To alleviate any need for  hardware  or  software

handshaking,  the  tnc  will  not  send  to  the  host  processor

unsolicited,  and  all  exchanges  are  limited  to  256   bytes.

Information transfers are fully transparent.

When host mode is enabled, the first byte sent  to  the  tnc

must be a channel number.  If  information  is  being  sent,  the

second byte must be a binary 0.  If a command is being sent,  the

second byte must be a binary 1.   The  third  byte  must  be  the

binary length of the actual information or  command,  decremented

by 1 (vacuous information or commands are  not  permitted).   The

actual  information  or   command   bytes   must   follow   last.

Information sent to channel 0 will be sent unproto.   Information

sent to an unconnected channel 1 - 4 will be discarded.  The  tnc

will respond to both information  and  commands  with  a  channel

number first, followed by a binary code of 0, 1, or 2, signalling

success or failure.  Codes of 1 or 2 will be followed by  a  null

terminated message.  Channels may be  interrogated  for  incoming

information or link status by using  the  'G'  command.   Monitor

headers and monitor information will always be sent to channel 0,

along with connect request link status messages.  All other  link

status messages will be sent to the  appropriate  channel,  along

with that channels connected information.  In response to  a  'G'

command, the tnc  will  respond  with  a  channel  number  first,

followed by a binary code of 0 if  nothing  is  available,  or  a

binary code of 3 - 7, identifying the bytes that follow.  A  code

of  4  indicates  the  monitored  frame  does  not   contain   an

information field.  A code of 5  indicates  the  monitored  frame

does contain an information field, and the next  'G'  command  on

channel 0 will return that information field, preceeded by a code

of 6.

Host to Tnc

-----------

CHANNEL   CODE           DESCRIPTION

-------   ----           -----------

n       0        Information (preceeded by length-1)

n       1        Command     (preceeded by length-1)

Tnc to Host

-----------

CHANNEL   CODE           DESCRIPTION

-------   ----           -----------

n       0        Success (nothing follows)

n       1        Success (message follows, null terminated)

n       2        Failure (message follows, null terminated)

n       3        Link Status (null terminated)

n       4        Monitor Header (null terminated)

n       5        Monitor Header (null terminated)

n       6        Monitor Information (preceeded by length-1)

n       7        Connect Information (preceeded by length-1)

Success messages

----------------

{channel status}

{parameter value}

CHANNEL NOT CONNECTED

Failure messages

----------------

INVALID CALLSIGN

MESSAGE TOO LONG

INVALID PARAMETER

INVALID BAUD RATE

NO SOURCE CALLSIGN

INVALID COMMAND: ?

NOT WHILE CONNECTED

INVALID VALUE: ?????

NO MESSAGE AVAILABLE

INVALID CHANNEL NUMBER

TNC BUSY - LINE IGNORED

CHANNEL ALREADY CONNECTED

STATION ALREADY CONNECTED

INVALID EXTENDED COMMAND: ?

Link Status messages

--------------------

BUSY fm {call} via {digipeaters}

CONNECTED to {call} via {digipeaters}

LINK RESET fm {call} via {digipeaters}

LINK RESET to {call} via {digipeaters}

DISCONNECTED fm {call} via {digipeaters}

LINK FAILURE with {call} via {digipeaters}

CONNECT REQUEST fm {call} via {digipeaters}

FRAME REJECT fm {call} via {digipeaters} (x y z)

FRAME REJECT to {call} via {digipeaters} (x y z)

x y z = FRMR information bytes

Monitor Header format

---------------------

fm {call} to {call} via {digipeaters} ctl {name} pid {hex}

Channel Status format

---------------------

a b c d e f

a = Number of link status messages not yet displayed

b = Number of receive frames not yet displayed

c = Number of send frames not yet transmitted

d = Number of transmitted frames not yet acknowledged

e = Number of tries on current operation

f = Link state

Possible link states are:

0 = Disconnected

1 = Link Setup

2 = Frame Reject

3 = Disconnect Request

4 = Information Transfer

5 = Reject Frame Sent

6 = Waiting Acknowledgement

7 = Device Busy

8 = Remote Device Busy

9 = Both Devices Busy

10 = Waiting Acknowledgement and Device Busy

11 = Waiting Acknowledgement and Remote Busy

12 = Waiting Acknowledgement and Both Devices Busy

13 = Reject Frame Sent and Device Busy

14 = Reject Frame Sent and Remote Busy

15 = Reject Frame Sent and Both Devices Busy

NOTE 1:  Only items a and b are displayed for channel 0.

NOTE 2:  Only states 0 - 4 are possible if version 1 is in use.

DEFAULT PARAMETERS

==================

In some instances, it  may  be  desirable  to  have  default

parameters which differ from the standard values.  To allow  easy

access, all default parameters have been placed at the  beginning

of the EPROM at location 0040H.  The  following  listing  defines

the layout of this area:

TYPE     VALUE          DESCRIPTION

----     -----          -----------

BYTE      1BH           COMMAND CHARACTER

BYTE    '      ',60H    SOURCE CALLSIGN (SEE NOTE 1)

BYTE    '      '        MNEMONIC IDENTIFIER

BYTE      04H           MAXIMUM CONNECTIONS

BYTE      03H           MONITOR MODE (SEE NOTE 2)

BYTE      01H           REPEATER DISABLE/ENABLE

BYTE      40H           P-PERSISTENCE VALUE

BYTE      0AH           SLOT TIME INTERVAL (10ms)

BYTE      1EH           TRANSMITTER DELAY (10ms)

BYTE      03H           FLOW CONTROL MODE

BYTE      01H           TRANSMITTER PTT DISABLE/ENABLE

BYTE      01H           AUTO LINEFEED DISABLE/ENABLE

BYTE      01H           ECHO COMMAND LINE DISABLE/ENABLE

BYTE      01H           VERSION 2 INITIATED DISABLE/ENABLE

BYTE      04H           MAXIMUM UNACKNOWLEDGED FRAMES

BYTE      0AH           MAXIMUM TRY COUNT

BYTE      04H           FRAME ACKNOWLEDGE INTERVAL

BYTE      00H           VALIDATE CALLSIGN ENABLE/DISABLE

BYTE      00H           FULL DUPLEX ENABLE/DISABLE

BYTE      00H           8-BIT CHARACTER ENABLE/DISABLE

***** SV7AIZ Multiuser-Port BBS v3.00 *****

This mailbox offers full service to all hams in North

and Northeast parts of Greece.

All other hams are welcome to use this mailbox.

You can send private-regular mails or bulletins.

There are 2 active ports.HF on 14.105/107 MHz,

VHF on 144.650 MHz.Tncs in both ports,tnc-2 with

NORD><LINK firmware.

For further infos,pse leave a msg to SV7AIZ

on this mailbox.

Enjoy !!!!

#ERROR TPU ###########�z#�#p###PKRESP  TPU ##########�#{#M3�###p-	�u#�R�.�~�&���##t#�r4#W�~�ǝ##W��#p-	�t

�~�&ǅz'##�"� =##u#�~�&��G1#t#�#�

�~�&ǅz'##�#��]�##U���#�#0#W�&#p-#�t#�6D1#����##/