This document explains how to use this bbs s/w under  the

BPQ packet switch internet s/w.

All the tests have been made by using BPQ v3.23,and  kiss

tncs.

We are making the whole work by simulating  pk232's  host

mode. We must configure the  bbs  cnfs  files  we  have  to

use,to handle pk232,as normal,by adding a B in the tnc type

description.

Note that, we  have  a normal bbs operation on the virtual

ports like the pk232 in real and we accept connects on  all

predefined channels.Bpq makes the deal with the  kiss  real

tncs we have and conserves the outer links.

In 'BPQCFG.TXT' we have to change:

HOSTMODE=1

so the first port in TNCPORTLIST,becomes PK232 virtual.

----------

The whole bbs operation remains almost the same,with  some

changes only.

changes:

i . Cnf file: In the port description  topic,we  must  use

the port we   have  defined  first   in   the   TNCPORTLIST

parameter minus 1.The same port  number  is  used  in  each

logical channel description which depends on  this  port.As

a tnctype we have to write PK232B instead  of  PK232  which

is for normal operation.

ii . We must first connect to SWITCH,to have access to the

local node which links with the outer world.Then if we wish

to connect to a station  we must give the port  number  and

callsign.

example  :  C 1 SV7AIZ

ii.Fwd file: We  must  add  one  more  line  in  each  fwd

station,that makes the link to switch.

example  :

---

2 SV7AIZ N

T 0023 <5

COSWITCH

S*C 1 SV7QI

R*Connected to SV7QI

---

** Note : We use port 2 on this example.It  means  that  we

have declared the first port in the  TNCPORTLIST  parameter

as 3.

TNCPORTLIST=3,4,5,6