mapampb1.doc

Jack Cavanagh, KB4XF

19 May 97

QB45 code to convert winAPRS maps to dosAPRS maps

ACKNOWLEDGEMENT:

This work would not have been possible without the publication

of the win/macAPRS map file format by Mike Musick and Mark Sproul.

Thanks guys.

INTRODUCTION

This is the beta test version of QB45 code to convert winAPRS

maps to dosAPRS maps.  The name is derived from the fact that it maps,

in the topological sense, coordinates from Mark-space to Bob-space.

In April 1997, the inventor of APRS, Bob Bruiniga, WB4APR, issued a

request on the APRSSIG for someone to write a map conversion code

so that we could all benefit from the win/macAPRS DCW maps.  The DCW (Digital Chart of the World) is a four CD package produced by the Defense Mapping Agency from 1:1,000,000 Aeronautical Navigation Charts and sold by USGS.  Unfortunately, the DCW is sold out at this time and the USGS web pages do not give any indication when it might be again available. DCW maps in WinAPRS format are available from the Sprouls in the form of a CD.  This is a way for APRS to go world wide.

This initial beta test vesion is distributed with three files:

1 - mapmapb1.exe, an dos executable code

2 - mapmapb1.bas, the QB45 source code

3 - mapmap.doc, this file that you are viewing (You didn't

print it out, did you?)

OPERATING:

For beta testing, running the source code under QB45 is preferred.

If the program bombs, we will know where the problem is rather than

some obscure address.  In either case, run the program and you will get

an opening screen and will be asked to supply a path to the winAPRS

maps that you want to convert.  If you just hit enter, it will default

to the current directory.  WARNING: this path information will be used

as a argument to the Basic CHDIR function.  This cannot change the

default disk drive.

The only other user input is the name of the actual file to convert. The screen will clear and you will be presented with a list of .map files on the default directory.  Select one of these with the arrow keys and hit ENTER. The mouse won't work here. Files that have been converted in this session are shown in blue.  Your current choice is in red. All others are in white.  The "STOP" option exists for the program to exit gracefully.

Next you will get a screen that contains the header information from the winPARS map file.  You can check to see if this is really the file that you want.  Since the winAPRS map files are binary files, this is about the only way to read the header.  Do the "hit any key" bit and the program does its thing and the results are written to an output file.  This beta version pauses after each  map so you can view the results.  Future versions will probably eliminate this pause.

When all dos maps have been made, the file selection menu appears.

You can then make more maps.  The process is fairly fast about 15-20 seconds per dos map on a 75 MHz Pentium.

OUTPUT FILES:

DWC files are huge.  The world is divided in 15 deg by 15 deg tiles

labeled A-Z in an west to east direction without the letters "I" and "O". They are labeled south to north likewise with the letters A-M.  Each tile is divided into nine 5 deg by 5 deg sub-tiles which are numbered 11,12,13,21,22,23,31,32, and 33 beginning in the SE corner of the tile.  The first number indicates the sub-tile column and second the sub-tile row.  Thus, sub-tile DK33 is the NW corner sub-tile of tile DK. Now 5x5 tiles are what make up winAPRS DCW derived maps, but this is too big for for dosAPRS.  I have chosen for this beta version to breakdown each sub-tile into sub-sub-tiles that are 1 degree in latitude (6o nautical miles n-s corresponding to a 30 mile map radius). The width is chose to fit a standard APRS screen.  This varies in longitude with latitude.  At the equator a sub-tile is broken into 5 rows by 4 columns that are 60 nm n-s and 80 miles e-w.  In northern Canada the shrinking of longitude spacing results in 5 row by 2 column sub-sub tiles that are still 60 by 80 nautical miles.  The sub-sub-tiles are identified by a two letter conbination with the first letter a-e inidcating the row and the second letter inidicating the column.   The dos map output file name is formed by chopping the first three characters from the winAPRS map file name; adding the row/column letters and the extension .dmf (This stands for Dos Map File - not very original, Jack) For example, a winAPRS file with named DCWDK11.map will produce dos files with names like dk11aa.dmf or dk11ec.dmf.

After you exit the program, view a file with the extension .log such as dk11.log.  This will show the dos map files names, their sizes in points and the lat/long of their origin. If the number of points is less than 3000, you can rename it as a .map file and move it to the APRS map subdirectory.  If the file has less than 18,000 points, load it into MAPFIX and follows Dr. Bob's prescriptions to decimate points, and split into 2999 point APRS size bites.

ALPHA testing:

At this time, only map vectors and text labels are converted.  Symbols conversion code has been written but cannot be tested until I can get a winAPRS map with symbols.

The pain associated with writing this code came from having to

learn new stuff at age 56 like TYPE....END TYPE, which is only briefly

documented by the minions of Bill Gates in the QB45 manual.  The

other trouble came from having to deal with the conversion of

big-endian Mac/Motorola numbers to little-endian PC/Intel numbers

given the limitation of Basic.  Now, Mark has written C++ code to do

this, but I am too ignorant to merge Basic and C++ code.  I am sure

that byte swapping is trivial in assembly language. Is there a software

guru out there who can write some efficient code?  An additional

opportunity exists for someone to show off their skills by writing better code with mouse assist to select files from a list.

I downloaded some winAPRS maps from the Sprouls's web site. These included CANADA.MAP, DCWDK.zip (British Columbia) and eraspk22.map (southern Greece). The general simple CANADA map and Greek map converted OK.  PKUNZIP reported some problems with DCWDK.zip and some of the nine files blow up with overflow errors.  I think this is a data file problem and not a code problem.  Unfortunately, the bad maps cover the cities of Vancouver, BC (where packet radio began before the FCC could spell ASCII) and Redmond, WA (where the man who gave us Windows resides).  It would have been nice to have maps that could have been compared with existing maps.

One of the features of the DCW maps retained by the Sprouls was populated places. This gives a nice outline of cities in magenta, but it fills up the maps with two pixel dots corresponding to every small town, village, hamlet, whistle stop and crossroad with one general store and a gas station. These are not copied to the dos files.  Now there are a awfull lot of lakes and streams in British Columbia that eat up precious dos map points, so we might want to make this and option in the future.  This is part of what beta testing is about.

Some things are left undone.  The map data could be put into the new line format for compaction and speed, but while testing is ongoing, it helps to have the output in something that is readable by mere mortals.  Colors need to be resolved.  All roads are green rather than the previous scheme of  white/red/green  for streets, highways and interstates.  Unfortunately, the road category information was not transferred from the DCW and we are stuck with only one color, but which one?  In addition, coastlines are in yellow we could make them blue but then polical boundaries would become blue. One annoying little feature is "neat lines".  These are the borders you see at the edges of printed maps.  We don't need them for APRS, but manual deletion in MAPFIX seems to be the only way to get rid of them.

During this period of beta testing, please let me know of your progress or lack of it.  One of the positive things that I like to know is the average size of the .dmf files.  If this is generally small enough, I could be psuaded to retain the city outlines but filter out the single points.  The more you can tell me about a problem the better chance I have of fixing it.  Use the QB de-bug functions, especially SHFT-F9, to figure out where the problem and what was the value of key variables.  Report good and bad news to me via e-mail: cjcavanagh@msn.com

Happy map making!

Jack