Instructions for PLNGRFnn.EXE

by W1EO

17 April 1997

This program will assist the (potential) server sysop in selecting the

best 2 meter packet user channel for a new server. It will also show

the closest packet servers for a new user of packet radio.

The (default) input data file is twometer.pln which may be edited and expanded

using the line format;

frequency callsign qth [gs]

as;

145.75 W1ON BedfordMA [FN42jm]

Notes:  1. Exactly one space between line elements and only three spaces

per line.

2. The GS, the last entry on each line, has the initial two characters

in upper case for those stations who report a GS in their beacons

and headers. These two characters are in lower case for stations for

which I estimated their GSs by using the APRS program map features.

3. There may be additional information immediately following the

GS entry. Such information will be included in curley brackets,

'{}' without any space separating it from the GS. The information

is power and antenna above mean sea level (AMSL) of the server

station. The format is '{50/400}' = {power/height}. This information

is used by the PLNGRFxx.EXE program to plot coverage circles.

I will publish updated versions of the data file as a packet bulletin

periodically as additions and changes are made. The text portion of the

bulletins can be edited to create a new twometer.pln file when received.

The program is called by typing "PLNGRFnn.exe" where "nn" is the latest

version number. The program reads the plan.ini file (if present).

Note: a sample ini file is included as 'sample.ini'. Please rename and

edit to fit your station.

The .ini file must have one of the following formats;

GridSquare

GridSquare {power/height}

GridSquare filename

GridSquare {power/height} filename

latitude longitude

latitude longitude {power/height}

latitude longitude filename

latitude longitude {power/height} filename

where;

GridSquare is the gridsquare of your station - FN42jm,

latitude and longitude of your station - 42.5 71.3333 format,

{power/height} is the transmit power and antenna height of your

station (AMSL), and

filename is a DOS file name where you want the results to be

put rather than the screen. [Used only by the PLANnn.exe program]

In addition, the plan.ini file line may terminate with a fully directorized

APRS map name, ie, \aprs\maps\flsouth.map as an example. The excellent APRS

program is available from many sources and its large number of ASCII format

maps are in the public domain.

This program will use the map filename if it is present in the specified

directory, as identified in the plan.ini file, to redirect the name of the

input data file to have the same name but with the extension of 'lst' as in

'flsouth.lst'. It will look for this file in the default directory where

PLNGRFnn.exe is located. PLNGRFnn.exe is, therefore, now generic and may be

used for any location for which APRS maps are available. The user must create

the data file using a matching name.

This program is a companion to the PLANnn.EXE program. They both use the

same data file. This program first allows you to select either one of the 25

standard packet frequencies or one of the 23 alternative 'even' packet

frequencies. Then it places all the servers it finds in the input data file

(default twometer.pln) which are listed as operating on the selected frequency

onto the displayed map. {default of New England.} If an APRS map file name is

included in the plan.ini file, that map will be displayed instead. You must

have the APRS map already on your harddrive - these programs do not include

them in the zipped package. The APRS program is shareware and is available

from many sources. Note that the data file name will also change to match the

map file name.

If a station location has been included in the plan.ini file (gridsquare or

lat/lon), that location will be plotted in red, if it is included in the

displayed map area. If a power/height entry is provided in the plan.ini

file, that information is used to plot a coverage circle in red around

the station. The default antenna height is 50 feet. If the station location

appears on the displayed map, the small red cursor will be placed initially

at that location.

The .lst file must contain information in specifically formatted lines. Any

number of initial comment lines can be used but each must begin with a '#'

character. Each of the subsequent information lines must contain exactly

four segments, each separated by exactly one space. The last segment of

each information line may consist of the location in either the gridsquare

format enclosed in square brackets, ie [fn42im], or in the latitude/longitude

format enclosed in angle brackets, ie <4230.50N/07122.33W>, using the angles

in degrees*100 + decimal-minutes per APRS convention. Note that the slashbar

and the leading 0 in the longitude in the latter format are mandatory.

The program first checks the format of each of the line entries in the .lst

file and aborts if any are found incorrect, leaving a copy of the offending

line on the screen.

You may cycle thru the 25 standard packet frequencies using the plus and minus

keys. You may also call the list of frequencies using the 'f' command at any

time. You may then move the cursor among the available packet frequencies using

the arrow keys. You select the frequency with the RETURN key.

You may move the map around using the cursor keys to select the new map

center and then typing the HOME key. You may change the size of the map

by typing either the PgUp or the PgDn key. Note: if you type a cursor (ARROW)

key less often than once a second, the cursor will move only one pixel for

each press. If you hold one of the ARROW keys down, the cursor will move

faster.

You may cause the major and minor grid square boundries to be plotted by

typing a 'g' character at any time;

You may cause all the YCCC, nodes, switches or BBSes to be plotted separately

for all frequencies by typing 'y', 'n', 's' or 'b' respectively;

Since I am a lighthouse buff, I have recently added a feature to display

the location of lighthouses on the APRS maps using the 'l' command.

Lighthouses can be included in the server list(s) by setting the frequency

to '0' and and including the 'LH' at the end of its name;

0 BostonLightLH BostonHarborMA <4220.00N/07051.88W>

Note the use of lat/lon enclosed with angle brackets since gridsquares are

not precise enough for the location of lighthouses. The lat/lon numbers

follow the APRS procedure of degrees*100 + decimal-minutes.

You may center the map on your location by typing a 'c';

You may remove all the coverage circles from all the servers and your

station location and display range circles centered on your station and on

the cursor by typing an 'r';

You may display a short help list by typing an 'h';

You may list each of the 48 packet frequencies and the type of server(s)

found in the data file by typing an 'f'. You may select a new frequency

from this list.

You may cause all the servers to be plotted for all frequencies by entering

a RETURN. Note: this one gets pretty crowded!

You may cause a list of all the servers at a specific location to be printed

at the top of the screen by placing the cursor on the symbol at the location

of interest and then typing a question mark, "?". If no servers are known at

that location, the top of the screen will so indicate.

You may cause any servers that reside on either of the adjacent frequencies

(+/- 10 Khz) to also be plotted but using the 'd' command. Each of these

servers (if any) will be a light white color and will have the appropriate

sign in front of the name.

You may cause the linkages between BBSes, nodes or switches to be plotted by

typing L (cap L, not lowercase l) when either the BBSes, nodes or switches

have been plotted.

You may add new servers to the *.lst file from within the program as of

version 43 by moving the cursor to the desired location and typing 'i' and

then answering the questions at the top of the screen. The *.lst file

will be sorted if the DOS program sort.exe can be located from your

enviornment statement, ie, path=\dos etc.

The top line of the display documents the cursor location in lat/lon and the

gridsquare. The second last entry on the top line is either the selected packet

frequency or a word describing the displayed station class (YCCCs, nodes,

switches, BBSes or lighthouses) in curly brackets. The last entry on the top

line is the status of the zoom function.

Each server location has a circle associated with it which approximates the

area in which it can be received. The simplifying assumption is that the

receiving station is at sea level on a 50 foot antenna and that the antenna

gain exactly offsets the line losses. If the receiving station is situated at

a higher elevation, it may be able to receive servers whose coverage circle

does not reach their location. The coverage circles do not take into account

any intervening obstructions like hills and mountains. Therefore, the coverage

circles are only indicative. If the information line for the server in the

input data file ends with a string formatted like '{pp/hhh}', this data will

be used for determining the radius of the coverage circle around the server

location. The pp value is the power and the hhh value is the antenna height

AMSL. The pp value has little effect on the radius but the hhh value is used in

the classic D=sqr(2xH) relationship. If no '{pp/hhh}' string is included on the

information line, default values of 20 watts and 50 feet (50 foot tower at sea

level) are assumed.

If there exists a plan.ini file, the PLNGRFnn.exe program will read the

location of your station from either the grid square or from the lat/lon

information and plot a small red circle on the displayed map at that

location. If the plan.ini file also contains the power/height information

for your station, it will use the antenna height values to calculate and

plot a red coverage circle around that location. Otherwise, the program

assumes a 50 foot antenna at sea level at your location. If the plan.ini

file does not exist, neither the small circle nor the coverage circles

will be plotted.

The various server locations are indicated with special symbols;

open square   =  BBS or switch (box)

closed circle =  node (lots of stuff inside)

closed square =  both a node and a BBS

'y'           =  YCCC

lighthouse    =  lighthouse

To exit the program, type the ESCAPE key.