LAN-LINK Version 2.32 PAGE 1

LAN-LINK Copyright (c) 1988 - 1994 by G3ZCZ

All Rights Reserved

Joe Kasser W3/G3ZCZ

POB 3419

Silver Spring

Md., 20918, U.S.A.

Voice (301) 593 6136

Data BBS (301) 593 9067

Internet jkasser@cap.gwu.edu

_______

____|__     | (tm)

--|       |    |-------------------

|   ____|__  | Association of

|  |       |_| Shareware

|__|   o   |   Professionals

-----|   |   |---------------------

|___|___|   MEMBER

LAN-LINK.DOC (c) G3ZCZ 1988 - 1994

LAN-LINK Version 2.32 PAGE 2

Table of Contents

Appendix 1 Introducing LAN-LINK ........................ 2

LAN-LINK PROGRAM HIGHLIGHTS............................ 4

Getting Started ....................................... 7

Bringing LAN-LINK Up the First Time ................... 8

Using the Quick Menu .................................. 9

Connecting to Other Stations .......................... 9

Using Different TNCs .................................. 9

LAN-LINK Memory Requirements ..........................10

PC to TNC Communications ..............................10

LAN-LINK Runs Open Loop ...............................10

In Case of Problems ...................................10

Logbooks ..............................................11

How To Get an Update ..................................11

Starting LAN-LINK in a Non Packet Mode ................11

Using LAN-LINK with a PacketCluster ...................12

Using ANSI Color for Transmitting Text Color Files ....13

Appendix 2 LAN-LINK Features, Menus and Commands ...... 14

1.0 LAN-LINK MESSAGE HANDLING FEATURES ................14

1.1 Answering Machine ..............................14

1.2 Mail Beacon (Annunciator) ......................14

1.3 How to Leave a Message .........................15

2.0 LAN-LINK WINDOWS ..................................15

3.0 HOT KEYS ..........................................19

3.1 Alt-A Alert Call (Packet) & AMTOR ARQ Call/Talk 19

3.2 Alt-B Send Break/AMTOR Changeover ..............19

3.3 Alt-C Call/Connect to Someone ..................19

3.4 Alt-D Disconnect or Receive ....................21

3.5 Alt-E Enter Call ...............................22

3.6 Alt-F Flush TNC Buffer .........................22

3.7 Alt-H Help Information for Function Keys .......22

3.8 Alt-J Jump to DOS ..............................22

3.9 Alt-K PacketCluster DX Alert....................22

3.10 Alt-L Log Contact .............................22

3.11 Alt-M Message Scan ............................22

3.12 Alt-N Next Stream .............................23

3.13 Alt-O Turn Robot On ...........................23

3.14 Alt-P Turn Printer ON/OFF .....................23

3.15 Alt-Q Call QRZ ................................23

3.16 Alt-R Retry Call/Connect ......................23

3.17 Alt-S Scan Log ................................24

3.18 Alt-W :QRA: ...................................24

3.19 Alt-X Exit to DOS .............................24

3.20 Alt-Y Zap PacketCluster/RTTY Transmit "RY"s ...24

3.21 Alt-Z Zap PBBS ................................24

3.22 Alt-= Toggle Miscellaneous Flag ...............26

3.23 End "> +?" [Over] .............................26

3.24 PgUp Scrollback (Scrolls back incoming text) ..27

LAN-LINK.DOC (c) G3ZCZ 1988 - 1994

LAN-LINK Version 2.32 PAGE 3

3.25 The Home Key ..................................27

3.26 The Down Arrow Key ............................27

3.27 Shift/Baudrate Control Keys ...................28

3.28 Control-Z (^Z) ................................28

3.29 Control T (^T) ................................28

4.0 FUNCTION KEYS .....................................28

4.1 Capture to Disk Toggle (F1) ....................29

4.2 Send Brag Tape and Talk (F2) ...................29

4.3 Transmit Callsign Sequence (F3) ................29

4.4 Send Brag Tape (F4) ............................30

4.5 Show Packet "MH" list and Point and Shoot

Connect (F5)...............30

4.6 Auto CQ Call (F6) ..............................30

4.7 Activate Type Ahead Buffer/AMTOR Relink (F7) ...31

4.8 Disconnect, Disengage or Receive (F8) ..........32

4.9 Put TNC in CMD Mode (F9) .......................32

4.10 Put TNC in CONVERSE/Transmit/AMTOR FEC (F10) ..32

4.11 Alternate Function Keys .......................33

4.12 Shift Function Keys ...........................33

4.13 Control Function Keys .........................33

5.0 THE MAIN MENU .....................................34

5.1 The Quick Menu .................................34

5.2 Auto CQ ........................................35

5.3 Send Brag Tape .................................35

5.4 Call Someone ...................................35

5.5 Log Menu .......................................35

5.6 Change Communications Mode .....................35

5.7 Call CQ ........................................35

5.8 Send/Talk File .................................35

5.9 QRZ ............................................36

5.10 Send/Talk LAN-LINK Files 001..010 .............36

6.0 THE PBBS/PacketCluster MENU .......................36

6.1 Download Text Files ............................36

6.2 Download Binary File (Xmodem) ..................37

6.3 Download Binary File (Ymodem) ..................37

6.4 Zap PacketCluster ..............................37

6.5 Zap PBBS .......................................37

6.6 Turn PacketCluster Mode ON/OFF .................37

6.7 Send DX Alert ..................................38

6.8 Clear All Search Flags .........................38

6.9 Set All Search Flags ...........................38

6.10 Set All CW Search Flags .......................38

6.11 Set All SSB Search Flags ......................38

6.12 Set/Clear Individual Search Flags .............38

6.13 Set/Clear Current Band Search Flags ...........38

7.0 THE CALL MENU .....................................39

7.1 Alert Call .....................................39

7.2 ReMember Path ..................................40

7.3 Call CQ ........................................40

7.4 Show Contents of Call Directory ................40

LAN-LINK.DOC (c) G3ZCZ 1988 - 1994

LAN-LINK Version 2.32 PAGE 4

7.5 Target Call ....................................40

7.6 Auto CQ ........................................41

7.7 Toggle DX Flag .................................41

7.8 Zap PBBS on Next Mail Beacon ...................41

8.0 EDIT MENU .........................................41

8.1 Edit Brag Tape .................................43

8.2 Edit Call Directory File .......................44

8.3 Edit Any Other File ............................44

8.4 Answer Capture-to-disk (Ctdsk) File

(Split Screen) ...........44

8.5 Answer Incoming Messages (Split Screen) ........44

8.6 Leave Note for Someone .........................44

8.7 Edit Capture-to-Disk file ......................45

8.8 Edit Two Files (Split Screen) ..................45

8.9 Edit Configuration File ........................45

8.10 Edit LAN-LINK.001 through LAN-LINK.010 Files ..45

8.11 Pick Capture-to-disk File to Edit .............45

8.12 Pick Message File to Edit .....................45

8.13 Leave Messages on your local PBBS .............46

8.14 Edit Incoming Message .........................46

8.15 Edit LAN-LINK.OUT File ........................46

8.16 Edit LAN-LINK.DMO File ........................46

8.17 Pick dEmo File to Edit ........................46

9.0 EVENT/CLOCK MENU ..................................46

9.1 Set Alarm ......................................47

9.2 Connect ........................................47

9.3 Display Event ..................................47

9.4 Periodic Event .................................47

9.5 Turn Robot ON/OFF ..............................47

9.6 Turn Contest ON/OFF ............................47

9.7 Turn Time Display ON/OFF .......................47

9.8 Cancel Event ...................................47

9.9 Zap PBBS .......................................47

10.0 FILES MENU .......................................48

10.1 Send Brag Tape ................................48

10.2 File Directory ................................49

10.3 Erase File ....................................49

10.4 Pick Capture-to-disk (Ctdsk) file to View .....49

10.5 Rename File ...................................49

10.6 Send File .....................................49

10.7 Send/Talk File ................................49

10.8 Send Uppercase File ...........................49

10.9 View File .....................................49

10.10 View Capture-to-disk (Ctdsk) File ............50

10.11 View Incoming Message File ...................50

10.12 Print File ...................................50

10.13 View Color File ..............................50

10.14 Turn Mode Files ON/OFF .......................50

10.15 Turn eXpand Files ON/OFF .....................50

11.0 HELP WITH FUNCTION KEYS ..........................50

LAN-LINK.DOC (c) G3ZCZ 1988 - 1994

LAN-LINK Version 2.32 PAGE 5

12.0 JUMP TO DOS SHELL ................................51

13.0 LAN-LINK MENU ....................................51

13.1 :QBU: External Protocol Command ...............51

13.2 :QBM: ASCII File ..............................52

13.3 :QDB: ASCII File ..............................52

13.4 :QIC: Page Operator ...........................52

13.5 :QMH: Monitor Heard ...........................53

13.6 :QRA: Who is "ON-LINE" ........................53

13.7 :QRU: Messages ................................53

13.8 :QSM: Message .................................53

13.9 :QSP: Message .................................54

13.10 :QTA: Messages ...............................54

13.11 :QTR: Ask for Time ...........................54

13.12 :QZD: Binary Files ...........................54

13.13 :QZU: Binary Files ...........................55

13.14 Demo Mode ....................................55

13.5 Set Single/Dual PK-900/DSP 2232 In Windows.....55

14.0 LOG MENU .........................................55

14.1 Append From Another LAN-LINK Log ..............56

14.2 Scan Log by Band ..............................56

14.3 Change Logbook File ...........................56

14.4 Delete Logbook File ...........................57

14.5 Find Date .....................................57

14.6 Purge Log .....................................57

14.7 Help Function Keys ............................57

14.8 Import From G3ZCZ's Contest Log ...............57

14.9 Jump to an Entry ..............................58

14.10 Pick a Log file ..............................58

14.11 Pack a Logbook ...............................58

14.12 Scan Log by Mode .............................58

14.13 Create a New Logbook .........................58

14.14 Print Log ....................................58

14.15 Re-index a Logbook ...........................58

14.16 Sort on Date + Time ..........................58

14.17 Turn Update Disk ON/OFF ......................59

14.18 Fuzzy Scan ...................................59

14.19 Turn Secondary Search ON/OFF .................59

14.20 Log Menu Function Keys .......................59

15.0 MESSAGE MENU .....................................61

15.1 PBBS Command Files ............................62

15.2 Current Messages ..............................62

15.3 View Message ..................................62

15.4 Leave Note ....................................62

15.5 Old Messages ..................................62

15.6 Time Tag ......................................62

16.0 TNC MENU .........................................63

16.1 Set TNC Date/Time .............................64

16.2 Flush TNC Buffer ..............................64

16.3 Initialize TNC ................................64

16.4 Show Calls Monitored List .....................65

LAN-LINK.DOC (c) G3ZCZ 1988 - 1994

LAN-LINK Version 2.32 PAGE 6

16.5 Show Connect Status ...........................65

16.6 Change CTEXT ..................................65

16.7 Activate TNC Mailbox ..........................65

16.8 Deactivate TNC Mailbox ........................66

16.9 TNC Modem Menu.................................66

16.10 Select Radio Port 1...........................66

16.11 Select Radio Port 2...........................66

16.12 Disable Radio Port 1..........................66

16.13 Disable Radio Port 2..........................66

16.14 Enable Radio Port 1...........................66

16.16 Enable Radio Port 2...........................66

16.17 Show ### Radio Port...........................66

17.0 AMSAT-OSCAR MENU .................................67

17.1 Full Duplex ON/OFF.............................67

17.2 SAREX MENU ....................................67

17.2.6 Set Attack Mode to Connect/CQ ...............69

17.3 SAREX Orbiter Features ........................69

18.0 PARAMETER MENU ...................................72

18.1 Turn Beacon Mode ON/OFF .......................72

18.2 Clear Connect Counter .........................73

18.3 TNC Custom Line (TNC Parameter) ...............74

18.4 Turn Beacon Control ON/OFF ....................76

18.5 Turn Command Echo ON/OFF ......................76

18.6 Change Keyboard Beep Count ....................76

18.7 Set Message Count .............................77

18.8 Turn Node Drop Link ON/OFF ....................77

18.9 Turn Printer ON/OFF ...........................77

18.10 Turn QTC Snatch ON/OFF .......................77

18.11 Turn :QSP: Relay Flag ON/OFF .................78

18.12 Turn Sound ON/OFF ............................79

18.13 Turn Contest Mode ON/OFF .....................79

18.14 Turn DCD Flag ON/OFF .........................80

18.15 Turn PBBS Snatch ON/OFF ......................80

18.16 Turn Time Id. ON/OFF .........................81

18.17 Turn Automatic Capture-to-Disk Flag ON/OFF ...81

18.18 Change PBBS Parameters .......................81

18.19 Change Callsigns .............................82

18.20 Change Directories ...........................83

18.21 Enter Common Calls ...........................84

18.22 Change File Names ............................84

18.23 Change LAN-LINK Parameters ...................84

18.24 Change PC Communications Parameters ..........88

18.25 Change Colors ................................89

18.26 Change SAREX Parameters ......................92

18.27 Update Configuration File ....................93

18.28 Verify Changes ...............................93

18.29 Change Scan Words ............................93

18.30 Turn Miscellaneous Flag ON/OFF ...............93

18.31 Change Band/Mode .............................95

18.32 Zap PBBS on Next Mail Beacon .................95

LAN-LINK.DOC (c) G3ZCZ 1988 - 1994

LAN-LINK Version 2.32 PAGE 7

18.33 PBBS Zap, Don't Disconnect After Sequence ....95

18.34 Control Send of LAN-LINK.OUT .................95

19.0 Q Codes [NC/L] ...................................95

20.0 COMMUNICATIONS MODES .............................96

20.1 VHF PACKET ....................................96

20.2 HF PACKET .....................................96

20.3 ASCII (RTTY) ..................................96

20.4 BAUDOT (RTTY) .................................96

20.5 MORSE (CW) ....................................97

20.6 AMTOR (STANDBY) ...............................97

20.7 AMTOR (MONITOR) ...............................97

20.8 PACTOR (STANDBY) ..............................97

20.9 PACTOR (MONITOR) ..............................97

20.10 SIGNAL .......................................98

20.11 NAVTEX .......................................98

20.12 G-TOR  .......................................98

20.13 MANUAL .......................................98

21.0 TERMINAL MODE MENU ...............................98

21.1 Packet Terminal Menu ..........................98

21.2 AMTOR Terminal Menu ..........................103

21.3 Baudot Terminal Menu .........................104

21.4 ASCII Terminal Menu ..........................106

21.5 CW Menu ......................................107

21.6 Pactor Terminal Menu .........................108

22.0 EXIT TO DOS .....................................109

23.0 THE ELMER MENU ..................................109

23.1 Change ELMER File ............................109

23.2 Turn ELMER ON/OFF ............................109

23.3 Load State Table (ELMER.QSO) .................109

23.4 Pick File to Edit ............................110

23.5 Edit State Table (ELMER.QSO) .................110

23.6 Show State Table .............................110

24.0 RADIO MENU ......................................110

24.1 Select VFO A/B................................110

24.2 Set Mode......................................111

24.3 Turn Flag QSY ON/OFF or N/A ..................111

24.4 Read VFO Frequency............................111

24.5 Set VFO Frequency.............................111

24.1 24.6 Speak Frequency..........................111

Appendix 3 LAN-LINK Disk Files ........................112

1.0 LAN-LINK.EXE .....................................112

2.0 LAN-LINK.SYS .....................................112

2.1 CTEXT Line ....................................115

2.2 ELMER Flag ....................................115

2.3 PC Port 4 Custom Address ......................115

2.4 PC Port 4 Custom IRQ ..........................115

2.5 VHF Stream Switch Character ...................115

2.6 HF Stream Switch Character ....................116

2.7 TNC Error Message .............................116

LAN-LINK.DOC (c) G3ZCZ 1988 - 1994

LAN-LINK Version 2.32 PAGE 8

2.8 Connect Timeout Delay .........................116

2.9 ELMER Configuration File ......................116

2.10 Expand Transmitted Files Flag ................116

2.11 First PacketCluster Command ..................116

2.12 Second PacketCluster Command .................116

2.13 TNC Mailbox ON Command .......................116

2.14 TNC Mailbox OFF Command ......................117

2.15 VHF Maxframe Command .........................117

2.16 HF Maxframe Command ..........................117

2.17 Inhibit VHF Logging Flag .....................117

2.18 Inhibit HF Logging Flag ......................117

2.19 Printer Permanent Flag .......................117

2.20 Log Disk Update Flag .........................117

2.21 Secondary Log Flag ...........................117

2.22 Auto Answer Flag .............................117

2.23 Ctdsk File Disk Update Flag ..................118

2.24 Default CW Speed .............................118

2.25 Secondary Logbook ............................118

2.26 BBS List Command .............................118

2.27 BBS List Mine Command ........................118

2.28 BBS Custom Command ...........................118

2.29 BBS Bye Command ..............................118

2.30 Start Up Packet Terminal Mode ................118

2.31 Page Time (:QIC: Seconds) ....................119

2.32 PacketCluster DX Alert Tone CW Dit time ......119

2.33 Packet Cluster DX Alert Tone CW Note .........119

2.34 Mode File Flag ...............................119

2.35 BAUDOT Color Change Text String ..............119

2.36 Dual Window Flag .............................119

2.37 TNC String for Modem 9600 baud ...............119

2.38 TNC String for Modem 1200 baud ...............119

2.39 TNC String for Modem RTTY ....................120

2.40 TNC String for Modem 300 baud ................120

2.41 TNC String for Modem RTTY and 1200 baud ......120

2.42 TNC String for Modem 300 baud and 1200 baud ..120

2.43 TNC String for Modem Fuji ....................120

2.44 Delay after a command line ...................120

2.45 Radio ........................................120

2.46 Radio Port ...................................120

2.47 Radio Port baud rate .........................120

2.48 Radio Port Bits ..............................121

2.49 Radio Port Stopbits ..........................121

2.50 Radio Port Parity ............................121

2.51 Radio Address ................................121

2.52 Flag QSY .....................................121

2.53 Flag DX Alert Prefix Check ...................121

2.54 Band parameters for DX Alert message

Log search ...............121

2.55 Modes for DX Alert message log search ........122

2.56 TNC Parameters ...............................122

LAN-LINK.DOC (c) G3ZCZ 1988 - 1994

LAN-LINK Version 2.32 PAGE 9

3.0 *.RUN ............................................122

4.0 *.DBF (HF.DBF and VHF.DBF) .......................122

5.0 LAN-LINK.DIR .....................................122

6.0 CONFIG.SYS .......................................123

7.0 *.MBX ............................................123

8.0 *.BBS ............................................124

9.0 LAN-LINK.001 - LAN-LINK.010 ......................125

10.0 QBU-RX.BAT ......................................126

11.0 QBU-TX.BAT ......................................126

12.0 LAN-LINK.QTA ....................................126

13.0 QZ*.BAT .........................................126

14.0 ELMER.QSO .......................................127

15.0 LAN-LINK.OUT ....................................127

16.0 *.DMO ...........................................127

Appendix 4 Local Area Network Capability ..............128

1.0 LAN Protocol (G3ZCZ Version) .....................128

2.0 Message Format ...................................129

3.0 ASCII File Uploading/Downloading .................129

4.0 Path Determination to a DX station ...............130

5.0 Binary File Transfers ............................131

6.0 Requirements for Transfer Protocols ..............131

7.0 The Zmodem Protocol ..............................132

8.0 The PCZ Zmodem Package ...........................132

9.0 BASIC NC/L DICTIONARY ............................133

9.1 :QBM: .........................................133

9.2 :QDB: .........................................134

9.3 :QMH: .........................................134

9.4 :QSM: .........................................134

9.5 :QSP: .........................................134

9.6 :QRA: .........................................134

9.7 :QRT: .........................................134

9.8 :QRU: .........................................134

9.9 :QNO: .........................................135

9.10 :QJG: ........................................135

9.11 :QRV: ........................................135

9.12 :QSL: ........................................135

9.13 :QTA: message ................................135

9.14 :QTC: Message List ...........................135

9.15 :QBU: External Protocols .....................136

9.16 :QZU: Zmodem Binary File Upload ..............136

9.17 :QZD: Zmodem Binary File download ............136

9.18 :QIC: Page Operator ..........................136

9.19 :QTR: Ask for Time ...........................136

Appendix 5 Using Different TNCs .......................137

1.0 TNC2 (MFJ 1270) ..................................137

2.0 TNC1 (HK-4040) ...................................137

3.0 Kantronics KPC-2 .................................137

4.0 Kantronics KAM ...................................138

LAN-LINK.DOC (c) G3ZCZ 1988 - 1994

LAN-LINK Version 2.32 PAGE 10

5.0 PK-232 ...........................................138

6.0 MFJ 1278 .........................................138

7.0 Heath HK-21 ......................................139

8.0 PK-900 and DSP 2232 ..............................139

Appendix 6 Anomalies and Bugs .........................140

1.0 Command/Converse Mode Anomalies ..................140

2.0 Buffer Messages in AMTOR Beacon CQ Mode ..........140

3.0 FEC +? Responses in AMTOR Beacon CQ Mode .........140

4.0 Using a Monochrome Monitor .......................140

5.0 The Packet Conference Mode .......................140

6.0 Packet Conference Mode Disconnects ...............141

7.0 Strange Events ...................................141

8.0 *** DISCONNECTED .................................141

9.0 *** CONNECTED ....................................141

10.0 Greek Characters and Other Garbage in the

Incoming Screen ..........142

11.0 Dumb TNC ........................................142

12.0 Intermittent Lockup .............................142

13.0 Automatic Functions Don't Work ..................142

14.0 BBS Zap Failure .................................142

15.0 LAN-LINK Forgets the Band/Power Information .....143

16.0 Zmodem Binary Transfer Problems .................143

Appendix 7 Updates, Revision History ..................144

Appendix 8 ELMER An Expert System Based on a Finite

State Machine ............145

1.0 INTRODUCTION .....................................145

1.1 Expert Systems ................................146

1.2 The User Interface ............................146

1.3 The Knowledge Base ............................146

1.4 The Inference Engine ..........................146

1.5 Features of an Expert System ..................146

1.6 Using an Expert System ........................147

1.7 The Semantic Network ..........................147

1.8 A State Machine ...............................148

1.9 The User Perception ...........................148

1.10 ELMER ........................................148

1.11 Responses to an Input ........................148

1.12 Syntactic Analysis ...........................149

1.13 Programming ELMER ............................149

2.0 The State Table (ELMER.QSO) File .................150

2.1 The Directory Path to the ELMER Text Files ....150

2.2 The Start Up File .............................150

2.3 Window Colors .................................150

2.4 The ELMER Initial State .......................151

2.5 State Machine Parameters ......................151

2.6 The State Machine File ........................152

2.7 Example of Programming the State Table ........152

LAN-LINK.DOC (c) G3ZCZ 1988 - 1994

LAN-LINK Version 2.32 PAGE 11

3.0 The Stand Alone Version Status Window ............153

4.0 The Stand Alone Menu .............................153

4.1 Edit Any File .................................153

4.2 Load State Table (ELMER.QSO) ..................153

4.3 Pick File to Edit .............................154

4.4 Edit State Table (ELMER.QSO) ..................154

4.5 eXit to DOS ...................................154

4.6 Show State Table ..............................154

Appendix 9 PCZ Binary file Transfers ..................155

Appendix 10 Other Products by Joe Kasser, W3/G3ZCZ.....168

1.0 PC-HAM 3.52 ......................................168

2.0 STARTREK The Computer Program ....................168

3.0 WHATS-UP 1.30 ....................................169

4.0 ELMER 1.00 .......................................170

5.0 BASIC PACKET RADIO ...............................170

Appendix 11 How shareware Works .......................172

Appendix 12 LAN-LINK 2.32 REGISTRATION FORM ...........174

INDEX .................................................175

LAN-LINK.DOC (c) G3ZCZ 1988 - 1994

Appendix 1 Introducing LAN-LINK Page 2

Appendix 1 Introducing LAN-LINK

LAN-LINK is distributed as a Shareware product. You may freely

copy and share the product for non commercial use, with your

friends, associates and other radio amateurs. If you decide to

use the product, become a registered user by completing the

registration form in Appendix 12, and sending it (or a copy), and

$45.00 or equivalent in foreign currency to the author

(Mastercard and Visa accepted). All checks should be drawn on US

banks. Canadians, please use postal money order. Upon receipt of

your registration, you will receive one free update disk,

telephone and mail (electronic and regular) support. If you are

not satisfied with the program after registering it, your money

will be refunded no questions asked, for up to 30 days from the

date your registration was acknowledged.

European Radio Amateur users may register (30.00 Pounds Sterling,

includes V.A.T.) and obtain support from Terry Dansey at

ReadyCrest Ltd., PO Box 75, Chatham, Kent, ME5 9DL, England.

ReadyCrest Ltd. accepts credit cards (Access, Visa, MasterCard

and Eurocard). Telephones: Voice 44 (0) 634-687168, FAX 44

(0)634- 687178, Data (BBS) 44 (0)634-200931.

LAN-LINK may not be sold or distributed with another product

without the express written permission of Joe Kasser, G3ZCZ. Joe

Kasser, G3ZCZ will only support unmodified copies of this

software.

Potential Commercial Users please contact Joe Kasser directly for

modifications and/or details of Site licensing.

LAN-LINK.DOC (c) G3ZCZ 1988 - 1994

Appendix 1 Introducing LAN-LINK Page 3

LAN-LINK is a Personal Packet Terminal Program for the TNC1,

TNC2, KPC-2, and, most of all, a smart multi-mode digital

communications controller for the KAM, MFJ1278, DSP-2232, PK-900

and the PK-232. LAN-LINK is designed to optimize the configura-

tion of the TNC in each communications mode and to provide some

smart terminal features. It is designed to allow anyone to use

and get the most out of their packet TNC as well as from the PK-

232, the MFJ1278, and the KAM for Morse, ASCII, BAUDOT, AMTOR,

Pactor as well as Packet Radio communications without having to

keep the manual handy.

LAN-LINK is a sophisticated program. In its basic state it allows

you to use the TNC in an optimal manner. It configures the TNC

(it types the commands) for you to maximize the communications

efficiency in the communications mode of your choice. That means,

for example, when working Packet on HF you need to program the

TNC parameters to different values than you would use on VHF to

make maximum use of the mode. One significant difference is the

length of the packet itself, for the longer it is, the greater

the probability of QRM destroying it. This program will adjust

the packet parameters for you.

Since the computer is now involved, other features have been

added to simplify operation, and several features have been

automated. All these operations are performed using menus and

function keys as documented below. It will take a while to learn

how to use this program in a manner which suits you. Read this

document and have fun. After all, isn't that one of the purposes

of Amateur Radio.

The LAN-LINK manual is organized by menu. It describes the

operation of each menu option and function key in each

Communications Mode. The manual tells you what LAN-LINK can do.

It does not tell you how to use LAN-LINK.

Any Terminal Node Controller (TNC) is a complex piece of

equipment. It can operate in a Command Mode, in which you tell it

to do something, or in a Converse Mode in which you are using it

to talk to other stations. Many people confuse the two when first

getting on Packet. If you monitor the channels you will recognize

Command Mode TNC instructions on the air, and when you use the

TNC you will receive the "error" reply when you type something

thinking you are in the Converse Mode but are really in the Com-

mand Mode. LAN-LINK is designed to make Packet operation simple

using Menus and high level (Function and Hot key combinations)

commands.

Morse, ASCII, BAUDOT communications modes are HALF DUPLEX. One

station is transmitting at any time. You thus have to use the

Transmit-Converse and Receive-Disconnect function keys to turn

LAN-LINK.DOC (c) G3ZCZ 1988 - 1994

Appendix 1 Introducing LAN-LINK Page 4

the transmitter on and off respectively.

AMTOR is also HALF DUPLEX but has some of the attributes of full

duplex. In this Mode you control who is talking through software.

Only one station can send data at a time, but you can type

characters in to the buffer and they will be transmitted

automatically when the other station turns the QSO over to you.

Once you have linked with someone, you do not use the

Receive/Disconnect function until the QSO is over.

Both stations are communicating automatically thanks to the TNC.

If you want the other operator to respond type the "+?" key

sequence (or use the "End" Key).

Packet Radio communications on the other hand is a full duplex

mode of communications. Once you have connected with someone, you

do not use the Receive-Disconnect function until the QSO is over.

Both stations are communicating automatically thanks to the TNC.

If you want the other operator to respond type the ">" key

followed by the "Enter" Key. Better yet, use the "End" Key.

Morse, AMTOR, ASCII and BAUDOT are character mode communications

in that each character you type is sent as you type them. In

Packet Communications, the TNC stores them up and sends out a

burst or packet when you type the "Enter" Key or you type enough

characters to fill up a packet. Studies have shown that the human

attention span when sitting at the keyboard is of the order of 2

seconds. If you are in a keyboard to keyboard packet radio

connect, remember this and don't forget to use the "Enter" Key

frequently, especially before you stop to think about what to

send next.

AMTOR, Pactor, and PACKET are full duplex modes of communica-

tions. When you connect with someone, there is no need to use the

transmit and receive function keys. In fact you will have

problems if you do. Do not disconnect or break the link until the

QSO is over.

LAN-LINK PROGRAM HIGHLIGHTS

*    First packet radio software to be flown in Space.

*    Function key and Menu driven.

*    Smart PacketCluster features, i.e. automatic QSY to the

frequency of a DX Alert (if in PacketCluster mode).

*    Automatic search of HF logbook in response to a Packet-

Cluster DX Alert Message. Customizable levels of

notification. Audio Notification of Prefix status.

*    Start Up Communications mode is customizable.

*    Start Up Packet terminal mode is customizable.

*    Allows you to search through 2 logbooks for previous

LAN-LINK.DOC (c) G3ZCZ 1988 - 1994

Appendix 1 Introducing LAN-LINK Page 5

contacts.

*    60 Different Bragtapes/files, 10 in each communications

mode.

*    Automatic logbook entries for Pactor, Packet and Mailbox

Beacon Mode AMTOR Connects, semiautomatic logbook entries

for other modes.

*    Provides artificial intelligence in the shape of a second

operator (ELMER).

*    Logbook file is dBASE compatible and can be processed by the

LOGBOOK Package of PC-HAM by G3ZCZ for indexed listings,

tracking of DXCC and other AWARDS, etc.

*    Contest operation, sends standard message and automatically

increments QSO count.

*    Automatic optimized configuration of the TNC for each commu-

nications mode.

*    All mode Function key "OVER" feature (End key).

*    There are 10 files with fixed names (LAN-LINK.001 through

LAN-LINK.010) which may be viewed and transmitted by means

of function keys. They may also be edited from the Edit

Menu.

*    Set up of TNC for AMSAT-OSCAR Telemetry reception.

*    Time display and event scheduler.

*    ASCII Text Editor.

*    Customizable Colors.

*    Access to the TNC Command Mode is provided in case the user

wishes to override any defaults.

In Packet Radio

*    LAN message store and forward capability.

*    Automatic capture to disk of all packet radio connects.

*    Automatic indication of the number of Packet connects.

*    Capable of automatic connect attempts to download a QTC from

another station in the LAN.

*    Capable of automatic connect attempts to a packet BBS to

download your incoming messages, when your callsign appears

on the BBS mail beacon annunciator.

*    Capable of automatically requesting Bulletins on subjects

that interest you from your local PBBS.

*    Digipeat monitoring and capture.

*    Alert signal to let you know when a predetermined call shows

up in a packet header on frequency.

*    Conference and Bridge modes in multi-connect situations.

*    Path determination to DX station via :QMH:.

*    Indicator that a specific station designated as the "target"

call connected in Packet Mode, or linked to AMTOR Beacon or

Mailbox while you were away.

*    Automatic NET/ROM and KA-Node path set up from directory

file.

*    Selective answering machine and MAILBOX using NC/L command

LAN-LINK.DOC (c) G3ZCZ 1988 - 1994

Appendix 1 Introducing LAN-LINK Page 6

dialogue.

*    Screen indication of connect by desired station (target

call).

*    Automatic Beacon Mode CQ caller.

*    Automatic contest (DX-pedition) mode.

*    Will call CQ repetitively and either work the connect and

keep going after disconnect or signal you when a reply is

received.

*    Zmodem binary file transfer capability

*    Customizable BBS Zap Commands.

*    CTEXT File (LAN-LINK.OUT) which can be sent to all incoming

connects.

*    ANSI Colors for Color "Brag Tapes".

*    SAREX Attack mode lets you digipeat/CQ via the SAREX station

as well as try a connect. Use the CQ feature to digipeat via

MIR, and the connect for the Space Shuttle.

*    Trigger to determine who else is on channel.

*    Single keystroke Packet Cluster connect.

In Pactor

*    Smart Answering Machine.

*    CTEXT File (LAN-LINK.OUT) which can be sent to all incoming

connects.

*    ANSI Colors for Color "Brag Tapes".

*    Automatic logging of connects.

In AMTOR

*    Automatic SELCAL determination.

*    Automatic CQ caller. Will call CQ repetitively and signal

you when a reply is received.

*    LAN message store and forward capability.

*    Selective answering machine and MAILBOX using NC/L command

dialogue.

*    Function key change from monitoring FEC CQs to QSOs in

progress (chirpcopy).

*    Automatic Beacon Mode CQ caller.

*    Automatic contest (DX-pedition) mode.

*    Color changes between incoming and outgoing (echoed as sent)

text.

In MORSE/BAUDOT/ASCII

*    Automatic CQ caller. Will call CQ repetitively and signal

you when a reply is received.

*    RTTY SELCAL.

*    NAVY MARS RTTY file transmitting protocols.

LAN-LINK.DOC (c) G3ZCZ 1988 - 1994

Appendix 1 Introducing LAN-LINK Page 7

Getting Started

MAKE A BACK UP COPY OF THE DISK, BEFORE YOU REMOVE THE WRITE

PROTECT TAB. If you are upgrading from a previous version of LAN-

LINK, the 2.32 configuration files are NOT compatible with

earlier ones. If you are running 2.00, 2.10, 2.20, or 2.20B use the

UPGRADE program to save your current settings. If you have a

previous version, you have to go through the install sequence.

Place the floppy disk in Drive A: Type "A:" (without the "")

followed by the "Enter" key so that you see the A:> prompt. The

floppy disk contains a batch file which will set up the correct

subdirectories on your hard disk. Type "SETUPLL", touch the

"Enter" key, and the batch file will go to work.

The Install program customizes the most often used parameters for

you. Section 18 of Appendix 2 describes each of the parameters.

Read it as you install LAN-LINK.

Install first asks you for the TNC you are using, reply with the

number associated with your TNC. If you don't find your TNC

listed, try the TNC2 option.

The main things you need to change are:

Your Callsign

Your SELCAL           (if you are using a Multi-mode TNC).

PacketCluster Call    the callsign of your local PacketCluster.

BBS Call              the callsign of your local BBS.

If you have not already connected to your local BBS, skip the

next few parameters, and re-run install at some later time when

you know which answers to reply with, or change them from the

LAN-LINK Parameter menu. These parameters are:

BBS Mail Trigger      text in your local PBBS mail beacon.

Read Request          the command you send to the PBBS to read a

message.

Bulletin Read         the command you send to the PBBS to read a

bulletin).

BBS Subject           the PBBS prompt for the subject of a

message you are sending.

BBS Message           the PBBS prompt for the text of a message

you are sending.

The next few parameters set up the serial port between the PC and

the TNC. Read the TNC Manual to see what the manufacturer's

defaults are. These parameters are:-

PC Com Port           A 0 disables serial I/O. Set to the Port

LAN-LINK.DOC (c) G3ZCZ 1988 - 1994

Appendix 1 Introducing LAN-LINK Page 8

number you wish to use.

PC Baud Rate          Enter the value you want to use.

Data Bits             7 or 8. Note: 7 will inhibit binary trans-

fers.

Parity Bit            Enter the correct letter.

Stop bits             Use 1 unless the baud rate is 300 or less.

Read Section 18 of Appendix 2 to learn what the remaining

parameters mean. You can change most of them, once you have

loaded LAN-LINK, by bringing up the Parameters Menu, and

following the prompts.

Bringing LAN-LINK Up the First Time

1    If you are upgrading from 2.00, 2.10 or 2.20, use the

UPGRADE program. Any other earlier version, or if this is

your first time, run the INSTALL program.

2    Turn on the TNC.

3    Load the program by typing LAN-LINK in the way you normally

load a program.

4    WAIT until the program says it has initialized the TNC and

shows the band and power in the status window at the top of

the screen.

5    If there is no battery in your TNC do what the TNC manual

tells you to do to synchronize the TERMINAL baud rate to

1200 baud.

6    After LAN-LINK has loaded, use the Parameters Menu to change

anything you still need to customize. Use the "U" option to

save the updated file. After reading the rest of this

manual, you may want to change other defaults to set up LAN-

LINK the way you like it.

7    Use the "N" and "I" options in the TNC Menu to configure the

TNC. Then select the desired mode of operation from the

displayed menu by pressing the appropriate key.

8    Exit from LAN-LINK by using the Alt-X Key or the Esc Key

followed by the X Key.

9    Reload LAN-LINK the same way you did before. This will load

the edited LAN-LINK.SYS file with the new settings. You are

now ready to use LAN-LINK. If this is your first time, try

the Quick Menu.

10   If you are updating from previous versions the following

applies. The LAN-LINK.SYS configuration files are incompat-

ible with any earlier version.

If you have batteries in the TNC, make sure the TNC is set for a

terminal baud rate of 1200, and try to run the program. If you

see Greek characters or it doesn't work, remove the batteries and

try again. You can then replace the batteries and from then on

the program will be correct. Make sure you leave the TNC on when

you replace the batteries or your initialization will revert to

LAN-LINK.DOC (c) G3ZCZ 1988 - 1994

Appendix 1 Introducing LAN-LINK Page 9

the TNC default settings - the batteries are there to power the

TNC RAM while your primary power is off.

If you have trouble with a PK232MBX, then, with LAN-LINK running,

turn the PK232MBX off, count to ten and turn it on again. Type

one * character. If that fails, repeat this a few times. If it

still fails, open the PK232 and remove the battery jumper. Count

to 10 slowly before replacing it and try again.

Using the Quick Menu

The Quick Menu is designed for new users and provides the most

commonly used functions in a single Menu. You bring up the Quick

Menu by touching the "Escape" key to access the Main Menu and

then selecting the "A" option.

The Quick Menu is described in Section 5.1 of Appendix 2.

Connecting to Other Stations

LAN-LINK treats incoming connects and outgoing connects

differently. To connect to another station, Use the Alt-C hot

key, Function key 5, or bring up the Quick Menu using the "Esc A"

character sequence as described in Section 5.1, or by bringing up

the Call Menu via the "Esc C" character sequence. DO NOT TYPE "C

Call sign" in the command mode. If you do, the connect will still

take place (the TNC won't know if you or LAN-LINK issued the

command) but LAN-LINK will treat it as an incoming connect and

issue the "[ZCZ] LAN-LINK 2.32>" handshaking signal (if the LAN-

LINK flag is in its default, or enabled state). If you connect to

a PBBS or a Node this way, you will get an error message back

from the PBBS or node, since it does not recognize LAN-LINK's

handshake.

When you use the menu, Alt-C or F5 approach you can also make use

of the "path memory" feature in the LAN-LINK.DIR file.

Using Different TNCs

There are differences between the command dialogue and the

features offered by the PK-232, PK-900, DSP 2232, TNC2 KAM, and

MFJ1278. LAN-LINK makes use of many of them. For example, the KAM

shows the HF and VHF packets as if they were separate streams.

LAN-LINK detects them and displays the different headers in

different colors. As this feature is not present in the PK-232,

LAN-LINK does not offer it for PK-232 users. From time to time,

as each manufacturer updates the firmware (EPROM) in the TNC,

LAN-LINK may be updated and new features added.

LAN-LINK is normally loaded by typing LAN-LINK followed by the

LAN-LINK.DOC (c) G3ZCZ 1988 - 1994

Appendix 1 Introducing LAN-LINK Page 10

"Enter" key. In this instance, LAN-LINK gets its configuration

information from the LAN-LINK.SYS file. LAN-LINK.SYS is the

default SYSTEM or configuration file. If you have more than one

TNC you may want to set up different configuration files for each

TNC.

To load LAN-LINK with the optional configuration file, type "LAN-

LINK" followed by the name of the file (without the filetype) on

the command line then press the "Enter" Key. For example, if you

have a PK-232 and a TNC2 and occasionally use the TNC2, set up

the LAN-LINK.SYS file for the PK-232. Then type "LAN-LINK" to run

the program normally. On the rare occasions when you want to use

the TNC2, you may type "LAN-LINK TNC2" to run LAN- LINK with the

TNC2.SYS configuration file. Make sure you configure that

TNC2.SYS file before you put any signals on the air.

LAN-LINK Memory Requirements

LAN-LINK 2.32 requires a minimum of 450k of RAM. You will need

more RAM if you wish to use the Editor. The Editor files will

each take a maximum of 64K additional RAM. If your memory is

marginal, you may get an error when you try to log a station or

edit a file/message. If you do not have enough memory working

from within LAN-LINK, you may use the EDITOR program from DOS.

PC to TNC Communications

All communications between the computer and the TNC normally use

the 8 BIT NO PARITY and 1 stop bit RS-232 Serial Port

configuration. If you want to limit the character set to the 7

bit ASCII set, set the TNC parameters accordingly. The baud rate

defaults to 1200, but you may set it to 2400, 4800 9600 or even

19200 if your CPU clock is fast enough and the TNC will respond

correctly.

LAN-LINK Runs Open Loop

IF YOU USE LAN-LINK, YOU STILL MUST READ THE TNC MANUAL. WARNING

THIS PROGRAM OPERATES IN AN OPEN LOOP MODE. IT REMEMBERS WHAT

INSTRUCTIONS OR COMMANDS IT GAVE THE TNC. IF FOR SOME REASON, THE

TNC IS NOT SYNCHRONIZED TO THE PROGRAM YOU WILL NOT GET CORRECT

DISPLAYS ON THE SCREEN.

In Case of Problems

In case you have problems, always check the default settings

using the "Verify Changes" option of the Parameters Menu. If you

work CW, read about the XMITOK flag, and set it for the way you

have wired your TNC to the radio. Problems can be caused by

incorrect settings of the TNC parameters. If all else fails,

LAN-LINK.DOC (c) G3ZCZ 1988 - 1994

Appendix 1 Introducing LAN-LINK Page 11

(hard) reset the TNC. Make sure XFLOW and FLOW are OFF.

Logbooks

LAN-LINK uses three dBASE 3 compatible logbook files. One is used

for VHF packet logging (VHF.DBF), the second for all HF modes

(HF.DBF) and a third designated as a secondary HF logbook. See

Appendix 2, Section 14 for more information.

How To Get an Update

There are two ways to get an updated version, as follows. When

you register your copy of LAN-LINK, if this version is still

current, your registration will be acknowledged, and the

following upgrade will be sent to you automatically at the time

it is released (in this case, if you move, make sure you notify

me of your new address). If a later version exists at the time

that you register, it will be sent to you when your registration

is acknowledged and you will not be put on the list for an

automatic upgrade.

If you are not on the list for an automatic upgrade (i.e., you

received a disk in the mail as a result of your registration),

then when you hear that a new release is out, do one of the

following:

Send in $5.00 and remind me what size disk to send you.

Download the new version from my land-line BBS or any place else

you find it (You do not need to reregister the upgrade in this

case).

Get it from a friend (You do not need to reregister the upgrade

in this case).

Collect together at least 300 kBytes of traffic on your local

VHF/UHF LAN by running LAN-LINK with the capture-to-disk

activated when you are not connected. If you can add to that,

connect to your local PBBS and dump all current bulletins with

full headers showing date/time into your capture-to-disk file. If

necessary compress the file (ZIP preferred). Send in the disk in

a mailer together with a label addressed to you, and sufficient

return postage, and the upgrade will be yours.

Starting LAN-LINK in a Non Packet Mode

You may start LAN-LINK in a non packet mode, such as Pactor or

Baudot from the DOS command Line. If you wish to do so, the

format of the command is 'LAN-LINK SYSFILE MODE', where SYSFILE

is the name of the LAN-LINK.SYS file or its equivalent, and MODE

LAN-LINK.DOC (c) G3ZCZ 1988 - 1994

Appendix 1 Introducing LAN-LINK Page 12

is the letter used in the Communications Menu to put the TNC in

the particular mode (See Appendix 2 Section 20). For example, to

start LAN-LINK in AMTOR from DOS, type 'LAN-LINK LAN-LINK A'. To

start up in Pactor, type 'LAN-LINK LAN-LINK P'.If you are using a

different configuration file, such as HF.SYS, type 'LAN-LINK HF

A' or 'LAN-LINK HF P'.

Using LAN-LINK with a PacketCluster

Connect to the PacketCluster using the Alt-Y key. When the

connection is achieved, and the PacketCluster sends you a ">"

character at the end of a line of text, LAN-LINK will issue the

first cluster command, followed by the second one, to the Packet-

Cluster and return to manual operation. The default commands are

"SHOW/WWV/1" command followed by a "SHOW/DX". Note: PacketCluster

is a specialized PBBS and conference node by Pavillion Software

for DX chasers.

LAN-LINK also sets an internal PacketCluster Flag. When this flag

is set, LAN-LINK will interpret DX announcements, check your hf

logbook and alert you to the status of that prefix in both sound

and color. If you want LAN-LINK to react to the DX Alerts and

search your HF logbook, to see if you have QSLd or Worked or Need

the prefix, customize the Packet Cluster DX Alert Mode in line

112 of the LAN-LINK.SYS file to your liking. The parts of the DX

Alert message will be shown in different colors.

Options in the PBBS/PacketCluster Menu allow you to customize the

check to set up which bands and which modes are checked. For

example if you never work the 160 Meter band, you may customize

LAN-LINK to ignore 160 Meter DX Announcements.

The results of a logcheck show up at the end of the DX Alert

message. LAN-LINK also changes the current band (see status

window) to the band of the call shown in the DX alert.

In the contest Mode, LAN-LINK checks the logbook for the

complete callsign.

"B" means worked on the same band

"W" means worked on another band

"Q" means that the callsign has been QSLd.

In the non contest Mode, LAN-LINK checks the logbook for the

prefix (2 or 3 letters) of the callsign.

"W" means worked on the same band

"w" means worked on another band and

"Q" means that the prefix has been QSLd on the same band

"q" means the prefix has been QSLd on a different band.

LAN-LINK.DOC (c) G3ZCZ 1988 - 1994

Appendix 1 Introducing LAN-LINK Page 13

In both contest and non contest modes, a check mark means the

logbook was checked and the prefix not found.

You may use the Alt-G key to grab a DX Alert from the incoming or

scrollback windows. LAN-LINK will set the Radio to frequency and

post the call. This is similar to the action of F10.

You can also enable an automatic QSY to the frequency of a DX

Alert via the Radio Menu.

Using ANSI Color for Transmitting Text Color Files

ANSI control codes are designed to allow you to control the

display at the other end of the link. LAN-LINK 2.32 reacts to

ANSI control codes including the color codes. This feature allows

you to send colorful bragtapes and other files using the text

mode graphics of the PC character set when operating packet,

Pactor or G-TOR.

ANSI color codes are a sequence of characters beginning with the

'Escape' Character (represented as "Esc" or "^[") followed by a [

character, one or two numbers follow and the sequence terminates

with the letter "m". Typical examples are 'Esc [37m' and 'Esc

[40m'. To save time, several codes may be combined using the

semi-colon character as in 'Esc [37;40m'.

When transmitting lines of color, the ANSI control codes add

characters to the line, so your line will probably contain more

than 80 characters. Make sure you set the correct TNC parameter

(i.e. SCREENLN) to 255 to ensure that the TNC doesn't add extra

'CR/LF' codes to your picture and mess it up.

LAN-LINK.DOC (c) G3ZCZ 1988 - 1994

Appendix 2 Page 14

Appendix 2 LAN-LINK Features, Menus and Commands

LAN-LINK provides an interface between you and the TNC. It works

in a client-server mode. It provides you with menus, and then

sends the TNC the commands that you would have had to type to

exercise the particular option. LAN-LINK also provides smart

features to smarten up your packet radio operations.

This appendix contains a summary of the commands and menus

available to you in LAN-LINK. The commands are organized by Menu.

Some function keys provide you with short cuts so you don't have

to go through the entire menus. These function keys are described

below.

1.0 LAN-LINK MESSAGE HANDLING FEATURES

This section describes the mail box and message handling features

built into LAN-LINK.

1.1 Answering Machine

LAN-LINK contains an AMTOR (Robot beacon CQ mode only) and PACKET

Mode SMART "answering machine" facility. You can leave messages

on your disk (in the same directory as the LAN-LINK.* files, if

you are using a hard disk), for different stations. When someone

connects to you, if you left a message for him, he (or she or

even it as the case may be) and only that station will receive it

automatically. No one else will normally be able to download that

message.

1.2 Mail Beacon (Annunciator)

To ensure that people know that you have left a message for them

a "MAIL for" list is loaded into your Packet Beacon and

transmitted every 30 minutes (Refer to the BTEXT command in the

TNC manual) as ":QTC:" followed by a list of calls. If no mail is

pending, or the only message in your system is one addressed to

you, the beacon transmissions are inhibited. This conforms to

good operating practice on crowded channels (at least inhibiting

the beacon does).

The mail beacon in the Pactor, G-TOR, and AMTOR Modes is

transmitted as part of the beacon AUTOCQ message. It is

automatically updated when a message is transmitted. If you don't

set the beacon (see below) you will not transmit a mail beacon

text in your AMTOR CQ message.

The Mail beacon text is set up either when you tell the computer

to set it up or when you disconnect a packet QSO from another

station.

LAN-LINK.DOC (c) G3ZCZ 1988 - 1994

Appendix 2 Page 15

In this manner you may update the list while the program is

running, for example, by using Borland's Sidekick or the included

Editor, to write a message, or preferably the NOTE feature.

Any station using LAN-LINK can be configured to automatically

attempt a connect (QTC-Snatch) when seeing their callsign in

someone else's LAN-LINK :QTC: beacon mail message list.

If for some reason the message got lost (e.g. because the link

was marginal), :QSM: will get you a repeat of that message.

1.3 How to Leave a Message

You can use the "note" feature or type the message as a non-

document file on the included Editor, or if you use another word

processor, you must leave the message as an ASCII text file. In

that case, you just name the message file as the callsign of the

station to whom you wish the message to be sent at connect time.

You must also give it the filetype of "OUT".

For example, a message for G3ZCZ would be stored on the disk as a

file named "G3ZCZ.OUT", and similarly a message to 4X6AA would be

stored on disk as "4X6AA.OUT".

Once the message has been sent, an entry to that effect will be

made in the LAN-LINK.RUN data capture file. The message file will

then be renamed and given the file extension ".OLD". At that time

any previously existing mail file with the file extension ".OLD"

will be deleted. This stops the addressee receiving the message

on successive connects if you can't delete it for some reason

(you forget) while at the same time, the message is still

available to you in case you need it.

You should use a separate utility program if you want the calls

in the mail beacon sorted. You may, for example, wish to sort

them in alphanumeric order, or in the order of the date that they

were left in your answering machine.

Incoming messages triggered by the QTC-Snatch will be left in a

message file named as YOURCALLSIGN.OUT, which can be scanned by

using the Alt-M Key combination. If you have non LAN-LINK

friends, train them to trigger the QTC-Snatch by simulating a

beacon, and prefixing and ending their text with lines containing

the ">" as the only character. Better yet, give them a copy of

LAN-LINK and tell them to use it and register it.

2.0 LAN-LINK WINDOWS

LAN-LINK displays information in different windows. The window at

LAN-LINK.DOC (c) G3ZCZ 1988 - 1994

Appendix 2 Page 16

the top of the screen is the status window. The big blank window

in the center is the incoming text window. The lower third of the

screen is the outgoing text window. All outgoing text are echoed

in this window. The bottom line is a prompt window.

The indicators in the status line depend on the state of the LAN

Link and are described in the sections that deal with the

functions they are associated with. Some typical status window

displays are shown below.

2 Meters    2 Watts PACKET 1200 W [R] TRFC ->N4QQ

20 Meters 100 Watts MORSE TEST[4]

20 Meters 100 Watts PACKET 300 N TRFC

CAPTURE TO DISK     PACKET 300 N [C1][Z] TRFC ->N4QQ

If the automatic Capture-to-disk is turned off, a yellow bug will

be flashing on the left side of the line.

If the Printer Flag is set, a yellow [P] will be flashing on the

left side of the line.

If the miscellaneous flag is set, an asterisk "*" followed by two

numbers and a "]" will be displayed. The first number is the PBBS

Sequence State, the second is the Connect Sequence State. These

numbers reflect the internal state of LAN-LINK and should be

quoted when describing problems.

If a message transfer is taking place, the callsign of the

station involved will be displayed.

When the Capture-to-Disk is turned on, the words "CAPTURE TO

DISK" will be displayed. If it is turned off, the operating band

(Meters) and power (Watts) will be shown. If a Zap PBBS is in

operation, the Bulletin Request Count will then be shown in

square brackets ("[]").

The communications mode will be shown next, it may be one of the

following;

PACKET AMTOR AMTOR-(MAIL) AMTOR-FEC

BAUDOT ASCII MORSE NAVTEX

SIGNAL ???? DEFAULT {boot up}

A flashing yellow musical note following the Communications Mode

indicates that the sound has been inhibited.

The next number to be displayed is the RF communications

baudrate. The letter "W" or "N" indicates that the Wide shift or

the Narrow shift has been selected. A "down arrow" will be shown

if the shift is normal. If the shift is reversed a flashing

LAN-LINK.DOC (c) G3ZCZ 1988 - 1994

Appendix 2 Page 17

yellow "up arrow" will be displayed.

In the BAUDOT mode, "USOS" will be displayed if unshift- on-space

is selected.

If the Contest Mode is selected, the word "TEST" will then be

displayed followed by the Contest QSO number in "[]".

In certain Robot and AutoCQ states, namely while the computer is

waiting for a reply, a flashing cumulative CQ count will be

displayed next.

If the RTTY SELCAL is active, the word "SELCAL" will be displayed

next.

If the Navy MARS BAUDOT RTTY protocol is enabled, a "[n]" will be

displayed.

In the packet and AMTOR Robot modes, if at least one station has

connected or linked, a flashing yellow "[C]" will be displayed

next. The number in the square brackets with the "C" is the

number of stations that have connected/linked. If the target call

station was one of those station, a happy face will also be

displayed in the square brackets.

If at least one QTC snatch has taken place a flashing yellow

"[Q]" will be displayed next. The number in the square brackets

is the number of QTC Snatches that have taken place.

In the packet communications mode, if a Zap PBBS, a QTC Snatch or

a PacketCluster Zap is in progress a flashing yellow "[Z]""will

be displayed next. However, if LAN-LINK is set to stay connected

when the sequence ends, the flashing indicator will be a "{Z}"

instead of "[Z]".

In the packet or AMTOR communications mode, a "[R]" will be

displayed next if LAN-LINK is configured for store and forward

(:QSP:).

In the packet and AMTOR communications modes, if the beacon mode

is enabled a "[B]" will be flashing yellow. If someone has

connected and sent a :QRT: sequence, the "[B]" will be flashing.

The "[B]" will be followed by a "[M]" if there are messages

pending. The number in the square brackets with the "[M]" is the

number of messages (*.OUT files) pending.

In the packet communications mode, if the SAREX mode is enabled,

a "[S]" will be shown next. The "[S]" will be flashing while

capture to disk is in progress. If SAREX packets have been copied

a happy face will be displayed in the square brackets together

LAN-LINK.DOC (c) G3ZCZ 1988 - 1994

Appendix 2 Page 18

with the "S". If the SAREX Orbiter robot is active a flashing

">>" will be displayed.

In the packet communications mode, if the digipeat Detect mode is

enabled, a "[D]" will be displayed next.

In the packet communications mode, if connected and ELMER is

active, an "[E]" will be displayed. The number with the "E" shows

ELMER's state.

In the packet communications mode, the terminal sub-modes are

then displayed as follows;

SOLO This lets you only see messages addressed to you.

CQ/B This lets you see beacons and CQ calls as well.

TRFC This lets you see packets containing messages as well.

EVRY This lets you see everything on channel, including the link

control packets.

MAIL This lets you monitor packets from a specific station. In

the PK-232, you can do it without displaying the headers.

Note each mode may not be available in each TNC.

The next item to be displayed is a callsign, under the following

conditions: the call of the station connected with, will be

prefixed by a "*>" when connected, and by a "->" when a connect

request is in progress. If a disconnect is in progress, the

callsign will be replaced by a "$$$$". The callsign will be seen

as flashing when a connect is in progress.

In the MAIL mode, the callsign of the station that you are

"reading the mail" on will be displayed. Otherwise, the callsign

displayed will be that of the one you are connected with.

If an Alert Call has been entered, it will be displayed next.

Similarly, if a Target Call has been designated, the target call

will be displayed next. The display will be slightly different of

the calls have been heard/worked. A "!!>" before the flashing

call indicates that the Alert function is active. A "<->" before

the Alert Call signifies that packets to or from that call have

been heard on frequency. A smiling face in front of the call,

signifies that it has been worked. A "->" is displayed before the

Target Call.

The next items to be displayed are the packet multi user mode

displays. If the conference bridge is up, a flashing yellow

bridge will be shown. If only the conference mode is set, the

letter "C" will be flashing, if the basic multi user mode is set,

the word "M" will be seen. The callsign will be prefixed by the

I/O channel number/letter, and will be in the color allocated to

LAN-LINK.DOC (c) G3ZCZ 1988 - 1994

Appendix 2 Page 19

the traffic on that channel. The number shown after the flashing

letter is the number of streams that LAN-LINK thinks that you are

connected on.

In the non packet modes, the callsign displayed will be that of

the station being worked (entered by the AMTOR robot, the

automatic CQ call recognition, or by the operator using the Alt-C

or Alt-E keys). In AMTOR, the SELCAL of the other station will

also be displayed sometimes, usually if you entered the call.

An "[L]" will be displayed in the non packet modes when a

callsign is entered. The "[L]" will flash to remind you (to log

it) until the QSO is logged.

3.0 HOT KEYS

LAN-LINK contains a number of "Hot" keys to allow you to perform

common operations with a minimum of key strokes. The keys perform

related but slightly different tasks in each communications mode.

A list of those available is shown in Table 3-1. Should you

require any more hot keys, I suggest you use an external program

with a Keyboard Macro capability such as Borland's Superkey.

Consider the action of each key in detail.

3.1 Alt-A Alert Call (Packet) and AMTOR ARQ Call/Talk

In the Packet Communications Mode, this key combination performs

the same task as the Alert Call option in the Call Menu.

In AMTOR, this key combination will attempt to send an ARQ call

to the desired station, transmit the callsign sequence when

linked and allow you to continue typing text (i.e. it will not

transmit the "+?" characters). Use this command option to log on

to an APLINK PBBS.

3.2 Alt-B Send Break/AMTOR Changeover

In the Packet Communications Mode, activating this key

combination sends a "BREAK" signal to the TNC. Use this key

combination to get out of the TNC's transparent mode.

In the AMTOR Communications Mode, use this key combination to

force a changeover. You can type text into the outgoing buffer

and then force a changeover without losing that text.

3.3 Alt-C Call/Connect to Someone

This key combination works in almost the same manner in both the

packet and non packet communications modes as described in the

following sections.

LAN-LINK.DOC (c) G3ZCZ 1988 - 1994

Appendix 2 Page 20

Table 3-1 LAN-LINK Hot Keys

Hot Key                  Communications Mode

_______________________________________________________________

Packet      AMTOR/Pactor/G-TOR   BAUDOT/ASCII    CW

____________________________________________________________________

A    Set ALERT Call      ARQ Call/Talk      ------        ------

____________________________________________________________________

B    Send 'Break to TNC'  Force Changeover  ------        ------

____________________________________________________________________

C    Connect             ARQ Call            Call          Call

____________________________________________________________________

D    Disconnect  Toggle Receive Modes   Receive Mode   Disengage Lock

____________________________________________________________________

E    Enter Call   Enter Call and Selcal    Enter Call Enter Call

____________________________________________________________________

F    Flush TNC Buffer (all communications modes)

____________________________________________________________________

G    Grab Call and frequency (PacketCluster Flag set)

____________________________________________________________________

H    Display function key Information Screen

____________________________________________________________________

J    Jump to DOS (all communications modes)

____________________________________________________________________

K    PacketCluster Spot

____________________________________________________________________

L    Edit Log Entry (all communications modes)

____________________________________________________________________

M    Scan Message (all communications modes)

____________________________________________________________________

N    Next I/O Stream

____________________________________________________________________

O    Turn Robot ON  Turn Robot ON            ------        ------

____________________________________________________________________

P    Turn Printer ON/OFF (all communications modes)

____________________________________________________________________

Q    call QRZ       call QRZ in FEC          call QRZ    call QRZ

____________________________________________________________________

R    Retry Connect  Retry last Call (entered).

____________________________________________________________________

S    Scan Logbook (all communications modes)

____________________________________________________________________

W    :QRA:         ----------           -------         ------

____________________________________________________________________

X    Exit to DOS (all communications modes)

____________________________________________________________________

Y    Zap PacketCluster   ----------------    Transmit RY's -----

____________________________________________________________________

Z    Zap BBS    Drop Link after         Receive mode after

after sending contents of buffer

LAN-LINK.DOC (c) G3ZCZ 1988 - 1994

Appendix 2 Page 21

____________________________________________________________________

3           Set foreground color in outgoing text

____________________________________________________________________

4           Set background color in outgoing text

____________________________________________________________________

3.3.1 Non Packet Modes

You use this key combination to call a station heard on the band.

In AMTOR the link request will be sent in the ARQ Mode to the

SELCAL of the desired station. (Note that this option sends the

"+?" after the callsign sequence).

3.3.2 Packet Mode

You use this key combination to attempt a connect with another

packet station. In a multiconnect situation the LAN-LINK will

auto-matically select a free I/O channel to try the connect.

3.3.2.1 Names, Handles and Paths: If you so desire, you can

create a directory file (default name is LAN-LINK.DIR)

and keep a list of names and calls. The computer will

look up a name and call the station by its callsign. For

example if you put "Joe G3ZCZ" as a line in the file,

when you tell the computer to connect to Joe, it will

try to connect to G3ZCZ. If you enter " 4X4HF 4X4HF V

4X1AA, 4Z4ZB, 4X4IL" on a line in the file, you only

need to type "4X4HF" to set up the correct connect path.

You can also use this feature to automate a NET/ROM

and/or a KA-Node path connection.

3.3.2.2 Loop Backs: In the Packet Communications Mode, if you

want to loop back through someone else you would have to

type "C YOURCALL VIA OTHERCALL" to the TNC. For example

if I wanted to loop back through 4X6AA, I would have to

type the command "C G3ZCZ VIA 4X6AA" to the TNC.

In LAN-LINK you just have to enter the "/" character

followed by the call of the station you want to loop

back through. For example if I wanted to loop back

through 4X6AA, I just need to enter "/4X6AA" as the call

to connect to at the correct prompt, and LAN-LINK will

tell the TNC to try to connect to G3ZCZ via 4X6AA,

namely issue the command "C G3ZCZ VIA 4X6AA" to the TNC.

3.4 Alt-D Disconnect or Receive

This key combination performs the same task as Function Key 8

(F8) EXCEPT in MORSE and AMTOR.

LAN-LINK.DOC (c) G3ZCZ 1988 - 1994

Appendix 2 Page 22

In the Morse communications mode, this key disengages the receive

speed lock in the PK-232 to unlock the PK-232 from the last

signal copied. It lets you lock onto another signal faster.

In AMTOR this key toggles the receive mode between ARQ and FEC

signals (QSO's in progress and CQ calls).

3.5 Alt-E Enter Call

This key combination lets you enter the call of any station

calling you into the program. If you type the Enter Key in

response to the prompt, you can later use Function Key (F3) to

transmit the sequence "DE YOURCALL". You may also use the Retry

feature to call that station.

In AMTOR, you use the feature to enter the callsign and SELCAL of

the station you are listening to and may want to call later.

3.6 Alt-F Flush TNC Buffer

This key combination performs the same task as the Flush TNC

Outgoing buffer option in the TNC Menu.

3.7 Alt-H Help Information for Function Keys

This key combination lets you display a brief reminder of the

action of each hot key. It performs the same function as the "H"

option in the Main Menu.

3.8 Alt-J Jump to DOS

This key combination lets you "Jump to a DOS Shell". It performs

the same function as the "J" option in the Main Menu.

3.9 Alt-K PacketCluster DX Alert

This key combination initiates the PacketCluster DX Alert sequence

described in the BBS Menu.

3.10 Alt-L Log Contact

This key combination appends the callsign for the current QSO to

the Log.

3.11 Alt-M Message Scan

This key combination performs the same task as the Message Scan

choice in the Message Menu.

LAN-LINK.DOC (c) G3ZCZ 1988 - 1994

Appendix 2 Page 23

3.12 Alt-N Next Stream

In the Packet Communications Mode, if you are using the multiple

connect capability of LAN-LINK, this key combination prompts you

to change I/O streams, and display the state of the I/O streams,

showing which ones were last used to send or receive something.

To change I/O streams, move the cursor to the desired stream then

push the "Enter" Key. The calls in the Next Stream window update

when stations connect and disconnect. In the event of the window

not being correct, you may manually enter or delete calls using

the "Insert" Key to enter a call, and the "Delete" Key to remove

a call.

The data about the streams will be shown in the same colors as

the multi-stream text colors.

3.13 Alt-O Turn Robot On

This key combination performs the same function as the Set ROBOT

option in the AMTOR Terminal Menu. The hot key combination

however, is active in both the Packet and AMTOR communications

modes.

This function key also puts the Packet and AMTOR Contest

operation into automatic.

3.14 Alt-P Turn Printer ON/OFF

This key combination performs the same function as the toggle

printer on/off option in the Parameter Menu.

3.15 Alt-Q Call QRZ

This key combination transmits the sequence "QRZ QRZ QRZ DE

YOURCALL" and turns the transceiver back to receive. In AMTOR, it

sends the sequence in FEC and adds the SELCAL.

3.16 Alt-R Retry Call/Connect

This key combination allows you to retry a connect that for some

reason did not go through the first time. It saves wear and tear

on your fingers particularly if the desired connect path is via a

number of digipeat relays or through a number of Nodes.

In the non packet modes, it allows you to retransmit a call to

another station without having to re-enter the call.

If you have previously used the "Enter Call" feature to put a

callsign in the program when you hear that station, you can use

the Retry command to call him later when the QSO in progress

LAN-LINK.DOC (c) G3ZCZ 1988 - 1994

Appendix 2 Page 24

terminates.

3.17 Alt-S Scan Log

This key combination performs the same function as the Scan

option in the LOG Menu.

3.18 Alt-W :QRA:

In the Packet Communications Mode, this key combination performs

the same task as the "W" option in the LAN-LINK menu. It puts the

TNC in the converse mode, sends a ":QRA:" and returns the TNC to

the command mode.

3.19 Alt-X Exit to DOS

This key combination performs the same task as the Exit to Dos

choice in the Main Menu. It exits the program and returns you to

DOS.

3.20 Alt-Y Zap PacketCluster/RTTY Transmit "RY"s

In the Packet Communications Mode, this key combination first

prompts you with the callsign of the PacketCluster. If you enter

a callsign, the entered callsign will become the new Packet-

Cluster call. LAN-LINK then issues a connect request to the

PacketCluster. In the RTTY communications mode, this key

combination causes a sequence of "RY"s to be transmitted.

3.21 Alt-Z Zap PBBS

In the Packet Communications Mode, the Alt-Z key initiates a PBBS

Mail Snatch. You use this key if you connect to a PBBS before you

hear its beacon, or if you have disabled the PBBS Mail Snatch

feature. If you are connected to a PBBS, the key must be used

BEFORE the first ">" is received. The first ">" triggers the

sequence. If you have already received that first ">", send the

PBBS any command, and the sequence should begin when the PBBS

signals that it has completed processing that command. If you are

not connected, LAN-LINK will first try to connect to the PBBS,

then perform the sequence.

When you use this command you will be asked if you want to upload

or download. If you want to download, just push the "Enter" Key.

A Download (Mail from the PBBS to you) will automatically be

followed by an Upload if the *.BBS file exists.

The download default sequence performed by the PBBS Mail Snatch

is as follows:

LAN-LINK.DOC (c) G3ZCZ 1988 - 1994

Appendix 2 Page 25

RM   To read your messages, (Configurable)

L    To let you know what is new on the PBBS,

LM   To list ALL your messages on the PBBS,

B    To log you off the PBBS.

LAN-LINK will wait for the ">" as the last character of the line

of text being received from the PBBS, before issuing the next

command in the sequence. Your messages will be placed in a

message file YOURCALL.OUT as if someone else had left a message

for you.

The "RM" may be customized in the LAN-LINK.SYS file to "RN" or

"VM" depending on the software in your local PBBS, and, on your

preference.

During the time that the PBBS is responding to the "L" command

with a list of new messages, LAN-LINK is scanning the lines

containing the messages to see if it finds one containing a Scan

Word. If it does it will queue a request to download that

message. The line of text will also be displayed in a color that

matches the colors those scan words are shown in the window

generated by exercising option W in the Parameter Menu.

The PBBS Mail Snatch can be initiated in two ways. It can be

initiated when your incoming screen displays a "Mail" beacon

(received from your local PBBS) with your call in the first line

following the header, or by LAN-LINK's Zap PBBS function (this

key combination). The automatic PBBS Zap will not take place if

LAN-LINK is in the multi-user mode.

In the event of a PBBS Mail-Snatch, if the *.BBS file exists for

that PBBS, the file will be processed and the contents sent to

the PBBS. Thus if you don't get a chance to send the mail before

LAN-LINK downloads the mail from the PBBS, it will send it for

you.

When the PBBS has accepted the commands the name of the file is

changed from "*.BBS" to "*.n", where "n" is a number increasing

sequentially each time the function is performed from a base

value set up in the LAN-LINK.SYS file. For example, the file name

KE8X.BBS will be changed to KE8X.001 the first time it happens.

In this way, you will have a copy of the outgoing file on your

disk until you choose to delete it.

In the real world however, message formats are nonstandard, and

can contain any set of characters. The algorithm developed here

transmits the next line in the *.BBS file when LAN-LINK sees a

"trigger" character string unless one of the "inhibitor"

character strings is also present on the line of the message text

received by LAN-LINK.

LAN-LINK.DOC (c) G3ZCZ 1988 - 1994

Appendix 2 Page 26

These "trigger" character strings are ">" (as the last character

of the line of text), and the customizable ones such as "Enter",

"Subject", "Sj:" and "Msg:". The "inhibitor" character strings

are "R:", "<" and "Message" ("R:" as in routing headers and "<"

and "Message" as in Message-Id: <message number> in message

headers).

This should cover most simulated manual PBBS message reading

(WA7MBL and W0RLI/VE3GYQ) and avoid false triggering due to ">"s

showing up in message and routing headers.

When LAN-LINK finishes processing the *.BBS file, it will send a

"B" command to the PBBS to terminate the session. This PBBS Zap

feature is designed to minimize the time you spend connected to

the PBBS by allowing you to scan the message list when automat-

ically receiving your mail, prepare your replies, and download

requests off-line and then later read the incoming messages or

files without tying up the PBBS. For this reason, it thus

disconnects you from the PBBS. You can, however, override the

automatic disconnect command for the duration of the connection

using the "F" option in the Parameter Menu.

3.22 Alt-= Toggle Miscellaneous Flag

This key combination performs the same function as the "+" option

in the parameters menu.

3.23 End "> +?" [Over]

In the Packet Communications Mode this command sends your

callsign followed by the ">" and "Carriage Return, Line Feed"

character combination. You may use this to signal "OVER" in a

packet contact.

In the AMTOR Communications Mode this command sends the "+?"

sequence of characters which is the software "OVER" signal. LAN-

LINK will change colors when it recognizes the '+?' in the

incoming window.

In the Pactor Communications Mode this command sends the "=>"

sequence of characters and switches the link to allow the other

station to send. LAN-LINK will change colors when it recognizes

the '=>' in the incoming window.

In the MORSE and RTTY (BAUDOT and ASCII) communications modes,

this command lets you transmit the "KN" sequence and returns you

to the Receive Mode. LAN-LINK will change colors when it

recognizes the 'KN' in the incoming window. In these modes there

is no standard for signaling 'over'. The 'KN' sequence may show

LAN-LINK.DOC (c) G3ZCZ 1988 - 1994

Appendix 2 Page 27

up in text (i.e. in the word KNOW) or in a callsign (i.e. G9KNA)

and cause a color change. If this "random" color change bothers

you in BAUDOT, change line 114 in the LAN-LINK.SYS file from KN

to some other pair of characters that should not show up in

normal text.

If you have typed something into the "type ahead buffer" and then

press this key, the contents of the buffer will be transmitted

followed by the turnover command.

For example, if you are 4X6AA, and you are working G3ZCZ, then

you would use Function Key 3 (F3) to transmit the sequence "G3ZCZ

de 4X6AA" and this key to send "over" and revert to receive.

3.24 PgUp Scrollback (Scrolls back incoming text)

This key lets you repeat the display of up to 412 lines of

incoming text which has scrolled out of the incoming window. You

may print it or capture it to disk during the repeat.

The bottom line menu displays your options when showing the

scroll back information. Use the "Escape" key to terminate the

Scrollback Mode.

You can save the contents scrollback buffer displayed on the

screen by turning the "Capture-to-Disk" on, and then writing text

to the window. Anything written will be captured. If the Capture-

to-disk is ON before or during scrollback, you will get two

copies of the text in the file.

3.25 The Home Key

The Home Key will clear the window screen that the cursor happens

to be in at the time that the key was depressed, and will then

position the cursor at the top left hand corner of that window.

3.26 The Down Arrow Key

The Down Arrow Key toggles the Normal/Reverse Sideband command in

all communications modes except MORSE. The sideband is depicted

in the status window by the "Up" and "Down" arrows. If the "Down"

Arrow is showing, you are set for Lower Sideband (LSB) which is

the one normally used.

If you set your transceiver to LSB, you will be set correctly for

all modes. If you have reversed the sidebands, a flashing "Up"

Arrow will be displayed. For CW operation you may set your rig to

the CW position and ignore the sideband setting. If you use FSK,

just set it so that it works for you.

LAN-LINK.DOC (c) G3ZCZ 1988 - 1994

Appendix 2 Page 28

3.27 Shift/Baudrate Control Keys

The following keys are used to control the Shift (Wide or narrow)

and the Radio link baudrate in the BAUDOT and ASCII

communications modes. In the BAUDOT and ASCII communications

modes all baud rates supported by the PK-232 may be selected.

3.27.1 RIGHT ARROW: This key speeds up the baud rate. In the CW

communications mode, the right arrow increases the CW

speed by 1 wpm.

3.27.2 LEFT ARROW: This key slows down the baud rate. In the CW

communications mode, the left arrow decreases the CW

speed by 1 wpm.

3.27.3 UP ARROW: This key toggles the shift from Wide shift (850

Hz) to Narrow shift (170 Hz) and back. If you are using

a KAM, it will also pass through the 425 Hz shift

position.

3.27.4 DELETE: This key toggles the USOS command (RTTY only).

The state of each of the above commands are displayed in the

status window at the top of the screen. If USOS is active, "USOS"

will be shown. The shift is shown as "W" or "N" for wide or

narrow respectively.

3.28 Control-Z (^Z)

This key combination is used in the Packet Communication Mode to

signal the end of a file or message, when loading messages into

another LAN-LINK system or into a PBBS. In the AMTOR Communica-

tions Mode, it is automatically translated into the sequence "

:EOF: +? " which can be used to terminate a message being stored

in another LAN-LINK AMTOR mailbox.

3.29 Control T (^T)

This key combination inserts the date-time-text string into the

outgoing buffer. A typical example is "11-Mar-93 01:39".

4.0 FUNCTION KEYS

Function keys are used to perform operations common to all

communications modes, or to allow fast performance of a function

without having to go through several layers of menus. The action

of each of the function keys is described in the following

sections. The function key number corresponds to the paragraph

number, namely 4.1 refers to Function Key 1 (F1), etc.

LAN-LINK.DOC (c) G3ZCZ 1988 - 1994

Appendix 2 Page 29

4.1 Capture to Disk Toggle (F1)

This key toggles the "capture-to-disk" command. When the

"capture-to-disk" is active, all incoming text (including "Echo

as sent" outgoing text) is stored on a disk (floppy or hard) in a

file called "YYMMDD.RUN" where YY is the last two digits of the

year, MM the month and DD the day (E.G. 900824.RUN). If this file

does not exist the first time that LAN-LINK is loaded, it is

created. From that time on, for the rest of that day, successive

activations append new data to the contents of the file.

Each time the command is activated a datestamp (or timetag) will

be written to the file.

When a Packet station connects "capture-to-disk" will be

activated automatically unless you instruct LAN-LINK not to do so

with the Capture-to-disk flag. Packet connects will thus be

recorded in their entirety automatically unless you turn off the

"Capture-to-disk" during the course of the QSO. "Capture-to-disk"

will be stopped at disconnect time.

You may later edit the file at your convenience with your

favorite word processor or the included Editor. However, if your

word processor can only handle files smaller than 64k, make sure

that the "capture-to-disk" file remains smaller. You can do this

by renaming the file to something like "LAN-RUN.001". LAN-LINK

will automatically close the "capture-to-disk" file when you

terminate the program and return to DOS.

You can make Capture-to-disk active when you scroll back. You can

use this feature to save data that has already scrolled by the

window.

4.2 Send Brag Tape and Talk (F2)

This command lets you transmit the Brag Tape (Default LAN-

LINK.TXT). In the non packet modes, it will leave the transmitter

on after sending the file. In the packet mode it will send a ^Z

after sending the file to notify the recipient that the file is

complete.

4.3 Transmit Callsign Sequence (F3)

This command lets you transmit the following sequence OTHERCALL

DE YOURCALL. For example, if you are 4X6AA, and you are working

G3ZCZ, then this command would transmit the sequence "G3ZCZ de

4X6AA". You may use this key at the start and end of a

transmission in any mode except Packet.

LAN-LINK.DOC (c) G3ZCZ 1988 - 1994

Appendix 2 Page 30

4.4 Send Brag Tape (F4)

This command lets you transmit the Brag Tape (Default LAN-

LINK.TXT). In the non packet modes, it will turn the transmitter

off after sending the file.

4.5 Show Packet "MH" list and Point and Shoot Connect (F5)

This command lets you see what calls the TNC has heard recently

on the Packet channels. You can see up to 18 callsigns, with the

date and time they were last logged by the TNC. Refer to the "MH"

command in the TNC manual for the particulars of the display.

The MH list shows up in a special window and freezes until you

depress any key. While the list is frozen, LAN-LINK is not

monitoring received data, so don't freeze it too long.

When you activate F5, LAN-LINK redirects all data coming from the

TNC into the MHwindow. If the MH list is blank, or information

comes in from the TNC between the time you entered the command

and the time the list is sent back, subsequent text from the TNC

will show up in the MH window until the next "Cmd:" appears or 18

lines of text have been received.

You can also use this command in a point and shoot method for

attempting a connect with a station you have heard. Bring up the

MH Window and move the cursor to the desired callsign. When the

cursor is positioned on any character in the call, press the

"Enter" Key and that call will be recognized as the callsign. If

the call is joined to the word "Cmd:" or the cursor is positioned

on a space or blank, the connect attempt will be inhibited.

LAN-LINK has no way of knowing when the MH list is complete.

Consequently, it sends two commands to the TNC to implement this

feature. The "MH" command instructs the TNC to start sending the

MH List, the "ZCZ" command forces an error response from the TNC.

LAN-LINK uses the error response to close the MH Window.

This command does not work on the TNC1.

4.6 Auto CQ Call (F6)

This command will send a CQ sequence, wait a presettable delay

and then repeat the CQ sequence until it receives your call back

from someone else. When it does so, it will alert you with a

Connect Alarm signal, reset the delay timer, and revert to the

normal or listening mode. In AMTOR it will revert to the mode in

which it is waiting for an ARQ call and is able to copy FEC

signals on frequency.

LAN-LINK.DOC (c) G3ZCZ 1988 - 1994

Appendix 2 Page 31

When the Automatic CQ sequence is in operation, an indication

will be shown in the status window. The indicator will be steady

while the transmitter is sending the CQ text, and will blink

while the receiver is listening for a reply.

The time delay between calls is set using the "Q" command in the

Parameters Menu (Set CQ delay time). It is also set as a defined

default by you when you setup your LAN-LINK.SYS file.

The automatic CQ command will only work in the TNC non packet

modes if the "Echo as Sent" parameter is set to "ON" per the LAN-

LINK.SYS file. In other words, you must be able to see the CQ

text slowly echo in the incoming window for the command to work.

This is because the "K K K" (or "+?" in AMTOR) sequence is used

to arm the software to look for a reply.

In AMTOR you will get the connect alarm only when the replying

station transmits the "+?" character combination.

The check for the detection of your callsign and the connect

alarm sequence only occur after a line feed character has been

received.

Under most conditions in AMTOR, many in RTTY, and some in MORSE,

LAN-LINK will recognize the callsign of the station answering

your CQ and display it in the status window. In the event that it

fails completely, the sequence "--?--" will show up in the status

window. Since the callsign recognition algorithm used just looks

for the first word following the "DE space" sequence and assumes

that it has the callsign, be careful to check it when operating

under QRM conditions. You can override the automatic call

selection with the Enter Callsign command in the Call Menu.

The automatic CQ sequence is canceled when an incoming call is

detected (only if the Beacon or Robot-Mailbox Mode is not

selected), if you push the disconnect command key, the "End" key,

F3, or if you change communications modes.

4.7 Activate Type Ahead Buffer/AMTOR Relink (F7)

This key is not available in the Packet Communications Mode. It

also functions somewhat differently in the AMTOR and other

communications modes as described in the following sections.

4.7.1 Activate Type Ahead Buffer

The use of this key sets LAN-LINK to the "Type Ahead" Mode. This

is necessary for the PK-232 only, the KAM and the MFJ1278 have

the feature built in. Any characters then typed at the keyboard

will be placed into the TNC transmit buffer. The contents of the

LAN-LINK.DOC (c) G3ZCZ 1988 - 1994

Appendix 2 Page 32

buffer will be transmitted over the air when the transmit command

is invoked with Function Key 10 [F10].

This feature allows you to "type ahead" as you can enter text

into the transmit buffer while viewing incoming words in the

INPUT window in the screen. Any characters typed from the

keyboard will be echoed in the OUTGOING window on the screen.

4.7.2 AMTOR Relink

If you return to the Command Mode when linked to another station

in AMTOR, use this key to get back to the Converse Mode. If the

link times out, you can try to relink with this key if you have

entered the SELCAL of the other station.

4.8 Disconnect, Disengage or Receive (F8)

This function key returns the station to the receive communi-

cations (TNC command) mode. It may also be used to abort the

automatic CQ sequence. If you use it to abort a CQ sequence while

the transmission is in progress, you may also want to flush the

PK-232 transmit buffer by using the Flush command in the TNC Menu

or the Alt-F hot key combination.

In a multi-connect situation the program will prompt you to tell

it which I/O channel to disconnect.

In AMTOR this command key toggles LAN-LINK between the Chirpcopy

Mode (listening to a QSO in progress) and monitoring FEC

transmissions (CQ calls).

4.9 Put TNC in CMD Mode (F9)

This command puts the TNC into the Command Mode. At this time,

the program is transparent and acts as a dumb terminal (except

for placing incoming and outgoing text in the relevant windows).

You use this Mode for overriding any of the TNC default

conditions.  (See I told you that you had to read the TNC

manual).

4.10 Put TNC in CONVERSE/Transmit/AMTOR FEC (F10)

This command places the TNC into the transmit or Converse Mode.

The transmitter is turned on and the contents of the transmit

buffer are transmitted. In the non packet communications modes,

the outgoing characters will be echoed back from the TNC and

displayed in the INCOMING window on the CRT as they are

transmitted on the air. In the AMTOR communications modes, this

key puts the TNC in the FEC Mode and turns the transmitter on.

LAN-LINK.DOC (c) G3ZCZ 1988 - 1994

Appendix 2 Page 33

4.11 Alternate Function Keys

These keys transmit the files with the fixed names LAN- LINK.001

through LAN-LINK.010 (the suffix number corresponds to the

function key). These files must be located in the same sub-

directory as the LAN-LINK.EXE program. The transmitter is left on

when the file has been sent.

4.11.1 Send/Talk file LAN-LINK.001

4.11.2 Send/Talk file LAN-LINK.002

4.11.3 Send/Talk file LAN-LINK.003

4.11.4 Send/Talk file LAN-LINK.004

4.11.5 Send/Talk file LAN-LINK.005

4.11.6 Send/Talk file LAN-LINK.006

4.11.7 Send/Talk file LAN-LINK.007

4.11.8 Send/Talk file LAN-LINK.008

4.11.9 Send/Talk file LAN-LINK.009

4.11.10 Send/Talk file LAN-LINK.010

If the Mode File Flag is set (see 10.14), the file names will

change according to the modes.

In the non packet communications modes, the transmitter is left

on when the file has been sent. In AMTOR, the changeover

character pair (+?) is not sent.

4.12 Shift Function Keys

These function keys show the contents of the files with the fixed

names LAN-LINK.001 through LAN-LINK.010 (the suffix number

corresponds to the function key). These files must be located in

the same sub-directory as the LAN-LINK.EXE program.

4.12.1 Show contents of file LAN-LINK.001

4.12.2 Show contents of file LAN-LINK.002

4.12.3 Show contents of file LAN-LINK.003

4.12.4 Show contents of file LAN-LINK.004

4.12.5 Show contents of file LAN-LINK.005

4.12.6 Show contents of file LAN-LINK.006

4.12.7 Show contents of file LAN-LINK.007

4.12.8 Show contents of file LAN-LINK.008

4.12.9 Show contents of file LAN-LINK.009

4.12.10 Show contents of file LAN-LINK.010

If the Mode File Flag is set (see 10.14), the file names will

change according to the modes.

4.13 Control Function Keys

These function keys show the first lines of the files with the

LAN-LINK.DOC (c) G3ZCZ 1988 - 1994

Appendix 2 Page 34

fixed names LAN-LINK.001 through LAN-LINK.010 (the suffix number

corresponds to the function key). These files must be located in

the same sub-directory as the LAN-LINK.EXE program.

If the Mode File Flag is set (see 10.14), the file names will

change according to the modes.

5.0 THE MAIN MENU

When you press the "Escape" Key, the Main Menu shown in Figure 5

appears on your screen and provides you with a number of choices

as described in the following sections. Type in the single letter

to make your choice.

Figure 5 Main Menu

A Quick Menu

B PBBS Menu

C Call Menu

D eDit Menu

E Event/Clock Menu

F Files Menu

H Help with Function keys

J Jump to DOS Shell

K Lan-linK Menu

L Log Menu

M Message Menu

N TNC Menu

O AMSAT-OSCAR Menu

P Parameter Menu

Q Q Codes [NC/L]

R Radio Menu

S communicationS Mode Menu

T Terminal Mode Menu

X eXit LAN-LINK

Z Elmer Menu

Different options will show up in each menu depending on the

connect state (in packet) or the type of TNC that you are using.

For example, if you are using a TNC that only supports packet,

the non packet menus will not be seen. The PBBS Menu is only

active when you are connected to another station.

5.1 The Quick Menu

The Quick Menu shown in Figure 5.1 is designed for new users and

provides the most commonly used functions in a single Menu. Type

in the single letter to make your choice. Each option is

described below.

LAN-LINK.DOC (c) G3ZCZ 1988 - 1994

Appendix 2 Page 35

Figure 5.1 The Quick Menu

A Auto CQ

B Send Brag Tape

C Connect to Someone

I Initialize PK232

L Log

M Change Mode

Q Call CQ

S Send File

T Send File/Talk

Z QRZ

1..0 Send/Talk 1..10

5.2 Auto CQ

This option initiates the automatic CQ sequence in which LAN-LINK

sends out a CQ call, listens for a reply, and if none is

received, sends out another CQ call, and so on, ad infinitum. It

is the same as using Function Key 6 (F6).

5.3 Send Brag Tape

This option transmits the Brag Tape is the same as using Function

Key 2 (F2).

5.4 Call Someone

This option is used when you want to call or connect to another

station.

5.5 Log Menu

This option invokes the logbook.

5.6 Change Communications Mode

This option invokes the Communications Mode Menu. You use it to

change TNC communications modes.

5.7 Call CQ

This option is used to initiate a single CQ call.

5.8 Send/Talk File

This option is used to send a file. When the file has been sent,

the transmitter will stay on. This option performs the same

function as Function Key 4 (F4).

LAN-LINK.DOC (c) G3ZCZ 1988 - 1994

Appendix 2 Page 36

5.9 QRZ

This option is the same as using Alt-Q Function Key (Alt-Q).

5.10 Send/Talk LAN-LINK Files 001..010

This option is used to transmit the contents one of the LAN- LINK

buffer files. You select the number, LAN-LINK does the rest. The

option is the same as that provided by the Alternate Function

Keys.

6.0 THE PBBS/PacketCluster Menu

The PBBS menu shown in Figure 6 gives you smart file transfer

capabilities with a PBBS. The menu is only active when you are

connected to another packet station. Type in the single letter to

make your choice.

Figure 6 PBBS Menu

D Download Text File

X Download Binary File (Xmodem)

Y Download Binary File (Ymodem)

C Zap PacketCluster

P Zap PBBS

K Turn PacketCluster Mode OFF

D Send DX Alert

1 Clear All Search Flags

2 Set All Search Flags

3 Set All CW Search Flags

4 Set All SSB Search Flags

5 Set/Clear Individual Search Flags

6 Set/Clear Current Band Search Flags

6.1 Download Text Files

This option lets you download a text file from the PBBS into a

separate text file on your disk. Enter the full PBBS download

command at the prompt line.

To download a file from a W0RLI PBBS such as FILENAME.TYP in the

"A" sub-directory you'd enter "DA FILE- NAME.TYP". To download

the same file from the PACKET sub-directory of a WA7MBL PBBS

you'd enter the command "D \PACKET\FILENAME.TYP".

If you use the "D" command to download a text file, it will end

up in a separate file on your disk ONLY IF a file of the same

name does not exist on your drive. If a file with the selected

name already exists on your disk drive you will get an error

message.

LAN-LINK.DOC (c) G3ZCZ 1988 - 1994

Appendix 2 Page 37

6.2 Download Binary File (Xmodem)

This option lets you download a binary file from the PBBS or

other packet station using the Xmodem protocol into a separate

text file on your disk. You have to configure the Xmodem Receive

Batch file before you use this command.

When you use this command, you first have to tell the PBBS the

name of the file you are going to download, and then bring up

this option to tell LAN-LINK that a binary file is on its way.

LAN-LINK uses an external binary file transfer protocol to do the

actual file.

6.3 Download Binary File (Ymodem)

This option lets you download a binary file from the PBBS or

other packet station using the Ymodem protocol into a separate

text file on your disk. You have to configure the Ymodem Receive

Batch file, before you use this command.

When you use this command, you first have to tell the PBBS the

name of the file you are going to download, and then bring up

this option to tell LAN-LINK that a binary file is on its way.

LAN-LINK uses an external binary file transfer protocol to do the

actual file.

6.4 Zap PacketCluster

This option is the same as the Alt-K function key.

6.5 Zap PBBS

This option is the same as the Alt-Z function key.

6.6 Turn PacketCluster Mode ON/OFF

This option turns the PacketCluster Mode ON and OFF. If the flag

is on LAN-LINK will scan the incoming lines for a DX Alert

message.

You may monitor a PacketCluster in the TRAFFIC mode. When a

DX Alert shows up, it is sent to each connected station in turn.

LAN-LINK will only interpret the first one.

LAN-LINK displays the parts of the DX Alert message in different

colors. The colors correspond to the Multi String Colors, shown

in the color option in the Parameter Menu.

LAN-LINK.DOC (c) G3ZCZ 1988 - 1994

Appendix 2 Page 38

6.7 Send DX Alert

This option is the same as the Alt-K function key. Use it AFTER

you have made the contact.

If you have a valid Radio, LAN-LINK will read the frequency from

the Radio. If not, LAN-LINK will prompt you to enter the

frequency. LAN-LINK will then prompt for the call of the DX

Station, and will assume that you want to send the call of the

one you have just logged.

6.8 Clear All Search Flags

This option clears all the band search flags. LAN-LINK will not

check any bands. These options are provided for fast temporary

changes when working a contest.

6.9 Set All Search Flags

This option sets all the band search flags. LAN-LINK will check

every band.

6.10 Set All CW Search Flags

This option sets all the CW band search flags. LAN-LINK will

check every CW band.

6.11 Set All SSB Search Flags

This option sets all the SSB band search flags. LAN-LINK will

check every SSB band.

6.12 Set/Clear Individual Search Flags

This option prompts you to set or clear each flag for each band

6.13 Set/Clear Current Band Search Flags

This option allows you to set or clear the search flags for the

band you are operating on. Use this option and the "clear all"

option in a contest to change settings when changing bands.

Notes:

1: Use the Update option in the Parameter Menu to save the state

of the individual flags permanently.

2: Set the debug flag on, to verify the prefix, if you are think

LAN-LINK is incorrect in its searches.

LAN-LINK.DOC (c) G3ZCZ 1988 - 1994

Appendix 2 Page 39

7.0 THE CALL MENU

The call menu is used to enter calls, either to call/connect/link

a station or to remember a call or path. Each time you are

prompted for a call, LAN-LINK will remember the previous 15 calls

and you may use the Up and Down arrows to move the cursor to a

remembered call. If you wish to capture a call from the incoming

text window, push Function Key 10 (F10) to enter the incoming

text window and move the cursor to the desired callsign. When the

cursor is positioned on any character in the callsign, push the

"Enter" Key and that word will be recognized as the callsign. Any

characters joined to the call (such as a ".") will be picked up

as part of the call. If the cursor is positioned on a space or

blank, the connect attempt will be inhibited.

In AMTOR, whenever you are asked to enter a callsign, LAN-LINK

will prompt you with its guess at the SELCAL. If you agree with

it, just press the "Enter" Key, if you disagree, enter your

choice over the prompt version.

The Call Menu is communications mode sensitive. It gives you

different choices in each communications mode. In the packet

communications mode it provides the choices shown in Figure 7.1.

In AMTOR, it provides the choices shown in Figure 7.2. Type in

the single letter to make your choice.

7.1 Alert Call

This option toggles the Alert call feature ON and OFF. If it is

off, you are prompted for the call to be monitored, and, if it is

on, it is turned off. The toggle is set when you enter a

callsign, and cleared when you depress the "Enter" Key instead of

a callsign.

Figure 7.1 The Packet Mode Call Menu

A Alert Call

D Show Directory

M ReMember Path

P Packet Cluster

T Target Call

Q Call CQ

U aUto CQ

! Zap PBBS [on Beacon]

LAN-LINK.DOC (c) G3ZCZ 1988 - 1994

Appendix 2 Page 40

Figure 7.2 AMTOR Mode Call Menu

D Show Directory

Q Call CQ

U aUto CQ

X Turn DX ON

The Alert call will be shown flashing in the status window when

in the disconnected state.

The Alert call feature is used in the Packet Communications Mode

when disconnected and the terminal set for "TRFC" or "CQ/BEACON".

LAN-LINK will scan the packet headers received from the TNC, and,

when it sees a packet originated (or digipeated if the MRPT

parameter in the TNC is set to "ON"), by the station whose call

you have entered as the "Alert" call, it will sound an alarm at

the console. The line containing the packet header will also be

displayed in the Alert call color. If the terminal is set for

"CQ/BEACON" the Alert will only sound if the specified station

transmits CQ or BEACON packets.

7.2 ReMember Path

You may use this option to put an entry in the LAN-LINK.DIR file.

7.3 Call CQ

You use this command key to call CQ. In the Packet Communications

Mode, the TNC will transmit a line of text along with the CQ

packet. The line of text is in line 3 of the LAN-LINK.SYS file.

In the AMTOR Communications Mode, your SELCAL will be transmitted

along with the CQ call in the FEC Mode.

7.4 Show Contents of Call Directory

This option displays the contents of the call directory file

(LAN-LINK.DIR). Use this option to verify the contents of the

file.

7.5 Target Call

This option toggles the Target call feature ON and OFF. If it is

off, you are prompted for the callsign to be detected. If it is

on, it is turned off. The toggle is set when you enter a

callsign, and cleared when you press the "Enter" Key instead of a

callsign.

The Target call feature is used in the Packet Communications Mode

when disconnected. LAN-LINK will monitor the stations connecting

to you. When it sees a connect by the station which you have

LAN-LINK.DOC (c) G3ZCZ 1988 - 1994

Appendix 2 Page 41

entered as the "Target" call, it will set the flashing Connect

Count display to show a "happy face". The Target call will be

shown flashing in the status window when in the disconnected

state.

7.6 Auto CQ

This option is the same as Function Key 6 (F6).

7.7 Toggle DX Flag

LAN-LINK allows you to call either "CQ" or "CQ DX". If this flag

is set, all CQ calls will be to "CQ DX". If it is cleared, all CQ

calls will be plain CQ's.

7.8 Zap PBBS on Next Mail Beacon

This option causes LAN-LINK to Zap your local PBBS next time it

beacons with a list of calls for which it has Mail. You set this

option active when you want to upload a set of messages, or are

not sure if there are any messages waiting for you on the PBBS. A

typical situation is where there are a lot of NTS messages

stacked up in the PBBS.BBS file waiting to go. This parameter

will be reset when the connect goes through, so if the Zap

attempt failed, it will try again on the following Mail beacon.

This will not work in the Multi-user mode.

8.0 EDIT MENU

LAN-LINK contains an ASCII text editor suitable for files less

than 64k in size. It is based on Borland's Turbo Pascal Editor

Toolbox and the commands are compatible with Sidekick and

Wordstar. Each time you invoke the editor, LAN-LINK checks that

enough RAM exists to contain the file to be edited (64k per

file). If you do not have enough RAM, LAN-LINK will give you a

"Heap Error" message.

LAN-LINK 2.32 is supplied with an external editor. Run this from

DOS and you will be able to edit larger files. Make sure you put

the EDITOR.EXE file in the LAN-LINK subdirectory.

The Edit Menu is shown in Figure 8-1. Type in the single letter

to make your choice.

A summary of the commands used in the Editor is presented in

Figure 8-2. The ^ sign in front of a character identifies that

character as a "Control" character. To activate it, hold down the

Control key AND the character key. Some of the commands require

two keystrokes. Use ^P to embed a control character in the text.

For example, if you are creating a file containing commands to be

LAN-LINK.DOC (c) G3ZCZ 1988 - 1994

Appendix 2 Page 42

sent to the TNC, to enter a Control-C character into the file,

use the ^P^C sequence of keystrokes.

Figure 8-1 The Edit Menu

A LAN-LINK.OUT

B LAN-LINK.TXT

D LAN-LINK.DIR

E Pick dEmo File to Edit

F Any other File

G Incoming Message

K Answer Capture-to-disk file

L Leave Messages >> N4QQ

M Answer Incoming Mail

N Leave Note

O Pick OUT File to Edit

P Pick Ctdsk File to Edit

R Capture-to-disk file

S LAN-LINK.SYS

T Two Files

Z LAN-LINK.DMO

1,2 - 0 LAN-LINK.001 - 010

Figure 8-2 Summary of Editor Commands

F10 Switch Windows (if more than one window open)

Backspace Delete left char

Del       Delete Char

Enter     New line

Ins       Toggle Insert mode

PgUp      Move cursor up one page

PgDn      Move cursor down one page

^A Move cursor left one word

^C Move cursor down one page

^D Move cursor right one character

^E Move cursor up one line

^F Move cursor right one word

^G Delete one character

^H Delete left one character

^I Tab

^L Find/replace next occurrence

^P Insert a control character into the text

^M New line

^N Insert line

^R Move cursor up one page

^S Move cursor left one character

^T Delete one word after cursor

^V Toggle insert mode

^W Scroll up

^X Move cursor down one line

LAN-LINK.DOC (c) G3ZCZ 1988 - 1994

Appendix 2 Page 43

^Y Delete line at cursor

^Z Scroll down

^K^B Mark beginning of block

^K^C Copy block to position of cursor

^K^D Save file and exit edit

^K^H Hide block marker

^K^K Mark end of block

^K^Q Abandon file and exit edit

^K^R Read file into screen

^K^T Mark single word as block

^K^V Move block to position of cursor

^K^W Write block to disk file

^K^Y Delete block

^K 1..9 Set marker 1 .. 9

^Q^A Find text and replace

^Q^B Move to beginning of block

^Q^C Move to end of file

^Q^D Move to right of line

^Q^E Move to top of window

^Q^F Find text

^Q^I Toggle autoindent mode

^Q^K Move to end of block

^Q^R Move to top of file

^Q^S Move to left of line

^Q^X Move to bottom of window

^Q^Y Delete to end of line

^Q 1..9 Jump to marker 1..9

The following options can be set in the Find/Find and Replace

(^Qf and ^Qa) operations.

# locates #th occurrence

G global replace

N replace without Y/N question

U ignore upper case/lower case

W match whole words only

While you are using the editor, LAN-LINK can't receive and

process characters from the TNC. All other features are inhibited

or locked out. Thus if someone connects to you, the TNC will know

but LAN-LINK will not. Be warned!!!

The Edit menu allows you to call up the various files for editing

as follows.

8.1 Edit Brag Tape

This option allows you to create and edit the Brag Tape File

(LAN-LINK.TXT).

LAN-LINK.DOC (c) G3ZCZ 1988 - 1994

Appendix 2 Page 44

8.2 Edit Call Directory File

This option allows you to create and edit the Call Directory File

(LAN-LINK.DIR).

8.3 Edit Any Other File

This option allows you to create and edit any other file.

8.4 Answer Capture-to-disk (Ctdsk) File (Split Screen)

This option allows you to edit the capture-to-disk file and reply

to incoming bulletins/messages in a split screen mode. The

contents of the incoming bulletins and messages will be displayed

in the top window, and the reply file in the lower window.

The reply file is assumed to be named according to the Zap

convention (*.BBS file). You will have to set up the replies/-

outgoing messages as if you were typing them by hand.

You put "SP Call @ PBBS" in the first line, then the "Subject of

the message" in the next. Enter the message, and end it with

":EOF:" , "/ex" or "/EX" as the first word on a separate line.

For example, a message to me could take the form shown below.

SP G3ZCZ@N4QQ

Test

This is a test

73 your name

/ex

8.5 Answer Incoming Messages (Split Screen)

This option allows you to answer incoming messages in a split

screen mode. The contents of the incoming messages will be

displayed in the top window, and the reply file in the lower

window. The reply file is assumed to be named according to the

Zap convention (LOCAL-BBS.BBS file). Reply messages are set up as

described in Section 8.4.

Use Function key 10 (F10) to switch windows. At this time you

cannot transfer text between windows.

8.6 Leave Note for Someone

In all modes you can leave a brief note (message) for another

station. When you invoke this option, you will be prompted for

the call of the station that you want to leave a note for. As you

enter the text it is stored on the disk in the usual format.

LAN-LINK.DOC (c) G3ZCZ 1988 - 1994

Appendix 2 Page 45

After you have left the note, if you are in the Packet Communi-

cations Mode, press Function Key 8 (F8) or Alt-D to load the

callsign of the recipient into the beacon.

When you leave a note, leave it for the root call of the

connectee, for example to leave a note for W3/G3ZCZ or G3ZCZ/W3

leave it for G3ZCZ, similarly to leave a note for WB8WKA-1 leave

it for WB8WKA. LAN-LINK is smart enough to extract the root call

from any callsign it links/connects with.

8.7 Edit Capture-to-Disk file

This option allows you to edit the Capture-to-Disk file as long

as it is smaller than 64k.

8.8 Edit Two Files (Split Screen)

This option allows you to create and edit any two files. Use

Function Key 10 (F10) to switch windows. At this time you cannot

transfer text between windows.

8.9 Edit Configuration file

This option allows you to edit the Configuration File (LAN-

LINK.SYS).

8.10 Edit LAN-LINK.001 through LAN-LINK.010 Files

This option allows you to create and edit the 10 files (LAN-

LINK.001 to LAN-LINK.010). Use the suffix number to access the

respective LAN-LINK.* file, use the number 0 to access the LAN-

LINK.010 file. If the Mode File Flag is set (see 10.14), the file

names will change according to the modes.

8.11 Pick Capture-to-disk File to Edit

This option presents you with a directory window containing a

list of capture-to-disk files (YYMMDD.RUN). Move the cursor down

to the desired file and touch the "Enter" key. If you don't want

to edit any of them, then move the cursor down to a blank line.

If you have more files than one screen full, use the Pgdn (Page

Down) key to show the next screen full.

8.12 Pick Message File to Edit

This option presents you with a directory window containing a

list of message files (*.OUT). Move the cursor down to the

desired message file and touch the "Enter" key. If you don't want

to edit any of them, then move the cursor down to a blank line.

If you have more files than one screen full, use the Pgdn (Page

LAN-LINK.DOC (c) G3ZCZ 1988 - 1994

Appendix 2 Page 46

Down) key to show the next screen full.

8.13 Leave Messages on your local PBBS

This option allows you to edit your LOCAL-BBS.BBS file in a

single window.

8.14 Edit Incoming Message

This option allows you to edit your incoming message file in a

single window.

8.15 Edit LAN-LINK.OUT File

This option allows you to edit the LAN-LINK.OUT file.

8.16 Edit LAN-LINK.DMO File

This option allows you to edit the Demo or Test file. This is a

file you can read into LAN-LINK and LAN-LINK treats it as if the

data came from the TNC. Use this option to test features and

custom settings.

8.17 Pick dEmo File to Edit

This option allows you to pick a demo file to edit.

9.0 EVENT/CLOCK MENU

LAN-LINK contains some time dependent features which allow you to

schedule events. These features are controlled from this

Event/Clock Menu shown in Figure 9. Type in the single letter to

make your choice.

Figure 9 Event/Clock Menu

A Set Alarm

C Connect ?

D Display Event

P Periodic Connect ?

R Turn Robot ON/OFF

S Turn ConteSt ON/OFF

T Turn Time Display ON/OFF

X Cancel Event

Z Zap PBBS

To set a scheduled event, first select the event, then enter the

time. You may set the event to occur once a day, or once an hour.

If you wish the event to occur once an hour, enter an hour value

greater than 23 when asked to enter the hour. Each time the event

LAN-LINK.DOC (c) G3ZCZ 1988 - 1994

Appendix 2 Page 47

occurs, the feature will be cleared.

9.1 Set Alarm

This option allows you to set an alarm clock.

9.2 Connect

This option allows you to set LAN-LINK to issue a call or connect

request at some later time.

9.3 Display Event

This option allows you to display the scheduled event.

9.4 Periodic Event

This option allows you to set a periodic connect request or call

event. You can set the start time, the end time and the time

interval between the connect/call attempts. The first connect

that goes through will clear the event scheduler.

9.5 Turn Robot ON/OFF

This option allows you to turn the AMTOR/Packet Robot on or off

at a fixed time.

9.6 Turn Contest ON/OFF

This option allows you to turn the AMTOR/Packet Automatic

Contester on or off at a fixed time. You will be prompted for the

time, a starting QSO number (in case you a restarting the

contest) and the Contest Text (in case you wish to change it).

9.7 Turn Time Display ON/OFF

This option allows you to turn the clock display on or off.

9.8 Cancel Event

This option allows you to cancel a scheduled event.

9.9 Zap PBBS

This option allows you to schedule a PBBS Zap at a predetermined

time (like in the middle of the night). You may for example set

it to attempt to connect to a PBBS at a fixed time or hourly at

any desired minute after the hour. Should the connect be made,

the event scheduler will be cleared and no further connect

attempts will be made.

LAN-LINK.DOC (c) G3ZCZ 1988 - 1994

Appendix 2 Page 48

10.0 FILES MENU

The Files Menu is shown in Figure 10. Type in the single letter

to make your choice.

Figure 10 The Files Menu

B Send Brag Tape

C View Color File

D File Directory

E Erase File

F priNt File

M Turn Mode Files ON/OFF

N priNt Ctdsk File

P Pick Ctdsk File to View

R Rename File

S Send File

T Send/Talk File

U send Uppercase File

V View File

Y View Capture-to-disk File

X Turn eXpand Files OFF

Z View Messages

10.1 Send Brag Tape

This command allows you to transmit a standard text file

historically known as a "Brag Tape". The default name of the brag

tape file is LAN-LINK.TXT, and it should be created with a word

processor in the non-document mode, or with LAN-LINK's editor.

During the file transfer, the outgoing text will appear in the

Outwindow. If the text is long enough and fills the TNC buffer,

you will see it stop, and then continue when the TNC is ready to

accept more text. LAN-LINK uses hardware flow control not

software (^S/^Q).

When you transmit a file you are returned to the receive mode

after the file is sent except in the Packet Communications Mode.

In the Packet Communications mode, when the file has been loaded

into the TNC the computer will add the ^Z character to the

outgoing text to signal to the recipient that the file transmis-

sion is over.

The computer will signal you (using the bell character) when the

file has been loaded into the TNC. That is not the same as the

time that the TNC has transmitted the file, because the TNC

contains a character buffer. LAN-LINK dumps the file into the TNC

at the baud rate used between the PC and the TNC. The TNC, then

LAN-LINK.DOC (c) G3ZCZ 1988 - 1994

Appendix 2 Page 49

transmits the file at the radio link data rate. Flow control is

used to regulate the passage of data between the various buffers

in the link.

10.2 File Directory

This command shows you the directory of the disk drive.

10.3 Erase File

This command allows you to erase or delete a file.

10.4 Pick Capture-to-disk (Ctdsk) file to View

This command allows you to view a *.RUN file. When you exercise

this command you will be presented with a list of capture-to-disk

files (YYMMDD.RUN). Move the cursor down to the one you want to

view and touch the "Enter" key to select it. Use this option, if

you must, on long files. On files containing less than 64k

characters, use the Edit Menu.

10.5 Rename File

This command allows you to rename a file. You could for example

rename a document file to a "CALLSIGN.OUT" file so that someone

will receive it automatically next time they connect.

10.6 Send File

This option lets you transmit an ASCII file. In the non packet

modes, it will turn the transmitter off after sending the file.

In the packet mode it will send a ^Z character when the file is

complete.

10.7 Send/Talk File

This option lets you transmit an ASCII file. In the non packet

modes, it will leave the transmitter on after sending the file.

10.8 Send Uppercase File

This option lets you transmit an ASCII file and converts each

line of text in the file to UPPERCASE before transmitting it. The

file itself is not changed. In the non packet modes, it will turn

the transmitter off after sending the file.

10.9 View File

This option lets you view the contents of a file. Use it if you

must on long files. On files containing less than 64k characters,

LAN-LINK.DOC (c) G3ZCZ 1988 - 1994

Appendix 2 Page 50

use the Edit Menu.

10.10 View Capture-to-disk (Ctdsk) File

This option lets you scroll forward through, or view, the

contents of the capture-to-disk file (YYMMDD.RUN) which you

specify by name. Use it if you must, on long files. On files

containing less than 64k characters, use the Edit Menu.

10.11 View Incoming Message File

This option lets you view the contents the Incoming message file

(yourcall.OUT). Use it if you must, on long files. On files

containing less than 64k characters, use the Edit Menu.

10.12 Print File

This option lets you print the contents of a text file.

10.13 View Color File

This option allows you to view an ANSI color file in glorious

color. If you use the regular edit or file view options, you will

see the ANSI color control characters inside the file. Use this

option to check on your color editing.

10.14 Turn Mode Files ON/OFF

This option lets you change the state of the Mode File Flag. If

this flag is set, LAN-LINK will change the name of the LAN-

LINK.001 to LAN-LINK.010 when you change modes. The name will

change to the mode followed by a number, i.e. BAUDOT.001 to

BAUDOT.002, or AMTOR.001 to AMTOR.002. This feature allows you to

use different files in different modes and still use the same Alt

key combination to transmit them. Thus, you can if you wish, set

up a Pactor file with all 128 characters, and an AMTOR file with

the AMTOR character set as MODE.002, (where MODE is the

communications mode) and transmit each using Alt-F2. If the Mode

File Flag is not set, then the file names never change.

10.15 Turn eXpand Files ON/OFF

This option lets you change the state of the eXpand File Flag. If

this flag is set, LAN-LINK will change the macro string beginning

with the # sign, to the equivalent text.

11.0 HELP WITH FUNCTION KEYS

This Main menu option will activate the HELP display which

presents you with a (very) brief reminder summary of the function

LAN-LINK.DOC (c) G3ZCZ 1988 - 1994

Appendix 2 Page 51

and hot keys. This window will remain on display until a

character is typed at the keyboard.

12.0 JUMP TO DOS SHELL

This Main menu option allows you to enter a DOS shell. If you do

not restore the system to the directory which contains LAN-LINK

before you EXIT from the shell, LAN-LINK will do so for you.

You may use the DOS shell while in QSO when you have loaded the

computer/TNC output buffers and output will continue to flow.

To exit from the DOS shell back to LAN-LINK, enter the command

"EXIT". LAN-LINK will automatically return you to the correct

sub-directory.

13.0 LAN-LINK MENU

The LAN-LINK menu provides different options depending on whether

you are connected to another station or not. Figure 13.1 shows

options that are available when connected and the PAcketCluster

flag is not set. Figure 13.2 shows the menu when not connected.

In each case, type in the single letter to make your choice.

Each of the features (except the :QYU:) may also be performed

manually by typing the full sequence of Network Control Language

character strings.

13.1 :QBU: External Protocol Command

This is a Sequence for external operations. It requires that the

batch file "QBU-TX.BAT" be present in the transmitting

(initiating) station, and the batch file "QBU-RX.BAT" be present

in the receiving station. When the QBU sequence is executed the

following occurs:-

Figure 13.1 LAN-LINK Menu When Connected

9 Demo Mode

B :QBU: Protocol

D :QZD: Binary File

G :QIC: Page Operator

H :QMH: Activity

M :QBM: ASCII File

P :QSP: Message

Q :QDB: ASCII File

R :QTR: Ask for Time

S :QSM: Message

T :QTA: Message

U :QRU: Messages

Z :QZU: Binary File

LAN-LINK.DOC (c) G3ZCZ 1988 - 1994

Appendix 2 Page 52

Figure 13.2 LAN-LINK Menu When Not Connected

9 Demo Mode

W Who is "ON-LINE"

The transmitting station sends a :QBU: The receiving station

sends a :QRV: and the amount of memory left on disk, enters the

transparent mode, jumps to dos and executes the batch file "QBU-

RX.BAT".

Upon receipt of the :QRV:, the transmitting station then puts the

TNC in the transparent mode and jumps to Dos and executes the

batch file "QBU-TX.BAT". The batch file should exit and return to

LAN-LINK. LAN-LINK then exits the transparent mode.

When the receiving station batch file finishes and returns to

LAN-LINK. LAN-LINK then exits the transparent mode and transmits

a :QSL: if the DOS Error level was 0 and a :QNO: error if it

wasn't. At this time both stations are back in the converse mode.

You may use this technique for transmitting pure binary files

using external (to LAN-LINK) protocols). You may change the

default names of the batch files in the LAN- LINK.SYS

configuration file.

13.2 :QBM: ASCII File

You use this option to download an ASCII file from another LAN-

LINK station. It is the same as manually typing ":QBM: FILENAME".

13.3 :QDB: ASCII File

You use this option to upload an ASCII file to another LAN- LINK

station. It is the same as manually typing ":QDB: FILENAME".

Using this option saves you typing the file name twice.

13.4 :QIC: Page Operator

This command allows you to page the operator at the other LAN-

LINK 2.1+ station. The sequence is:- you send :QIC: to another

LAN-LINK 2.1 station. Use the LAN-LINK Menu or type it by hand.

Upon receipt, LAN-LINK begins to page the operator if the sound

flag is on. At that time it sends you a :QRX: followed by a

number, i.e. ':QRX: 30'. The number is the number of seconds it

will page the operator. At the end of the paging sequence it will

send the prompt i.e. 'G3ZCZ>'. If the sound is inhibited it will

send ':QRX: QRT' to signal that it is in a silent configuration.

LAN-LINK.DOC (c) G3ZCZ 1988 - 1994

Appendix 2 Page 53

13.5 :QMH: Monitor Heard

This command causes your copy of LAN-LINK to issue the :QMH:

command. When the other station's copy of LAN-LINK receives a

:QMH: command, it drops back to the TNC Command Mode and issues a

"MH" command to the TNC (except to a TNC1 which does not support

it). It stores the list received in a temporary buffer, and then

returns to the Converse Mode and transmits the list back to you.

13.6 :QRA: Who is "ON-LINE"

You use this option (when unconnected and monitoring the traffic

on a channel) to see who is "ON-LINE" on the LAN. It is the same

as going manually into the Converse Mode and transmitting an

unconnected packet containing the character string :QRA:. You may

also use the Alt-W function key to transmit the same sequence.

13.7 :QRU: Messages

The NC/L QRU command allows you to upload a number of messages to

the other station. You use it when you have messages pending on

your system and want to pass them to a third computer for storage

in the LAN, while you take your computer off-line.

You use this option to begin the upload sequence to transfer

messages for a number of stations to another LAN-LINK station. It

is the same as manually typing :QRU:.

You can only use this option with a station which you have

previously designated as a store and forward mail box, for

specific stations. You must also have set up a CALLSIGN.MBX file

for that station.

When the upload sequence is over, the computer sending the

messages will transmit the NC/L command word :QJG: to signal that

the sequence is complete. Should you receive it before you get

any messages, then there weren't any pending.

If you want to upload messages to another LAN-LINK system,

connect to it and type the sequence :QRU: followed by the "Enter"

character. That system will then respond to you as if the

operator there had initiated a QRU command.

13.8 :QSM: Message

You use this option to request a repeat of a message. It is the

same as manually typing ":QSM:".

LAN-LINK.DOC (c) G3ZCZ 1988 - 1994

Appendix 2 Page 54

13.9 :QSP: Message

You use this option to store a message on another LAN-LINK

station. It is the same as manually typing ":QSP: CALLSIGN".

LAN-LINK contains store and forward routines for messages in a

Local Area Network (LAN).

When connected to another LAN-LINK station, you may use the NC/L

:QSP: command to leave a message at that station. This QSP

command automates that feature for you. It lets you just enter

the callsign of the station you wish to leave a message for. It

will then send the text sequence :QSP: <callsign>. The other LAN-

LINK HOST station will either reply :QNO: followed by a number or

:QRV: <callsign>. The "QNO" message means that the other computer

cannot accept the message, the "QRV" message means that it is

waiting for you to send it.

If you get the :QRV: reply, go ahead and type the message, or

send the text file containing the message. If you type the

message, use a Control-Z (^Z) character and carriage return to

terminate the message. (In AMTOR, LAN-LINK will translate the ^Z

to :EOF: +? to terminate the message). You will then receive

either a :QSL: from the other station meaning that the message is

accepted, or a :QNO: followed by a message that something went

wrong and the message was not stored.

13.10 :QTA: Messages

If you have a bad path and your messages are not automatically

deleted by the other station after they have been sent to you,

you use this option to delete your message. It is the same as

manually typing ":QTA:".

13.11 :QTR: Ask for Time

This command allows you to get the date-time-text string from the

other LAN-LINK 2.1+ station. Don't confuse it with :QRT:.

13.12 :QZD: Binary Files

This command uses the external batch files to download a Binary

file from another LAN-LINK Station using the Zmodem Protocol. If

you attempt to download a file which exists in your computer,

LAN-LINK will use ZModem's "restore after interrupt" capability

to pick up the transfer at the point when (it assumes) the

transfer was interrupted. You must configure all four QZ*.BAT

files before trying to :QZD: as described in Section 24 (Install

takes care of the configuration in most instances).

LAN-LINK.DOC (c) G3ZCZ 1988 - 1994

Appendix 2 Page 55

13.13 :QZU: Binary Files

This command uses the external batch files to Upload a Binary

file to another LAN-LINK Station, making use of the Zmodem

Protocol. LAN-LINK will not let you upload a file to the other

system, if a file with that name already exists on the system.

This stops you overwriting existing files. If the link drops out

while you are uplinking a file, and you wish to try again later,

rename the file you are sending, then reconnect and try again.

Don't forget to :QSP: the other station and tell the operator

what happened. You must configure all four QZ*.BAT files before

trying to :QZU: as described in Section 24.

13.14 Demo Mode

Use this option to enter the demo mode, and simulate incoming

lines of text from a disk file.

13.5 Set Single/Dual PK-900/DSP 2232 In Windows

This option in only active for the PK-900 and DSP 2232. It allows

you to separate the incoming text from the two radio ports into

two windows. When operating hf/vhf the hf window is usually the

middle incoming window.

14.0 LOG MENU

LAN-LINK allows you to have up to 3 logbooks on-line. One is

designated as the HF logbook, the second is the VHF logbook, and

the last one is known as the Secondary Logbook. They can be

different or the same.

LAN-LINK performs automatic log book entries for Packet, Pactor

and AMTOR Beacon mode Connects, and semi-automatic log book

entries for other modes. The Log files are in dBASE 3 format and

can be processed by the DBASE Log book Package in PC-HAM for

indexed listings, tracking of DXCC and other AWARDS, etc. This is

ideal for special event stations and DX-peditions, for the whole

QSL process and further statistical analyses of their operation.

The structure of the logbook files are as shown in Table 14.

The size of the COMMENTS field is a compromise. Each field is a

fixed size so that each log entry takes up a minimum of 62

characters worth of space on the disk even if all the character

space is unused. You may change the size of the field if you

wish, but to do that you will have to use dBASE. Note also, that

if you do make the comments field longer, the display will be

screwed up.

For the logging feature to work, the logbook files must be

LAN-LINK.DOC (c) G3ZCZ 1988 - 1994

Appendix 2 Page 56

present on the disk.

Table 14 Structure of the Logbook Database File

Field     Field Name       Type         Length

1              DATE      Character      8

2              TIME      Character      4

3              BAND      Character      3

4              CALL      Character      10

5              RX        Character      3

6              TX        Character      3

7              MODE      Character      4

8              POWER     Character      4

9              QSLSENT   Character      1

10             QSLRX     Character      1

11             COMMENTS  Character      20

When an AMTOR beacon mode, G-TOR, Pactor, or packet connect

occurs, the connect is automatically logged (if the automatic

logging feature is enabled). In the Packet Mode, the connect path

is placed in the comments section of the logbook entry. In the

non packet modes, the log data entry is set up for you with

default 599 values for the reports exchanged.

When you bring up the Logbook you will be shown the last screen

or page of log entries. The last entry will be highlighted. At

this time a number of function keys can be used, or you may

depress the "Escape" key to bring up the Main Log Menu. When you

bring up the Main Log Menu, it provides the choices shown in

Figure 14.1. Type in the single letter to make your choice.

14.1 Append From Another LAN-LINK Log

This option lets you append entries from another LAN-LINK

logbook. Use it after a contest, to merge the contest log into

the main logbook.

14.2 Scan Log by Band

This option lets you scan the logbook file and display QSOs made

on a particular band.

14.3 Change Logbook File

This option lets you temporarily change the logbook file. If you

want to change it permanently, change it in the LAN- LINK.SYS

file. When you enter the new name, LAN-LINK will assume that it

is a filetype DBF. For example, if you have different logbooks

for different operations on-line, such as 4X.DBF or W8.DBF, you

only need answer "4X" in response to the question, to change to

LAN-LINK.DOC (c) G3ZCZ 1988 - 1994

Appendix 2 Page 57

the 4X.DBF logbook. In accordance with the dBASE convention, you

cannot use any filetype other than DBF.

Figure 14.1 The Log Menu

A Append from Another LAN-LINK Log

B Scan Log by Band

C Change Log

D Delete Logbook

F Find date

G Purge Log

H Help Function Keys

I Import From G3ZCZ's Contest Log

J Jump to an Entry

K PicK Log

L pack Logbook

M Scan Log by Mode

N Create New Blank Log

P Print Log

R Re-index Logbook

T Sort on Date + Time

U Turn Update Disk OFF

Z fuZzy scan

2 Turn Secondary Search OFF

14.4 Delete Logbook File

This option lets you delete both the logbook (.DBF) and its

associated index file (.NDX).

14.5 Find Date

This option lets you locate and move the highlight to entries for

a particular date. If no entries are found for the desired date,

the first entry after that date will be highlighted. You can also

scan for parts of a date, such as "90/01", for the first entry in

January 1990.

14.6 Purge Log

This command allows you to purge one callsign from the log. Use

it to delete the callsign of your local PBBS if the logbook file

gets too big.

14.7 Help Function Keys

This option brings up a menu listing the log mode function keys.

14.8 Import From G3ZCZ's Contest Log

This option lets you import logs from earlier releases of LAN-

LAN-LINK.DOC (c) G3ZCZ 1988 - 1994

Appendix 2 Page 58

LINK (up to and including Version 1.56). It can also be used to

import a log generated by the Contest Program in PC-HAM.

14.9 Jump to an Entry

This option lets you jump to an entry by its record number. This

is the number displayed in the first column.

14.10 Pick a Log file

This command allows you to pick a *.DBF file. When you exercise

this command you will be presented with a list of log files

(*.DBF). Move the cursor down to the one you want to use and

touch the "Enter" key to select it.

14.11 Pack a Logbook

This function key allows you to pack the logbook files. All

entries marked for deletion will really be deleted at this time.

14.12 Scan Log by Mode

This option lets you scan the logbook file and display QSOs made

using a particular mode.

14.13 Create a New Logbook

This command allows you to create a blank logbook (*.DBF) file.

14.14 Print Log

This command prints the contents of the logbook file in a

formatted manner to the printer.

14.15 Re-index a Logbook

This command allows you to index logbooks that you have ported

over from other DBASE formats, or to re-index logbooks that have

had their index files damaged in some manner.

14.16 Sort on Date + Time

This command allows to you create a new logbook from the current

one with the entries is sequential order sorted on date and time.

You use this option after entering old QSOs manually. Make sure

there is space on the disk for a temporary logbook before evoking

this operation.

LAN-LINK.DOC (c) G3ZCZ 1988 - 1994

Appendix 2 Page 59

14.17 Turn Update Disk ON/OFF

If this parameter is set ON, LAN-LINK will close the logbook and

re-open it after every entry. If the parameter is set OFF, LAN-

LINK saves a few entries in memory and only writes them to disk

when the buffer fills, or the logbook is closed. Setting the

parameter guarantees you will not lose any entries if the power

fails, at the expense of a slightly slower logging operation.

14.18 Fuzzy Scan

This option allows you to scan the callsign or the comment field

(your choice) for a character string. For example you could scan

the comments field for all entries with the word "Joe". You could

scan the callsigns for all calls with the suffix of "ZCZ".

14.19 Turn Secondary Search ON/OFF

This option allows you to turn the secondary search mode on and

off. If it is on, and if the secondary logbook exists, every time

you command a search of the logbook to find a prefix or callsign,

LAN-LINK will also search the secondary log. Use this feature if

you have more than one log, as a result of operating from more

than one location or for any other reason.

14.20 Log Menu Function Keys

The Function Keys shown in Figure 14.2 are active in the Log

book. The operation of each function key is described below.

Figure 14.2 Function Keys Active in Log book

A  Append Entry

E  Edit Log Entry

H  Scan for Log Entry

S  Scan for Call

U  Toggle Delete Mark

X  eXit Log Menu

Ins  Toggle Insert Mode

End  Show Last Page

Home Show First Page

PgUp Page Up

PgDn Page Down

Up   One Entry

Down One Entry

14.20.1 Alt-A Append Entry

This function key allows you to manually append an entry to the

log. You can also use this command to enter the odd SSB contact

LAN-LINK.DOC (c) G3ZCZ 1988 - 1994

Appendix 2 Page 60

into the logbook file. If you do, use the characters "SSB" or

"FM" for those voice modes to remain compatible with the rest of

the DBASE logbook package.

When appending or editing an entry, LAN-LINK will automatically

enter the date, time, callsign of the other station, band, mode

and QSL information in upper case.

The comments field in the LAN-LINK logbook file is restricted to

a maximum of 20 characters. If you try and enter more than 20,

the surplus will be ignored.

14.20.2 Alt-E Edit Log Entry

This function key allows you to edit the highlighted entry. Move

the cursor with the arrow keys, and end the edit process by

depressing the "Escape" key.

You may use the "QSL" entry to tag the fact that you have written

out or received a QSL card.

14.20.3 Alt-H Scan Log by Call

This function key allows you to scan the logbook by callsign

prefix for the callsign in the highlighted entry in the logbook.

This command displays the contents of the logbook file in a

formatted manner on the screen. It requests the callsign of the

log entry to be displayed. If you want to see entries for

particular callsigns or parts of a callsign, enter those

callsigns or the front parts of the callsign. Valid entries are

G, G3, G3Z etc. If you want to see every entry in the log, don't

use this command, use the Page Up and Page Down keys from the

Main Log menu.

14.20.4 Alt-S Scan Log by Call

This function key allows you to scan the logbook by callsign

prefix for the callsign entered with the Alt-C or Alt-E keys.

This command displays the contents of the logbook file in a

formatted manner on the screen. It requests the callsign of the

log entry to be displayed. If you want to see entries for

particular callsigns or parts of a callsign, enter those

callsigns or the front parts of the callsign. Valid entries are

G, G3, G3Z etc. If you want to see every entry in the log, don't

use this command, use the Page Up and Page Down keys from the

Main Log menu.

14.20.5 Alt-U Toggle Delete Mark

This function key allows you to mark an entry for deletion, or

LAN-LINK.DOC (c) G3ZCZ 1988 - 1994

Appendix 2 Page 61

unmark it in case you change your mind. Entries marked for

deletion will be flagged by an "*" character next to the record

number.

14.20.6 Alt-X eXit Log

This function key lets you exit from the Logbook.

14.20.7 Ins Toggle Insert Mode

This function key allows you to turn the insert mode ON and OFF

for the edit/append functions.

14.20.8 End Show Last Page

This function key moves you to the last callsign on the last page

of the logbook.

14.20.9 Home Show First Page

This function key moves you to the first callsign on the first

page of the logbook.

14.20.10 PgUp Move Up One Page

This function key moves the display up one page of the logbook.

14.20.11 PgDn Move Down One Page

This function key moves the display down one page of the logbook.

14.20.12 Up Arrow Move Up One Entry

This function key moves the highlight up one entry in the

logbook.

14.20.13 Down Arrow Move Down One Entry

This function key moves the highlight down one entry in the

logbook.

15.0 MESSAGE MENU

The Message Menu is shown in Figure 15. Type the single letter to

make your choice.

Figure 15 The Message Menu

B PBBS Command Files

C Current Messages

L Leave Note

LAN-LINK.DOC (c) G3ZCZ 1988 - 1994

Appendix 2 Page 62

N Print Message

O Old Messages

T Time Tag

V View Messages

15.1 PBBS Command Files

This option shows you a list of the PBBS command files on the

default directory. In other words, all files with the file

extension of ".BBS".

15.2 Current Messages

This option shows you a list of the outgoing messages that have

not been sent. In other words, all files with the file extension

of ".OUT".

15.3 View Message

This feature allows you to view messages in the computer. After

depressing the key you will be prompted for INcoming or OUTgoing

(I/O) messages. If you request "I" (Incoming) messages you will

see the messages addressed to your callsign. If you request "O"

(for OUTgoing) you will be presented with a directory window

containing a list of messages. Move the cursor down to the

desired message, and touch the "Enter" key. If you don't want to

view any of them, then move the cursor down to a blank line. If

you have more messages than one screen full, use the Pgdn (Page

Down) key to show the next screen full.

15.4 Leave Note

This feature allows you to leave a note for another station on

the LAN in your system. It works using the editor in the manner

described in Section 8.

15.5 Old Messages

This feature shows you a list of the messages that have been

sent. In other words, all files with the file extension of

".OLD".

15.6 Time Tag

This option is used to place a date time header on a message file

previously prepared by a word processor. The name of the file

must be in the format "CALLSIGN.OUT" (i.e. G3ZCZ.OUT). It

requests the callsign of the message file. It then opens a new

file, writes the header to the new file and copies the contents

of the original "*.OUT" file to the new one. When all is done,

LAN-LINK.DOC (c) G3ZCZ 1988 - 1994

Appendix 2 Page 63

the original filename is changed to "*.BAK" and the new one

becomes "*.OUT".

16.0 TNC MENU

The TNC Menu depends on the TNC. For example the MFJ1278 provides

the choices are shown in Figure 16-1. The PK-232 TNC menu is

similar as shown in Figure 16-2. The actual menu you see also

depends on the mode. In the non packet modes, the non packet

options will be locked out. For example, in the PK-232 Baudot

mode, the menu will appear as shown in Figure 16-3.

The DSP-2232 and PK-900 support simultaneous dual port operation.

LAN-LINK provides for support of this feature, so the TNC Menu

for those TNCs has additional options as shown in Figure 16-4.

Figure 16-1 MFJ1278 TNC Menu

D Set MFJ1278 Date/Time

E Disable MFJ1278 Port

F Flush MFJ1278 Buffer

I Initialize MFJ1278

M calls Monitored List

S Show Connect Status

T Change cText

X Activate MFJ1278 Mailbox

Y Deactivate MFJ1278 Mailbox

Figure 16-2 The PK-232 TNC Menu

D Set PK232 Date/Time

E Disable PK232 Port

F Flush PK232 Buffer

I Initialize PK232

M calls Monitored List

S Show Connect Status

T Change cText

X Activate PK232 Mailbox

Y Deactivate PK232 Mailbox

Figure 16-3 The PK-232 TNC Menu in BAUDOT

D Set PK232 Date/Time

E Disable PK232 Port

F Flush PK232 Buffer

I Initialize PK232

Figure 16-4 The PK-900 TNC Menu in Packet

D Set PK900 Date/Time

LAN-LINK.DOC (c) G3ZCZ 1988 - 1994

Appendix 2 Page 64

E Disable PK900 Port

F Flush PK900 Buffer

I Initialize PK900

M calls Monitored List

N PK900 Modem Menu

S Show Connect Status

T Change cText

X Turn PK900 Mailbox ON

Y Turn PK900 Mailbox OFF

1 Select Radio Port 1

2 Select Radio Port 2

3 Disable Radio Port 1

4 Disable Radio Port 2

5 Enable  Radio Port 1

6 Enable  Radio Port 2

7 Show ## Radio Port

Whichever menu is displayed, type in the single letter to make

your choice.

16.1 Set TNC Date/Time

This feature puts the TNC into the Command Mode and loads the

date into it from the computer time of day clock. This command

does not work on the TNC1.

16.2 Flush TNC Buffer

This option is used to flush LAN-LINK's incoming or outgoing text

buffer and that of the PK-232, PK-900 or DSP 2232. When used with

other TNCs it will only flush contents (if any) of the buffers in

the PC. If you flush the incoming buffer, you will only flush the

buffer in the PC. Any characters still in the TNC buffer will

still come through. If that happens, bring up the function but

wait a while before activating it.

16.3 Initialize TNC

This feature sets up the defaults for the TNC from the file LAN-

LINK.SYS. If you have put batteries in the TNC, you should

normally only have to run this command when you first power up

the TNC or change the batteries. However, the batteries must be

removed initially since they are there to retain the default

settings of the TNC (particularly the PK-232). Once the

initialization is complete, you can replace the batteries but

leave the TNC on since the battery provides power to the RAMs

after the TNC power is turned off.

The function will also set the date and time within the TNC from

the PC computer clock, and, when it has loaded the configuration

LAN-LINK.DOC (c) G3ZCZ 1988 - 1994

Appendix 2 Page 65

file, may prompt you to set the TNC Terminal mode. If you are

using a TNC1 or a TNC2, it will not prompt you because it assumes

that they only operate on VHF packet. If you are using a KPC-2,

you may select "H" or "V", and of course, if you have a PK-232,

you may choose any of the options.

16.4 Show Calls Monitored List

This option performs the same operation as Function Key 5 (F5).

16.5 Show Connect Status

When you are in the Packet Communications Mode, this command lets

you see the state of the connection. It will show you if there

are any unacknowledged packets en route to the other station. If

you have been sending text, it is advisable to use this command

before you disconnect, to ensure that all the data has been

received and acknowledged by the other party. You may use this

command in either the TNC Command or the TNC Converse Mode.

In a multi-connect situation, it will show you the status on all

the I/O channels.

You may also use this command to reset the status window display

after a relink or any other event which was not reflected in the

status window.

16.6 Change CTEXT

This feature allows you to change the message that will be

received by another packet station when it connects to you. Refer

to the CTEXT command in the TNC manual.

16.7 Activate TNC Mailbox

The mailbox in your TNC provides a convenient way for people to

leave you a message when your PC is switched off or is in use for

something other than packet radio. If you do choose to make use

of the personal PBBS in the mailbox, make sure that you have a

battery in the TNC. You will have to interrogate the mailbox in

the command mode to view any messages. Type the commands listed

in your TNC manual.

This menu option allows you to activate the personal mailbox

built into your TNC to read any messages stored in it, or, before

shutting down, if you have not set the shut down mode to 5.

To save your local full service PBBS SYSOP some hassles, it is

strongly recommended that you let LAN-LINK do your outgoing and

incoming message and bulletin store and forwarding. Use the "L"

LAN-LINK.DOC (c) G3ZCZ 1988 - 1994

Appendix 2 Page 66

option in the Edit menu to enter any messages off-line, the "M"

and "K" options in the Edit menu to answer any incoming messages

or comment on any bulletins. Once the messages are written into

the LOCALPBBS.BBS file, let the PBBS Zap take care of

transferring the messages to the PBBS.

16.8 Deactivate TNC Mailbox

This option allows you to deactivate the personal mailbox built

into your TNC.

16.9 TNC Modem Menu

This option applies to the PK-900 and DSP 2232. It interrogates

the TNC and displays list of the modems in the TNC.

16.10 Select Radio Port 1

This option applies to the PK-900 and DSP 2232. It sets outgoing

text to Radio Port 1.

16.11 Select Radio Port 2

This option applies to the PK-900 and DSP 2232. It sets outgoing

text to Radio Port 2.

16.12 Disable Radio Port 1

This option applies to the PK-900 and DSP 2232. It disables all

input from Radio Port 1.

16.13 Disable Radio Port 2

This option applies to the PK-900 and DSP 2232. It disables all

input from Radio Port 2.

16.14 Enable  Radio Port 1

This option applies to the PK-900 and DSP 2232. It disables all

input from Radio Port 1.

16.16 Enable  Radio Port 2

This option applies to the PK-900 and DSP 2232. It disables all

input from Radio Port 1. You might want to use it to temporarily

disable the vhf port when receiving an ANSI color picture on the

HF port.

16.17 Show ## Radio Port

LAN-LINK.DOC (c) G3ZCZ 1988 - 1994

Appendix 2 Page 67

This option applies to the PK-900 and DSP 2232. It displays the

current TNC connect and radio port status.

17.0 AMSAT-OSCAR MENU

This menu allows you to configure the KAM, MFJ1278, or the PK-232

to copy signals from AMSAT-OSCAR Spacecraft telemetry beacons

depending on the TNC. The following options are available.

D Full Duplex ON/OFF

S SAREX Menu

Type in the single letter to make your choice.

17.1 Full Duplex ON/OFF

This option allows you to turn the FULLDUPLEX parameter in the

TNC ON and OFF. Set it to OFF (default) when working a station

simplex (same uplink and downlink frequencies) i.e. MIR. Set

FULLDUPLEX to ON when working a station duplex (different uplink

and downlink frequencies) i.e. SAREX and Fuji-OSCAR 20.

17.2 SAREX MENU

This menu shown in Figure 17-1, is only active in the Packet

Communications Mode. It is designed for use with the Shuttle

Amateur Radio Experiment (SAREX), MIR, and the packet radio

Microsats, or any terrestrial station you want to connect with as

soon as they appear on frequency. Apart from the Attack Mode, it

can also be used as a diagnostic LAN tool for monitoring LAN

traffic.

Figure 17-1 The SAREX Menu

A Set Attack Mode ON

C Set SAREX Call

D Edit SAREX Capture File

L Turn SAREX Capture ON

M set Attack Mode Connect/CQ

Z Turn Zap MIR-1 ON

0 Multiple User Connects

17.2.1 Attack Mode ON/OFF

If the Attack Mode is set, this option will cause LAN-LINK to

issue a connect request to the SAREX Call whenever a packet sent

to or from it is heard. The mode, is cleared when the connect is

made (and does not retry out) or when the "A" option is selected

a second time, or when the Alert Call is cleared. If this mode is

enabled, the Alert/SAREX Call prefix shown in the Status Window

LAN-LINK.DOC (c) G3ZCZ 1988 - 1994

Appendix 2 Page 68

will indicate accordingly.

A happy face will be displayed next to the ">" before the call

once the connect has been achieved.

Be careful using this feature, as it has the potential to cause a

great deal of QRM. It can also be cleared by another station

connecting to you and telling you to ":QRT:".

In the SAREX configuration you are listening on one channel while

transmitting on an other. If this feature is used on a simplex

channel (everyone transmits and listens on the same channel)

aimed at a DX-pedition or the MIR space station, there is a

potential lockout mode, in which any one station sending a

connect request to the desired/SAREX call will trigger the other

ones on frequency who will in turn trigger the first. If this

pile up situation occurs, I hope the DX-station will QSY and

leave the automatic stations to do their thing. IN any event, if

you use this feature and cause QRM, anyone can connect to you

with the callsign of the DX station (as a pirate) and shut you

down. You will then not get the coveted DX QSL.

17.2.2 Change SAREX Call

This option lets you temporarily change the SAREX call.

17.2.3 SAREX Packets Capture to Disk ON/OFF

This option allows you to configure LAN-LINK to capture to disk

any packets addressed to or from the SAREX callsign. A "[S]" will

be displayed in the status window when this option is active. The

capture to disk file is opened by a packet header containing the

SAREX call, and closed by another packet header not containing

the call. When the file is open, the "[S]" will blink. Packet

headers are considered to be lines with a ">" character in them.

LAN-LINK thus considers both of the lines below as packet

headers.

N4QQ*>G3ZCZ

N4QQ PBBS>

If you use this feature for monitoring PBBS traffic, the PBBS

prompt lines ("E.G. N4QQ PBBS>") will not be captured to disk. Do

not get your SYSOP to remove the ">" from the prompt line, or the

ZAP feature will then not work.

A happy face will be displayed in the "[S]" after the SAREX call

has been heard.

If you want to see an ORBITER menu, temporarily change the SAREX

LAN-LINK.DOC (c) G3ZCZ 1988 - 1994

Appendix 2 Page 69

call to be the same as your call and bring up the AMSAT-OSCAR

Menu.

17.2.4 Edit SAREX Capture-to-disk File

This option lets you edit the SAREX Capture-to-disk file as long

as it is less than 64 kBytes.

17.2.5 Zap SAREX ON/OFF

This option combines the "Set SAREX Attack Mode" and the "SAREX

Packets Capture to Disk ON/OFF" options in one fast operation.

17.2.6 Set Attack Mode to Connect/CQ

This option allows you to configure the SAREX Attack mode to

digipeat or CQ via the SAREX station as well as to try for

connect. Use the CQ feature to digipeat via MIR, and the connect

for the Space Shuttle. If the Attack Mode is set, LAN-LINK will

either try for a connect or send an unproto packet containing the

CQ text. If you want it to go through the SAREX Callsign you must

configure the UNPROTO parameter in the TNC. For example, to

attempt a digipeat via R2MIR, set 'UNPROTO CQ via R2MIR' from the

command mode.

17.3 SAREX Orbiter Features

This was the menu as seen by WA4SIR in orbit on STS-35. You need

the SAREX ROBOT EPROM for this menu to actually do anything to

the TNC. The EPROM may be available from The Radio Amateur

Satellite Corporation (AMSAT) for DX-pedition use. Other options

which control LAN-LINK parameters may be used without the SAREX

ROBOT EPROM.

In the Orbiter mode, LAN-LINK does not scan incoming text for the

"*** CONNECTED" and "*** DISCONNECTED" messages when the SAREX

Robot is active. This inhibits the bells and the capture-to-disk.

It also means that the Robot must be OFF for any :Q: code uploads

or downloads to occur.

17.3.1 Status Window Display

A flashing ">>" appears in the status window when the SAREX robot

is active. The SAREX Configuration boots up assuming that the

SAREX Robot is active (Default).

17.3.2 Alt-O Function key

The Alt-O Function Key brings up the Orbiter menu. This is a

faster route than the "Esc O" sequence.

LAN-LINK.DOC (c) G3ZCZ 1988 - 1994

Appendix 2 Page 70

17.3.3 The Metabeacon

The Metabeacon is loaded from the contents of one of the META.00n

files. These files may be created by the LAN-LINK editor. When

loading, LAN-LINK will insert a pass character (^V) into the text

before a carriage return, so as not to end the packet before at

least 240 characters, and not more than 255 characters have been

output to the TNC. The maximum number of characters that can be

loaded into the Metabeacon is 1776. The maximum number of packets

in a Metabeacon is 7. You can also force a hard carriage return

or "end of packet" in the Metabeacon by inserting the "\"

(backslash) character, which will not be transmitted, into the

text.

17.3.4 SAREX Orbiter Menu

The following commands are active in the Orbiter Menu.

17.3.4.1 Activates Beacons/SAREX Robot: This option activates the

SAREX Robot and loads the Meta Beacon. This command is

designed for use when the SAREX is returned to the

regular frequency.

17.3.4.2 Set QRZ/QSL Beacon Interval: This command allows you to

change the beacon timer.

17.3.4.3 Set SAREX Call: This command allows you to change the

SAREX call. Don't use this command on orbit (it's for

ground use only).

17.3.4.4 Turn SAREX ROBOT OFF: The Robot must be turned ON and

OFF this way to synchronize LAN-LINK to the TNC. When

the Robot is turned OFF the following commands are sent

to the TNC by LAN-LINK.

USERS 1

|A

RO OFF

CMSG OFF

NO OFF

17.3.4.5 Set SAREX ROBOT Heard Count: This command lets you set

the Heard Count number.

17.3.4.6 Show I/O Status: Shows the callsign of the station

connected on each stream, the SAREX Robot state, the

number of USERS, the beacon timer value, the metabeacon

timer value and the CTEXT.

LAN-LINK.DOC (c) G3ZCZ 1988 - 1994

Appendix 2 Page 71

17.3.4.7 ChecK Count: Shows both heard and worked counts.

17.3.4.8 List Heard Calls: Lists the callsigns of the last few

stations heard.

17.3.4.9 Load Metabeacon: This command loads the Metabeacon. You

will be prompted for a text file number in the META.00n

series. Enter a number 1-9 corresponding to META.001-

META.009. The 0 value is for file META.010. Note the

delays after loading the file to allow the TNC to

stabilize.

17.3.4.10 Show Worked Calls: Shows the last 35 or so callsigns

worked.

17.3.4.11 Turn SAREX ROBOT ON: The SAREX Robot must be turned ON

and OFF via the menu. When the Robot is turned ON the

following commands are sent to the TNC by LAN-LINK.

USERS 9

CONOK ON

CMSG ON

RO ON

NO ON

17.3.4.12 Stop Metabeacon: This command stops the metabeacon

transmission. When activated, the contents of the beacon

are dumped to the UNPROTO address (default SAREX) by the

TNC, rather like a regular PBBS dumping packets when a

user disconnects abnormally.

17.3.4.13 Set Metabeacon Interval: This command allows you to set

the metabeacon timer. It will be loaded into the TNC the

next time that the metabeacon is loaded. Even though

loaded into the TNC immediately, it does not take effect

till then.

17.3.4.14 Set SAREX ROBOT Worked Count: Lets you set the starting

number.

17.3.4.15 Set Connect TeXt: This is the text (CTEXT) that will be

transmitted to an incoming station when the ROBOT mode

is off. This option lets you set up a new text string to

go with the QSO number.

17.3.4.16 Selects Silent Mode: This command turns off the

beacons, the SAREX Robot and also stops the metabeacon.

This command is designed to be used just before any

prearranged packet activity.

LAN-LINK.DOC (c) G3ZCZ 1988 - 1994

Appendix 2 Page 72

17.3.4.17 Restart TNC: This command RESTARTs the TNC, loads the

date from the computer's clock and turns the "EC OFF".

It seems to have no effect on the SAREX ROBOT and

counters, but does kill the metabeacon.

17.3.4.18 Capture QSL/QRZ Calls: This command initiates the log-

to-disk operation. It turns the capture-to-disk "on" and

sends the following commands to the TNC:- "DA", "SERW",

"SERH","MW", "MH". When the calls have stopped

scrolling, use Function key 1 to turn the capture-to-

disk off.

18.0 PARAMETER MENU

A typical Parameter Menu is shown in Figure 18. The colors used

for the option letter differentiate between those options that

are "remembered" in the SYS file and those that are only set

temporarily for the session. Type in the single letter to make

your choice.

Figure 18 A Typical Parameter Menu

A Send of LAN-LINK.OUT is ON   0 Change Band & Power

B Beacon/Robot Mode is OFF     1 Change PBBS/PacketCluster Parameters

C Clear Connect Count          2 Change Callsigns

D Digipeat Detect is OFF       3 Change Directories

E Command Echo is ON           4 Enter Common Calls

F BBS Zap Drop Link is ON      5 Change File Names

I Packet Beacon Control is ON  6 Change LAN-LINK Parameters

L Automatic Ctdsk is ON        7 Change PC Communications Parameters

M Set Message Count            8 Change Colors

N NODE Drop Link is OFF        9 Change SAREX Parameters

P Printer is OFF

Q QTC Snatch is ON             U Update LAN-LINK.SYS

R :QSP: Relay Flag is ON       V Verify Changes

S Sound is ON                  W Change Scan Words

T Contest Mode is OFF          Z Time Id. is OFF

X DCD Flag is OFF              ! Zap PBBS On Next Beacon is OFF

Y PBBS Snatch is ON            + Misc. Flag is OFF

18.1 Turn Beacon Mode ON/OFF

This command toggles the beacon option for the automatic CQ

caller in the AMTOR, Pactor, G-TOR and Packet Communications

Modes. When it is active a "[B]" will be flashing in the status

window. In this mode, the Automatic CQ sequence will resume after

the AMTOR link is dropped or a Packet Communications Mode

disconnect.

In the Beacon Mode, the last few logbook entries will be

displayed in the logbook window which overlays the outgoing text

LAN-LINK.DOC (c) G3ZCZ 1988 - 1994

Appendix 2 Page 73

window at the bottom of the screen.

In the Packet Mode, the CTEXT message is the only transmission

from you that the other station will receive, so make it good and

don't forget to set "CMSG ON".

In the AMTOR Mailbox/Beacon Mode, the contents of the LAN-

LINK.TXT file will be transmitted to the station who called you.

The capture-to-disk will then be turned on to record one

transmission from that station. The QSO will then be logged to

the logbook file. Don't forget to check the entries in the

logbook and text in the LAN-LINK.RUN files in case the computer

made a mistake in recognizing the call sign of the other station.

The Beacon mode is designed to encourage people to QSY off the

crowded HF packet channels. Do not use it on a crowded HF channel

or on an active VHF channel. If you do, you will soon find that

other stations have set their TNCs to ignore your packets, and

you will not work anyone. You will just be a source of QRM and

will be known as such.

The Beacon mode is also designed for robot beacons on the higher

frequencies. For example, if you leave it running on 6 or 10

meters, you never know when a short opening will allow a QSO with

some part of the world, a DX contact you would otherwise have

missed. Don't leave it running on 20 Meters when the band is wide

open. Further applications are in meteor scatter and other

"exotic" communication modes.

If you are using the Beacon Mode in a special event station or

DX-pedition you may type characters into the transmit buffer,

while the other station is sending to you. When he, she, or even

it, turns it back to you, you will transmit those characters

before the sign-off sequence.

In all communications modes, the robot may be shut down by

stations connecting to you and telling you to QRT.

18.2 Clear Connect Counter

You use this option to reset the PACKET and AMTOR Beacon Mode

"connect has taken place indicator" and QTC Snatch counters.

These are the flashing "[C]" and "[Q]" displays in the status

window. Whenever someone else connects or links up to you the

"[C]" display will be incremented. The number shown with the "C"

is the number of connects that have taken place since the

Indicator was last reset. If a "happy face" is flashing along

with the connect count, your "target station" was amongst those

connecting. The "[Q]" indicator alerts you to the fact that LAN-

LINK tried to connect to another station after recognizing your

LAN-LINK.DOC (c) G3ZCZ 1988 - 1994

Appendix 2 Page 74

call in a mail beacon. Both are reset by this option. If you are

monitoring Packet traffic on a channel, you may not notice that a

connect has previously taken place. These indicators alert you to

check the contents of the Incoming mail file ("yourcall.OUT") for

any incoming messages.

If the "Q" count is much greater than the "C" count, you may be

copying a beacon from a station that you cannot connect with

directly.

18.3 TNC Custom Line (TNC Parameter)

This line has different effects depending on which TNC you are

using. To set the line, add the numbers up for the parameters you

want, and put the total in line 32.

18.3.1 All TNC's

TNC Sync shift (128)

LAN-LINK normally ensures that the Transmitter and Receiver

data sense are in phase. This presupposes AFSK operation. If

you are running FSK and need them out of phase, set this

byte to 128. If this line includes the number 128, LAN-LINK

will NOT set the receiver shift when you change modes.

Zap Stay Up (64)

If you include the number 64 in this line, LAN-LINK will

default to staying connected to a PBBS when the Zap sequence

ends. Use this parameter setting if the messages contain a

lot of ">" characters at the end of lines and LAN-LINK is

faked into thinking the messages ended prematurely.

TNC initialize Date (32)

LAN-LINK normally sets the time of day clock in the TNC from

the PC during initialization. If you don't want LAN-LINK to

set the TNC time of day clock, include the value 32 in this

line.

18.3.2 PK-232, PK-900 and DSP 2232

PK-232 Digipeat Display Flag (1)

Set this line to 1 to activate this feature, 0 to inhibit

it. If the Digipeat Display is active, any signals digi-

peated through you will be stored to the LAN-LINK.RUN file

and a "beep" will be sounded for each packet being

digipeated. The status window will also contain a "[D]".

PK-232 Converse Mode Default (2)

LAN-LINK runs the PK 232 is command mode in the RTTY an CW

modes. Function key 7 (F7) allows you to set the PK 232 into

LAN-LINK.DOC (c) G3ZCZ 1988 - 1994

Appendix 2 Page 75

the converse mode. The MFJ 1278 and KAM operate in the

Converse mode in all communications modes. If you set this

bit to 2, LAN-LINK will run the PK-232 in the converse mode

in all modes, and you will not need to use F7 to enter the

converse mode before transmitting.

PK-232 Serial Port Initialization (4)

If this bit is set to 4, LAN-LINK will assume there are no

batteries in the PK-232 and issue the '* * *' sequence to

the PK-232 on start up. If this bit is not set, LAN-LINK

will assume the PK-232 contains batteries and will not try

to synchronize the PC to PK-232 serial port.

This feature was removed from LAN-LINK in 2.00 in response

to numerous users who had batteries in their PK-232's and

didn't want to see the initialization sequence. Then came

the complaints from the users who didn't have batteries in

their PK-232's.

PK-232 XMITOK Flag (8)

In the CW communications mode, you may have to change the

value of the PK-232 XMITOK Parameter depending on how you

connect the PK-232 to your transmitter. This LAN-LINK

parameter determines if LAN-LINK instructs the PK-232 to

turn the XMITOK PK-232 parameter on and off when entering

and leaving the CW communications Mode. You may temporarily

change the value of the XMITOK Flag in the CW menu to set

things up.

As an example, to configure LAN-LINK to initialize the PK-

232 serial port (4) and default to the Converse Mode (2),

Set this parameter to 4+2 or 6.

18.4.2 KAM

KAM Set LFADD (2)

If this bit is set, LAN-LINK will send a 'LFADD ON' command

to the KAM when you enter the BAUDOT communications mode,

and send a 'LFADD OFF' command when you exit that mode to

another one.

KAM Pre 3.0 EPROM (4)

If your KAM contains an EPROM with a Revision number of less

than 3.0, then set this bit. LAN-LINK will issue NODE

commands when selecting SOLO mode. You must set this bit if

you plan to use the mailbox in the KAM, or the traffic on

frequency will fill the memory in the KAM, and people will

not be able to leave you any messages.

LAN-LINK.DOC (c) G3ZCZ 1988 - 1994

Appendix 2 Page 76

18.4.3 MFJ 1278

MFJ Radio Port (1)

This bit allows MFJ users to set the default radio port for

VHF. Set this bit to 1 to assign Radio 1 to VHF (Radio 2 to

HF). Set this bit to 0 to assign Radio 2 to VHF (Radio 1 to

HF).

MFJ Echo ON/OFF Flag (2)

Some versions of the MFJ firmware require 'ECHO ON' in

AMTOR. IF bit 2 is set, LAN-LINK will command 'ECHO ON'

before entering AMTOR, and will command 'ECHO OFF' when

leaving AMTOR, if you switch modes via the Menu.

MFJ AMTOR Converse Flag (4)

This bit configures the MFJ to send the CONVERSE Command

when someone connects in the AMTOR Mode.

18.4 Turn Beacon Control ON/OFF

This option toggles the LAN-LINK Handshake features. When they

are on (default), the Connect handshake message

[ZCZ] *** LAN-LINK 2.32>

will be transmitted on incoming connects. The beacon text and

time will be controlled by LAN-LINK as a function of the messages

you have posted on your system. When they are off, LAN-LINK does

not do anything to your beacon, nor does it transmit the

handshake.

18.5 Turn Command Echo ON/OFF

This option toggles the LAN-LINK "Command ECHO flag" ON and OFF

(not the ECHO parameter in the TNC). If the Command Echo flag is

set, the commands and (contents of) files LAN-LINK sends to the

TNC will be displayed in the outgoing window. Set this flag to

see how LAN-LINK commands the TNC in the different modes.

18.6 Change Keyboard Beep Count

LAN-LINK counts the number of characters entered on a line at the

keyboard. When it gets to a preset value it will beep, signaling

you to start a new line. If you have the Word wrap option set, it

will also automatically insert a "carriage return/line feed"

character sequence into the outgoing text for you. This option

changes the value that the keyboard beep counter is compared with

to sound the audio signal.

LAN-LINK.DOC (c) G3ZCZ 1988 - 1994

Appendix 2 Page 77

18.7 Set Message Count

As an alternative to Function Key 8 (F8) or Alt-D, using this

option will also load a QTC "Mail" list into your Packet Communi-

cations Mode Beacon and initiate a beacon transmission every 30

minutes (Refer to the BTEXT command in the TNC manual). If no

messages are present, or the only message is to you (incoming or

snatched), beacon transmissions are inhibited. This conforms to

good operating practice on crowded Packet channels.

18.8 Turn Node Drop Link ON/OFF

When connecting via a Node, the normal way of operating is to

disconnect if the connect attempt fails. You may configure the

Node connect attempt flag to stay connected at the node where the

failure occurred. Use the normal mode when trying to get through

on a known path, and the stay alive mode when exploring Node

circuits.

18.9 Turn Printer ON/OFF

This option toggles the printer ON and OFF. When the printer is

turned on, any incoming data will be printed in real time. If you

intend to print files received at 1200 baud in the Packet

Communications Mode, use a print spooler or you may lose data.

The printer is automatically turned off in the Packet Mode when a

disconnect occurs.

18.10 Turn QTC Snatch ON/OFF

This option toggles the QTC Snatch flag ON and OFF. If the QTC

Snatch flag is enabled, LAN-LINK scans the contents of packets

appearing in the Inwindow on the screen while the terminal is

disconnected. Should it recognize the three text strings "LAN-

LINK", ":QTC:", and "YOURCALLSIGN" in the contents of a packet,

it will then issue a connect request to the callsign of the LAN-

LINK station that originated the beacon transmission. If the

connect is made, the other station will send you the message

automatically which LAN-LINK will "capture to disk". The connect

will time out and terminate a few minutes later.

The normal connect request will be made DIRECT to the station. If

you expect messages from stations you can only connect with via a

digipeater, place the path in your LAN-LINK.DIR directory file.

It should be there anyhow if you connect to the station

regularly. For example, if you expect messages from 4Z4ZB whom

you can only work if digipeated via 4X1AA then place an entry

like "4Z4ZB 4Z4ZB V 4X1AA" in the LAN-LINK.DIR file.

The QTC Snatch leaves your messages in a message file

LAN-LINK.DOC (c) G3ZCZ 1988 - 1994

Appendix 2 Page 78

YOURCALL.OUT. It is triggered by the first ">" in the initial

connect response of "LAN-LINK 2.00R>" and terminated by the

"over" at the end of the message e.g. G3ZCZ/W8>. If you train

stations not using LAN-LINK who want to leave you messages to

trigger your QTC-Snatch, by transmitting the following un-

addressed packet

LAN-LINK :QTC: YOURCALL

your system will connect to them. (For example to leave a message

for me, they would send

LAN-LINK :QTC: G3ZCZ

as an un-addressed packet [in Converse Mode when not connected,

or in their beacon text].)

If they then send you a ">" on a line by itself, followed by the

message itself, and terminated by another ">" as the last

character of the line, the message will end up in a message file

rather than in the LAN-LINK.RUN file and you can scan it fast

using Alt-M (Message Scan Menu Options). It goes without saying

that the message itself should not contain the ">" character at

the end of a line. If it does, by the way, the first part of the

message will be in the message file, while the remainder will be

in the LAN- LINK.RUN file.

18.11 Turn :QSP: Relay Flag ON/OFF

This option toggles the Store and forward Message Relay

Capability of your system ON and OFF. If the Message Relay

Capability is active (toggled on), any one who connects can store

a message in your system using the ":QSP: CALLSIGN" NC/L command.

If the Message Relay Capability is not active anyone who attempts

to ":QSP:" a message will receive a ":QNO: error" message. The

status display window will show a "[R]" when the Message Relay

Capability is active. The default state for the Message Relay

Capability is loaded from the LAN-LINK.SYS file.

When the Store and forward capability is active, the suffix "R"

will be added to the "[ZCZ] *** LAN-LINK 2.32". i.e., "[ZCZ] ***

LAN-LINK 2.32R" message received by anyone connecting to your

system to notify them that they can store messages. In this

manner they don't have to try to leave a message and then get

":QNO:" error message to find out that the feature is toggled

off.

Remember, if you configure your computer to store and forward

messages, you have a moral obligation to deliver them.

LAN-LINK.DOC (c) G3ZCZ 1988 - 1994

Appendix 2 Page 79

18.12 Turn Sound ON/OFF

This option toggles the sound ON and OFF. If the sound is off, a

flashing indicator will be displayed in the Status Window. When

the sound is toggled on again, a chime will sound to verify the

audio.

18.13 Turn Contest Mode ON/OFF

This option first toggles the Contest Mode. If the Contest Mode

is active, the word "TEST" and the number of the next QSO will be

displayed in the status window. When the Contest Mode is toggled

on, this command prompts you to set the starting QSO number, and

the standard message that will be transmitted as part of the

exchange of messages during the contest. The QSO number will be

incremented each time you log a contact.

The contest feature is toggled off the next time you select this

feature.

While these features do give you some semiautomatic contest

operation by using command keys, there is no checking of

callsigns for duplicates built into the program. You may do that

by using the Alt-S hot key.

In a contest, you enter the callsigns and call the other station

in the normal manner, then use the "End" Key to transmit the

exchange (QSO number and standard text). LAN-LINK will only

transmit the contest text in this manner. If you want to add a

"K" or "KN", you will need to put it in the contest text string.

It is also advisable to use a different log file for each contest

to cut down the search time. The contest QSO number increments

each time you log the QSO (Alt-L).

You may put the Contest Mode into automatic if you are operating

AMTOR or packet. To do so, use the Alt-O function key.

The following special character sequences can be entered in the

contest text string.

18.13.1 ##

A "##" will be replaced by the QSO number when the text is

transmitted.

18.13.2 #1

A "#1" will be replaced by the callsign of the station being

worked in the contest text string.

LAN-LINK.DOC (c) G3ZCZ 1988 - 1994

Appendix 2 Page 80

18.13.3 #T

A "#T" will be replaced by the date and time as read from the PC

time-of-day clock and converted to Universal Coordinated Time

(UTC). A sample time text is "03-Mar-92 03:13".

For example, use the following entries for the following

contests;

"##1 UR ##A" ... in the Sweepstakes,

"5905" ... in the WAZ,

"59 MD" ... in the ARRL DX, and

"59 ##" ... in the WPX.

As an example, if I (W3/G3ZCZ) am in the Sweepstakes contest, and

my Contest text line is,

"##1 UR ##A W3/G3ZCZ 68 MDC".

If the next QSO number is 345, and I am working KW3C, when I

touch the "End" key, LAN-LINK would transmit the following line

"KW3C UR 345A W3/G3ZCZ 68 MDC".

18.14 Turn DCD Flag ON/OFF

If you are using a PK-232 then this flag should be set to ON. If

you are using a KAM, this flag should be set to OFF. If you are

using another TNC, and if you can set the TNC so that pin 8 on

the RS-232 interface changes state when connected, do so, and set

this flag to ON. If you can't, then set this flag to OFF. If this

flag is OFF, the multiple-connect modes will not work so don't

attempt to invoke them. If this flag is OFF, the words "***

CONNECTED" and "*** DISCONNECTED" when seen in the LAN may cause

spurious apparent connects and disconnects in LAN-LINK.

18.15 Turn PBBS Snatch ON/OFF

When the PBBS Mail Snatch is enabled, and a PBBS sends a packet

announcing "Mail", if your callsign is in the first line of that

message, LAN-LINK will issue a connect request (again via the

path in LAN-LINK.DIR) to that PBBS. When connected, it will act

as if you have initiated a Zap PBBS command. The Zap counter will

be incremented each time a PBBS Mail Snatch takes place.

If the PBBS Mail beacon has an extra line in it (between the

header and the list of calls) as follows, the PBBS Mail snatch

will not work.

On the other hand, if N4FG is monitoring the LAN, and a message

is posted for any callsign containing the character string "N4FG"

such as N4FGA, N4FG will respond to the message. This gives rise

to a lock out situation, because N4FG will respond to the beacon,

LAN-LINK.DOC (c) G3ZCZ 1988 - 1994

Appendix 2 Page 81

go through the sequence described in the Zap PBBS section and

disconnect. The beacon message will not be cleared because it was

not read, so the next mail beacon from the PBBS will contain the

call N4FGA, and N4FG will once again respond to it.

Should you be unlucky and happen to be in this situation, disable

the PBBS Snatch flag and set a Periodic Event to Zap your PBBS

sometime in the middle of the night. Alternatively, when you come

into the shack and the PBBS is in use, set the "Zap PBBS on Next

Mail Beacon" parameter to ON.

18.16 Turn Time Id. ON/OFF

This option allows you to determine if the callsign sequence

transmitted using Function key 3 (F3) will contain a date/time

stamp. If the Id. is set ON a date/time stamp will be

transmitted.

18.17 Turn Automatic Capture-to-Disk Flag ON/OFF

This option allows you to turn the automatic "capture-to-disk"

feature ON and OFF.

18.18 Change PBBS Parameters

This sub-menu allows you to change the PBBS Parameters, as

described in the following paragraphs.

18.18.1 Local PBBS Callsign

This option lets you change the call of the local PBBS. This is

the call that LAN-LINK will show in the prompt for the "Zap"

command.

18.18.2 Mail-Snatch Trigger Word

You can set the Mail-Snatch Trigger in the PBBS Mail beacon. Most

PBBSs use word "Mail" and a string of calls to indicate the

presence of messages. If your PBBS uses something else, you may

customize LAN-LINK on this line to tell it what word your PBBS

uses. This word is case sensitive. If your PBBS uses "QTC"

instead of "Mail", you may end up calling yourself if you

disconnect while traffic is flowing on the LAN.

18.18.3 Read Request Command

This is the command LAN-LINK issues to the PBBS during a Zap

sequence to request your mail. The default value is "RM" for Read

Mine. On a WA7MBL type PBBS you may change it to "VM" for a

Verbose Read of the full packet headers. On a W0RLI PBBS you may

LAN-LINK.DOC (c) G3ZCZ 1988 - 1994

Appendix 2 Page 82

change it to RN to only request new messages. If you do change

it, use the "V" option in the Parameters Menu to make sure it is

correct. If it isn't correct, the Zap PBBS feature will not work.

18.18.4 Read Bulletin Request Command

This is the command LAN-LINK issues to the PBBS during a Zap

sequence to read bulletins identified by the scan words. The

default value is "R" for Read. For a Verbose Read of the full

packet headers on a WA7MBL type PBBS, change it to "V"; on a

W0RLI type PBBS change it to "RH".

18.18.5 PBBS Subject String

When you enter a message into a PBBS, you first use the "S"

command. The PBBS then sends you a reply asking you for some

information about the PBBS; typically the subject of the message.

When you have entered the subject, the PBBS then asks you for the

rest of the message. This line is used in the Zap sequence and is

a unique word seen in the text that the PBBS sends you asking for

the subject.

18.18.6 PBBS Message String

When you enter a message into a PBBS, you first use the "S"

command. The PBBS then sends you a reply asking you for some

information about the PBBS; typically the subject of the message.

When you have entered the subject, the PBBS then asks you for the

rest of the message. This line is used in the Zap sequence and is

a unique word seen in the text that the PBBS sends you asking for

the text of the message.

18.19 Change Callsigns

This sub-menu allows you to change the Callsign and AMTOR SELCAL,

as described in the following paragraphs.

18.19.1 Callsign

The default callsign is G8BTB. Please do not use G8BTB. Replace

G8BTB with yours. This is the callsign that will be used in the

non packet modes. You may use "portable designators" such as

G3ZCZ/A, G3ZCZ/W3, W3/G3ZCZ or G3ZCZ/4X, which you cannot do in

Packet.

This is the call that will be monitored for in the Digipeat

Monitoring Mode. In this Mode, which is toggled on and off by the

selection of the "D" Option included in the Parameter Menu (PK-

232 only), if someone uses you as a digipeater while you are not

connected to anyone else, the event will be logged to disk. This

LAN-LINK.DOC (c) G3ZCZ 1988 - 1994

Appendix 2 Page 83

Mode will only work if your callsign as stored in this line is a

valid (uppercase only) packet callsign.

For example, in the PK-232, if the callsign is G3ZCZ then the

sequence >G3ZCZ> will be recognized as a digipeat and the event

logged. Thus 4X6AA*>G3ZCZ>4Z4ZB would be logged as a digipeat

event.

Store your callsign using UPPERCASE characters if you want the

"DIGIPEAT DETECT" option to work (i.e. G3ZCZ). If you use lower

case (i.e. g3zcz), recognition will never take place. The TNC

displays all callsigns in upper case in the non packet communica-

tions modes.

If you are using an SSID (e.g. G3ZCZ-8) in the Packet

Communications Mode, you should enter the whole call INCLUDING

the SSID.

18.19.2 The SELCAL

This is your AMTOR SELCAL. LAN-LINK will try to guess it and

prompt you with a SELCAL configured for your callsign. If you

want a different one, overwrite the prompted version.

If you are not using a PK-232 or KAM or the MFJ1278, you don't

have to change anything, but the SELCAL line must still be in the

Configuration file.

18.19.3 The PacketCluster Call

This option allows you to change the default PacketCluster Call

without connecting to it.

18.20 Change Directories

This sub-menu allows you to change the Directories used by LAN-

LINK, as described in the following paragraph.

18.20.1 Capture-to-disk files Directory

This is the drive and directory that LAN-LINK will put the *.RUN

capture-to-disk files. Note: the last "\" MUST be present if you

are using a different sub-directory.

18.20.2 QBM Download Directory

This is the directory the :QBM: command will search to find a

file to be downloaded. The default directory for the files is

\LAN-LINK\FILES\. Note: the last "\" MUST be present.

LAN-LINK.DOC (c) G3ZCZ 1988 - 1994

Appendix 2 Page 84

18.21 Enter Common Calls

Each time you call another station, LAN-LINK remembers it (up to

the last 16). When you update the SYS file, those calls will be

saved to disk. You can use this option to add calls, delete calls

or edit calls in the list. Use the "Delete" key to delete a call.

18.22 Change File Names

This sub-menu allows you to change the default file names used by

LAN-LINK, as described in the following paragraphs.

18.22.1 VHF Logbook File (VHF.DBF)

This is the name of the file to be used as your VHF Packet

logbook.

18.22.2 HF Logbook File (HF.DBF)

This is the name of the file to be used as your HF logbook. It is

used for all HF modes, including HF packet.

18.22.3 Path Directory File (LAN-LINK.DIR)

This is the name of the file used as your call directory.

18.22.4 Brag Tape (LAN-LINK.TXT)

This is the name of the file used as your brag tape.

18.22.5 QBU Upload File Name (QBU-TX.BAT)

This is the name of the batch file used to initiate a QBU.

18.22.6 QBU Download File Name (QBU-RX.BAT)

This is the name of the batch file used when a :QBU: command is

received from a remote station.

18.23 Change LAN-LINK Parameters

This sub-menu allows you to change the LAN-LINK Parameters, as

described in the following paragraphs.

18.23.1 Change CQ Text

This feature allows you to change the contents of the CQ text

message string. This is the message string that is transmitted

along with the CQ call. Think of this message as a CQTEXT line.

LAN-LINK.DOC (c) G3ZCZ 1988 - 1994

Appendix 2 Page 85

18.23.2 Set CQ Delay Time (Seconds)

This option allows you to change the time (seconds) that the LAN-

LINK waits for a response after a CQ call until the following one

is transmitted. The default value (in minutes and seconds) is

loaded from the LAN-LINK.SYS file.

18.23.3 Set CQ Delay Time (Minutes)

This option allows you to change the time (minutes) that the LAN-

LINK waits for a response after a CQ call until the following one

is transmitted. The default value (in minutes and seconds) is

loaded from the LAN-LINK.SYS file.

18.23.4 Set Connect Time Timeout (Minutes)

This option allows you to change the time (minutes) that the LAN-

LINK waits for a response in a packet connect before issuing a

Disconnect command. The default value is loaded from the LAN-

LINK.SYS file.

18.23.5 Set Number of Lines in a Non-Contest CQ Call

This option allows you to change the number of lines in a non-

contest CQ call from the default value of 3 to any number you

wish. The default value is loaded from the LAN-LINK.SYS file.

18.23.6 Keyboard Beep Character Count

When your typing at the keyboard gets to a point where you may

overrun the normal line length, LAN-LINK issues a beep to remind

you to touch the "Enter" key. This option allows you to change

the number of characters you can type in a line before the beep

sounds. Note the beep will not sound if the sound flag is

inhibited.

18.23.7 Word Wrap Mode

This option allows you to change the word wrap mode. It is really

more of a "force carriage return" parameter, rather than "word

wrap". There are four options as follows:

18.23.7.1 Off: Word wrap is off.

18.23.7.2 AutoCR: LAN-LINK will insert an "Enter" character for

you into the outgoing text when you enter the first

space character after the beep has sounded.

18.23.7.3 Word: LAN-LINK will hold all entered text until you

either type a space character or touch the "Enter" key.

LAN-LINK.DOC (c) G3ZCZ 1988 - 1994

Appendix 2 Page 86

At that time LAN-LINK will send the word to the TNC.

18.23.7.4 Line: LAN-LINK will hold all entered text until you

either touch the "Enter" key or the beep sounds. At that

time LAN-LINK will send the word to the TNC.

Set this parameter value as follows,

0 = OFF,

1 = AutoCR,

2 = Word,

3 = Line.

18.23.8 Set Shut Down Mode (QRT Flag)

This option allows you to customize the shut down and start up

sequences of the TNC as described in the following sections. If

the Shut down mode is not set to 1, you also have the option to

customize the packet terminal mode.

18.23.8.1 State 0: On shut down, turns the TNC into a VHF Digi-

peater and turns CONOK OFF (CFROM NONE for the PK-232).

In this condition the TNC will reject connect requests.

On start up turns CONOK or CFROM on.

18.23.8.2 State 1: Does not send any commands to the TNC on shut

down or start up.

18.23.8.3 State 2: On shut down, sends CMSG ON to the TNC. On

start up, sends CMSG OFF to the TNC.

18.23.8.4 State 3: On shut down, turns on the mailbox in the TNC.

On start up turns off the mailbox.

18.23.8.5 State 4: On shut down, turns on the mailbox in the TNC

and puts the TNC into the SOLO Mode so that the internal

buffer in the TNC does not fill with on-the-air traffic.

On start up turns off the mailbox.

18.23.9 Set Default Users

This option allows you to set the maximum number of multiple

users that can be connected to your TNC at the same time, and to

configure the default start up mode of the program. If this

parameter is set to 1, LAN-LINK will configure itself to the

single user mode, and set the maximum number of users to 9. If

this parameter is set to any number other that 1, LAN-LINK will

assume that value to be the maximum number of allowed connects,

and configure itself in the Multiple user Mode (equivalent to the

TNC Menu Option 0).

LAN-LINK.DOC (c) G3ZCZ 1988 - 1994

Appendix 2 Page 87

18.23.10 Set UTC Offset

This parameter contains the difference in hours between Universal

Co-ordinated Time (UTC) and your local time. For example, if you

are located on the east coast of the U.S.A., and it is winter,

you are in the Eastern Standard Time (EST) Zone, which is 5 hours

behind UTC. In this case, the value of the UTC Offset parameter

is 5. Don't forget to change this parameter when the clocks are

adjusted each spring and autumn.

18.23.11 VHF Packet Length

This option allows you to change the default VHF packet length.

The TNC has a maximum permissible value of 255. Set the parameter

to 127 or 255.

18.23.12 HF Packet Length

This option allows you to change the default HF packet length.

The default value of about 30 seems to be optimal.

18.23.13 Beacon Time

This option allows you to change the default beacon time. The KAM

requires a value in minutes, all other TNCs in tens of seconds.

Use good operating practice and don't beacon too often. Every 30

minutes on a well used channel is more than enough.

18.23.14 Custom Prompt before "Over"

This option allows you to add a word or two (up to 20 characters)

before the ">" in packet or the "+?" in AMTOR. If you are working

an AMTOR APLINK PBBS, clear this feature by changing it to a

space character.

18.23.15 Packet Cluster DX Alert Mode

This parameter determines the action LAN-LINK takes when you are

connected to a PacketCluster, and a DX Alert message shows up.

The states are 0, 1, 2, or 3.

If it is 0, LAN-LINK takes no action at all. In all other states,

LAN-LINK splits the prefix from the DX callsign, searches the HF

logbook for the prefix and posts a message in the status window

(in the Alert Call color) as follows: Prefix, Logbook Status, DX

callsign. The logbook status shows up as 'Q' for QSLd, 'W' for

worked and '?' for not in log. You can configure LAN-LINK to

generate an audible alarm for each condition to save you

monitoring the screen. If the Sound is inhibited, the audible

alarm will also be inhibited. The visual display and audio alarms

LAN-LINK.DOC (c) G3ZCZ 1988 - 1994

Appendix 2 Page 88

are controlled by this parameter as shown below.

State     QSLd (Q)            Worked (W)          Not in Log (?)

0       No Action           No Action                No Action

1       Audio + Display     Audio +Display      Audio + Display

2       Display             Audio + Display     Audio + Display

3       Display             Display             Audio + Display

You can customize the CW tone and speed, by changing the states

of the "Packet Cluster DX Alert Tone CW Dit time" and "Packet

Cluster DX Alert Tone CW Note" parameters in the LAN-LINK.SYS

File.

In this version of LAN-LINK, the prefix scan is simple minded, so

set this parameter to 3 to get used to it, then set it to 1.

18.24 Change PC Communications Parameters

This sub-menu allows you to change the Communications Parameters

between the PC and the TNC, as described in the following

paragraphs.

18.24.1 The Communications Port

This is the default Communications port. The default value is 1.

You may set it at any value between 1 and 4. If you only have one

serial port in your PC, set the parameter to "1" even if your

card is configured as Com2. LAN-LINK will ALWAYS assume Com1 for

a single card. If you use a different port instead of Com1, and

plan to do binary transfers, don't forget to customize the

QZ*.BAT files.

The default Com3 and Com4 default PC port values are as follows.

Port  Address(Hex)            Address (Decimal)     IRQ

Com3       3E8                      1000             4

Com4       2E8                       744             3

If you set the port number to 0, LAN-LINK will not accept or

transmit any character to the serial port. All other features

will work.

18.24.2 The Terminal Baud Rate

This Parameter tells LAN-LINK what baud rate the TNC is set to.

This is the baud rate on the RS-232 line between the TNC and the

PC. Valid baud rates for LAN-LINK are 300, 1200, 2400, 4800, 9600

and 19,200. If you specify something else LAN-LINK will default

to 1200.

LAN-LINK.DOC (c) G3ZCZ 1988 - 1994

Appendix 2 Page 89

You may experience problems when using baud rates greater than

1200. It depends on the CPU clock rate, any TSR's in the PC, and

the I/O card in the PC. LAN-LINK scans the incoming text

constantly, something which requires time. My PK-232, when used

with my XT Clone at 10MHz, does not like baud rates faster than

2400, and sometimes not even that. The PK-232 seems to drop the

first character of some lines at 4800 baud. Use 1200 until you

have tested, and are sure that a faster one will work.

This program has been tested on a TNC2 and an MFJ1278 using a

terminal baud rate of 9600 baud and worked correctly. It also

works correctly with a KAM at 2400 Baud.

If you use a different rate instead of 1200, and plan to do

binary transfers, don't forget to customize the QZ*.BAT files.

18.24.3 TNC Data Bits

You can set the number of data bits on the RS-232 line between

the PC and TNC to 7 or 8. Use 7 with a TNC2 if you have problems

with Greek characters. Set this value to either 7 or 8.

18.24.4 TNC Parity

Use this parameter to set the Parity bit on the RS-232 line

between the PC and TNC. Enter the letter corresponding to the

desired parity. N = None, O = Odd, E = Even, M = Mark, S = Space.

18.24.5 Number of TNC Stop Bits

You can set the number of stop bits on the RS-232 line between

the PC and the TNC to 1 or 2. Use 1 at all terminal (not packet)

baud rates except 300. Set this value to 1 or 2.

18.25 Change Colors

This option lets you change the default colors. A display of all

of the combinations is shown on the left side of the screen. You

will then be prompted to sequence through each window/message

color that can be customized. Enter the number shown in the

combination window followed by the "Enter" key to set a value. If

you don't wish to make a change, just press the "Enter" Key.

The following identifications are made to the color choices:

18.25.1 OutWindow Color

This is the color of the text (typed at the keyboard) in the

outgoing window.

LAN-LINK.DOC (c) G3ZCZ 1988 - 1994

Appendix 2 Page 90

18.25.2 StatusWindow Color

This is the color of the Status Window.

18.25.3 PromptWindow Color

This is the color of most of the menus.

18.25.4 LogWindow Color

This is the color of the text in the log window.

18.25.5 InWindow Color

This is the color of the text in the window showing the incoming

characters.

18.25.6 ScrollBack Window Color

This is the color of the text in the Scroll back window.

18.25.7 MHWindow Color

This is the color of the text in the monitor heard calls (MH)

window.

18.25.8 Alarmwindow Color

This is the color of the text in the Error and Alarm window.

18.25.9 BottomWindow Color

This is the color of the text in the prompt line at the bottom of

the screen.

18.25.10 Emphasis Color

This is the color of the emphasized text in the Menu windows.

18.25.11 Prompt Color

This is the color of the prompt text in the Menu windows.

18.25.12 Option Color

This is the color of selected option text in the Menu windows.

18.25.13 Connected Text Color

This is the color of the text in the incoming window when

LAN-LINK.DOC (c) G3ZCZ 1988 - 1994

Appendix 2 Page 91

connected in the packet communications modes.

18.25.14 Computer Color

This is the color of the computer generated text in the outgoing

window when the Command Echo Flag option is enabled.

18.25.15 VHF Header Color

This is the color of the VHF Header lines in the Incoming window

when not connected.

18.25.16 HF Header Color

This is the color of the HF Header lines in the Incoming window

when not connected.

18.25.17 Alert Call Color

This is the color of the packet header lines containing the Alert

call in the Incoming window when not connected. It is also the

color of the non-permanent Parameter Menu prompts.

18.25.18 Outgoing Text Color

This is the color of the outgoing text echoed in the Incoming

window in the AMTOR Communications Mode.

18.25.19 Multi I/O Stream Color[1]

This is the color of the text lines in the Incoming window in the

Multiconnect Mode when connected to I/O stream 1.

18.25.20 Multi I/O Stream Color[2]

This is the color of the text lines in the Incoming window in the

Multiconnect Mode when connected to I/O stream 2.

18.25.21 Multi I/O Stream Color[3]

This is the color of the text lines in the Incoming window in the

Multiconnect Mode when connected to I/O stream 3.

18.25.22 Multi I/O Stream Color[4]

This is the color of the text lines in the Incoming window in the

Multiconnect Mode when connected to I/O stream 4.

LAN-LINK.DOC (c) G3ZCZ 1988 - 1994

Appendix 2 Page 92

18.25.23 Multi I/O Stream Color[5]

This is the color of the text lines in the Incoming window in the

Multiconnect Mode when connected to I/O stream 5.

18.25.24 Multi I/O Stream Color[6]

This is the color of the text lines in the Incoming window in the

Multiconnect Mode when connected to I/O stream 6.

18.25.25 Multi I/O Stream Color[7]

This is the color of the text lines in the Incoming window in the

Multiconnect Mode when connected to I/O stream 7.

18.25.26 Multi I/O Stream Color[8]

This is the color of the text lines in the Incoming window in the

Multiconnect Mode when connected to I/O stream 8.

18.25.27 Multi I/O Stream Color[9]

This is the color of the text lines in the Incoming window in the

Multiconnect Mode when connected to I/O stream 9.

18.25.28 Multi I/O Stream Color[10]

This is the color of the text lines in the Incoming window in the

Multiconnect Mode when connected to I/O stream 10.

When you change the colors, LAN-LINK does not overwrite all

existing windows. If you want to be sure of seeing the changed

colors, save the color changes using the Update option, and then

exit from and reload LAN-LINK.

18.26 Change SAREX Parameters

This sub-menu allows you to change the SAREX Ground Station

parameters, as described in the following paragraphs.

18.26.1 SAREX CallSign

This is the callsign of the SAREX station.

18.26.2 Change SAREX Capture to Disk File

This option lets you change the name of the SAREX Capture- to-

disk file. Don't change file names while "capture-to-disk" is in

progress (when a flashing "[S]" in present in the status window),

or the old one will stay open and may not close properly when

LAN-LINK.DOC (c) G3ZCZ 1988 - 1994

Appendix 2 Page 93

LAN-LINK is terminated, resulting in data loss.

18.27 Update Configuration File

Use this option to save the changes (permanently) in the

configuration file (LAN-LINK.SYS). Activate it twice to see most

of the changes.

18.28 Verify Changes

This feature lets you display how LAN-LINK was configured by the

LAN-LINK.SYS file. It also displays the state of any flags you

have set/reset if they are not already shown in the Status window

or Parameter Menu.

18.29 Change Scan Words

In the ZAP PBBS feature you can ask the program to scan the lines

read from the PBBS and if it sees any of the scan words in the

line, it will queue a request for that bulletin by number. You

can only set 16 such Scan words (should be enough) and request up

to 255 messages/bulletins at a time.

Color in the scan words. The colors shown in the Scan Word window

match the colors the lines containing those scan words are

written in during a BBS Zap. The colors are the multi stream

colors.

Use the "Delete" key to delete a word from the list.

Words that show up in the PBBS prompt line cannot be used as scan

words. For example, if your PBBS prompts you with

N4QQ PBBS>

then "N4QQ" and "BBS" should not be used as Scan Words.

Don't leave a blank line in the Scan Word section of the

configuration file.

18.30 Turn Miscellaneous Flag ON/OFF

The Miscellaneous Flag performs several different functions in

the different Communications modes as follows.

18.30.1 All Modes:- Debug or state-sequence numbers

Shows two debug or state-sequence numbers in left hand corner of

the status window in the format N-M]. N is the BBS State and M is

the Communications State. These are used to figure out what went

wrong in the automatic sequences. If an automatic sequence fails

LAN-LINK.DOC (c) G3ZCZ 1988 - 1994

Appendix 2 Page 94

to work, set the miscellaneous flag, and observe the state in

which LAN-LINK seems to hang. It will be waiting for something,

and you should get a clue as to what, from Table 18.30.1. BBS

State (N) numbers are shown in Table 18.30.1-A and the

Communications states are shown in Table 18.30.1-B.

Table 18.30.1-A BBS States

1    Connected to BBS waiting for first '>'

2    Receiving incoming messages

3    Receiving list of new messages

4    Receiving your messages (in case missed some)

5    Custom command (i.e. KM)

6    LAN-LINK QTC Snatch in progress

7    Receiving Mail going to YOURCALL.OUT

8    :QDB: waiting for QRV/QNO

9    :QDB: sending file waiting for QSL/QNO

10   AUto UPload Opening BBS.BBS command file

11   AUto UPload sending one line command

12   AUto UPload sending subject line of message

13   AUto UPload sending message

14   AUto UPload waiting for last '>' before disconnecting

22   Receiving file

25   Packet cluster connect waiting for first '>'

26   Packet cluster connect waiting for first '>'

Table 18.30.1-B Communications States

0    Disconnected packet mode or normal non packet mode.

1    Connected in packet mode

2    Trying to connect in packet mode

3    Trying to disconnect in packet mode

4    Sending mail file in packet mode

5    ARQ called someone, or in QSO

6    Connected in autocq mode

7    Calling autocq in packet

8    Sending file in autocq

9    Waiting for reply in autocq packet

10   Autocq non packet sending CQ message

11   Autocq non packet waiting for reply

12   SELCAL waiting for ZCZC or YOURCALL

13   SELCAL waiting for NNNN

14   Autocq AMTOR sending bragtape

15   Autocq AMTOR receiving message

16   Autocq AMTOR requesting repeat of his_call

17   Autocq MFJ1278/KAM waiting for 73 to complete and sign off

21   QRU mode packet

22   Calling someone in pactor

25   AMTOR sending :QQQ: response

26   QRU mode packet

LAN-LINK.DOC (c) G3ZCZ 1988 - 1994

Appendix 2 Page 95

18.30.2 BAUDOT Mode:- Navy MARS Protocol Verify

When the flag is set, and the BAUDOT Navy MARS option is enabled,

the character string ":CR:" will be displayed in the outgoing

window each time a carriage return character is received in the

incoming window. Similarly, every time a line feed character is

received, a ":LF:" will be displayed in the incoming window.

You may use this feature to verify that another station is

sending files using the correct protocol. If you do, make sure

that you tell the PK-232 not to add a line feed to incoming

carriage returns by setting the ALFD parameter to OFF in the Com-

mand Mode. You will have to set "ALFD ON" again when you change

to other communications modes, or all incoming lines will

overwrite each other.

18.31 Change Band/Mode

This option is used to set the band and radio mode without

invoking the Log Menu. You can use it when changing bands on HF

before logging a new QSO. The value is not remembered when you

shut LAN-LINK down, unless you log the QSO.

18.32 Zap PBBS on Next Mail Beacon

This parameter causes LAN-LINK to Zap your local PBBS next time

it beacons with a list of calls for which it has Mail.

18.33 PBBS Zap, Don't Disconnect After Sequence

This parameter stops LAN-LINK from issuing the automatic

disconnect when the Zap sequence is over. You can set it during

the Zap sequence. Use it to stop the disconnect if the sequence

gets out of sync with the messages, or if there is something

interesting you wish to do to the PBBS. This parameter resets

when the Zap sequence terminates.

18.34 Control Send of LAN-LINK.OUT

THe LAN-LINK.OUT file is a file that you can configure LAN-LINK

to transmit to an incoming connect (packet, AMTOR, Pactor and G-

TOR modes). Think of this file as a CTEXT File. If this flag is

set, the file will be transmitted. If this flag is OFF, the file

will not be transmitted. Use this file as a welcome or help file

to other stations when using LAN-LINK in a special event station.

19.0 Q Codes [NC/L]

This option displays help information about the use of the

LAN-LINK.DOC (c) G3ZCZ 1988 - 1994

Appendix 2 Page 96

Network Communications Language (NC/L) "Q" codes.

20.0 COMMUNICATIONS MODES

This menu allows you to change the communications mode of the

TNC. If your TNC hardware cannot support those modes, don't

bother to try them. This is software not hardware, and in spite

of what people tell you, software can't do everything. The commu-

nications modes shown in Figure 20 are available. Type in the

single letter to make your choice. While the KAM ports are set,

and the MFJ-1278 ports are defaulted in LAN-LINK (1 = VHF, 2 =

HF), don't forget that YOU HAVE to select the correct radio port

on the PK-232.

Figure 20 PK-232 Communications Mode Menu

V VHF PACKET

H HF PACKET

A AMTOR (STANDBY)

B BAUDOT (RTTY)

C MORSE (CW)

M AMTOR (MONITOR)

P PACTOR (STANDBY)

Q PACTOR (MONITOR)

S ASCII (RTTY)

N NAVTEX

G SIGNAL/G-TOR

X Manual

20.1 VHF PACKET

This is the 1200 baud VHF Packet Mode. LAN-LINK will optimize the

TNC link parameters for this mode.

20.2 HF PACKET

This is the 300 baud HF Packet Mode. LAN-LINK will optimize the

TNC link parameters for this Mode.

20.3 ASCII (RTTY)

This is the ASCII RTTY Mode. LAN-LINK will configure the TNC

parameters for this Mode at 110 baud.

20.4 BAUDOT (RTTY)

This is the BAUDOT RTTY Mode. LAN-LINK will configure the TNC

parameters for this Mode at 45 baud.

LAN-LINK.DOC (c) G3ZCZ 1988 - 1994

Appendix 2 Page 97

20.5 MORSE (CW)

This is the CW Mode. LAN-LINK will configure the TNC parameters

for this Mode at the default CW speed (20 for the PK-232 and MFJ-

1278, 10 wpm for the KAM).

In this Mode the ALT-D Key will disengage the receive speed lock

inside the PK-232 to facilitate locking onto another signal and

Function Key 8 (F8) will turn the transmitter off.

LAN-LINK has a flag in the configuration file that lets you

configure the XMITOK parameter in the TNC. This flag allows you

to set XMITOK to "OFF" when you enter the CW communications mode,

and turn it "ON" again when you exit from the CW communications

mode.

20.6 AMTOR (STANDBY)

When you select the AMTOR Standby Mode you are placed into the

"Standby Receive" Mode. If you are in AMTOR Standby Mode, you

will not be able to display signals of the Chirpcopy Mode (which

means that you will not be able to display a QSO in progress).

The Alt-D Key and Function Key 8 (F8) both toggle between the

AMTOR STANDBY and MONITOR communications modes.

20.7 AMTOR (MONITOR)

If you want to read the mail on an existing AMTOR QSO in

progress, select the AMTOR (MONITOR) or Chirpcopy Mode.

The Alt-D key and Function key 8 (F8) both toggle between the

AMTOR STANDBY and MONITOR communications modes.

20.8 PACTOR (STANDBY)

When you select the Pactor Standby Mode you are placed into the

"Standby Receive" Mode. If you are in Pactor Standby Mode, you

will not be able to display signals from a QSO in progress.

The Alt-D Key and Function Key 8 (F8) both toggle between the

Pactor STANDBY and MONITOR communications modes.

20.9 PACTOR (MONITOR)

If you want to read the mail on an existing Pactor QSO in

progress, select the Pactor STANDBY or Chirpcopy Mode.

The Alt-D key and Function key 8 (F8) both toggle between the

Pactor MONITOR and STANDBY communications modes.

LAN-LINK.DOC (c) G3ZCZ 1988 - 1994

Appendix 2 Page 98

20.10 SIGNAL

Use this option to select the PK-232 SIGNAL Mode. Read the PK-232

documentation carefully to understand how this mode works. Once

the PK-232 has identified the digital signal it is tuned to, use

the Communications Mode Select Menu a second time to select the

relevant mode and then set the baud rates and shift to display

the data if necessary.

20.11 NAVTEX

Use this option to select the NAVTEX Mode. Read the TNC

documentation carefully to understand how this Mode works.

20.12 G-TOR

If you are using a KAM, you may use this option to select the G-

TOR Mode. Read the TNC documentation carefully to understand how

this Mode works. If you are not using a KAM, this option will not

show up on your menu.

20.13 MANUAL

This option selects the HF logbook and copies the previous mode

entry in subsequent logging. Use this mode to log SSB or FM

contacts when monitoring the PacketCluster and working stations

spotted therein.

21.0 TERMINAL MODE MENU

This menu allows you to set the Terminal Configuration Mode. The

Terminal Menu provides different choices depending on which

communications mode you have selected.

21.1 Packet Terminal Menu

The Packet Terminal Mode of operation will be shown in the Status

window at the top of the screen. If you are reading the mail on a

station, the callsign of that station will be shown next to the

word "MAIL". If you are connected to someone, the call (and path)

will be shown prefixed by the "*>" sequence.

In the Packet Communications Mode, you are presented with the

choices shown in Figure 21.1. Type in the single letter to make

your choice.

21.1.1 Change Packet Baud Rate

In the PACKET Mode you may change the RF baud rate between 300,

LAN-LINK.DOC (c) G3ZCZ 1988 - 1994

Appendix 2 Page 99

600, 1200 and 2400 baud. While LAN-LINK allows these baud rates,

the TNC may not.

21.1.2 CQ/Beacons

In this Mode, you will see CQ and BEACON packets on the channel.

This Mode only works on the PK-232 and the KAM.

Figure 21.1 Packet Terminal Menu

B Change Packet Baud Rate

C CQ/Beacons

E Everything

I Information

L Turn MCON ON

M Read the Mail

O Turn Meta Beacon ON

S Just My Station (Solo)

T All Traffic

0 Multiple User Connects

F Turn Conference ON

21.1.3 Everything

You can set the TNC Mode to "Everything", i.e. show all packets

on the LAN.

21.1.4 Read the Mail

You can read the mail on a PBBS or other station and get

bulletins without connecting to that station yourself. You use

this Mode to do so. You will be prompted for the call whose

packets you wish to monitor. Remember to enter the call of the

station sending the packets, not the call of the one receiving

them. This mode only works without showing the packet headers on

the PK-232. On the other TNCs the headers will also be displayed.

You can also use this command to monitor both sides (read the

mail) on a QSO on a busy packet channel.

You may want to turn the Digipeat Data Display Mode off when

reading the mail.

21.1.5 Just My Station (Solo)

In this Mode, you will only see messages addressed to you. You

will only get messages from people who connect to you.

21.1.6 All Traffic

In this Mode you will see all of the data packet traffic on

LAN-LINK.DOC (c) G3ZCZ 1988 - 1994

Appendix 2 Page 100

channel. You can use this Mode to check that the TNC is working.

21.1.7 Information Packets

In this Mode you will see all of the packet traffic on channel

that contain information. You can use this Mode to check that the

TNC is working.

21.1.8 Single User Connects

This is the normal one-on-one Mode of operation (USERS 1).

21.1.9 Multiple User Connects

When you are in a Multiple Connect Mode, an asterisk will be

shown in the status window next to the word "PACKET". Each I/O

stream will show up in the Inwindow in a different color.

LAN-LINK supports two multiple connect modes, Individual and

Conference.

An extract of an actual "off the air" CRT display (showing

received text only) in a multi-user mode is shown below. Each

line of text as received by the TNC is prefixed (by the PK-232)

with the I/O channel number and callsign of the station its

connected with.

1:4X4IL: *** CONNECTED to 4X4IL via 4X4IC, 4X6JP

0:4X4IC: joe do u read?

2:4Z4ZB: *** CONNECTED to 4Z4ZB

1:4X4IL: Look for me on PR via 4X6JP 73s and Shalom.

2:4Z4ZB: GE JOE I WILL DIS THEN U CAN GO VIA THE REP>

1: *** DISCONNECTED: 4X4IL via 4X4IC, 4X6JP

0:4X4IC: UNPROTO CQ

2:4Z4ZB: GO AHEAD BOYS...DE ALON..>

0:4X4IC:

2: *** DISCONNECTED: 4Z4ZB

This is the normal Multi-connect Mode as described in the TNC

manual. Here you are connected to up to 10 stations and will send

different traffic to each of them. Each time you wish to send

something to a particular station, you must select the I/O

channel the station is connected on before typing the text or

sending the file.

The default stream change character for all TNCs in the VHF

multiple connect modes is "|" ($7C). In the KAM, it is "\" on HF

and "|" on VHF. They are both on the same key but one is in the

shift case. Be careful on HF as the "\" shows up in file

directories and confuses the computer. These values can be

LAN-LINK.DOC (c) G3ZCZ 1988 - 1994

Appendix 2 Page 101

customized. However to do so, they must be customized both in

LAN-LINK and in the TNC at the same time.

You select the I/O channel using the Alt-N key and moving the

cursor to the desired stream, or by typing the "|" character

followed by (in the TNC) the channel letter/number. For example

to do it manually, if you are connected to G8BTB on I/O channel 0

and to G3ZCZ on I/O channel 1, then to talk to G8BTB you would

type "|0" before any text, and similarly to send something to

G3ZCZ you would type a "|1" before the text you"re going to send

me. If you are not using a PK-232, read the TNC manual to deter-

mine if the I/O streams are identified by numbers or by letters.

The TNC2 standard configuration uses letters ("|A" etc.).

Use the Alt-N key and move the cursor up and down to select the

I/O channel automatically.

21.1.10 Conference

In the Conference Mode on the other hand, everything that you

type at the keyboard is automatically transmitted to each station

that you are connected with. Thus if you are linked to two

stations each line will be packeted twice by the TNC. You don't

have to worry about sending the wrong thing to the wrong person,

as they will all get the stuff.

You enter this Mode by selecting the "F" option in the Terminal

Menu. You terminate this Mode by selecting the "1" option.

You connect in the usual manner using Alt-C or the Connect Option

in the Call Menu. You must wait for a connection to be

established, or to time out, before attempting to connect to the

next station. If you do not, you will get an error message back

from the TNC.

In the Conference Mode, as the same text is transmitted on all

I/O channels, the throughput will be reduced. Slow down your

typing speed or use short packets. Should you check the link

status or call monitored list, you may not be able to get back

into the Converse Mode if you have too many packets outstanding.

If that happens, wait a while and then try to set the Converse

Mode with Function Key 10 (F10). You may have to try it a few

times, depending on how good your links are.

Don't try to send a file in the conference mode, it is not

designed for it. Keep the packets short (use short lines). Be

also warned that if you use the conference bridge on bad links

(such as crowded HF channels, or VHF/UHF channels with lots of

retries, the keyboard may seem to lock up. This is because LAN-

LINK has to wait for space to appear in the TNC buffer before it

LAN-LINK.DOC (c) G3ZCZ 1988 - 1994

Appendix 2 Page 102

can output the text.

The Conference Mode will only work in the PK-232 if DCDCONN is

set to ON. So if you are using a PK-232, LAN-LINK will do it for

you when you enter the Conference Terminal Mode in case you

forgot or/and haven't set the LAN-LINK.SYS file to include the

line "DCD ON". At the same time set LAN-LINK.SYS default states

to show the calls of other stations on each incoming line in the

Multiconnect modes (read the TNC manual on how to do this).

The Conference Mode will only work in the TNC2 if DCDCONN is

jumpered so that Pin 8 of the RS-232 Interface changes state when

you connect to someone else. If you get error messages and see

the characters "|J" on your screen, then the jumper probably

isn't set. The Conference Mode does not work on the TNC1.

In the Conference Mode, if any one of the stations disconnects,

the TNC will return to the Command Mode, and you must use

Function Key 10 (F10) to return to the Converse Mode before you

type another character of text.

21.1.10 Conference Bridge ON/OFF

This option toggles the Conference Bridge ON and OFF. In the

Bridge Multiconnect Mode you are acting as a central switch. You

can link together up to 10 stations in one multi-way connection.

In this Mode any packets arriving on any stream are echoed to all

other streams that you have "connects" on except the one that

they came in on. Anything you type will be sent to all connected

streams.

In this manner, all stations will not see their own text echoed

back. Each line of text output by LAN-LINK in this Mode will

contain the callsign of the station originating the packet. Your

packets will contain your call as entered in line 1 of the LAN-

LINK.SYS file. Thus for example, if your call was KB8RP/4X and

normally shows up in the packet header as KB8RP here it will show

in the data as KB8RP/4X.

Just remember that the Bridge puts out a lot of packets, so do

multi-way connects on a quiet frequency. You must be in the

Conference Mode to turn the bridge on, and any disconnect will

take the bridge down. Should that happen, toggle it on again. A

bridge will be shown in the status window whenever the Conference

Bridge is up.

21.1.11 Turn Metabeacon ON/OFF

This option lets you turn the LAN-LINK Metabeacon ON and OFF.

LAN-LINK provides you with a metabeacon, that gives you the

LAN-LINK.DOC (c) G3ZCZ 1988 - 1994

Appendix 2 Page 103

capability to transmit a long beacon message at periodic

intervals. The Metabeacon is loaded from the contents of one of

the LAN-LINK.00n files. These files may be created by the LAN-

LINK editor. When loading, LAN-LINK will insert a pass character

(^V) into the text before a carriage return, so as not to end the

packet before at least 240 characters, and not more than 255

characters have been output to the TNC. The maximum number of

characters that can be loaded into the Metabeacon is 1776. The

maximum number of packets in a Metabeacon is 7. You can also

force a hard carriage return or "end of packet" in the Metabeacon

by inserting the "\" (backslash) character, which will not be

transmitted, into the text.

The frequency of Metabeacon transmissions is the same as that of

the automatic CQ timer.

The MetaBeacon is designed for special event stations. PLEASE do

not abuse it.

21.2 AMTOR Terminal Menu

In the AMTOR Communications Mode, you are presented with the

choices shown in Figure 21.2. Type in the single letter to make

your choice.

Figure 21.2 AMTOR Menu

L Force LETTERS Case

K Turn SELCAL ON/OFF

R Set ROBOT/Mailbox

W CW Identification

Z Set ROBOT Infinite Delay

21.2.1 Force LETTERS Case

This option lets you force the PK-232 receiver to the letters

case if for some reason you appear to be in the numbers case. It

will return you to the Converse Mode.

If your TNC allows you to use a control character (^L) to force

the letters case, you don't need to go through all these menu

layers to do the job.

21.2.2 SELCAL ON/OFF

This command key toggles the SELCAL feature ON and OFF. When the

SELCAL is activated, as shown in the status window, the incoming

text from the TNC is scanned. When either the letters "ZCZC" or

"your callsign" (as entered in line 1 of the LAN- LINK.SYS file)

is received, the "capture-to-disk" is turned on. All subsequent

LAN-LINK.DOC (c) G3ZCZ 1988 - 1994

Appendix 2 Page 104

text will be stored in the LAN-LINK.RUN file until the letters

"NNNN" are received. At that time the "capture-to-disk" is turned

off.

21.2.3 Set ROBOT/Mailbox

This option lets you turn on the Mailbox during a regular AMTOR

QSO so that the other station can leave a message or download a

file remotely. You must only use it when the other station is

sending text.

21.2.4 CW Identification

This option only works on the PK-232, PK-900 and DSP 2232. You

use it to send a CW identification (CWID). The CWID is sent in

the FEC Mode. The CWID consists of the sequence "DE YOURCALLSIGN"

where YOURCALLSIGN is that entered into the PK-232 as "MYCALL".

You use this feature if your license requires you to identify in

Morse code as well as in the operating mode.

21.2.5 Set ROBOT Infinite Delay

You use this option to prevent the Robot from ever transmitting

when not linked. In the normal manner, the Robot counts down the

CQ delay parameter, and, when it gets down to 0, puts a CQ call

on the air. If the Infinite Delay is set, the CQ call will be

inhibited. This option thus puts the Robot into an indefinite

wait mode where it thinks it has sent out a CQ call and is

waiting for a reply.

You would use this Mode if you want to set up a MailBox on a

known frequency for some group or organization.

21.3 Baudot Terminal Menu

In the Baudot Communications Mode, you are presented with the

choices shown in Figure 21.3. Type in the single letter to make

your choice.

21.3.1 Set Baud Rate

This feature allows you to set a desired communications baud rate

directly. You will be prompted to enter the desired rate. Use

this command to avoid having to shift through too many intermedi-

ate states, if for example, you want to make a direct change from

45 to 100 baud.

LAN-LINK.DOC (c) G3ZCZ 1988 - 1994

Appendix 2 Page 105

Figure 21.3 BAUDOT Menu

B Set Baud Rate

F Select Shift

N Turn NAVY MARS Message Protocol ON

U USOS OFF

K Turn SELCAL ON/OFF

W CW Identification

21.3.2 Select Shift

This feature lets you select Wide (850 Hz) or narrow (170 Hz)

shift directly. The PK-232 will copy 170 Hz signals using its

internal 200 Hz circuitry. If you have a KAM, you may also select

the Commercial Shift (425 Hz).

The shift is displayed in the status window at the top of the

screen. It is shown as "W"", "N" or "C" for wide, narrow or

commercial respectively.

21.3.3 NAVY MARS Message Protocol

The Navy MARS option toggles the Navy MARS Mode ON and OFF. When

the Navy MARS option is active, an "[N]" will be displayed in the

status window, and the feature performs the protocol described

below.

21.3.3.1: Any Carriage Return or Enter (CR) from the keyboard is

sent as CR, CR. (two sequential CR characters.

21.3.3.2: In a file transmission lines are terminated by a CR,

CR, LF (Line Feed) sequence.

21.3.3.3: In a file transmission, blank lines are sent as a LF

(without a CR).

21.3.3.4: In a file transmission, a plus sign "+" in the file is

replaced by a sequence of 12 "letters" characters.

There can be more than one + sign on a line.

21.3.3.5: In a file transmission, the sequence "NNNN" is trans-

mitted followed by a sequence of 12 "letters" charac-

ters. There can only be one "NNNN" sequence on a line.

21.3.4 USOS ON/OFF

This option toggles the Unshift on space (USOS) feature ON and

OFF. When the USOS is on, the TNC reverts to the letters shift

each time a "space" character is received. It helps when

displaying noisy copy.

If USOS is active, "USOS" will be shown in the status window at

the top of the screen.

LAN-LINK.DOC (c) G3ZCZ 1988 - 1994

Appendix 2 Page 106

21.3.5 SELCAL ON/OFF

This command key toggles the SELCAL feature ON and OFF. When the

SELCAL is activated, as shown in the status window, the incoming

text from the TNC is scanned. When either the letters "ZCZC" or

"your callsign" (as entered in line 1 of the LAN-LINK.SYS file)

is received, the "capture-to-disk" is turned on. All subsequent

text will be stored in the LAN- LINK.RUN file until the letters

"NNNN" are received. At that time the "capture-to-disk" is turned

OFF.

You may use this feature when monitoring commercial RTTY

broadcasts to record news and other information, or when calling

CQ to start recording all incoming text and keep the QSO on disk.

You may use Function Key 1 (F1) to turn the capture to disk

permanently on to override the automatic turnoff if you wish.

21.3.6 CW Identification

This option is used to send a CW identification. You use this if

your license requires you to identify in Morse Code as well as in

the operating mode.

The identification consists of the sequence "DE YOURCALLSIGN"

where your callsign is that entered into the TNC as "MYCALL".

21.4 ASCII Terminal Menu

In the ASCII Communications Mode, you are presented with the

choices shown in Figure 21.4. Type in the single letter to make

your choice.

Figure 21.4 ASCII Menu

B Set Baud Rate

F Select Shift

K Turn SELCAL ON

W CW Identification

21.4.1 Set Baud Rate

This feature allows you to set a desired communications baud rate

directly. You will be prompted to enter the desired rate. Use

this command to avoid having to shift through too many intermedi-

ate states. For example, if you want to make a direct change from

110 to 1200 baud.

LAN-LINK.DOC (c) G3ZCZ 1988 - 1994

Appendix 2 Page 107

21.4.2 Select Shift

This feature lets you select Wide (850 Hz) or narrow (170 Hz)

shift directly. The PK-232 will copy 170 Hz signals using its

internal 200 Hz circuitry. If you have a KAM, you may also select

the Commercial Shift (425 Hz).

The shift is displayed in the status window at the top of the

screen. It is shown as "W", "N" or "C" for wide, narrow or

commercial respectively.

21.4.3 SELCAL ON/OFF

This command key toggles the SELCAL feature ON and OFF. When the

SELCAL is activated, as shown in the status window, the incoming

text from the TNC is scanned. When either the letters "ZCZC" or

"YOURCALLSIGN" (as entered in line 1 of the LAN-LINK.SYS file) is

received, the "capture-to-disk" is turned on. All subsequent text

will be stored in the LAN-LINK.RUN disk file until the letters

"NNNN" are received. At that time the "capture-to-disk" is turned

off.

You may use Function Key 1 (F1) to turn the capture to disk

permanently on to override the automatic turnoff if you wish.

21.4.4 CW Identification

This option is used to send a CW identification. You use this if

your license requires you to identify in Morse code as well as in

the operating mode.

The identification consists of the sequence "DE YOURCALLSIGN"

where your callsign is that entered into the TNC as "MYCALL".

21.5 CW Menu

In the CW Communications Mode, you are presented with the Menu

shown in Figure 21.5. Type in the single letter to make your

choice.

Figure 21.5 CW Menu

L Lock to Receive Signal

S Set Morse Speed

U Unlock Morse Speed

X Turn XMITOK Flag ON

21.5.1 Lock to Receive Signal

This option locks the TNC to the speed of the incoming CW signal.

LAN-LINK.DOC (c) G3ZCZ 1988 - 1994

Appendix 2 Page 108

21.5.2 Set Morse Speed

This option lets you set the CW speed.

21.5.3 Unlock Morse Speed

This option unlocks the TNC from the incoming CW signal. It

performs the same task as the Alt-D option keys. It is used to

speed up capture of a new signal or to break the lock when the

TNC appears to have locked up at the wrong speed.

21.6 Pactor Terminal Menu

Pactor applies to the KAM and PK-232 only, assuming they contain

Pactor firmware. In the Pactor Communications Mode, you are

presented with the choices shown in Figure 21.6. Type in the

single letter to make your choice. Do not use any of these

options when linked to another station or you may drop the link.

21.6.1 AUTO BAUD

This option configures the TNC to switch between 100 and 200 baud

on the HF Pactor link as determined by propagation Conditions.

Figure 21.6 Pactor Menu

0 AUTO BAUD

1 Force 100 BAUD

2 Force 200 BAUD

H Enable Huffman

K Disable Huffman

21.6.2 Force 100 Baud

This option forces the TNC to use 100 baud on the HF Pactor link

as determined by propagation Conditions.

21.6.3 Force 200 Baud

This option forces the TNC to use 200 baud on the HF Pactor link

as determined by propagation Conditions. This option is only

available for the KAM.

21.6.4 Enable Huffman

This option enables Huffman compression of the transmitted text.

LAN-LINK.DOC (c) G3ZCZ 1988 - 1994

Appendix 2 Page 109

21.6.5 Disable Huffman

This option disables Huffman compression of the transmitted text.

22.0 EXIT TO DOS

When you exit the program and return to DOS, if the Shut down

(QRT) flag is set to 0, the TNC is put into the VHF default mode

in which it rejects calls from any other stations. (Read about

the CFROM and/or CONOK command in the TNC manual). You can, if

you wish, leave the TNC powered, and, if you also leave the VHF

radio powered up, you will be active as a digipeater, but at the

same time anyone trying to connect with you will receive a "busy

signal". The MBX command (Reading the Mail) is also cleared at

this time.

If you want to leave the TNC set up to accept messages when LAN-

LINK is not active, customize the shut down state as described in

the Section 18.23.

23.0 THE ELMER MENU

ELMER is a smart server or second operator feature described in

Appendix 7. The ELMER Menu presents you with the choices shown in

Figure 23. Type in the single letter to make your choice.

Figure 23 The ELMER Menu

C Change ELMER File

E Turn ELMER ON/OFF

L Load ELMER.QSO

P Pick File to edit

S Edit ELMER.QSO

T show state Table

23.1 Change ELMER File

This option allows you to change the ELMER state table file.

23.2 Turn ELMER ON/OFF

This option allows you to turn ELMER ON and OFF. If the ELMER

state table file (ELMER.QSO) is not present, you will not be able

to activate ELMER.

23.3 Load State Table (ELMER.QSO)

This option allows you to reload the state table. The name of the

state table will appear in the menu.

LAN-LINK.DOC (c) G3ZCZ 1988 - 1994

Appendix 2 Page 110

23.4 Pick File to Edit

This option allows you to pick and edit, a file with the default

filetype of "TXT".

23.5 Edit State Table (ELMER.QSO)

This option allows you to edit the state table.

23.6 Show State Table

This option allows you to see the contents of the state machine.

It is used in debugging the logic to ensure that the contents of

the table match what you thought you put into it. A typical set

of contents appears as shown in Figure 23.1. The keywords have

been capitalized and set to 15 characters wide. None existent

characters have been represented as periods.

Figure 23.1 Typical ELMER State Table

L   C   N  R   E      KEYWORD    U       FILE

0   1   2  0   0 WHAT........... 0 *

1   2   3  0   0 ENGLISHMAN..... 0 *

2   2 100  0   0 SYSTEMS........ 0 *

3   3   4  0   0 DOING.......... 0 *

4   4   1  0   0 COUNTRY........ 0 ME-USA.TXT

5   1   1  0   0 EQUIPMENT...... 0 PK232COM.TXT

24.0 RADIO MENU

The Radio Menu provides you with the capability to exercise the

interface to a Kenwood Radio. The options available are shown in

Figure 24-1. Type the single letter to make your choice. All

commands will be ignored if the radio is not enabled. The radio

interface is automatically enabled at LAN-LINK initialization

time.

Figure 24-1 The Kenwood Radio Menu

F Select VFO A/B

M Set Mode

Q Turn Flag QSY ON/OFF or N/A

R Read VFO A Frequency

S Set Frequency

V Speak Frequency

24.1 Select VFO A/B

This option sets the VFO that will be interrogated or set by LAN-

LAN-LINK.DOC (c) G3ZCZ 1988 - 1994

Appendix 2 Page 111

LINK.

24.2 Set Mode

This option sets the radio mode (USB, LSB, FM, CW etc.). You will

be prompted to supply the mode.

24.3 Turn Flag QSY ON/OFF or N/A

This option set the QSY Flag as follows:

N/A  Radio is not enabled.

ON   QSY to the frequency of a DX Alert (if in PacketCluster

mode).

OFF  Don't QSY in response to a DX Alert.

24.4 Read VFO Frequency

This option reads and displays the frequency of the selected VFO.

24.5 Set VFO Frequency

This option sets the frequency of the selected VFO. You will be

prompted to supply the frequency.

24.6 Speak Frequency

This option sends a command to the radio to announce the

frequency of the selected VFO. If the radio is equipped with a

voice annunciator the radio will announce the frequency. If the

radio is not equipped in such a manner, the command will be

ignored.

LAN-LINK.DOC (c) G3ZCZ 1988 - 1994

Appendix 3 Page 112

Appendix 3 LAN-LINK Disk Files

The following disk files are associated with the LAN-LINK

program.

LAN-LINK.COM

LAN-LINK.SYS

*.RUN

LAN-LINK.LOG

LAN-LINK.DIR

CONFIG.SYS

*.MBX

*.BBS

LAN-LINK.001 .. LAN-LINK.010

QBU-RX.BAT

QBU-TX.BAT

LAN-LINK.QTA

QZ*.BAT

ELMER.QSO

LAN-LINK.OUT

*.DMO

The "*" represents a "wildcard" character in the manner of DOS.

Consider each file in turn.

1.0 LAN-LINK.EXE

This is the program. You activate it from DOS by typing the word

"LAN-LINK" followed by "ENTER" in the normal manner of invoking

DOS commands.

2.0 LAN-LINK.SYS

This file contains the configuration information for both the

program and the TNC. Use the customization options in the

parameters menu to make any changes not taken care of in the

INSTALL program.

The contents of the default file together with an item (line)

number are shown below.

1. Callsign

2. Selcal

3. CQ Text

4. CTEXT Line

5. ELMER Flag

6. CQ Delay (seconds)

7. CQ Delay (minutes)

8. :QSP: Message Relay Flag

9. PC Serial Communications Port Number

LAN-LINK.DOC (c) G3ZCZ 1988 - 1994

Appendix 3 Page 113

10. TNC Type

11. PC to TNC Baud Rate

12. VHF Log Book File

13. HF Log Book File

14. GMT Offset

15. Path Directory File name

16. Brag Tape File name

17. PBBS Snatch Mail Trigger Word

18. QTC Snatch Flag

19. PBBS Snatch Flag

20. Shut Down Mode (QRT Flag)

21. PC to TNC Data Bits

22. PC to TNC Parity Bit

23. PC to TNC Stop Bits

24. DCD Flag

25. QBM Download Sub-directory

26. Run File Directory Path

27. PBBS Read Request Command

28. Local PBBS Callsign

29. SAREX Callsign

30. SAREX Capture to Disk File name

31. PacketCluster Call

32. TNC Specific Parameters

33. Node Drop Link Flag

34. Sound Flag

35. Display Computer Generated Commands Flag

36. Date/timestamp in Call Sequence Flag

37. Keyboard Character Beep value (number of characters).

38. :QBU: Receiving Station Batch file (QBU-RX.BAT)

39. :QBU: Originating Station Batch file (QBU-TX.BAT)

40. Capture-to-disk Flag

41. Beacon Control/LAN-LINK handshake Flag

42. Read Bulletin Character

43. PBBS Subject String

44. PBBS Message String

45. VHF Paclen

46. HF Paclen

47. Beacon Time

48. Number of CQ Lines

49. Word wrap

50. PC Port 4 Custom Address

51. PC Port 4 Custom IRQ

52. VHF Stream switch character

53. HF Stream switch character

54. TNC Error message

55. Connect Timeout Delay

56. Custom Prompt

57. ELMER Configuration File

58. Expand Transmitted File flag

59. Input Message Base Count

LAN-LINK.DOC (c) G3ZCZ 1988 - 1994

Appendix 3 Page 114

60. Default users

61-88. Custom Colors

89. First PacketCluster Command

90. Second PacketCluster Command

91. TNC Mailbox ON Command

92. TNC Mailbox OFF Command

93. VHF Maxframe Command

94. HF Maxframe Command

95. Inhibit VHF Logging Flag

96. Inhibit HF Logging Flag

97. Printer Permanent Flag

98. Log Disk Update Flag

99. Secondary Log Flag

100. Auto Answer Flag

101. RUN File Disk Update Flag

102. Default CW Speed

103. Secondary Logbook

104. BBS List Command

105. BBS List Mine Command

106. BBS Custom Command

107. BBS Bye Command

108. Start Up Packet Terminal Mode

109. Page Time (:QIC: Seconds)

110. Packet Cluster DX Alert Tone CW Dit time

111. Packet Cluster DX Alert Tone CW Note

112. Packet Cluster DX Alert Mode

113. Mode File Flag

114. BAUDOT Color Change Text String

115. Default RTTY Baud Rate

116. Dual Window Flag

117. TNC String for Modem 9600 baud packet

118. TNC String for Modem 1200 baud packet

119. TNC String for Modem RTTY

120. TNC String for Modem 300 baud packet

121. TNC String for Modem RTTY and 1200 baud packet

122. TNC String for Modem 300 baud and 1200 baud packet

123. TNC String for Modem Fuji

124. Delay after a command line

125. Radio

126. Radio Port

127. Radio Port baud rate

128. Radio Port Bits

129. Radio Port Stop Bits

130. Radio Port Parity

131. Radio Address

132. Flag QSY

133. Flag DX Alert Prefix Check

134. start of band parameters for DX Alert message log search

135. * end of band parameters; modes for DX Alert message log

search

LAN-LINK.DOC (c) G3ZCZ 1988 - 1994

Appendix 3 Page 115

136. Start of Scan Words (MUST HAVE AT LEAST ONE)

137. * end of Scan Words

138. Start of Common Calls (MUST HAVE AT LEAST ONE)

139. * end of Common Calls

140. Remaining TNC start up Commands.

The contents of each line are described in the section of the

Parameters menu which deals with customizing them. The others,

described below, can only be changed by using the Editor. The

remaining lines contain commands to the TNC. For further informa-

tion about them refer to the TNC manual. LAN-LINK assumes

different defaults (in some instances) to those set in the TNC.

The LAN-LINK.SYS file contains only those that are different to

the TNC default reset state.

2.1 CTEXT Line

This parameter is the CTEXT line changed in the TNC Menu.

2.2 ELMER Flag

This parameter is the ELMER Flag. If it is a "1", ELMER is primed

for activity. ELMER is only active in the Packet communications

mode, when you are connected to another station. If the ELMER

flag is a "0", or the ELMER state table file is not present in

the default directory, LAN-LINK will never activate ELMER.

2.3 PC Port 4 Custom Address

This parameter is the decimal value of the address of your Port

4. Use this parameter if you are using a non standard Serial I/O

card. Configure the PC Port to 4. The PC default address for Com4

is 2E8 (hex) or 744 (decimal).

2.4 PC Port 4 Custom IRQ

This parameter is the decimal value of the IRQ of your Port 4.

Use this parameter if you are using a non standard Serial I/O

card. Configure the PC Port to 4. The PC default IRQ for Com4 is

3.

2.5 VHF Stream Switch Character

This parameter is the VHF stream switch character. If you wish to

change it, you must enter the ASCII representation in this line,

and the equivalent stream switch command in the LAN-LINK.SYS

file.

LAN-LINK.DOC (c) G3ZCZ 1988 - 1994

Appendix 3 Page 116

2.6 HF Stream Switch Character

This parameter is the HF stream switch character. If you wish to

change it, you must enter the ASCII representation in this line,

and the equivalent stream switch command in the LAN-LINK.SYS

file. The KAM is currently the only TNC that has different HF and

VHF stream switch characters.

2.7 TNC Error Message

This parameter is the error message your TNC generates. The PK-

232 generates "?What?". Others may generate "EV?" or "?EH".

2.8 Connect Timeout Delay

This parameter is the time, in minutes, LAN-LINK lets you stay

connected to another packet station, after the receipt of the

last incoming character.

2.9 ELMER Configuration File

This parameter is the name of the default ELMER Configuration

file. It is changed through the ELMER Menu.

2.10 Expand Transmitted Files Flag

The Flag tells LAN-LINK to scan the outgoing text when a file is

transmitted and perform the same conversions it does to the

contest text. You can insert the '##', "#1' and "#T" parameters

in the files to transmit, your call sign, the other station's

call, and the time of day. Set this parameter to 1 to enable the

feature, 0 to disable it.

2.11 First PacketCluster Command

This parameter is the first command (up to 20 characters) LAN-

LINK sends to the PacketCluster after receiving a ">" character

at the end of line from the PacketCluster.

2.12 Second PacketCluster Command

This parameter is the second command (up to 20 characters) LAN-

LINK sends to the PacketCluster after receiving a ">" character

at the end of line from the PacketCluster.

2.13 TNC Mailbox ON Command

This parameter is the command (up to 15 characters) your TNC

expects to turn its internal mailbox ON.

LAN-LINK.DOC (c) G3ZCZ 1988 - 1994

Appendix 3 Page 117

2.14 TNC Mailbox OFF Command

This parameter is the command (up to 15 characters) your TNC

expects to turn its internal mailbox OFF.

2.15 VHF Maxframe Command

This parameter is the command (up to 15 characters) your TNC

expects to set the VHF MAXFRAME parameter.

2.16 HF Maxframe Command

This parameter is the command (up to 15 characters) your TNC

expects to set the HF MAXFRAME parameter.

2.17 Inhibit VHF Logging Flag

When set to 1, automatic VHF Logging is disabled.

2.18 Inhibit HF Logging Flag

When set to 1, automatic HF Logging is disabled.

2.19 Printer Permanent Flag

When clear (set to 0), if you leave the printer on during a

packet connect, it will stop printing when you disconnect. This

stops the printer generating lots of garbage. If you set this

parameter to 1, the printer will not stop printing when you

disconnect.

2.20 Log Disk Update Flag

If this flag is set to 1, LAN-LINK writes each log entry to disk

as you enter it. If this flag is not set (=0), LAN-LINK writes

the log entry to a buffer and updates the disk when the program

is terminated. Set the flag ON to ensure that rare DX contact is

logged, Set it OFF for speed, or if power dropouts are not a

problem in your area.

2.21 Secondary Log Flag

If this Flag is set to 1, LAN-LINK allows you to search both the

current log book and a secondary logbook for the call of a

station.

2.22 Auto Answer Flag

If this flag is set to 1, AND you have a LAN-LINK.OUT file on

your system, LAN-LINK will transmit the contents of the LAN-

LAN-LINK.DOC (c) G3ZCZ 1988 - 1994

Appendix 3 Page 118

LINK.OUT file to every incoming connection.

2.23 Ctdsk File Disk Update Flag

If this flag is set to 1, LAN-LINK will close the capture-to-disk

file (YYMMDD.RUN) between received messages. If the flag is not

set, LAN-LINK will close the file when all messages have been

received, or a DISCONNECT takes place.

2.24 Default CW Speed

This is the default CW speed for the multi-mode TNC (KAM, PK-232

or MFJ1278).

2.25 Secondary Logbook

This is the name of the secondary logbook.

2.26 BBS List Command

This is the BBS List Command. The default value is 'L'.

2.27 BBS List Mine Command

This is the BBS List Mine Command. The default value is 'LM'.

2.28 BBS Custom Command

This is a configurable BBS List Command. The default value is

'KM'.

2.29 BBS Bye Command

This is the BBS Bye Command. Change this parameter when the

default 'B' command only drops you to the Node.

2.30 Start Up Packet Terminal Mode

This is the Packet Terminal Mode when LAN-LINK is first

activated. Set this to the letter corresponding to the terminal

mode you like being in. See Section 21.1. Use the corresponding

letter, as shown in Table 2.30.

Table 2.30 Start Up Terminal Mode

Letter    Mode                              Actions

I         INFO lets you only see packets containing information.

S         SOLO lets you only see messages addressed to you.

C         CQ/B lets you see beacons and CQ calls as well.

T         TRFC lets you see packets containing messages as well.

LAN-LINK.DOC (c) G3ZCZ 1988 - 1994

Appendix 3 Page 119

E         EVRY lets you see everything on channel, including the

link control packets.

2.31 Page Time (:QIC: Seconds)

This is the number of seconds the system pages the operator in

response to a QIC command, if the sound is turned on.

2.32 PacketCluster DX Alert Tone CW Dit time

This is the time period for 1 dit sound in the PacketCluster

audible DX alert message.

2.33 Packet Cluster DX Alert Tone CW Note

This is the tone for the PacketCluster audible DX alert message.

2.34 Mode File Flag

If this flag is set, LAN-LINK will change the name of the LAN-

LINK.001 to LAN-LINK.010 when you change communications modes.

The name will change to the mode followed by a number, i.e.

BAUDOT.001 to BAUDOT.002, or AMTOR.001 to AMTOR.002.

2.35 BAUDOT Color Change Text String

This is a pair of characters that LAN-LINK will scan text

appearing in the incoming window for. When LAN-LINK recognizes

the pair, it will change colors. This parameter is customizable

to allow you to override apparent random changes of color, since

there is no agreed standard way for signaling 'over' in BAUDOT.

2.36 Dual Window Flag

This flag applies to the PK-900 and DSP 2232 only. If this flag

is set, LAN-LINK separates out the text streams coming from each

port and directs them to two separate windows. If the flag is

reset, the text streams are combined into a single incoming

window.

2.37 TNC String for Modem 9600 baud

This item applies to the PK-900 and DSP 2232 only. This is the

command used to select the modem for working 9600 baud packet.

2.38 TNC String for Modem 1200 baud

This item applies to the PK-900 and DSP 2232 only. This is the

command used to select the modem for working 1200 baud packet.

LAN-LINK.DOC (c) G3ZCZ 1988 - 1994

Appendix 3 Page 120

2.39 TNC String for Modem RTTY

This item applies to the PK-900 and DSP 2232 only. This is the

command used to select the modem for working HF RTTY.

2.40 TNC String for Modem 300 baud

This item applies to the PK-900 and DSP 2232 only. This is the

command used to select the modem for working 300 baud HF packet.

2.41 TNC String for Modem RTTY and 1200 baud

This item applies to the PK-900 and DSP 2232 only. This is the

command used to select the modem for working simultaneous HF RTTY

and VHF 1200 baud packet.

2.42 TNC String for Modem 300 baud and 1200 baud

This item applies to the PK-900 and DSP 2232 only. This is the

command used to select the modem for simultaneous HF 300 baud

packet and 1200 baud VHF packet.

2.43 TNC String for Modem Fuji

This item applies to the PK-900 and DSP 2232 only. This is the

command used to select the modem for working Fuji 1200 baud PSK

packet.

2.44 Delay after a command line

This item is the number of milliseconds to wait between sending

lines to the TNC output buffer.

2.45 Radio

This item is the name of the Radio. Kenwood has been tested, Icom

may work (coded but not tested). If this is not set to Kenwood or

Icom, the Radio functions are disabled.

2.46 Radio Port

This item is the serial port for the Radio (1 to 4). If set to 0,

the Radio functions are disabled.

2.47 Radio Port baud rate

This item is the serial port baud rate for the Radio. Check the

correct value in the Radio manual.

LAN-LINK.DOC (c) G3ZCZ 1988 - 1994

Appendix 3 Page 121

2.48 Radio Port Bits

This item is the number of data bits on the serial port for the

Radio. Check the correct value in the Radio manual.

2.49 Radio Port Stop Bits

This item is the number of stop bits on the serial port for the

Radio. Check the correct value in the Radio manual.

2.50 Radio Port Parity

This item is the parity value on the serial port for the Radio.

Check the correct value in the Radio manual.

2.51 Radio Address

This item is ignored for Kenwood radios. For Icom, it is the

decimal value of the radio address. Check the correct value in

the Radio manual.

2.52 Flag QSY

This flag determines of LAN-LINK tunes the radio to the frequency

of the PacketCluster DX Alert message automatically.

2.53 Flag DX Alert Prefix Check

This flag determines how LAN-LINK responds to a DX Alert message.

If the flag is set, LAN-LINK checks to see of the prefix is in

the log.

2.54 Band parameters for DX Alert message log search

These lines contain the band, reference frequency (Hz), low and

high band search flags. They can be changed via the Radio Menu.

Typical values are:

10, 28300000.0,Y,Y

12, 24930000.0,Y,N

15, 21200000.0,Y,Y

17, 18110000.0,Y,Y

LAN-LINK checks the frequency of the DX Alert message to see if

it matches on in these lines. If it does, then LAN-LINK compares

the frequency with the value on the corresponding line to

determine if the DX Alert frequency is higher or lower than the

reference frequency.

If the frequency is lower than the reference, and the low band

search flag is "Y", LAN-LINK will check to logbook to see if the

LAN-LINK.DOC (c) G3ZCZ 1988 - 1994

Appendix 3 Page 122

prefix is in the log.

Similarly, if the frequency is higher than the reference, and the

high band search flag is "Y", LAN-LINK will check to logbook to

see if the prefix is in the log.

If LAN-LINK does not find a match, the check mark is displayed.

2.55 Modes for DX Alert message log search

This line marks the end of the band parameters for DX Alert

message log search with the *. It then contains the

communications modes corresponding to the low and the high parts

of the band. Typical values are:

*,CW,SSB

2.56 TNC Parameters

The remaining lines contain parameters loaded into the TNC at

initialization.

If you are not using a TNC1 you MUST set the HEADERLN parameter

to ON. The packet headers and the text of the packets must show

up on your screen on separate lines. You must also have the "Echo

as Sent" feature set ON ("EAS ON" for PK-232 and "XM" ON " for

the KAM).

3.0 *.RUN

These files contain incoming data from the TNC recorded while the

"capture-to-disk" function was active. LAN-LINK names them as

YYMMDD.RUN automatically (YY = Year, MM = Month, DD = Date, i.e.

920903.RUN is a file for 3 Sept 1992).

4.0 *.DBF (HF.DBF and VHF.DBF)

These file contain the log entries. They are Dbase 3 and 4

compatible. The format of the logbook file is the same as that

used by the LOGBOOK package of the PC-HAM software package, with

a little additional space for comments.

5.0 LAN-LINK.DIR

This is the directory file as described in Appendix 2, Section

6.1.2. You use it to store names, handles, NET/ROM, TheNet, KA-

Node, ROSE or digipeat paths as well as name/callsign relation-

ships. Sample lines are shown below.

LAN-LINK.DOC (c) G3ZCZ 1988 - 1994

Appendix 3 Page 123

Joe G3ZCZ

Alon 4Z4ZB V 4X6AA

Milt 4X6AA

LR 4X6LR

hf-il 4x4hf v 4z4zb 4x4il

hf-rj 4x4hf v 4z4zb 4z4rj

K1HTV ELK v WB4APR-5! SNJ3! NNJ! WMA! SCCT!K1HTV

w9tnn-2 jrw !% w3eax-7 !& w9tnn-2

You create this file with the Editor or by using your word

processor in its non-document mode. You must leave at least one

space character between the first (key) word and the connect

path. When you type in the key word, the program does not care

about upper or lower case.

You may also use the "reMember Path" option in the Call Menu to

make entries in this file.

If you already have a YAPP.DIR or a PK232COM.DIR file just rename

it to LAN-LINK.DIR and add any NET/ROM and/or KA-Node paths.

All path entries in LAN-LINK.DIR must be on a single 80 column

line.

6.0 CONFIG.SYS

You must have a CONFIG.SYS file on the boot disk if you are going

to use the disk logging features. This file must contain the line

FILES = 20.

See the DOS manual for more information about the CONFIG.SYS

file.

7.0 *.MBX

These are the files associated with the stations you have

designated as LAN store and forward mailboxes. Other stations can

only :QRU: with you if you have designated them as QRU stations

and have a "CALLSIGN.MBX" file on your disk. For example, if you

want to use 4Z4ZB as a store and forward mailbox, you will have

to create an ASCII file called 4Z4ZB.MBX using Editor, or the

non-document mode of your word processor. In this file you will

put a list of the call signs of the stations 4Z4ZB can connect

with reliably. For example, if you want 4Z4ZB to be able to store

messages from you to 4X6AA, 4X6LR, G3ZCZ, ON8IK, F0WN,and G8BTB,

you would put those call signs in the file, one call on a line as

follows.

LAN-LINK.DOC (c) G3ZCZ 1988 - 1994

Appendix 3 Page 124

4X6AA

4X6LR

G3ZCZ

ON8IK

F0WN

G8BTB

When the QRU function is invoked either by you manually or by

4Z4ZB connecting to you and sending you the command :QRU:, any

message files present on your system addressed to any of those

stations (E.G. G3ZCZ.OUT and ON8IK.OUT) will be transferred from

you to 4Z4ZB just as if you had tried to :QSP: the messages

manually or as if G3ZCZ or ON8IK had connected directly to you.

The file names on your system will be changed to *.OLD.

You must create one file for each designated store and forward

station, i.e. 4Z4ZB.MBX, G3ZCZ.MBX etc.

8.0 *.BBS

These are LOCAL-BBS.BBS AUTO-Upload Command files. Each file is

named for the callsign of the BBS that you wish to access. It

must have exactly the name as the callsign of the BBS. If the BBS

is W9TNN-1 the name of the file must be W9TNN-1.BBS, or if the

BBS is 4Z4ZB then the file must be called 4Z4ZB.BBS. The file

must be an ASCII file. that means the word processor must be used

in the non-document mode to create it.

This file contains the command sequence that you would have typed

at the keyboard if you were accessing the BBS manually. The ONE

difference is that the text of a message you are uploading with

the S command must be followed by the character strings ":EOF:"

or "/EX" (on a separate line) as illustrated by the example

below.

If you use the "D" command to download a text file, the file will

end up in a separate file on your disk ONLY IF a file of the same

name does not exist on your drive.

Do delete "read" mail from your local PBBS. Don't clutter the

PBBS's disk with your read mail. The "LM" command is built into

LAN-LINK to remind you of what you have out there.

The PBBS "S" command (including its variations such as "SP") is

the only multiple line command allowed. You can put all the

single line commands such as "D", "H", "?", "L", and "R" in the

PBBS.BBS file.

A "multiple command" is defined in this instance as a command

which allows you to enter multiple lines before the PBBS will

LAN-LINK.DOC (c) G3ZCZ 1988 - 1994

Appendix 3 Page 125

send back a ">" character (i.e. one that the PBBS tells you to

type in text and terminate it with the Control-Z (^Z) character).

A typical message could be as shown below.

SP G3ZCZ @ N4QQ

LAN-LINK

I think it is great. However, why don't you change the

following...... (add your comments here).

73 de your name

:EOF: {last line may also be "/EX"}

k 1234

d \net\lan-link.doc

s g8btb

Test Message

This is a test message.

:EOF:

lm

r 567

r 593

9.0 LAN-LINK.001 - LAN-LINK.010

These are 10 different text files with fixed names. They can be

transmitted using the Alternate function keys. Their contents can

be viewed using the Shift function keys. In all cases the

function key to use for a particular file corresponds to the

number in the file suffix. For example, use Alt/Shift Function

Key 3 (Alt-F3) to transmit/view file LAN-LINK.003. You may also

use the Edit Menu to edit and/or create the files.

In case you forget what you put in each of the files, LAN- LINK

allows you to take a peek into all of them at the same time. Use

Control Function key 1 (Control-F1) to look at the first line of

all the LAN-LINK.001 - LAN-LINK.010 files and the Brag Tape.

If the Mode File flag is set, LAN-LINK changes the files as you

change modes. You thus have 10 files in EACH mode, accessible via

the same set of control function keys. The file names are shown

below.

COMMUNICATIONS MODE Mode Flag                FILE NAMES

All                 OFF            LAN-LINK.001 to LAN-LINK.010

Packet              ON             LAN-LINK.001 to LAN-LINK.010

AMTOR               ON             AMTOR.001 to AMTOR.010

BAUDOT              ON             BAUDOT.001 to BAUDOT.010

ASCII               ON             ASCII.001 to ASCII.010

CW                  ON             MORSE.001 to MORSE.010

Pactor              ON             PACTOR.001 to PACTOR.010

G-TOR               ON             G-TOR.001 to G-TOR.010

LAN-LINK.DOC (c) G3ZCZ 1988 - 1994

Appendix 3 Page 126

10.0 QBU-RX.BAT

This is the command or batch file used in the :QBU: originating

station.

11.0 QBU-TX.BAT

This is the command or batch file used in the :QBU: receiving

station.

12.0 LAN-LINK.QTA

You can leave a message on your system for someone else in a file

named CALLSIGN.OUT. When the station with that callsign connects

to you, the message will be sent, and the filename will be

changed from CALLSIGN.OUT to CALLSIGN.OLD. This is fine as long

as the path between the stations good. If the path is marginal,

it is possible that your station will transmit the message and

change the filename, but the link will drop out before the

message is received at the other station. If the other station

does not connect to you and :QSM:, the message may get lost. If

this situation occurs, you can tell LAN-LINK that there is a bad

path between you and that station, and not to change the file

names without an explicit :QTA: instruction from that other

station. You do this by putting the call of the station(s) having

a bad path in an ASCII file called LAN-LINK.QTA. Put one call on

each line in the form

N9BE

G8BTB

You must use the root call of the station without an SSID. that

means, if the station is N9BE-4, just put N9BE in the file.

LAN-LINK will automatically acknowledge incoming messages from

stations with callsigns in the LAN-LINK.QTA file with a ":QTA:".

13.0 QZ*.BAT

As any LAN-LINK station can either receive or originate a :QZD:

and a :QZU: command, LAN-LINK uses four batch files to control

Zmodem transfers. These files are as follows.

QZU-RX.BAT Batch file implemented after receipt of :QZU:

QZU-TX.BAT Batch file implemented after transmission of :QZU:

QZD-RX.BAT Batch file implemented after receipt of :QZD:

QZD-TX.BAT Batch file implemented after transmission of :QZD:

Each LAN-LINK user must configure the QZ*.BAT files for their

LAN-LINK.DOC (c) G3ZCZ 1988 - 1994

Appendix 3 Page 127

systems. The port number in the batch file should be set to the

PC I/O Port used. The data rate in the batch file should be set

to the data rate used between the PC and the TNC, NOT the data

rate used on the radio link.

For example, the default QZU-RX.BAT file of

"pcz 1 1200 rz %1" should be changed to

"pcz 2 1200 rz %1" if you are have interfaced your TNC to Port

2 on your PC. If, the baud rate between the TNC is 2400 baud, the

file should be changed to "pcz 2 2400 rz %1".

Each of the four batch files have to be customized for each user

who is not using PC Port Com1 and 1200 baud between the PC and

their TNC. The INSTALL program takes care of that chore. The high

level of customization required here is because different Zmodem

implementations require different sequences of instructions and

LAN-LINK does not want to limit you to a specific one.

14.0 ELMER.QSO

This is the state table (configuration file) for ELMER. The

contents of the file are as described in Appendix 8. If this file

is not present in the same sub-directory as LAN-LINK, you will

not be able to activate ELMER.

15.0 LAN-LINK.OUT

This file will be transmitted to all incoming connects (except

when in the multi user mode), if the file is present, and if the

Auto Answer flag is enabled. You can think of this file as a

CTEXT File.

16.0 *.DMO

These files are demonstration or test files. When LAN-LINK is put

into the Demo Mode (LAN-LINK Menu), LAN-LINK treats the lines of

text read from these files as if they came in from the TNC via

the serial port. Use these files for demonstrations and/or tests

of different features.

LAN-LINK.DOC (c) G3ZCZ 1988 - 1994

APPENDIX 4 Page 128

Appendix 4 Local Area Network Capability

VHF Packet radio systems can be considered as part of a Local

Area Network (LAN) in which messages can be left by one station

in the computer belonging to a second station. At HF the same is

true, but the area becomes greater. The basic problem here is

that people can only send and receive messages to or from you

when you are on-line. To compensate for this, BBS stations were

developed which allowed both messages and bulletins to be stored

by anyone for later retrieval.

LAN-LINK allows you to store messages in your computer so as to

use it as an automatic answering machine. By storing the messages

in a file called by the name of callsign, with the file extension

".OUT", anyone connecting to the computer will get their message

without having to request it. LAN-LINK also extends that concept

to cover the eventuality of you wanting to take your computer

off-line for some reason. You may now load the message into any

other computer you can connect to using elements of the Q code

adapted to the High Level Network Communications Language (NC/L)

first proposed in my book Software for Amateur Radio, (Book

number 1560) published by TAB Books, Blue Ridge Summit, Pa. 17214

U.S.A.

1.0 LAN Protocol (G3ZCZ Version)

The protocol is as follows. When connected to someone who has

their LAN-LINK configured as a host, if you want to store a

message on that system, you use "QSP". You send the other station

":QSP: CALLSIGN" where "CALLSIGN" is the callsign of the station

that the message is for, not the callsign of the host station in

whose computer you are storing the message. [Note use only one

space character after the :QSP:].

For example if you want to store a message for 4Z4ZB in 4X6AA's

computer which is configured as a LAN-LINK Store and Forward

system, you would first connect to 4X6AA and then send the

command as ":QSP: 4Z4ZB". Better still use :QSP: option in the

Lan-Link Menu to automate the sequence.

The computer at 4X6AA will respond either with a statement saying

that it is ready for you to go ahead, or send a message saying

that it can't comply. If it is ready you get a positive reply

which will take the form :QRV: CALLSIGN which if you know the Q

code, means "I am ready to accept a message for CALLSIGN".

At this time you may go ahead and send the message. If you type

the message at the keyboard, you may use either a Control-Z (^Z)

character or the character sequence ":EOF:" followed by a

carriage return (the "Enter" key) to terminate the message. If

LAN-LINK.DOC (c) G3ZCZ 1988 - 1994

APPENDIX 4 Page 129

you have first written the message into a text file you may then

send it using the "Send File" option of the Files Menu and the

termination sequence is automatic.

Once you have completed the message, the other (host) computer

will either reply that the message has been successfully stored

or give you an error message.

If the message is stored and ready to be sent next time the

addressee connects to that computer, you will see the message

:QSL: on your screen. If something went wrong, you will get back

a negative message taking the form :QNO: followed by a number.

The number tells you why the operation failed.

2.0 Message Format

The message is stored just as if you had left it in your system

(except that a header is added identifying the time of reception

and the call of the sending station). It will contain a header

showing the date and time it was received and your callsign.

Should a message for that station already be in the system, yours

will be appended to it. In the event your upload is aborted, the

amount of text received before the abort occurred will be stored

as the message.

When you disconnect from the other LAN-LINK host station, its

mail beacon will be updated.

Once the message is loaded in the host, it can only be deleted by

the operator of the host station. When the addressee connects to

the host and receives the message, the file type will change from

".OUT" to ".OLD". Note QTA stations require positive confirmation

of receipt of message for the renaming to take place.

3.0 ASCII File Uploading/Downloading

There comes a time when you want to leave a file on your system

for someone to download later. You can do this by using NC/L to

control the transfer. You must leave the file in the file

download sub-directory defined in the LAN-LINK.SYS FILE (default

value is C:\LAN-LINK\FILES). You must first create the sub-

directory, if it does not exist. The station who wants the

message then asks for it using :QBM: FILENAME.TYP. They do not

need to use the full sub-directory path. They must, however,

leave a single space character between :QBM: and the filename. In

AMTOR the Mailbox must be turned on for remote functions.

For example, you have the latest ARRL DX bulletin, and you want

to pass it on. You could pass it to selected people by copying

the file to individual messages with the filetype ".OUT", e.g.

LAN-LINK.DOC (c) G3ZCZ 1988 - 1994

APPENDIX 4 Page 130

4X6AA.OUT, 4Z4ZB.OUT or even G3ZCZ.OUT if you want me to get it,

which wastes a lot of disk space.

On the other hand you could tell people that the file was

available for downloading, either in the CTEXT connect message

line which everyone gets when connecting to you by configuring

LAN-LINK accordingly, or in individual (".OUT") messages.

If you include the following in your CTEXT message

"ARRLDX.015 now available"

people will know that to download it, they just have to type the

line ":QBM: ARRLDX.015" (one space character between :QBM: and

the file name) to get the file. LAN-LINK is not designed as a

BBS, however, if you copy the contents of the \LAN-LINK\FILES

sub-directory into a file called DIR (USE DIR > DIR in DOS),

anyone can get a list of the files you have stored in it for

downloading by typing :QBM: DIR. Since they do not get the real

directory, just what was in it at the time you copied it into the

file "DIR", you may keep "hidden" files in this sub-directory.

You may thus leave a file for someone to download, tell them that

its there by leaving them a message (which they will get

automatically when they connect) and no one else connecting will

know that it is there.

If you want to upload a file, ":QDB:" allows you to upload text

or ASCII files.

If you use the "lan-linK" option of the Packet Terminal Menu,

LAN-LINK will automate the QDB and QBM features for you.

4.0 Path Determination to a DX station

If you want to establish a digipeat path to a station somewhat

out of your direct range, you need to know which of the stations

that you can connect to can hear that desired DX station. If you

could get a call monitored (MH list) from the stations that you

connect to, you would be able to see if the station you are

connected to has heard your desired DX station.

LAN-LINK uses the :QMH: command to request such a list. When LAN-

LINK receives a :QMH: command, it drops back to the TNC Command

Mode and issues a "MH" command to the TNC (except to a TNC1 which

does not support it). It stores the list received in a temporary

buffer, and then returns to the Converse Mode and transmits the

list back to the station it is linked to.

By judicious use of :QMH: you can determine paths to other

stations. Note however, that just because one station can hear

LAN-LINK.DOC (c) G3ZCZ 1988 - 1994

APPENDIX 4 Page 131

another station, it does not mean that it can work it. For

example, the station you are connected to may be using a power

level of 1 watt or so, while the station 200 miles away that it

heard was using 100 watts. Test the path yourself, or/and leave a

message asking how reliable the connect path between those two

stations is.

If you and a small group use a non standard packet channel for

inter-group communications, you may also use the :QMH: function

to find out who the station you are connected with, has heard

recently, when you first sign on the LAN channel.

5.0 Binary File Transfers

In the PC world most binary file transfers seem to be taking

place using the YAPP protocol built into the YAPP user terminal

program and the BBS software written by Jeff Jacobsen, WA7MBL. As

Jeff stated in the documentation, YAPP was written to provide a

terminal program for use in testing the WA7MBL BBS code. However

for many radio amateurs, YAPP was their first terminal program.

The YAPP binary transfer protocol is similar to the Xmodem

protocol used on telephone line BBSs. In order to transmit a

file, the sending station breaks the file down into blocks of

characters. Each block is transmitted in sequence. After a block

has been transmitted, the sending station waits for the receiving

station to acknowledge that block and verify that it was received

without errors.

When two stations are alone on a frequency, they can transfer

data quite quickly. So back in 1986 when YAPP was written, it

provided the few hams using packet radio with a binary file

transfer capability. Today, however, the situation is different.

In most urban areas, each packet radio channel is occupied by

many stations, and the requirements for binary file transfers are

somewhat different.

6.0 Requirements for Transfer Protocols

The AX.25 protocol provides for a basic verification that packets

have been received error free. If the binary file transfer

protocol could establish the connection, start sending blocks,

and then keep sending data until one of three things happen; the

link drops out, the receiving station signals that some of the

data were received with errors, or the whole file is transmitted

correctly.

If the whole file is transmitted, then the sending station just

has to wait for an acknowledgment from the receiving station that

the file was received. If the circuit is busy with several

LAN-LINK.DOC (c) G3ZCZ 1988 - 1994

APPENDIX 4 Page 132

stations on the channel, or the path goes through one or more

Nodes, there will be varying delays in the delivery time of the

packets. By not having to acknowledge each block of data, the

file can get through much faster than it would have, in the case

in which each received block has to be individually acknowledged.

If the link drops out, it would be nice if the computers, could

determine how much of the file was received, then pick up the

transmission at that point next time they link up.

If the receiving station signals that some data were received in

error, the sending station should then begin to retransmit from

the start of the error.

7.0 The Zmodem Protocol

This situation is not unique to the amateur radio packet network.

It is also present in the commercial packet switching

environment, and the Zmodem protocol was developed to provide the

capabilities described above. Zmodem thus seems to be an almost

ideal binary file transfer protocol for use on radio amateur

packet channels, and is incorporated in LAN-LINK 2.00+.

LAN-LINK uses the :QZD: and :QZU: commands to provide the

capability to remotely download and upload binary files. :QZD:

allows you to download a file from another station, while :QZU:

allows you to upload a binary file. :QZD: is configured so that

if the link drops outs during the transfer, when the connection

is once again established, a subsequent :QZD: for the same

filename, will cause the transfer to pick up where it left off

when the link dropped out. This feature is not allowed in the

upload :QZU: command to inhibit people from injuring files that

already exist on the other computer. Each command may be

initiated from either end of the link, but to gain the most from

the recovery capability, files should be downloaded by the

receiving station.

8.0 The PCZ Zmodem Package

LAN-LINK implements the Zmodem protocols using batch files in DOS

that call up external programs. This approach allows stations to

try different versions of Zmodem and experiment to find the

optimal one for their area. LAN-LINK comes with the (PCZ) Public

Domain implementation of the Zmodem, Ymodem, Xmodem, Xmodem-1K

and Sealink protocols by Drue Kennon and Gary Smith.

Since any LAN-LINK station can either receive or originate a

:QZD: and a :QZU: command, LAN-LINK comes with four default batch

files to control Zmodem transfers. These files are as follows.

LAN-LINK.DOC (c) G3ZCZ 1988 - 1994

APPENDIX 4 Page 133

QZU-RX.BAT Batch file implemented after receipt of :QZU:

QZU-TX.BAT Batch file implemented after transmission of :QZU:

QZD-RX.BAT Batch file implemented after receipt of :QZD:

QZD-TX.BAT Batch file implemented after transmission of :QZD:

Each pair of users can experiment with other protocols and Zmodem

implementations using the :QZD: and :QZU: commands after reading

the PCZ documentation. The :QBU: command can also be used if the

corresponding DOS batch files (QBU-RX.BAT and QBU-TX.BAT) are

created.

LAN-LINK contains an output buffer to store data being output to

the TNC. The TNC contains a buffer which stores data about to be

sent on the radio link. In the normal course of events, the two

buffers fill up, so that the Zmodem package used by LAN-LINK may

think that all the data has been transmitted, when in fact there

are still characters left in the buffers, or the last packet has

not yet made it to the destination station. In this case, you may

see a message on your screen that the system has sent all the

data and is waiting for the final acknowledgment, while at the

same time you can see from the LEDs on the TNC that you still

have outgoing data in your system. Have patience and wait a while

to let the two systems synchronize.

In the event that the link is bad and errors occur, this

buffering may cause problems which require some of the Zmodem

timing parameters to be changed. Should this situation arise,

read the Zmodem package documentation.

The flexibility offered by the Zmodem protocol has many

advantages when used in a packet radio network. LAN-LINK 2.00+

implements Zmodem by the use of an external public domain

protocol package which provides for the maximum degree of user

customization. This capability is provided at the cost of having

the individual user customize four batch files using the text

editor built into LAN-LINK.

9.0 BASIC NC/L DICTIONARY

Messages can be transferred into any other LAN-LINK computer you

can connect with using elements of the Q code adapted for the

purpose. NC/L command words are summarized in the following

paragraphs. There is no command word to request your message. To

receive your message, you do nothing. You will receive your mail

automatically when connecting/linking with a LAN-LINK station.

You cannot normally read messages addressed to another person.

9.1 :QBM:

To download an ASCII file, send ":QBM: FILENAME.TYP". The

LAN-LINK.DOC (c) G3ZCZ 1988 - 1994

APPENDIX 4 Page 134

FILENAME.TYP is the file you want. i.e. ":QBM: DIR".

9.2 :QDB:

To upload ASCII files to another LAN-LINK system send ":QDB:

FILENAME.TYP". Files will be uploaded into the same sub-directory

as LAN-LINK, and will have a message header inserted at the top

of the file to allow the operator to know who uploaded the file.

If the operator then wants to make the file available for

downloading, he or she can move it to the \LAN- LINK\FILES sub-

directory. The ".TYP" following the filename is optional. If you

attempt to upload a file which has the same name as one that

already exists on the remote system, you will receive an "error"

message. This command is the reverse of :QBM:.

9.3 :QMH:

To request a call monitored list ("MH") from the LAN-LINK station

that you are connected with, send ":QMH:".

9.4 :QSM:

To request a repeat of a message from a station using LAN- LINK,

send ":QSM:". This command will be valid as long as the host

station has not deleted the YOURCALL.OLD file on his disk. If the

file does not exist, you will receive a :QNO: "error" response.

9.5 :QSP:

To leave a message, send ":QSP: callsign". The callsign is that

of the station you wish to leave a message for.

9.6 :QRA:

When disconnected, to see what stations (using LAN-LINK 1.53+)

are on the LAN, or which LAN-LINK stations have messages pending,

send ":QRA:". All LAN-LINK stations will reply with a mail beacon

within a few seconds. The Alt-W key implements this feature.

9.7 :QRT:

To shut down a LAN-LINK AMTOR/Packet mode Mailbox or beacon

station which is causing QRM, link or connect to the station and

send ":QRT:".

9.8 :QRU:

To upload messages from one LAN-LINK/PK232COM (1.38+) system to

your computer, send ":QRU:".

LAN-LINK.DOC (c) G3ZCZ 1988 - 1994

APPENDIX 4 Page 135

You may only use the QRU function with stations designated as

Store and Forward mailboxes. Put a list of stations that you can

connect to reliably in your CALLSIGN.MBX file and send it to

other stations in your LAN also using LAN-LINK. They will then be

able to dump their mail messages on you and you on them. If they

do the same and send you their file, then you will be able to

send messages via them for stations that they can work.

While QRU gives you the capability to bulk upload messages to

another station in your local area, when you take your machine

off line, it may also be used to transfer messages between two

LANs via well sighted gateway digipeaters.

9.9 :QNO:

:QNO: and error message text is a response to a request.

9.10 :QJG:

:QJG: is a response to :QRU:. It confirms that the QRU sequence

is complete. There are no more messages pending.

9.11 :QRV:

:QRV: callsign is a response to :QSP: callsign. The computer is

ready to send the message. End the message with a control Z (^Z)

character, or the sequence :EOF:

9.12 :QSL:

:QSL: is a response to a command. It confirms receipt of message

to that callsign or that a file has been successfully uploaded

and stored to disk. It is also used to acknowledge receipt of a

":QRT:" command.

9.13 :QTA: message

If LAN-LINK receives :QTA: and a message exists for the

connecting station, LAN-LINK will change the filetype from "OUT"

to "OLD" and respond with a "QSL". If a message does not exist,

it will respond with a "QNO".

9.14 :QTC: Message List

If LAN-LINK receives :QTC: it will respond with the beacon text

showing the mail queue. :QTC: precedes a list of callsigns for

whom messages are stored up on a computer. It is used in Packet

Beacon transmissions or AMTOR Beacon mode CQ calls.

LAN-LINK.DOC (c) G3ZCZ 1988 - 1994

APPENDIX 4 Page 136

9.15 :QBU: External Protocols

This feature is supplied to allow experimentation with external

protocols.

9.16 :QZU: Zmodem Binary File Upload

:QZU: is used to implement an upload of a binary file from

another station using an external Zmodem protocol via the QZ*.BAT

files.

9.17 :QZD: Zmodem Binary File download

:QZD: is used to implement a remote download of a binary file

from another station using an external Zmodem protocol via the

QZ*.BAT files. In the event that the link drops out during the

transmission, when the link is remade, and the :QZD: retried, the

transfer will pick up from the point at which it was interrupted,

if the batch files provided are used.

9.18 :QIC: Page Operator

Use :QIC: to page the operator at any LAN-LINK 2.10+ station. The

sequence is:- you send :QIC: to another LAN-LINK 2.10 station.

Use the LAN-LINK Menu or type it by hand. Upon receipt, LAN-LINK

begins to page the operator if the sound flag is on. At that time

it sends you a :QRX: followed by a number, i.e. ':QRX: 30'. The

number is the number of seconds it will page the operator. At the

end of the paging sequence it will send the prompt i.e. 'G3ZCZ>'.

If the sound is inhibited it will send ':QRX: QRT' to signal that

it is in a silent configuration.

9.19 :QTR: Ask for Time

The :QTR: command allows you to get the date-time-text string

from the other LAN-LINK 2.10+ station. Don't confuse it with

:QRT:.

LAN-LINK.DOC (c) G3ZCZ 1988 - 1994

Appendix 5 Page 137

Appendix 5 Using Different TNCs

The Packet Mode capabilities can be used on all TNCs. There are

minor differences in the command dialogue between different TNCs.

The dialogue differences affecting the operation of LAN-LINK are

built into the program and set when the TNC type is read from the

LAN-LINK.SYS file at the program load time. The non packet modes

will obviously result in error messages if attempted on a packet

mode only TNC.

The following notes apply to different TNCs or digital

communications controllers.

1.0 TNC2 (MFJ 1270)

The following functions do not work because the TNC does not

support them.

All NON PACKET modes.

Software controlled HF/VHF switching.

Software controlled baudrate switching.

CQ/BEACON Packet terminal mode.

TNC Transmit Buffer flush.

N2GNJ noticed that he needed "LC ON" when using a Tiny 2 EPROM

Version 2.0 in the MFJ 1270B to make the automatic snatch

features work.

2.0 TNC1 (HK-4040)

The Packet Communications Mode header colors are not supported in

LAN-LINK for this TNC.

The following functions do not work because the TNC does not

support them.

All NON PACKET modes.

Day/Time functions.

MH and remote :QMH: functions.

MAIL Packet Terminal Mode.

CQ/BEACON Packet Terminal Mode.

TNC Transmit Buffer flush.

Conference and multiconnect modes.

3.0 Kantronics KPC-2

The Packet Communications Mode header colors are not supported in

LAN-LINK for this TNC.

The following functions do not work because the TNC does not

LAN-LINK.DOC (c) G3ZCZ 1988 - 1994

Appendix 5 Page 138

support them.

All NON PACKET modes.

MAIL Packet Terminal Mode.

CQ/BEACON Packet Terminal Mode.

TNC Transmit Buffer flush.

4.0 Kantronics KAM

LAN-LINK switches colors when it sees the /V or /H following the

callsign in the packet header, or in the calls monitored ("MH")

list. If you are monitoring both HF and VHF simultaneously, the

colors will show you which radio you heard which signal on. If

the Mail-Snatch or the QTC-Snatch are triggered, LAN-LINK will

change modes and attempt the connect on the correct radio but

will not change the band/power information or the logbook to

reflect that change in the logbook or in the status window

display. Turn the DCD flag OFF for these automatic features to

work.

The following functions do not work because the TNC does not

support them.

TNC Transmit Buffer flush.

The SIGNAL Communications Mode.

The Alt-D key in the MORSE Communications Mode.

The AMSAT-OSCAR Menu does not allow UoSAT option, as the KAM does

not have 1200 baud ASCII capability.

5.0 PK-232

Turn BBS OFF, if you want the Alert Call, packet communications

mode header colors, and Digipeat Detect features to work,

otherwise turn BBS ON.

6.0 MFJ 1278

LAN-LINK assumes that the Radio 1 port is connected to the

VHF/UHF packet transceiver, and the Radio 2 port is connected to

the HF transceiver with the multi mode communications capability

(you may customize this however).

The MFJ1278 suffers from one major deficiency as far as LAN-LINK

is concerned. The Echo-As-Sent only works in the AMTOR

Communications Mode. That means that you will not see the

contents of a file echo in the incoming window as the file is

being transmitted in the RTTY mode.

The following functions do not work because either the TNC does

not support them or does not do so in a consistent manner.

LAN-LINK.DOC (c) G3ZCZ 1988 - 1994

Appendix 5 Page 139

Pactor

Type Ahead Buffer

TNC Transmit Buffer flush.

The SIGNAL Communications Mode.

The AMSAT-OSCAR Menu does not allow UoSAT option.

CW Identification.

7.0 Heath HK-21

K3WGF noticed that his Heath HK-21 TNC didn't seem to have the

proper circuitry to support true DCD on pin 8 of the RS232

connector. Even though the pin was labeled DCD it didn't signal

properly. Set the DCD function in LAN-LINK to OFF to make the

Capture-to-Disk close out normally rather than continue to remain

engaged.

8.0 PK-900 and DSP 2232

LAN-LINK supports dual windows, and some of the

communications modes. Otherwise, support is as if they were PK-

232s. Sometimes, LAN-LINK may take a few characters to

synchronize windows.

LAN-LINK.DOC (c) G3ZCZ 1988 - 1994

Appendix 6 Page 140

Appendix 6 Anomalies and Bugs

This section discusses various anomalies and bugs.

1.0 Command/Converse Mode Anomalies

Since the Program has no way to check the command/converse mode

status of the TNC, you may see various "Cmd:" and other "error

messages" from the TNC when using the menus or function key

commands. Ignore them for now. If you get an error message, try

using the function key again.

2.0 Buffer Messages in AMTOR Beacon CQ Mode

In the AMTOR Beacon CQ Mode, if the other station drops the link

while you are recording the text, BEFORE the callsign has been

recognized as a valid callsign, LAN-LINK will time out before

logging the QSO and restart the CQ sequence. If someone else

calls in before the time-out has occurred, the text from both

stations should be captured to disk, but the log entry may not

show both stations. You should compare the callsigns in the

logbook with the "RUN" file for QSO purposes.

LAN-LINK in the Beacon/Mailbox Mode, normally logs the contact at

the end of the line in which it recognized the callsign as a

valid callsign.

3.0 FEC +? Responses in AMTOR Beacon CQ Mode

In the AMTOR Beacon CQ or Mailbox mode, if another station sends

a +? in the FEC Mode, the beacon will attempt to send the LAN-

LINK.TXT file. The PK-232 however is not in the Converse Mode at

that time and will interpret the text of the file as commands. If

by some chance, the text corresponds to a valid command the PK-

232 will execute it and put the PK-232 in an unknown state and

you may not notice that it did. As such I recommend that the

first character of each line in the file LAN-LINK.TXT be a non

valid command character such as a number or a period (.) or

another punctuation sign.

4.0 Using a Monochrome Monitor

If you are using a monochrome monitor, the text you type into the

Outwindow may be fainter than the incoming text. This is because

of the default colors used. Change the colors using the Parameter

Menu.

5.0 The Packet Conference Mode

The Conference Mode has been tested on a PK-232, KAM, a SANYO

LAN-LINK.DOC (c) G3ZCZ 1988 - 1994

Appendix 6 Page 141

MBC-675 which has a clock speed of 4.7MHz and other PC clones

with faster clock speeds. If you have problems with intermittent

selection of I/O channels try slowing down your CPU clock. There

is a software delay loop in the program that is used to allow the

TNC time to switch I/O streams after receipt of the stream change

command. This delay is there because LAN-LINK tests for a connect

on each channel before outputting any text, so that it does not

send un-addressed packets on non connected I/O channels.

6.0 Packet Conference Mode Disconnects

Any Packet conference Mode disconnect returns the TNC to the

Command Mode, even if you remain connected to other stations.

7.0 Strange Events

If the TNC does not use an interrupt driven serial port you may

experience problems when using baud rates greater than 1200. The

PK-232 DOES NOT LIKE LAN-LINK to use RS-232 baud rates faster

than 2400, and sometimes not even that. Use 1200 until YOU have

tested and ARE SURE that a faster one will work.

If you have any strange problems and are using an RS-232 terminal

baud rate between the PC and the TNC which is greater than 1200,

set the Terminal Baud Rate to 1200 and see if the problem goes

away.

Make sure that your configuration is set the way you think it is.

Verify it using the Parameter Menu.

You must have at least one common call or scan word in your SYS

file, or you will get errors when you update the SYS file.

8.0 *** DISCONNECTED

In the Packet Communications Mode, when you are connected to

someone and the TNC sends "*** DISCONNECTED" to the computer,

LAN-LINK thinks you have been disconnected and resets. This

normally only happens when a true disconnect occurs. However, if

the text shows up in the middle of a line being sent to you, it

will have the same effect. LAN-LINK will think that you have

disconnected if the DCD Flag is not set in the SYS file, and the

TNC does not use the DCD signal on pin 8 of the RS-232 interface.

9.0 *** CONNECTED

In the Packet Communications Mode, if you are monitoring traffic

on the LAN and someone sends a packet containing "*** CONNECTED",

LAN-LINK thinks you have been connected, if the DCD-Flag is not

set, and the TNC does not use the DCD signal on pin 8 of the RS-

LAN-LINK.DOC (c) G3ZCZ 1988 - 1994

Appendix 6 Page 142

232 interface.

10.0 Greek Characters and Other Garbage in the Incoming Screen

If you see Greek characters in the Incoming screen you probably

have the wrong parity set in the TNC. First make sure that the

RS-232 baud rate is correct. If it is, take the batteries out of

it and reset it. If that fails, play with the parity. LAN-LINK

sets the serial port as configured in the TNC.SYS file for each

TNC. For example, for the PK-232 it is 8 bits, no parity. For the

TNC2, it is 7 bits. If you still can't get LAN-LINK to talk to

the TNC, set the interface to 7 bits in the configuration file.

11.0 Dumb TNC

If the TNC seems to respond to commands but is not sending

anything back, you may have one of two problems. You may have a

bad RS-232 cable with a wire disconnected. If the computer can

operate the TNC in another communications program, try to reset

the TNC, take the batteries out, etc. You may also have a not-so-

compatible PC.

12.0 Intermittent Lockup

Watch out for flow control problems on the cable between the TNC

and the PC and between the PC and the Display.

Versions of LAN-LINK up to and including 1.55 used software

handshaking. Anytime the program received a Control-S (^S)

character from the TNC it waited until it subsequently received a

Control-Q (^Q). These characters can appear on the LAN in a

binary packet, either when binary file transfers are in progress,

or in a Inter NET/ROM message. To avoid this lockup, a line

containing "MFILTER 19" was added in the LAN-LINK.SYS file for

the PK-232 to make the TNC filter the ^S character out and not

pass it to LAN-LINK. LAN-LINK 1.56 and onward use hardware hand-

shaking so this problem should not appear. However if the FLOW

and XFLOW parameters are not set to OFF in your TNC, LAN- LINK

can appear to lockup. Make sure that they are both set OFF in

your TNC.

13.0 Automatic Functions Don't Work

This usually happens with the KAM. Set the DCD Flag off, and the

automatic functions should work. Note that in this case the

multi-connect functions will not work.

14.0 BBS Zap Failure

If the BBS Zap hangs up in the middle of sending a message, the

LAN-LINK.DOC (c) G3ZCZ 1988 - 1994

Appendix 6 Page 143

probability is that the LAN-LINK is waiting for the PBBS to send

it a word it recognizes. You have probably not configured the

"PBBS Read Subject" and "PBBS Message" Strings (lines 43 and 44)

in the LAN-LINK.SYS File (See Appendix 2, Section 18.18).

15.0 LAN-LINK Forgets the Band/Power Information

LAN-LINK picks up the band and power information from the

logbook. You can set them via the "0" option in the Parameters

Menu. If the logbook is empty, LAN-LINK will not remember the

settings when it is switched off. Put at least one entry in each

logbook.

16.0 Zmodem Binary Transfer Problems

Some stations perform binary transfers with no problems, others

seem to have problems. If you are having problems with the binary

file transfers make sure:

*    You have enough memory left over after loading LAN-LINK to

run the external communications protocol.

*    The QZ*.BAT files have the correct port and baud rate

values.

*    The TNC is set for 8 bit transfers.

*    Both stations are using the same data rate between the PC

and the TNC.

*    The TNC does not filter any characters out of the data

stream, i.e. MFILTER or the equivalent parameter in your TNC

is not set.

*    Make sure you try the binary transfers via the LAN-LINK

menu.

*    Read the PCZ Documentation file and set the Environmental

variables correctly.

*    If all else fails, try a different external protocol.

LAN-LINK.DOC (c) G3ZCZ 1988 - 1994

Appendix 7 Page 144

Appendix 7 Updates, Revision History

If you are updating from a previous version of LAN-LINK, the

following changes were made in recent versions of LAN-LINK.

2.00

ELMER, Alt-W, word wrap, G option in Edit menu, Strswitch chars &

error_st in sys file, added. KAM cw/cmd bug fixed, AMTOR sets

narrow shift, connect timeout configurable, KAM RX/TX inv added,

bell filtered out of connectee/log call, KAM cw and other

parameter changes, log power/band picked up from log, custom

word[20] in prompt, Elmer added, event timer 2/6 reset change, MH

fire delay for KAM, ^T filtered out in all tncs, #T in contest

text, NAVTEX and navy MARS enabled for KAM/MFJ1278, flag_XMITOK

added, base count added to SYS file, CTEXT saved.

2.10

PACTOR (KAM, PK-232) added. Converted to TP6, Bug fixes Init:

7/8bits, log package changed, Elmer.qso loaded at start-up, Elmer

RUN commmand, BBS custom commands, Flag_Inhibit_Logging, Logging

package, FLAG_Printer_Perm, TNC_parameter, mode at start up, QIC,

QMH in menu, QTR, LAN-LINK.OUT, ANSI Colors, PacketCluster Log

Search mode added. ELMER resets on disconnect, MFJ1278 radio port

customzable, Shut down flag 5 deleted, BBS upload states changed.

2.20

Converted to TP7. Scan word colors, PacketCluster DX Alert

colorized. 2.2 was really a bug fix version for 2.10 which should

never have been released and was replaced within a few days.

2.30

Support for PK-900 and DSP 2232. Better ANSI support. G-TOR in

KAM. Manual Mode, Z and B options added to BBS Menu. Expanded

PacketCluster support. Demo option added in LAN-LINK Menu. Radio

Menu added for Kenwood and Icom (untested). EDIT.EXE and

UPGRADE.EXE external programs added.

LAN-LINK.DOC (c) G3ZCZ 1988 - 1994

Appendix 8 Page 145

Appendix 8 ELMER An Expert System Based on a Finite State Machine

There are two versions of ELMER supplied with LAN-LINK. One is

built in for use in packet mode communications, when connected to

someone else. The second version is a stand alone version for use

in developing the text files and the logic. The stand alone

version lets you program and debug your own personal ELMER on

your own. The stand alone version is a separate product and

requires separate registration.

Most initial amateur radio contacts (QSO) are somewhat

repetitive, in that they send the same information each time. In

digital mode communications hams don't type exactly the same

information in every QSO, but in general the information

exchanged is a subset of a standard set. Hams have noticed this

phenomenon and have each created and used files containing

standard information. This fact was noticed years ago, and the

first so called "brag tapes" were invented to serve that need

back when they were using teletypewriters and generated paper

tapes for the standard information.

ELMER takes that concept a little further and provides expert

system. By parsing the incoming text, ELMER will recognize

certain words and transmit text files containing replies or

further questions, just as if the operator was at the keyboard.

Hams can use this feature to build a smart server they can put

on-line on the LAN. The server will prompt people and will serve

an educational purpose. ELMER can be set up to teach people about

packet radio, license upgrades, or anything else. ELMER can also

be programmed to recognize different languages and hold a

conversation in those languages. In fact, apart from the

dedicated educational and informational server applications,

ELMER may be configured with such a variety of vocabulary files

so as to pass the Turing test in most typical amateur radio

digital mode conversations.

1.0 INTRODUCTION

ELMER is a program which allows you to build an expert system. It

uses the ELMER.QSO file to set up the state machine.

The ELMER contains an array of words to be matched, current

states, next states, and files to be loaded.

The ELMER starts up in an initial state. When a line of text is

received, it begins to match an array of words against the

incoming line of text. It starts matching the words in the order

that they are present in the array. It only matches words that

are enabled for the current state or for state 0. If it finds a

LAN-LINK.DOC (c) G3ZCZ 1988 - 1994

Appendix 8 Page 146

match, it advances the next state, and if a valid file name is

present (and exists), will output that file. The ELMER will then

continue scanning the line of text from the point that the last

match was obtained. At the end of the line, it waits for the next

line.

1.1 Expert Systems

Before discussing ELMER, here's some information about expert

systems. An expert system contains knowledge about a particular

field to assist human experts or provide information to people

who do not have access to an expert in the particular field. It

is an information system that can pose and answer questions

relating to information borrowed from human experts and stored in

the system's knowledge base. Although they vary in design, most

expert systems have a user interface, a knowledge base, and an

inference engine.

1.2 The User Interface

The User Interface to the expert system is the way that the user

interacts with the system to extract information from the system.

1.3 The Knowledge Base

The Knowledge Base of an expert system contains both declarative

and procedural knowledge. The facts describing the situations,

events and objects are called declarative knowledge. Procedural

knowledge is the information about courses of action and the

rules governing the actions. There are various kinds of rules

that may be employed.

1.4 The Inference Engine

The Inference Engine controls how and when the information in the

knowledge base is applied. It determines how the rules in the

knowledge base are to be applied to the problem.

1.5 Features of an Expert System

The following list of features are desirable in any expert

system.

1.5.1 Useful

The system should meet a specific need.

1.5.2 Usable

The system should function so that even a novice computer user

LAN-LINK.DOC (c) G3ZCZ 1988 - 1994

Appendix 8 Page 147

finds it simple to use. It should be able to respond to simple

questions.

1.5.3 Educational

The system should allow non experts to increase their expertise.

In a similar vein, the system should be able to explain the

reasoning behind its advice to allow the user to determine the

validity of the advice.

1.5.4 Adaptable

The system should be able to learn new knowledge.

1.6 Using an Expert System

Expert systems take the form of software packages residing on a

hardware platform (computer). The most common use of an expert

system is via a man-machine dialogue. The user types something at

the keyboard and the system replies. The user interface accepts

the input. The input is parsed in some manner. The inference and

knowledge engines process the user input in a predetermined

manner and an output appears. The parsing function may be a

simple pattern matching method commonly called keyword analysis,

or a more complicated function using syntactic analysis. Keyword

analysis is a logical function in which the presence of various

keywords are detected. When a keyword is found, the system

responds in the manner in which it has been programmed. For

example, if the system detects the words "the wx" in a question,

it may respond with " the wx here is sunny".

In syntactic analysis, a sentence is analyzed according rules

which allow the system to respond differently to keywords which

appear in different sequences. For example, the system may

respond differently to the question "what is the wx" and "the wx"

if it recognizes those words in a sentence.

1.7 The Semantic Network

The most general and oldest artificial intelligence scheme for

representing knowledge is the semantic network. A semantic

network is a collection of objects called nodes. The nodes are

connected together by links. Ordinarily, both the links and the

nodes are labeled. A drawing of a semantic network contains

bubbles to represent the nodes, and lines connecting the nodes to

represent the links. Both nodes and links are labeled. The

drawing looks just like a PERT or CP/M chart. It is also the

drawing used to represent a state machine.

LAN-LINK.DOC (c) G3ZCZ 1988 - 1994

Appendix 8 Page 148

1.8 A State Machine

A state machine is a system that exists in several states. The

system makes a transition from one state to another as a result

of a stimulus. All states and transitions are defined at the time

the system is designed. The links in the semantic network are the

transitions in the state diagram.

1.9 The User Perception

The user perception of an expert system is that the system

appears to be in a rest state. It receives an input from the user

and enters a transition to a state in which it processes the data

it has just received. After processing the input, it generates an

output and returns to what seems to be the same rest state. In

reality, the two rest states (initial and final) are different.

1.10 ELMER

ELMER is an expert system for amateur radio. ELMER is based on a

finite state machine using table driven software. The declarative

knowledge is stored in the form of separate ASCII text and

command files. The procedural knowledge is stored in the state

table. The Data files in ELMER correspond to the Knowledge Engine

(and the Inference Engine). The State Tables correspond to the

Inference Engine. ELMER also contains an ASCII editor for text

files, and a tool to view the state table. These tools allow the

user to "teach" ELMER new knowledge.

1.11 Responses to an Input

ELMER has three kinds of reaction to a word match. It can do

nothing, output a test file, run a command or overlay a new state

table. ELMER's state table contains four entries:-

1.11.1 Current State

This is the state that the string match is performed in. ELMER

can have up to 63 different states.

1.11.2 Next State

This is the state that the ELMER will advance to if a string

match is found.

1.11.3 Repeat Flag

This is a flag to allow or disallow repeats. If it is a 0 repeats

are allowed. If it is a 1, they are not.

LAN-LINK.DOC (c) G3ZCZ 1988 - 1994

Appendix 8 Page 149

1.11.4 Command Flag

This is a flag to tell the ELMER how to treat the file. If it is

a 0, load a text file; a 2, overlay the current state table with

the defined one.

1.11.5 Keyword

This is the text string to match in the syntactic analysis of the

input text. The text string can be up to 15 characters long and

can include spaces. The match is case insensitive, i.e. there is

no difference between upper and lower case.

1.11.6 Data File

This is a text file to be loaded or a new configuration file.

1.12 Syntactic Analysis

ELMER performs syntactic analysis on the input text in the manner

described below. ELMER performs a string match on the line of

received text against each keyword entry in the state table.

ELMER starts with the first entry. If a match is made, ELMER

changes state and performs the transient function defined by the

Command Flag. All words in the input text including the keyword

are then deleted. ELMER then continues the string match until

either the input line of text has been reduced to zero length,

or, the line of input text has been tested against all keywords

in the state table.

1.13 Programming ELMER

Programming ELMER is simple. The knowledge base is broken out in

terms of states and transitions. States can be passive or active.

A passive state is one in which ELMER does nothing other than

accept/parse further input. An active state is one in which ELMER

loads and displays a text file containing information or shells

to DOS and runs an external program.

ELMER contains a set of tools for programming and debugging

purposes. A state table for the procedural knowledge is drawn up

using pencil and paper and a large sheet of graph paper. The

built in text editor is then used to enter information into the

table. The individual text files containing the knowledge are

also created using the editor. The state table viewing tool is

used to check the entries. The state table loading capability is

used to exercise ELMER and verify the logic. Because the

procedural knowledge is contained in the state table, all sorts

of rules and linkages may be used.

LAN-LINK.DOC (c) G3ZCZ 1988 - 1994

Appendix 8 Page 150

2.0 The State Table (ELMER.QSO) File

These are the contents of the ELMER.QSO file.

2.1 The Directory Path to the ELMER Text Files

This item is the directory path to the sub-directory containing

the ELMER text files. For example "c:\LAN-LINK\ELMER\" or

"c:\ELMER\" or just "c:", if you put them in the same sub-d-

irectory as ELMER.EXE. If you do use a path you MUST have the

last "\" character at the end of the sub-directory name because

the program concatenates the directory path to the ELMER.

2.2 The Start Up File

This line is not used in LAN-LINK's ELMER. It is only used in the

stand alone version. It contains the name of the text file to

output the first time ELMER is invoked (if present);

2.3 Window Colors

These seven lines are only used in the stand alone version. They

are the numbers which correspond to the colors of the seven

different windows displayed on the screen of the stand alone

version, in the following order.

2.3.1 Status Window Color

The color of the window at the top of the screen.

2.3.2 OutWindow Color

The color of the window in which the replies are displayed.

2.3.3 InWindow Color

The color of the window in which your keystrokes are displayed.

2.3.4 BottomWindow Color

The color of the bottom line of the screen.

2.3.5 Promptwindow Color

The color of the window in which menus are displayed.

2.3.6 Option Color

The color of the highlighted characters in the menus and prompts.

LAN-LINK.DOC (c) G3ZCZ 1988 - 1994

Appendix 8 Page 151

2.3.7 Alarmwindow Color

The color of the window in which error and alarm messages are

displayed.

2.4 The ELMER Initial State

This item is included to allow you to have different state

machines, each starting with a different state number.

2.5 State Machine Parameters

The next few lines contain the state machine parameters.

2.5.1 Current State

This is the state the string match is performed in. LAN-LINK

allows up to 63.

2.5.2 Next State

This is the state that the ELMER will advance to if a string

match is found.

2.5.3 Repeat Flag

This is a flag to allow or disallow repeats. "0" means that

repeats are allowed. "1" means they are not.

2.5.4 Command Flag

This is a flag to tell the ELMER how to treat the file as shown

below.

Flag                Operation

0    Load/display a text file.

1    Execute a BAT, COM, or EXE file.

2    Overlay the current state table with the new one.

2.5.5 Keyword

This is the text string to match. It can be up to 15 characters

long and can include spaces. The match is case insensitive, i.e.

there is no difference between upper and lower case.

2.5.6 Data File

This is either a text file to be loaded or a command file to be

executed, depending on the state of the Command File Flag. An "*"

character defines a "do nothing" state in which the Data File is

LAN-LINK.DOC (c) G3ZCZ 1988 - 1994

Appendix 8 Page 152

ignored.

The two character sequence "#1" will cause ELMER to clear the

screen before displaying the rest of the text in the data file.

2.6 The State Machine File

An example of some state machine parameters is shown in this

typical extract from an ELMER.QSO file.

1, 2,0,0,what,*

2, 3,0,0,englishman,*

3, 4,0,0,doing,*

4, 1,0,0,country,me-usa.txt

1, 1,1,0,equipment,pk232com.txt

1, 1,0,0,wx,localwx.txt

1, 5,0,0,kenwood,kenwood.txt

5, 1,0,0,811,811.txt

1, 1,0,0,hello,hello.txt

0, 1,0,0,name,joe.txt

An "*" character defines a "do nothing" state in which the Data

File is ignored.

2.7 Example of Programming the State Table

Consider a typical question that I get asked.

"what's an englishman like you doing in this country ?"

followed by

" The equipment here is ....."

Assuming that the ELMER is in State 1, and that the first few

lines in the ELMER.QSO file are as shown above. The ELMER

recognizes the word "what" and advances to State 2. Since the

file name is an "*" character no file is sent. The ELMER then

recognizes the word "englishman" and advances to state 3. The

ELMER machine then recognizes the word "doing" and advances to

State 4. The ELMER then recognizes the word "country" advances to

State 1 and transmits the text file "me-usa.txt", because the

Command Flag is a 0. It then recognizes the word "equipment",

remains in State 1 and transmits my brag tape file called

pk232com.txt. Since the Repeat Flag is a 1, the ELMER will not

retransmit pk232com.txt should it once again recognize the word

equipment while in State 1.

Since the ELMER scans sections of the line, the same effect will

be achieved if the incoming text had looked like :-

LAN-LINK.DOC (c) G3ZCZ 1988 - 1994

Appendix 8 Page 153

"what's an englishman like you

doing in this country ?

The equipment here is ....."

You can only have 63 lines in any one state machine array. You

can define up to 64,000 different states. Each state must be an

integer. State 0 is a wild card state. This means that any word

in the array coded for state 0 will be matched against the

incoming text whichever state the machine is in. The entries in

the state machine file must end with a line that contains state

666. For example

666, end of file

The number of lines in the array and the current state at the end

of each line scanned are shown in the status window.

3.0 The Stand Alone Version Status Window

The Status Window provides you with some diagnostic information

about the state machine.

C:        shows the current state.

T:        shows the number of states (lines) in the state table.

ELMER.QSO represents the name of the current state machine.

4.0 The Stand Alone Menu

You bring up the menu by touching the "Escape" character. You are

then presented with the options shown in Figure 8-4. To activate

an option touch the highlighted key.

Figure 8-4 ELMER Menu

E Edit Any File

L Load ELMER.QSO

P Pick File to edit

S Edit ELMER.QSO

X eXit to DOS

* Show State Table

4.1 Edit Any File

This option allows you to edit text files using the LAN-LINK

compatible text editor.

4.2 Load State Table (ELMER.QSO)

This option allows you to reload the state table, or load a

LAN-LINK.DOC (c) G3ZCZ 1988 - 1994

Appendix 8 Page 154

different one. If a different state table is loaded, its name

will appear in the menu.

4.3 Pick File to Edit

This option allows you to pick a file with the default filetype

of "TXT", and edit it.

4.4 Edit State Table (ELMER.QSO)

This option allows you to edit the state table (configuration)

file.

4.5 eXit to DOS

This option allows you to exit to DOS.

4.6 Show State Table

This option allows you to see the contents of the state machine.

It is used in debugging the logic to ensure that the contents of

the table match what you thought you put in.

LAN-LINK.DOC (c) G3ZCZ 1988 - 1994

APPENDIX 9 PCZ 4.06.90 Nov. 27, 1990 Page 155

Appendix 9 PCZ Binary file Transfers

LAN-LINK uses the PCZ public domain external protocol for binary

file transfers by shelling to DOS and executing the QZ*.BAT batch

files. The QZ*.BAT files are set up for you by the INSTALL

program. This appendix contains the documentation for PCZ as

supplied by the developers of PCZ, in case you have problems

making binary file transfers work in your system.

PCZ - A Public Domain implementation

Zmodem, Ymodem, Xmodem, Xmodem-1K, Sealink

for the IBM PC family

PcConnect Zmodem by Drue Kennon and Gary Smith

PCZ is a public domain Zmodem implementation that will allow

users of communication software that does not implement internal

Zmodem to access it through an external program. It is a full-

featured version of Zmodem, but does not have the large volume of

bells, whistles & options provided by Omen Tech's DSZ. However, I

do believe that it will be suitable for the average demanding

BBS'r.

PCZ provides support for the basic Xmodem variants: Xmodem,

Xmodem-1K, Ymodem & SEAlink.

The following is a description of how to start PCZ for file

transfers.

LEGEND

Data enclosed in "[" "]" are optional.

Data enclosed in "<" ">" are required.

Send Command line:

PCZ <1-8> <300-19200> [m] [f] [d] [s] <s?> [r] [@]<filespec>

Recv Command line:

PCZ <[1-8]> <[300-19200]> [m] [f] [k] [d] [s] <r?> [r]

[filespec]

Ref:

1-8 - the number 1 - 4, -(8) << PS/2 >>

300-19200 - the number 300, 1200, 2400 etc...

m - front-end menu option (see MENU)

f - enable FOSSIL comm driver routines

k - rx keep aborted received files

d - disable video memory writes (use dos)

s - request enable Super_Z transfer mode (zmodem only)

r?,s?  - transfer type sz, rz, sx, rx etc...

LAN-LINK.DOC (c) G3ZCZ 1988 - 1994

APPENDIX 9 PCZ 4.06.90 Nov. 27, 1990 Page 156

r - resume/recover aborted transfer (zmodem only)

@ - command file transfer alert flag (SEAlink, Z,Ymodem)

filespec - file name w/extension {complete file spec

allowed}     {drive:\path\filename.ext}

Flags should be placed as shown for standardization purposes.

i.e. "f" before "d"; "d" before "k"; "rz,sz" before "r" etc...

Due to the nature of the beast, no support is provided for a

default port & no default port speed is assumed.

NOTE * See environment variables below for more on default

port/speed

I personally never enter "port" or "speed", but always enter

their values. i.e. pcz 1 9600 f sz xxxxxxx.xxx The batch files

included reflect this method.

"ZMODEM RECEIVE"

<< SINGLE >>

PCZ 1 9600 rz myfile.ext - assume your specified file name.

<< BATCH >>

PCZ 1 2400 rz - Receive ALL sender files.

PCZ f d k rz r - Receive ALL sender files

assume PCZPORT & PCZSPEED values

use FOSSIL routines

use standard DOS screen output

KEEP all aborted files.

RESUME any incomplete previous files.

one file mode:

PCZ 1 9600 rz INFILE.ARC - Receives one file and names it

INFILE.ARC

PCZ 1 9600 rz r INFILE.ARC - Recovery effort on file

INFILE.ARC

NOTE * A word about command line switches might be in order here.

The switch flags " f, d, k " are global. That is they control PCZ

and not any particular protocol. The "r & s" switch is isolated

within zmodem, but is applied to both sender and receiver. If you

want PCZ's resume code enabled when sending a file then the

sender must specify "r" on the command line. Accordingly if the

receiver requests resumption of an aborted transfer, the sender

must have "r" specified or we won't resume anything. This was

requested by BBS Sysops who track download bytes, transfer points

etc... Probably so you aren't charged twice or the such like.

LAN-LINK.DOC (c) G3ZCZ 1988 - 1994

APPENDIX 9 PCZ 4.06.90 Nov. 27, 1990 Page 157

"ZMODEM SEND"

<< SINGLE >>

PCZ 1 9600 sz OUTFILE.ARC - Send one file to receiver.

<< BATCH >>

Create a TEXT command file with one complete file spec per line,

which includes drive and directory information for each file that

is not in the directory PCZ was called from.

e.g. C:\MYDIR\THISFILE.ZIP

There is NO limit to the number is allowed. Have no blank lines

in the file. Place the "@" symbol as the first character of the

filename.

Pass it to PCZ on the command line as follows:

PCZ 1 2400 sz @OUTFILE.TXT

What you name the command file is your business. It simply must

be a standard ASCII file with no software or printer control

codes present.

An alternative to creating the command file for sending 2 or 3

files may be used. Simply type the file names on the dos command

line separating each one with a space.

Remember the receiver must be set to accept batch uploads or you

might be spinning your wheels. Some BBS packages are just now

coming on-line with batch upload features using PCZ. They

normally indicate (BATCH) if it is supported.

BBS << BATCH >>

One method for implementing batch uploads on a bbs is to use the

log file and examine the transfer status character. The first

character of each line in the log file denotes how the transfer

finished. A "1" for good transfer & "0" for a failed transfer.

Armed with this information, simply setup to call PCZ in batch

receive mode. After PCZ has finished and your bbs code has

control, open the logfile and read in the line(s) of information.

You may now format a string to send the user about the status of

each file he/she sent, prompt for a file description etc..

After you have dealt with each entry in the logfile, simply close

LAN-LINK.DOC (c) G3ZCZ 1988 - 1994

APPENDIX 9 PCZ 4.06.90 Nov. 27, 1990 Page 158

the file and delete it. PCZ will create a new file for the next

transfer session. If you desire a continuous record of the

transfers then you should create your own and append each entry

you read from the pczlog to your continuous log file.

The information below shows exact start positions for each piece

of data. The x's denote the length of each element, the first (x)

shows start position as PCZ will write it to the log.

e.g. Below, the file size is 47877. PCZ will always write the

file size as a 7 character string which begins at position 21 in

the line. Hence, read from pos (21) to pos (27), trim any blanks

& convert to decimal value etc..

Log extract w/char positions:

1 2 3 4 5

1234567890123456789012345678901234567890123456789012345

x xx xxxxxxxxxxxx xxxxxxx xxxxxx xxxx x

1 sz PCZ40x.ZIP 47877 @ 2400 bps @ 237 cps 0

6 7 8

6789012345678901234567890

xxxxxxxxxxxxx

errors Fri 10/5/1990

"XMODEM VARIANTS"

To receive files using any of the Xmodem variants, the format is:

PCZ <port> <speed> rs filename - SEAlink receive

PCZ <port> <speed> rs - SEAlink Batch receive

PCZ <port> <speed> rx filename - Xmodem receive

PCZ <port> <speed> r1 filename - Xmodem-1K receive

PCZ <port> <speed> ry filename - Ymodem receive

PCZ <port> <speed> ry - Ymodem Batch receive

Remember the "k" option works for all of the receive modes. Hence

you can receive a file via SEAlink, have it abort on errors, and

then later RESUME the transfer using Zmodem.

To send files using any of the Xmodem variants, the format is:

PCZ <port> <speed> sx filename - Xmodem

PCZ <port> <speed> s1 filename - Xmodem-1K

PCZ <port> <speed> sy filename - Ymodem *

PCZ <port> <speed> ss filename - SEAlink *

* Refer to Zmodem Batch sending for batch transfers via Ymodem

and SEAlink

NOTE: The bytes and cps rate reflect the actual number of bytes

sent to, or received from the IOBuffer as a transfer progresses.

LAN-LINK.DOC (c) G3ZCZ 1988 - 1994

APPENDIX 9 PCZ 4.06.90 Nov. 27, 1990 Page 159

It does not necessarily reflect any specific "BLOCKSIZE". When

sending, as a transfer nears it's end, you will see the cps

decline. That's because PCZ uses buffered output vs a simple

"sector size" buffer. This will let you see actually how PCZ is

progressing in the transmission of data. In the best case "Full

Streaming Zmodem", you will see the time for actual file data;

worse case time will reflect the transmission of data headers and

block acks. It is different, but I hope everyone can appreciate a

more accurate account of what is happening inside the powerful

zmodem protocol.

** : The "Transfer Time" reflects PORT speed, (NOT MODEM) speed!

If speed conversion is being used, FORGET the transfer time.

"ENVIRONMENT VARIABLES"

PCZ supports several environment variables which can make setting

up a comm pkg. or BBS more "user friendly". These variables are

as follows and should be placed in the autoexec.bat file as I

have them shown here or modify the setenv.bat file included to

accommodate your system.

"DRIVE" refers to the letters "A".."Z"

"YOURDIR" refers to a complete directory specification.

i.e. \COMM\ARC\

"YOUR.FIL" refers to a filename with extension that you want PCZ

to use vs it's default name.

"DIRRX"

set DIRRX=drive:\yourdir\

This variable provides the ability for all your received files to

be placed in a single location. It is very useful for bbs

purposes as well as general comm program usage. If you specify a

directory spec on the command line, it will override DIRRX.

Therefore you can easily set the variable and then just forget

about it. If DIRRX isn't set and no spec. is given on the command

line, your receive file will be created in the current active dos

directory.

"PCZLOG"

set PCZLOG=DRIVE:\YOURDIR\

In the log file's case, it specifies the directory you want your

PCZ.LOG file to be kept in. If "PCZLOG" is not found in the

environment then no log file is generated.

LAN-LINK.DOC (c) G3ZCZ 1988 - 1994

APPENDIX 9 PCZ 4.06.90 Nov. 27, 1990 Page 160

set PCZLOG=DRIVE:\YOURDIR\YOUR.FIL

As an option to the log file name "PCZ.LOG" you may specify your

own filename in the filespec. This provides greater flexibility

with some bbs packages such as RBBS which expects it's own naming

convention to be used, and for multitasking environments etc...

If your "YOUR.FIL" exist, it overrides filename "PCZ.LOG" in all

cases.

"PCZPORT"

set PCZPORT=(1-8)

PCZ will use the port number (1-8) as the default port.

"PCZSPEED"

set PCZSPEED=(300-19200)

PCZ will use the speed (300-19200) as the default speed.

NOTE ** If you specify the PORT or SPEED on the command line, it

will override a default set by the environment variables.

"PCZDIR"

set PCZDIR=DRIVE:\YOURDIR\

If you experience any problems with PCZ not being able to access

it's overlay file, setting this variable will cure the problem.

DRIVE:\YOURDIR\ should point to the directory where you have

located PCZ.EXE

IMPORTANT NOTE:

NO SPACES are allowed when setting an environment variable!

i.e. {WRONG}

set PCZLOG = C:\COMM\ will not work.

That's not my fault. It is just DOS....

i.e. {ahhhh}

set PCZLOG=C:\COMM\

"PORT & IRQ"

It has seemingly become necessary, instead of interrupt sharing,

to reassign Comm. PORT addresses and use seldom triggered IRQ

lines e.g. "5 & 7" to allow multiple "more than two" ports on one

machine. Efforts to support this trend have been successful with

PCZ. Referring back to our PCZPORT environment variable:

LAN-LINK.DOC (c) G3ZCZ 1988 - 1994

APPENDIX 9 PCZ 4.06.90 Nov. 27, 1990 Page 161

set PCZPORT=1

If you have reassigned your IO card addresses etc..., you can

force PCZ to comply with your wishes.

Lets say you want to use PORT 3 @ 2E8h on irq 5. You set PCZPORT

in this manner.

set PCZPORT=3,2E8,5

| | |

| | |____ IRQ line number

| |_______ PORT address

|__________ PORT number

If you specify an invalid address or one your card isn't set for,

then PCZ will simply abort during it's port validation routine.

This method should provide the support needed for those who like

to match their wits against the cpu's.

"Super_Z" (tm)

Always in search for faster from fast. Super_Z takes zmodem to

the ragged edge! If you are one of the blessed, and enjoy

relatively noise free transmission lines, you should see an

increase of 2 to 3 % efficiency. XON/XOFF MUST be disabled and

the modem must pass these characters. You may activate Super_Z

via the command line switch "s". As is usually the case, Super_Z

can be called by the receiver or by the sender. That is to say,

that either can flag for Super_Z and if the remote version

supports Super_Z, we're doing it.

Regarding noise, a seldom thought of fact is that just because

your local transmission lines are great to one place, someone

else's local lines may be the pits. A good example is.... When I

log on to the "Corner" from Texas on a "596" extension I rarely

ever get a line hit. When I use the "995" extensions I am prone

to hits anytime. A reverse situation might occur in your

particular area. It simply depends on the quality of any of the

many exchanges you might encounter. I can only hope you are one

of the lucky ones.

"NETWORK SUPPORT"

PCZ has been tested completely in a network environment. It will

allow multiple users to read the same file via use of DOS's

shareable attribute. It will also lock any files that it is

currently writing to. This has been tested using Novell Netware,

LAN Manager, and CBIS Network-OS.

LAN-LINK.DOC (c) G3ZCZ 1988 - 1994

APPENDIX 9 PCZ 4.06.90 Nov. 27, 1990 Page 162

"MENU FRONTEND"

Many fine programs exist which allow protocol engines to be

called from a communications package. In an effort to make PCZ

all the more user friendly, there is now a simple menu system

built into the program. It is controlled by the file PCZ.CFG

which is included in this zip file. Take a few moments to read

over it. It may be that it is just what you are looking for in

the way of a nice user interface. The design and layout of the

system is very simple and straight forward. You control the

options and protocols you wish to have active. PCZ.CFG comments

explain the various functions you can use. To use the menu system

simply call PCZ in the following manner.

PCZ m

Make your menu selections and complete the command line with your

filespecs.

"FOSSIL SUPPORT"

FOSSIL is an acronym for "Fido/Opus/SEAdog Standard Interface

Layer". This version of PCZ implements the use of FOSSIL

communications support. To date only one, "X00.SYS" driver has

been tested to work with PCZ. X00.SYS is a product of Raymond

Gwinn. It has proven itself in many environments and I recommend

it to you without reservation. X00.SYS was chosen because it

provides "FIFO" etc... support for the 16550 UART chip. My

internal routines support the chip but not all it's features.

Also X00.SYS provides enhanced ability to work in a multitasking

environment such as DeskView. If DeskView is active PCZ gives

idle processor time back to the cpu for smoother operations. For

a brief description of setting up X00.SYS on your system read the

enclosed file PCZFOSSL.DOC

Making use of the driver with PCZ is as simple as adding "f" to

the command line. It should be placed after the baudrate in all

transfer types.

i.e. PCZ 1 9600 f sz OUTFILE.ZIP

When PCZ starts, it will check for the presence of the driver in

memory. If it is resident all will proceed as planned. If

however, the driver is not in memory, PCZ will gracefully shut

down with a message stating this fact. I gave careful thought to

jumping to the internal routines, but decided against it for a

couple of reasons. In some cases the ports may be scrambled which

will surely cause a system lockup on PCZ's part. A problem may

exist with the driver you are using, giving false info which

PCZ's routines would not like very much. A "dumb" terminal might

LAN-LINK.DOC (c) G3ZCZ 1988 - 1994

APPENDIX 9 PCZ 4.06.90 Nov. 27, 1990 Page 163

have charge of things and not want another routine hanging

around, etc...

"PHONE SUPPORT"

Phone support can be provided when you require it, if you call

while I am home. I umpire for our local women's softball league

on Mon. & Tues. nights. I play golf on Wed. & Thurs. evenings.

Fri. night I do what Maureen tells me to do! Sat. is up for grabs

and Sun. morn & evenings I am in church. Please try me sometime

other than those mentioned ...... Seriously though, any night

(late) will be fine. Just try and make it before 10p.m. CST if

possible.

Note from the author:

Over the last year PCZ has received some very good reviews. More

users rely on the program for their transfers now than I thought

would ever be interested. For this I am especially grateful and

would thank each one if possible. Gary & I will continue to grow

with the needs of the BBS community. We ask for your continued

input and support, with our assurance that your IDEAS, "cheers &

jeers" are taken with sincerity.

Thanks

Drue

History

-------

V1.01.88 - Initial release, no known bugs. Please report any

problems to Drue Kennon.

V2.01.89 - Extended version that contains Xmodem, Xmodem-1K,

Sealink, and Ymodem Batch in addition to Zmodem. Also provides

full file sharing for use in networking environments.

V2.11.89 - Fixed several reported bugs. Enabled hardware flow

control for use with MNP type modems. Enhanced port speeds up to

19.2k, and tightened some code for more efficient operation.

Properly set errorlevel 1 in event of transfer failure.

V3.03.90 - Fixed a problem with SEAlink xfers not seeing the

last EOT. Batch sending for SEAlink per request. Enhanced some

screen data writes.

V3.06.90 - Fixed a problem with SEAlink xfers created by making

the last fix. Zmodem command file name can't be fouled due to

misunderstanding the docs.

NOTE:

LAN-LINK.DOC (c) G3ZCZ 1988 - 1994

APPENDIX 9 PCZ 4.06.90 Nov. 27, 1990 Page 164

Unless forced into it by the users, I do not plan to support 7

bit data transfers. Nor can I figure why any IBM machined BBS

would? Did I miss something along the way?

V3.08.90 - Got with the program and added support for FOSSIL

driver. e.g. Gwinn's Communications Controller, X00.SYS add "f"

to cmd line params for fossil). Removed internal support for

default port and speed which caused problems for multitaskers.

Defaults are now specified through environment variables. Added

environment variable for default receive dir. Reinstated environ-

ment variable for pcz log file.

V3.09.90 - By popular demand, namely "Gary", I added yet another

multitasker enhancement. Since the cpu is much faster than the

port, idle time occurs during output. This time is now "given

back" to the cpu, allowing the "other side" to operate smoother

and more efficiently.

V3.10.90 - Some users mentioned that PCZ would wait forever in a

transfer if the carrier was lost. I couldn't dupe the problem,

but added several more checks nonetheless. The logfile has

changed so that you may specify a full filespec. including

filename for you logfile in the env. var. The first char of each

line in the file represents pass or fail. "0" = failed xfer "1" =

good xfer. Keep those cards and letters coming!

V3.11.90 - ArrrrG! %#^^#%%@#*$(%*#$%($$%&*( the &*&^*&^ woes of

changes really &#$^& my &(&^#$.

V3.12.90 - Forget 3.11.90 ever existed! Several routines got a

face lift! You should see more efficiency with this release. I

can't begin to go through the entire chase with you here, but I

need a new can of RAID! Seems there is never a total end with a

bug problem, in programming as well as the home! I must offer a

special word of thanks to Dick Melville from San Marino for his

the help with RBBS interface problems. Dick was an excellent BETA

Buster! And to Steve Clancy who finally helped me see there was

NO carrier on the line!

V4.00.90 - {BETA} Version 4.00 represents a major revision

change to PCZ. As you have noticed, if you've used prior

versions, there are two PCZ files now instead of one. PCZ.EXE is

the global executable file and PCZ.OVR contains the actual

protocol functions. The decision to go with overlay functions was

made due to the fact that more Sysops are using PCZ with their

BBS's in a multitasking environment. Memory usage has been

drastically reduced. Additionally, each transfer type function

i.e. "sz,sx(1),ss,sy" & "rz,rx(1),rs,ry" is totally self

contained which simply means once the program loads and a

transfer type is called, no other overlay read is needed. The

LAN-LINK.DOC (c) G3ZCZ 1988 - 1994

APPENDIX 9 PCZ 4.06.90 Nov. 27, 1990 Page 165

function completes and exits. Simply put; execution time has been

enhanced.

V4.01.90 - {BETA} Dick & Steve

V4.02.90 - {BETA} Dick & Steve

V4.03.90 - {BETA} PcConnect, Dick & Steve

V4.04.90 - {BETA} PcConnect

V4.05.90 - Major revision V4.xx.90 released on the world!

V4.06.90 - Some enhancements added here include; Super_Z (tm) by

PC#�#, offers enhanced transfer times when used with error

correcting modems and relatively noise free transmission lines.

No limit to the number of files allowed in a batch transfer

command file. Non-standard PORT, IRQ line assignments supported

via environment setup. Frontend menu system for comm package use.

PCZ is supplied as a ".EXE" file which when executed breaks it's

code out into overlays. The largest executable code segment will

run well inside of a 64k memory segment.

Personal contact may be made by one of the following methods.

Drue Kennon Gary J. Smith

Rural Route #2 Box 54 6122 Cedar Wood Drive

Center, TX. 75935 Columbia, MD 21044

Voice 409-598-3809

The Programmer's Corner

Home of

"PcConnect" & "PCZ"

Data/BBS: 301-596-1180 Data/BBS: 301-995-3744

Data/BBS: 301-621-4636

10 roll over lines in place

with over 1.5 gigabytes of

storage space

DSZ, ZCOMM, Professional-YAM are Copyrights of Omen Technology

Inc, all rights reserved.

X00.SYS Copyright (c) by Raymond L. Gwinn, All Rights Reserved.

PCZ Copyright (c) by Drue Kennon, All Rights Reserved.

Super_Z(tm) is a trademark of PC#�#

** If you are the author of a communications package that has

zmodem implemented internally and would like to enable Super_Z in

you program for PCZ compatibility, look over SuperZ.Doc. If you

have any questions or problems then just drop me a line or call

and we will be glad to help you out.

LAN-LINK.DOC (c) G3ZCZ 1988 - 1994

APPENDIX 9 PCZ 4.06.90 Nov. 27, 1990 Page 166

The PCZ Menu Transfer System

This is the PCZ.CFG file.

;To make use of this transfer menu system PCZ should be called in

the following manner.

;PCZ m <ENTER>

;We assume in this config file that you are using SETENV.BAT to

preset your PCZ parameters for PORT and SPEED. If this isn't the

case then simply add them on the protocol lines below in their

proper place. You should be able to see from the listings the

format used here. i.e.

;SEND    without the ";" begins send protocol block

;END     without the ";" end the protocol block

;The protocol goes on a line followed by the ":" char followed by

PCZ's command line options for that protocol. You will be

prompted to complete the command line with your filespec(s) once

the program is started etc... pretty simple huh?

;Simple yes! but simple minded as well, and if you fail to follow

these guidelines.... who knows what type of protocol you might be

trying to use for a transfer. Just to satisfy your cats curiosity

you may uncomment the first line after SEND. Run PCZ using "m"

menu option, select upload and see what happens. No your pc won't

bark but it might barf if you select "A" as your protocol type.

;The following represent the send parameters. Using the ";" at

the beginning on a line comment out any protocol you do not wish

to use during you file transfer sessions.

SEND

;flag for beginning of send protocols

;TOSSIT:  sp  ;This is a curiosity test line

;XMODEM:  sx

;send a file using xmodem

;X1K:     s1

;send a file using xmodem 1k blocks

;YMODEM:  sy

;send a file using ymodem with file header information

ZMODEM:  s sz

;send a file using Super_Z zmodem

LAN-LINK.DOC (c) G3ZCZ 1988 - 1994

APPENDIX 9 PCZ 4.06.90 Nov. 27, 1990 Page 167

SEALINK: ss

;send a file using sealink

END

;end of send protocols

:The following represent the receive parameters.

RECEIVE

;flag for beginning of receive protocols

;XMODEM:  rx

;recv a file using xmodem

;X1K:     r1

;recv a file using xmodem 1k blocks

;YMODEM:  ry

;recv a file using ymodem with file header information

ZMODEM:  s rz

;recv a file using Super_Z zmodem

SEALINK: rs

;recv a file using sealink

END

;end of receive protocols

;You may add any of the other command line switches you wish to

the above. They are

; "f" use FOSSIL

; "d" no direct screen writes

; "k" keep all aborted downloads

;Thanks and have fun!

LAN-LINK.DOC (c) G3ZCZ 1988 - 1994

Appendix 10 Other Products by Joe Kasser, W3/G3ZCZ, Page 168

Appendix 10 Other Products by Joe Kasser, W3/G3ZCZ

1.0 PC-HAM 3.52

PC-Ham contains the following suite of programs.

1.1 LOGBOOK

Full blown logging package. With automatic check of logs for

awards such as DXCC. Allows you to recall any entry by call sign

within seconds. Indexed displays, QSLing, Contest mode QSLing

(prints the lot) and lots more. Although written in dBASE3 the

package contains a compiled version (LOGBOOK.EXE), so you don't

need dBASE to run it. The source code is ONLY given to registered

users. It can convert your exported LAN-LINK.LOG file to LAN-

LINK.DBF to put this package to work. Ideal for DX-peditions or

for DX robot users to handle QSLing and log statistics.

1.2 CONTEST

Keeps Dupes in memory, logs QSO's to disk in format which can be

processed by the LOGBOOK package. Now compiled in Turbo BASIC,

source code is supplied so that you can modify the program to

meet your requirements.

1.3 CQSS

Sweepstakes game compiled in Turbo BASIC. Work the ARRL

Sweepstakes contest on your computer. You are located just

outside Washington DC. A propagation model is built in to the

program. This program is REQUIRED training for all sweepstakes

operators. Earlier version of the program is described in detail

in Software for Amateur Radio by Joe Kasser G3ZCZ, published by

TAB Books, Blue Ridge Summit, PA. 17214.

1.4 WHATSON

Predict HF Propagation for given days. Contest mode with printout

to whole world at hourly intervals. Needs BASIC.

2.0 STARTREK The Computer Program

An ideal task for the beginner to learn a language on, is a

simulation game which is written to run on a computer that the

beginner has access to. For in that case, there is complete

control of all inputs and outputs. This kind of game (in which

the player makes decisions based on the information available to

him or her available at the time), can be made sufficiently

sophisticated and complex so as to make writing it an adequate

challenge for anybody.

LAN-LINK.DOC (c) G3ZCZ 1988 - 1994

Appendix 10 Other Products by Joe Kasser, W3/G3ZCZ, Page 169

The techniques used in writing a good game are the same that

programmers use in professional activities. Writing a good game

poses a challenge that allows you to develop good habits and

techniques for programming and also allows you to learn a

language in an interesting manner. By taking an orderly approach

to the game design, complex operations may be clearly understood

and converted to computer code with the aid of a language

reference manual, irrespective of the language being used.

This product teaches the techniques for writing such a game using

the STARTREK game as an example, and the BASIC language as the

programming language in which to write the code.

Registration fees.

Single Copy $15.00

10 - 50        Copies $12.00 per registered copy.

50 - 100       Copies $10.00 per registered copy.

100 +          Copies $8.00 per registered copy.

3.0 WHATS-UP 1.30

WHATS-UP is a tool for experimenting with orbital dynamics and

Telemetry Decoding and display Program for the UoSAT-OSCAR 2,

AMSAT-OSCAR 13, Fuji-OSCAR 20 and the AMSAT Microsat Spacecraft

(OSCARs 16, 17, 18 and 19). It is table driven via the

configuration files to allow maximum flexibility.

WHATS-UP contains the following features:

Can beacon [APRS] and/or automatically attempt a connect to

MIR/SAREX: Can automatically set Kenwood Radios to spacecraft

beacon frequency when satellite comes over your horizon: Can

automatically read the frequency from Kenwood Radios to a disk

file for Doppler frequency measurements: Can automatically

configure a PK-232 for each spacecraft data format: Reads 2 Line

and AMSAT format orbit element data files: Provides antenna

azimuth and elevation pointing data: Real time, Playback and Dumb

Terminal mode: Automatic logging of AOS in LAN-LINK compatible

logs for AOS time analysis: Automatic Capture-to-disk of raw

telemetry. Extracts telemetry channel data to a database or

spreadsheet readable file for further analysis: Link quality

measurement on packet telemetry:

Capability to print the raw telemetry as it is received: Up to 16

user configurable display pages (screens): You set the position

on the page (width of engineering unit field, and number of

decimal places) that a parameter is displayed at: Wild card page

(parameter shows up on all pages): Selectable display of

Engineering units or raw byte for each display page: Display of

LAN-LINK.DOC (c) G3ZCZ 1988 - 1994

Appendix 10 Other Products by Joe Kasser, W3/G3ZCZ, Page 170

raw packets (i.e. STATUS) Color changes if a parameter value

changed between successive frames: Audio and visual alarms if a

telemetry value exceeds, falls below or falls outside a preset

limit value(s): Dumb split screen terminal mode (in the manner of

LAN-LINK): Customizable colors, PC to TNC baud rate, data parity

and stop bits: Default spacecraft configuration files: Time of

day clock display (in HH:MM:SS format). Real time analysis

requires a receiver and demodulator. Analysis of playback data

can be performed with just the computer and WHATS-UP.

If you would like a copy of WHATS-UP and have captured any

telemetry to disk, especially daytime or southern hemisphere

passes, please put them on a disk and send it in with your

request, a mailer and return postage. Don't let the individual

data files become too long. I suggest that you rename them every

day in the format YYMMDD.SC, such as 900824.D17, 900824.F20, etc.

IF YOU ARE INTERESTED IN OSCAR AND SPACECRAFT TELEMETRY, THEN

THIS PROGRAM IS A MUST.

4.0 ELMER 1.00

ELMER is an Expert System Based on a Finite State Machine. There

are two versions of ELMER supplied with LAN-LINK. One is built in

for use in packet mode communications, when connected to someone

else. The second version is a stand alone version for use in

developing the text files and the logic. The stand alone version

lets you program and debug your own personal ELMER by yourself.

The stand alone version is a separate product and requires

separate registration.

5.0 BASIC PACKET RADIO

Basic Packet Radio explains how the computer can be used to

smarten up your use of packet. Not only does the book explain

packet radio, it comes with a full feature version of LAN-LINK

(for the PC) on disk and contains the manual for the program.

The topics covered in the book are outlined below.

Getting Started in Packet Radio, What Packet Radio Is, The Shared

Radio Channel, The Virtual Channel, The Equipment Needed to Hold

a Packet QSO, The TNC, TNC Modes, The Terminal Program.

Connecting the PC to the TNC, Command Convention, Making the PC

Communicate with the TNC, Determining and Changing the State of a

Parameter, Setting the Carriage Returns and Line Feed Display

Controls, Flow Control, Connecting the TNC to the Radio, Delays

in the Transceiver.

LAN-LINK.DOC (c) G3ZCZ 1988 - 1994

Appendix 10 Other Products by Joe Kasser, W3/G3ZCZ, Page 171

The Local Area Network, How TNCs Communicate, Controlling the

Display of Monitored Packets, Setting Your TNC to Display or

Ignore Packets, The Wireless LAN, Use of Packet Beacons, The

Alert Call, The Target Call, LAN Answering Machine, Mail Beacon

(Annunciator), Remote Beacon Shutdown.

Connecting to Someone, Connecting to Other Stations the LAN-LINK

Way, The Point and Shoot (MH List) Way, Using the Call Menu, Loop

Backs, Names, Handles and Paths, Connecting via NET/ROM and

TheNet, Connecting via KA-Nodes, The SAREX Call and Working MIR.

Extending the Range, Digipeating, Using NET/ROM and TheNet Nodes,

Using G8BPQ Nodes, Using KA-Nodes, Using MSYS Nodes, Using ROSE

Switches, Using LAN-LINK to Communicate via Nodes, Path

Determination to a DX Station, Finding Packet Signals, Using

Communication Satellites.

The Packet Bulletin Board System, How to Connect to the PBBS,

Connected to the PBBS, Connected, But What Next ?, Telling it

Your Name, Getting Information about the PBBS Itself, The Command

Prompt Line, Changing the Prompt Line, Listing the Messages and

Bulletins, Reading Messages, Sending Messages, Sending Messages

Around the World, Sending a Bulletin, Forwarding Bulletins,

Sending NTS Traffic, The Parts of the NTS Message, Killing

Messages, Using LAN-LINK to Send NTS Traffic, Handling NTS

Traffic, Files, Talk to the SYSOP, Connecting Elsewhere via the

PBBS, Obtaining On-Line Help, Logging Off the PBBS, The F6FBB

PBBS Features, Automating Message Reading, Message Headers.

The PacketCluster, Summary of commands and features, Smartening

Up PacketCluster Access With LAN-LINK.

Other Applications of Packet, Data Base Servers, Using a REQFIL

Server, Using a REQDIR Server, Using a REQQTH Server, Electronic

Newspapers, Non Real-Time QSOs, Automatic Beacons, Robots and

Contests, Propagation Research, Transmission Control

Protocol/Internet Protocol (TCP/IP).

This is a book to read once, then use as a reference over and

over again. If you decide you don't like the book, send it back

within 30 days for a swift refund.

LAN-LINK.DOC (c) G3ZCZ 1988 - 1994

Appendix 11 Page 172

Appendix 11 How Shareware Works

The Association of Shareware Professionals (ASP) has established

standards for its members and for any organization which has "ASP

Approved" status. The ASP wants to make sure the shareware

principle works for you. If you are unable to resolve a problem

with an ASP member or organization (other than technical

support), the ASP may be able to help. Please write to

The ASP Ombudsman, 545 Grover Road,

Muskegon, MI. 49442-9427, USA.

You are encouraged to copy the floppy disk and share it freely

with others. You have the luxury of trying out the product at

your own pace and in the comfort of your own home or workplace.

After you have used the material for a reasonable evaluation

period (30 days), you should either discontinue use of the

material or register your copy. Your support is important and

greatly appreciated. With it, Shareware authors are encouraged to

design and distribute new products. Without it, a great deal of

high quality, low cost software will cease to be available.

Why pay at all?

You receive support from the author. You receive a CURRENT copy

of the program. Your input and ideas help shape future products.

You have a sense of pride and ownership in having honestly

participated in the Shareware revolution. You help to keep

software prices down by supporting a distribution method which

doesn't depend on expensive advertising campaigns.

Be aware of the following restrictions, designed to protect the

community of Shareware users and to prevent greedy people from

taking unfair advantage of the trust, hard work and good will of

Shareware authors.

1. No price or consideration may be charged for the material.

However, a distribution cost may be charged for the cost of

the diskettes, shipping and handling, not to exceed $6.

2. The files and programs on the disks may not be modified or

deleted.

3. The material cannot be sold as part of some other more

inclusive package.

4. The material cannot be "rented" or "leased" to others.

5. The end user must be told clearly in writing on the outside of

LAN-LINK.DOC (c) G3ZCZ 1988 - 1994

Appendix 11 Page 173

the package and in all advertising that the diskette(s) are

"Shareware."

6. The package must contain a written explanation that the disk

is for evaluation purposes, and that an additional

"registration fee" is expected by the author, if the material

is used beyond an initial evaluation period.

7. In the case of distribution via any telecommunications link,

the following must be done:

* An error checking protocol must be used.

* The individual files must be combined into, and transferred

in a library or archive format.

8. Shareware distribution is permitted only in the United States,

Canada, England, and Australia.

LAN-LINK.DOC (c) G3ZCZ 1988 - 1994

Appendix 12 Page 174

Appendix 12 LAN-LINK 2.32 REGISTRATION FORM

PLEASE COPY this form and MAIL to

Joe Kasser G3ZCZ, P.O. BOX 3419, SILVER SPRING, MD 20918.

CALL ________________ TODAY'S DATE ________________

NAME ______________________________________________

ADDRESS ___________________________________________

___________________________________________________

CITY ______________________________________________

STATE _________ POSTCODE ____________ TNC TYPE_____

FIRMWARE REV ____ HOME BBS ______________

DISK SIZE 5.25 _____ 3.5 ____

Please register me as a user of LAN-LINK. I am currently using

LAN-LINK Version ______ which I obtained from _______. Please

send me the latest version of LAN-LINK or if a more recent one

does not exist at this time, QSL my registration and add my name

to the list to receive a free update when it becomes available.

I also enclose an additional amount for evaluation copies of

ELMER _, WHATS-UP _, PC-HAM _ and Startrek TCP _ ($5 for 1

program, $15.00 for the set). If I like them, I plan to register

them in due course.

Basic Packet Radio Book (29.95)                   __.__

Shipping for book $3.50 US/Canada, $6.00 overseas __.__

LAN-LINK Registration ($45.00)                    __.__

ELMER Registration ($45.00)                       __.__

Evaluation software                               __.__

____________________________________

Maryland Residents please add 5% sales tax. __.__

Total Enclosed                                    __.__

MasterCard/Visa Number ___________________________Expires __/__

My favorite operating modes are ________________________.

Many additions come into LAN-LINK as a result of user

suggestions, so here's your chance to get some input in. Write

them on this piece of paper.

LAN-LINK.DOC (c) G3ZCZ 1988 - 1994

Index Page 175

INDEX

Accessible, 125

AFSK, 74

Alarmwindow, 90, 151

Alt-A, 2, 19, 59

Alt-B, 2, 19

Alt-C, 2, 9, 19, 60, 101

Alt-D, 2, 21, 45, 77, 97, 108, 138

Alt-E, 2, 19, 22, 60

Alt-F, 2, 22, 32, 125

Alt-F, 50

Alt-G, 13

Alt-H, 2, 22, 60

Alt-J, 2, 22

Alt-K, 2, 22, 37, 38

Alt-L, 2, 22, 79

Alt-M, 2, 15, 22, 78

Alt-N, 2, 23, 101

Alt-O, 2, 23, 69, 79

Alt-P, 2, 23

Alt-Q, 2, 23, 36

Alt-R, 2, 23

Alt-S, 2, 24, 60, 79

Alt-U, 60

Alt-W, 2, 24, 53, 134, 144

Alt-X, 2, 8, 24, 61

Alt-Y, 2, 12, 24

Alt-Z, 2, 24, 37

AMTOR, 2, 3,  7, 10, 3, 4, 5, 6, 12, 14, 16, 17, 19, 20, 21,  22,

23, 26, 28, 30, 31, 32, 33, 39, 40, 47, 50, 54, 55,  56,

72,  73,  76, 79, 82, 83, 87, 91, 94, 95, 96,  97,  103,

104, 119, 125, 129, 134, 135, 138, 140, 144

AMTOR-FEC, 16

Announce, 111

Announcements, 12

Annunciator, 2, 5, 14, 111, 171

Anomalies, 10, 140

ANSI, 2, 6, 13, 50, 66, 144

AOS, 169

APLINK, 19, 87

APRS, 169

ARQ, 2, 19, 20, 21, 22, 30, 94

ASCII, 5, 7,  9, 3, 4, 5, 6, 10, 15, 16, 20, 26, 28, 41, 49,  51,

52,  96,  106, 115, 116, 123, 124, 125, 126,  129,  130,

133, 134, 138, 148, 157

ASP, 172

Attempt, 15,  19, 21, 30, 39, 41, 47, 54, 69, 77, 80,  134,  138,

140, 169

Audio, 4, 76, 79, 87, 88, 170

LAN-LINK.DOC (c) G3ZCZ 1988 - 1994

Index Page 176

Auto, 3, 4, 8, 30, 35, 39, 40, 41, 94, 108, 114, 117, 127

AUTO-Upload, 124

AUTOCQ, 14, 17, 94

AutoCR, 85, 86

Autoexec, 159

Battery, 8, 9, 64, 65

BAUDOT, 7,  8, 3, 4, 6, 11, 16, 17, 20, 26, 27, 28, 50,  63,  75,

95, 96, 104, 105, 114, 119, 125

BBS, 1, 8, 9, 10, 2, 5, 6, 7, 11, 20, 22, 24, 25, 26, 41, 44, 46,

62,  66, 72, 93, 94, 112, 114, 118, 124, 128, 130,  131,

138, 142, 144, 156, 157, 159, 160, 163, 164, 165, 174

BPQ, 171

BTEXT, 14, 77

Bulletin, 7, 16, 65, 82, 93, 113, 129, 171

Callsigns, 6, 30, 59, 60, 71, 72, 79, 82, 83, 126, 135, 140

Capture-to-disk,  4, 6, 11, 16, 27, 29, 42, 44, 45, 48,  49,  50,

69,  72, 73, 81, 83, 92, 103, 104, 106, 107,  113,  118,

122, 139, 169

CFROM, 86, 109

Checked, 12, 13

Chirpcopy, 6, 32, 97

Client-server, 14

CLOCK, 4, 10, 34, 46, 47, 64, 72, 74, 80, 89, 141, 170

Cluster, 8, 6, 12, 39, 87, 88, 94, 114, 119

CMSG, 70, 71, 73, 86

Colorized, 144

COMMENTS, 55, 56, 59, 60, 122, 125, 162

Compares, 121

CONFIG, 9, 112, 123, 166

CONOK, 71, 86, 109

Contest, 4,  5, 6, 5, 6, 12, 13, 17, 23, 38, 46, 47, 56, 57,  58,

72, 79, 80, 116, 144, 168

Control-C, 42

Control-F, 125

Control-Q, 142

Control-S, 142

Control-Z, 3, 28, 54, 125, 128

CONVERSE, 3, 10, 3, 24, 32, 52, 53, 65, 74, 75, 76, 78, 101, 102,

103, 130, 140

CQ, 3, 4, 6,  10, 6, 14, 17, 18, 19, 22, 30, 31, 32, 35, 39,  40,

41, 67, 69, 72, 84, 85, 94, 99, 100, 103, 104, 106, 112,

113, 118, 135, 137, 138, 140

CQs, 6

CQTEXT, 84

CR, 13, 95, 105

Ctdsk, 4, 8, 42, 44, 48, 49, 50, 72, 118

CTEXT, 6, 7,  6, 63, 64, 65, 70, 71, 73, 95, 112, 115, 127,  130,

144

LAN-LINK.DOC (c) G3ZCZ 1988 - 1994

Index Page 177

CW, 3, 7, 8,  10,  20, 27, 28, 36, 38, 74, 75, 88, 96,  97,  103,

104,  105, 106, 107, 108, 111, 114, 118, 119, 122,  125,

139, 144

CWID, 104

Date-time-text, 28, 54, 136

Daytime, 170

DBASE, 5, 11, 55, 57, 58, 60, 122, 168

DCD, 6, 72, 80, 102, 113, 138, 139, 141, 142

DCD-Flag, 141

DCDCONN, 102

DEmo, 4, 5, 42, 46, 51, 52, 55, 127, 144

Demonstration, 127

Demonstrations, 127

Digipeat, 5, 6, 18, 23, 69, 72, 74, 82, 83, 99, 122, 130, 138

Digipeater, 77, 82, 86, 109

Directory,  3, 4, 10, 5, 14, 21, 39, 40, 44, 45, 48, 49, 51,  62,

77, 83, 84, 113, 115, 122, 130, 150, 157, 159, 160

Directs, 119

Disconnect,  2, 3, 7, 4, 6, 14, 18, 20, 21, 23, 26, 29,  31,  32,

65, 72, 77, 81, 85, 94, 95, 102, 117, 118, 129, 141, 144

DMO, 4, 9, 42, 46, 112, 127

Downlink, 67

Download, 3,  9, 5, 11, 14, 24, 25, 26, 36, 37, 52, 54,  83,  84,

104, 113, 124, 129, 130, 132, 133, 136, 156

Dropouts, 117

DSP, 5, 10, 3, 9, 55, 63, 64, 66, 67, 74, 104, 119, 120, 139, 144

DSZ, 155, 165

Dual, 5, 8, 55, 63, 114, 119, 139

Duplex, 6, 3, 4, 67

Duplicates, 79

Echo, 6, 29, 31, 72, 76, 91, 122, 138

Echo-As-Sent, 138

Editor, 5,  10,  15, 29, 41, 42, 43, 48, 62, 70, 103,  115,  123,

133, 148, 149, 153

ELMER, 7, 8, 9, 10, 11, 5, 18, 34, 109, 110, 112, 113, 115,  116,

127,  144, 145, 146, 148, 149, 150, 151, 152, 153,  154,

170, 174

Ended, 74

Ensures, 74

EOF, 28, 44, 54, 124, 125, 128, 135

Equipped, 111

EVENT, 4, 5, 23, 25, 31, 34, 46, 47, 55, 65, 68, 73, 81, 82,  83,

95, 103, 129, 133, 136, 144, 163

EXpand, 4, 8, 48, 50, 113, 116

Expires, 174

Faked, 74

FEC, 3, 10, 6, 20, 22, 23, 30, 32, 40, 104, 140

LAN-LINK.DOC (c) G3ZCZ 1988 - 1994

Index Page 178

Flashing, 16, 17, 18, 19, 27, 40, 41, 69, 72, 73, 79, 92

FSK, 27, 74

Fuji, 8, 114, 120

Fuji-OSCAR, 67, 169

FULLDUPLEX, 67

G-TOR, 7, 13, 14, 20, 56, 72, 95, 96, 98, 125, 144

Garbage, 10, 117, 142

Gateway, 135

Grab, 13, 20

Greek, 10, 8, 89, 142

Header, 5, 25, 40, 62, 68, 80, 91, 102, 129, 134, 137, 138,  166,

167

HEADERLN, 122

Huffman, 108, 109

Icom, 120, 121, 144

Incoming, 2, 4, 10, 5, 6, 9, 13, 15, 16, 25, 26, 27, 29, 31,  32,

37, 39, 42, 44, 46, 50, 55, 62, 64, 65, 66, 69, 71,  74,

76, 77, 89, 90, 91, 92, 94, 95, 102, 103, 106, 107, 108,

116,  118, 119, 122, 126, 127, 138, 140, 142, 145,  152,

153

Interrogated, 110

Interrogates, 66

IRQ, 7, 88, 113, 115, 160, 161, 165

KA-Node, 5, 21, 122, 123

KAM, 9, 3, 9, 28, 31, 67, 75, 80, 83, 87, 89, 94, 96, 97, 98, 99,

100, 105, 107, 108, 116, 118, 122, 138, 140, 142, 144

KM, 94, 118

KN, 26, 27, 79

KPC, 9, 3, 65, 137

Larger, 41

LF, 13, 95, 105

LFADD, 75

LM, 25, 118, 124, 125

Log-to-disk, 72

Logcheck, 12

Mail-Snatch, 25, 81, 138

Marks, 122

MARS, 6, 17, 95, 105, 144

MBX, 9, 53, 109, 112, 123, 124, 135

MCON, 99

Metabeacon, 70, 71, 72, 102, 103

Meter, 12

MFILTER, 142, 143

MFJ, 9, 10,  3, 9, 31, 63, 67, 75, 76, 83, 89, 94, 96,  97,  118,

LAN-LINK.DOC (c) G3ZCZ 1988 - 1994

Index Page 179

137, 138, 144

MH, 3, 30, 53, 72, 90, 130, 134, 137, 138, 144, 171

MHwindow, 30, 90

Milliseconds, 120

MIR, 6, 67, 68, 69, 169, 171

MRPT, 40

Multi-connect, 5, 32, 65, 100, 142

Multi-mode, 3, 7, 118

Multi-stream, 23

Multi-user, 25, 41, 100

MYCALL, 104, 106, 107

NAVTEX, 7, 16, 96, 98, 144

NET, 5, 21, 122, 123, 125, 142, 171

NNNN, 94, 104, 105, 106, 107

Node, 6, 3, 9, 12, 72, 75, 77, 113, 118

NTS, 41, 171

Outwindow, 48, 89, 140, 150

Overseas, 174

PacketCluster, 2, 3, 8, 4, 7, 12, 17, 20, 22, 24, 36, 37, 51, 72,

83, 87, 98, 111, 113, 114, 116, 119, 121, 144, 171

PACTOR, 7,  3, 4, 5, 6, 11, 12, 13, 14, 20, 26, 50, 55,  56,  72,

94, 95, 96, 97, 108, 125, 139, 144

PBBSs, 81

PC#�#, 165

PCZ, 9, 11,  127,  132, 133, 143, 155, 156, 157, 158,  159,  160,

161, 162, 163, 164, 165, 166

PCZDIR, 160

PCZPORT, 156, 160, 161

PCZSPEED, 156, 160

PgDn, 42, 45, 59, 61, 62

PgUp, 2, 27, 42, 59, 61

Post, 13

Prematurely, 74

Presupposes, 74

Printer, 2, 6, 8, 16, 20, 23, 58, 72, 77, 114, 117, 144, 157

PromptWindow, 90, 150

PSK, 120

QBM, 5, 9, 51, 52, 83, 113, 129, 130, 133, 134

QBU, 5, 9, 51, 52, 84, 113, 126, 133, 136

QBU-RX, 9, 51, 52, 84, 112, 113, 126, 133

QBU-TX, 9, 51, 52, 84, 112, 113, 126, 133

QDB, 5, 9, 51, 52, 94, 130, 134

QIC, 5, 8, 9, 51, 52, 114, 119, 136, 144

QJG, 9, 53, 135

QMH, 5, 9, 5, 51, 53, 130, 131, 134, 137, 144

QNO, 9, 52, 54, 78, 94, 129, 134, 135

LAN-LINK.DOC (c) G3ZCZ 1988 - 1994

Index Page 180

QQQ, 94

QRA, 2, 5, 9, 20, 24, 53, 134

QRM, 3, 31, 68, 73, 134

QRT, 9, 17, 52, 54, 68, 73, 86, 109, 113, 134, 135, 136

QRU, 5, 9, 51, 53, 94, 123, 124, 134, 135

QRV, 9, 52, 54, 94, 128, 135

QRX, 52, 136

QRZ, 2, 3, 20, 23, 35, 36, 70, 72

QSL, 9, 52, 54, 55, 60, 68, 70, 72, 94, 129, 135, 174

QSM, 5, 9, 15, 51, 53, 126, 134

QSO, 7, 9, 10, 11, 4, 5, 14, 17, 19, 22, 23, 29, 32, 47, 51,  71,

73,  79,  80, 94, 95, 97, 99, 104, 106, 109,  110,  112,

127, 140, 144, 145, 150, 152, 153, 154, 170

QSP, 5, 6, 9, 17, 51, 54, 55, 72, 78, 112, 124, 128, 134, 135

QSY, 7, 8, 4, 13, 68, 73, 110, 111, 114, 121

QTA, 5, 9, 51, 54, 112, 126, 129, 135

QTC, 6, 9, 5, 14, 15, 17, 72, 73, 77, 78, 81, 94, 113, 135

QTC-Snatch, 15, 78, 138

QTR, 5, 9, 51, 54, 136, 144

QYU, 51

QZD, 5, 9, 51, 54, 126, 132, 133, 136

QZD-RX, 126, 133

QZD-TX, 126, 133

QZU, 5, 9, 51, 55, 126, 132, 133, 136

QZU-RX, 126, 127, 133

QZU-TX, 126, 133

Random, 27, 119

Receive-Disconnect, 3, 4

REQDIR, 171

REQFIL, 171

REQQTH, 171

Reregister, 11

Results, 12

Retry, 2, 20, 22, 23, 67

Reverse, 27, 134, 161

Robot, 2, 4, 14, 17, 18, 19, 20, 23, 46, 47, 69, 70, 71, 72,  73,

103, 104, 168

Robot-Mailbox, 31

RTTY, 2, 7, 8, 6, 17, 24, 26, 28, 31, 74, 96, 106, 114, 120, 138

RY, 2, 24, 158, 164, 167

SAREX, 6, 17, 18, 67, 68, 69, 70, 71, 72, 92, 113, 169, 171

Scrollback, 2, 13, 27, 90

SELCAL, 6,  7,  17, 19, 20, 21, 22, 23, 32, 39, 40, 82,  83,  94,

103, 105, 106, 107, 112

Separates, 119

Simulate, 55

Simultaneous, 63, 120

SOLO, 18, 75, 86, 99, 118

LAN-LINK.DOC (c) G3ZCZ 1988 - 1994

Index Page 181

Speak, 7, 110, 111

Spot, 20

Spotted, 98

SSID, 83, 126

State-sequence, 93

StatusWindow, 90

Staying, 74

Stopbits, 8

Subdirectory, 41

Supply, 111

Target, 4, 5, 6, 17, 18, 39, 40, 41, 73, 171

TheNet, 122, 171

Therein, 98

Timeout, 8, 85, 113, 116, 144

Transparent, 19, 32, 52

Tunes, 121

Unproto, 69, 71, 100

Unshift, 17, 105

Untested, 144

UPGRADE, 7, 8, 11, 144

Uplink, 67

USB, 111

Verbose, 81, 82

VFO, 7, 110, 111

Welcome, 95

XFLOW, 11, 142

XMITOK, 10, 75, 97, 107, 144

Xmodem, 3, 36, 37, 131, 132, 155, 158, 163, 166, 167

XOFF, 161

XON, 161

YAPP, 123, 131

Ymodem, 3, 36, 37, 132, 155, 156, 158, 163, 166, 167

Zap, 2, 3,  4, 6, 7, 10, 6, 16, 17, 20, 24, 25, 26, 36,  37,  39,

41, 44, 46, 47, 66, 67, 68, 69, 72, 74, 80, 81, 82,  93,

95, 142

Zmodem, 9, 10, 6, 54, 55, 126, 127, 132, 133, 136, 143, 155, 156,

157, 158, 159, 161, 163, 165, 166, 167

LAN-LINK.DOC (c) G3ZCZ 1988 - 1994

########################################################################################