TUCSON AMATEUR PACKET RADIO

Packet Radio

Terminal Node Controller

TNC 2, Rev 2

ASSEMBLY MANUAL

Tucson Amateur Packet Radio                 TNC 2 Assembly Manual

Copyright (c) 1985, 1991

Tucson Amateur Packet Radio Corporation

P.O. Box 12925

Tucson, AZ  85749

(602) 749-9479

FAX  (602) 749-5636

All rights reserved.

Reproduction  or translation of any part of this work beyond that

permitted  by Sections  107  or 108  of  the 1976  United  States

Copyright  Act  (or  its  legal successor)  without  the  express

permission of the  copyright owner  is unlawful  except as  noted

below.     Requests  for  permission  to   copy  or  for  further

information should  be addressed  to Tucson Amateur  Packet Radio

Corporation.

Except  as  noted  above,  permission is  hereby  granted  to any

non-profit group  or individual to reproduce any  portion of this

document provided  that: the reproduction is not sold for profit;

the  intent  of  the   reproduction  is  to  further  disseminate

information on packet  radio; the  reproduction is  not used  for

advertising  or  otherwise   promoting  any  specific  commercial

product;  and full credit is given to Tucson Amateur Packet Radio

Corporation  (including  address)  as   the  original  source  of

information.

June 1985

Revised June 1991

i

Tucson Amateur Packet Radio                 TNC 2 Assembly Manual

SPECIAL EDITION FOR ASSEMBLY OF PC BOARDS

EXCLUSIVE OF THE COMPLETE KIT

1991

The  information contained  in this  document has  been carefully

checked and is  believed to  be entirely reliable.   However,  no

responsibility is  assumed  for  inaccuracies.    Tucson  Amateur

Packet  Radio  Corporation  (TAPR)  reserves the  right  to  make

changes  to  any products  to  improve  reliability, function  or

design  without obligation to  purchasers of  previous equipment.

TAPR does not assume any liability arising out of the application

or use of any  product or circuit described herein;  neither does

it convey  any license under its  patent rights or  the rights of

others.

This document was  originally prepared for  assembly of the  TAPR

TNC 2  kits.  These kits  are no longer available  from TAPR, nor

are any of the parts, with the exception of programmed EPROMS for

the state machine and  firmware.  This document has  been revised

for use with bare printed circuit boards furnished by TAPR.  Some

of the  assembly  techniques described  and  parts shown  may  be

unique to the  parts contained in the TAPR TNC 2  kit.  This does

not imply that  substitutions are not possible,  however TAPR has

not evaluated all of the possible effects  of parts substitution,

and   assumes  no   responsibility   for  any   peculiarities  in

performance.  In selecting  parts for use with these  boards, you

should become familiar with  both the layout of the  board itself

and the  assembly techniques  described herein, in  selecting the

proper  part for the application, prior to assembly.  Included in

the  revision is  the deletion  of  the MF-10  switched capacitor

filter,  which results in increased  dynamic range for the XR2211

demodulator and the upgrade to 32K RAM, which was not part of the

original kit, but is required for use with the current version of

TAPR  firmware.  In addition,  it is HIGHLY  RECOMMENDED that you

modify the DCD circuit as described by Eric Gustafson, N7CL in an

article published  in the  Proceedings of  the 7th  ARRL Computer

Networking Conference  (American  Radio Relay  League,  225  Main

Street, Newington, CT 06111).  TAPR offers a kit for this purpose

which makes the modification fairly painless.

ii

Tucson Amateur Packet Radio                 TNC 2 Assembly Manual

Parts List

Resistors

1/4 watt, 5%

( ) 1  47 ohm    (yellow-violet-black-gold)

( ) 6  100 ohm   (brown-black-brown-gold)

( ) 1  200 ohm   (red-black-brown-gold)

( ) 5  470 ohm   (yellow-violet-brown-gold)

( ) 1  560 ohm   (green-blue-brown-gold)

( ) 7  1k ohm    (brown-black-red-gold)

( ) 2  4.7k ohm  (yellow-violet-red-gold)

( ) 1  5.6k ohm  (green-blue-red-gold)

( ) 26 10k ohm   (brown-black-orange-gold)

( ) 1  27k ohm   (red-violet-orange-gold)

( ) 4  47k ohm   (yellow-violet-orange-gold)

( ) 15 100k ohm  (brown-black-yellow-gold)

( ) 1  150k ohm  (brown-green-yellow-gold)

( ) 2  470k ohm  (yellow-violet-yellow-gold)

( ) 1  510k ohm  (green-brown-yellow-gold)

( ) 3  1M ohm    (brown-black-green-gold)

1/8 watt, 1%

( ) 1  16.2k ohm (brown-blue-red-red-brown)

( ) 1  22.1k ohm (red-red-brown-red-brown)

( ) 1  30.1k ohm (orange-black-brown-red-brown)

( ) 1  47.5k ohm (yellow-violet-green-red-brown)

( ) 1  100.k ohm (brown-black-black-orange-brown)

Trimpots

( ) 2  10k ohm   (3299W-10K)

( ) 1  20k ohm   (3299W-20K)

( ) 1  50k ohm   (3299W-50K)

Capacitors

Mylar or Monolithic

( ) 1  10 pF     (10, 10J or 100)

( ) 1  100 pF    (101)

( ) 1  220 pF    (221)

( ) 1  0.001 uF  (.001 or 102)

( ) 1  0.0015 uF (.0015 or 152)

( ) 1  0.0022 uF (.0022 or 222)

1

Tucson Amateur Packet Radio                 TNC 2 Assembly Manual

( ) 1  0.0047 uF (.0047 or 472)

( ) 8  0.01 uF   (.01 or 103)

( ) 2  0.022 uF  (.022 or 223)

( ) 29 0.1 uF    (.1 or 104)

( ) 1  0.15 uF   (.15 or 154)

Electrolytic (and Trimmer)

The electrolytic capacitors may have  voltage ratings equal to or

greater than those listed below, but not less.

( ) 1  1 uF, 16v Radial TANTALUM

( ) 4  1 uF, 16v Radial

( ) 8  10 uF, 16v Radial

( ) 1  100 uF, 16v Radial

( ) 1  1000 uF, 16v Axial

( ) 1  60 pF Trimmer

Integrated Circuits

Integrated circuits  may come from various  manufacturers and may

have differing  prefixes and/or  suffixes.   For example,  if the

part is  listed  as a  74LS00,  it  may be  marked  SN74LS00N  or

MC74LS00P or DM74LS00N or  F74LS00P or a similar variation.   The

key is  that the sequence 74LS00  appears in the part  number.  A

four-digit number, such as  8515, indicates the year and  week of

manufacture  and  should not  be confused  with the  part number.

Typical parts are shown in many instances.

WARNING:    Exercise extreme  caution  when handling  ICs.   They

should  be  kept in  a black  foam  carrier or  other anti-static

protective container until you  are ready to install them.   This

precaution cannot be overemphasized!

( ) 1    324 Quad Op Amp                        LM324

( ) 1    358 Dual Op Amp                        LM358

( ) 1    556 Oscillator/Timer                   LM556

( ) 1    2206 FSK Generator                     XR2206

( ) 1    2211 PLL Demodulator                   XR2211

( ) 1    2764/27C64 EPROM, Programmed, "STATE MACHINE"

( ) 1    27C256 CMOS EPROM, Programmed, "TAPR FIRMWARE"

( ) 1    32K CMOS Static RAM                    43256C-15L

( ) 1    8440/84C40 SIO/0 Dual Serial Port       Z8440

( ) 1    84C00 or 70008 CMOS Z80 CPU            Z84C00

( ) 1    4040 CMOS Divider                      CD4040B

( ) 1    74HC4066 Quad CMOS Switch              74HC4066

( ) 1    74HCT04 Hex Inverter                   74HCT04

2

Tucson Amateur Packet Radio                 TNC 2 Assembly Manual

( ) 3    74HC14 Hex Schmitt Inverter            74HC14

( ) 1    74HC/HCT107 Dual J-K Flip Flop         74HC107

( ) 1    74HC/HCT139 Dual Decoder               74HC139

( ) 1    74HCT374 Octal Latch                   74HCT374

( ) 1    74HC/HCT393 Dual Binary Divider        74HC393

Sockets

( ) 1   08-pin DIP Socket

( ) 11  14-pin DIP Socket

( ) 5   16-pin DIP Socket

( ) 1   20-pin DIP Socket

( ) 3   28-pin DIP Socket

( ) 2   40-pin DIP Socket

Connectors

( ) 5   2-pin male header

( ) 2   3-pin male header

( ) 1   5-pin male header with "wall"

( ) 1   14-pin male DIP header

( ) 1   16-pin male DIP header

( ) 1   5-pin DIN female, PC right angle

( ) 1   5-pin DIN male, cable type, with boot *

( ) 1   DB25S 25-pin female PC right angle

( ) 1   2.1mm Power PC right angle

( ) 1   2.1mm Power Cable w/connector

( ) 3   Jumper, Push-On

* The DIN5-180P consists of the following items:

1    Body, Connector

1    Boot, Insulating

1    Shield, Upper, Metal

1    Shield, Lower, Metal

1    Ferrule, Metal

5    Pins, Male

Diodes

( ) 2  1N754, Zener, 6.8v

( ) 6  1N4001, Rectifier, 1A

( ) 7  1N4148, Switching

( ) 1  1N4746, Zener, 18v

( ) 1  1N4752, Zener, 36v

( ) 5  LED, Red, Rectangular

3

Tucson Amateur Packet Radio                 TNC 2 Assembly Manual

Transistors and Voltage Regulator

( ) 1  TX101 or VN10KM VFET

( ) 9  2N3904 NPN Switching

( ) 3  2N3906 PNP Switching

( ) 1  7805CT +5v Regulator

Miscellaneous

( ) 2   4-40 x 3/8 screw

( ) 2   #4 Star Washer, Internal Tooth

( ) 2   4-40 hex nut

( ) 1   DIP Switch, 8-position, Right Angle

( ) 1   Switch, DPDT, Push-On/Push-Off

( ) 1   Cap, Grey, Push Button

( ) 1   4.9152 MHz Crystal, HC-18/U

( ) 1   10 uH inductor, molded,large

( ) 2   10 uH inductor, molded, small

( ) 5   Self-tapping screws, Phillips head

( ) 4   Stick-on rubber feet

( ) 1   Clip-on Heat Sink

( ) 1   Shielded Jumper Wire

( ) 1   Battery, Lithium, 3-volt

NOTE:  The Lithium  battery used in TNC 2  must be

handled with  extreme care.   NEVER ALLOW  A SHORT

CIRCUIT  ACROSS  THE  BATTERY  TERMINALS   OR  THE

BATTERY  COULD EXPLODE  AND CAUSE  SERIOUS INJURY!

Leave the battery in its envelope until called for

in the instructions.

( ) 1   Printed Circuit Board, TNC 2, Rev 3

4

Tucson Amateur Packet Radio                 TNC 2 Assembly Manual

TAPR TNC 2 Assembly Directions

This  section will  proceed  in a  step-by-step fashion.   Please

mount and solder the components to the board in the  order given.

It  has been  tested on  numerous units  and found  to be  a very

efficient order for assembly.

When mounting  axial lead discrete components,  such as resistors

and certain  diodes, grasp the body  of the part in  one hand and

bend the  leads of the device with the other, such that the leads

will pass through the holes provided on the board.  Note that all

axial  lead components (except C12 and R80)  are on 0.5 inch lead

centers.   Place the  part so the  body is parallel  to, and flat

against, the  circuit board  unless instructed otherwise.   Next,

bend the leads on the bottom  of the board slightly to secure the

part.  Then proceed to  mount the next component.  Do  not solder

the part until directed.

NOTE:  The tab of regulator Q3 (7805CT) can be used as a  bending

jig!  Simply place the resistor or diode across the tab on Q3 and

form the leads around the tab.  The leads will  then be spaced at

0.5".

Soldering

When soldering parts  to the  printed circuit board,  use only  a

very  small tipped  controlled temperature  (700 degrees  F., 371

degrees  C.   maximum)  soldering  iron  and   fine  (.050  inch)

rosin-core  solder.   63/37 (eutectic)  solder is  preferred, but

60/40 is acceptable.

To  ensure the  joint is  properly soldered,  the iron  should be

placed so that it contacts the pad  on the board and the lead  to

be soldered.   Solder should then  be applied to the  pad and the

opposite side  of the lead  from the iron.   Thus, the  iron must

heat both the pad and the lead to cause the solder to melt.  This

helps  prevent cold solder joints.  Also, keep a wet sponge handy

and wipe the soldering iron tip  on it frequently to keep it (and

your connections) clean.

Care must be taken to  ensure that the pad is not  overheated, or

it may lift  from the PC  board!  Apply heat  only when ready  to

solder.  Do not  apply heat longer than  necessary to complete  a

good joint, but be sure to heat adequately for the solder to flow

completely.

NOTE:  All modern  components are designed for soldering  and you

should not worry about 5-10 seconds of heating.

5

Tucson Amateur Packet Radio                 TNC 2 Assembly Manual

Board Mapping

Place  the circuit board on  a clean working  surface. Orient the

board  so the  lettering  is in  normal  reading position.    For

purposes of description, the board  consists of four sectors. The

top of  the board is the  edge nearest the large  axial capacitor

(C12).  The bottom, left and right follow in natural  order.  The

upper left is quadrant one, the upper right is  quadrant two, the

lower  left is  quadrant three  and the  lower right  is quadrant

four.  The board assembly will proceed by quadrants.

Board Modification

The TNC  2 was originally designed to use two (2) 6264 static RAM

ICs, giving a total memory of 16K.  TAPR firmware revisions 1.1.4

and later require 32K  of RAM.  This  uses a single 32K  RAM chip

(U24 is not used).   This changes requires a  slight modification

to the PC board.

( ) Locate JMP12 on the board.  It is in quadrant 2 near the

bottom and  to  the  right  of  U6 with  the  board  oriented  as

described above.

(  ) Turn  the board over  so that  the circuit  side can be

seen.   There is a  trace connecting the  top and center  pads of

JMP12.  Cut this trace.

(  )  Add a  jumper between  the bottom  and center  pads of

JMP12.

( ) Turn the board  over again so that it is in the position

described above.

IC Sockets

NOTE:  If  any socket  pins are bent,  carefully straighten  them

with a pair of long-nose pliers.

When installing  IC sockets, tack-solder  two diagonally opposite

corners  first (such  as pin  1 and  pin 8  on a  14-pin socket).

Double check to ensure that the socket is seated properly against

the board with  the notch, bevelled corner or   "1" nearest pin 1

(pin 1  is the upper left  corner when viewed from  the component

side  of the board on all  socketed ICs)  and  that all IC socket

pins are  showing on the solder  side of the board.   Then solder

the remaining pins of  that socket before proceeding to  the next

one.

6

Tucson Amateur Packet Radio                 TNC 2 Assembly Manual

NOTE:  Take care now to avoid solder bridges!

If you find  a socket is difficult to install,  remove it, as you

are probably bending a pin!

Now begin installing sockets.

(quadrant 1)

( )  U1   16-pin

( )  U2   14-pin

( )  U7   14-pin

( )  U8    8-pin

( )  U3   14-pin

( )  U9   14-pin

(quadrant 2)

( )  U4   14-pin

( )  U10  14-pin

( )  U11  14-pin

( )  U5   20-pin

( )  U6   28-pin

(quadrant 3)

( )  U15  14-pin

( )  U16  16-pin

( )  U19  16-pin

( )  U20  14-pin

(quadrant 4)

( )  U21  40-pin

( )  U22  40-pin

( )  U12  16-pin

( )  U13  14-pin

( )  U14  14-pin

( )  U23  28-pin

( )  U25  28-pin

Now check your work.  All leads should be soldered.  There should

be no solder bridges (a  blob of solder that shorts two  adjacent

soldered connections) or cold (grey and/or grainy-looking) solder

connections.  The positions on the printed circuit board for U17,

U18  and U24 will  be empty.   You should have  one (1) 16-pin IC

7

Tucson Amateur Packet Radio                 TNC 2 Assembly Manual

socket remaining.

( )       OK so far.

Since  we  have deleted  the  MF10 switched  capacitor  filter, a

jumper  must be connected to  route the received  audio signal to

the demodulator.

( )  Solder an insulated wire from the  plated through holes

of  U17 pin  8 to  U18 pin  1 (no  sockets are  mounted in  these

positions).

Resistors

Install the following 5% resistors on the board:

(quadrant 1)

( )  R1   47 ohm    (yellow-violet-black-gold)

( )  R2   10k ohm   (brown-black-orange-gold)

( )  R3   100k ohm  (brown-black-yellow-gold)

( )  R5   100k ohm  (brown-black-yellow-gold)

( )  R7   10k ohm   (brown-black-orange-gold)

( )  R8   1k ohm    (brown-black-red-gold)

WARNING:   Be  careful when  clipping leads,  as they may  have a

tendency to fly towards your eyes!   Take appropriate precautions

(grasp leads and provide eye protection).

( )       Solder and clip the leads (12 total)

( )  R14  100 ohm   (brown-black-brown-gold)

( )  R15  100 ohm   (brown-black-brown-gold)

( )  R16  100 ohm   (brown-black-brown-gold)

( )  R17  100 ohm   (brown-black-brown-gold)

( )  R18  100 ohm   (brown-black-brown-gold)

( )  R19  100 ohm   (brown-black-brown-gold)

( )  R20  100k ohm  (brown-black-yellow-gold)

( )  R21  100k ohm  (brown-black-yellow-gold)

( )  R22  10k ohm   (brown-black-orange-gold)

( )  R23  10k ohm   (brown-black-orange-gold)

( )  R6   100k ohm  (brown-black-yellow-gold)

( )       Solder and clip leads (22 total)

( )  R24  100k ohm  (brown-black-yellow-gold)

( )  R25  27k ohm   (red-violet-orange-gold)

( )  R26  10k ohm   (brown-black-orange-gold)

8

Tucson Amateur Packet Radio                 TNC 2 Assembly Manual

( )  R27  100k ohm  (brown-black-yellow-gold)

( )  R28  10k ohm   (brown-black-orange-gold)

( )       Solder and clip leads (10 total)

( )  R36  1k ohm    (brown-black-red-gold)

( )  R37  100k ohm  (brown-black-yellow-gold)

( )  R38  10k ohm   (brown-black-orange-gold)

( )  R39  150k ohm  (brown-green-yellow-gold)

( )  R40  100k ohm  (brown-black-yellow-gold)

( )  R41  470k ohm  (yellow-violet-yellow-gold)

( )       Solder and clip leads (12 total)

( )  R42  100k ohm  (brown-black-yellow-gold)

( )  R43  100k ohm  (brown-black-yellow-gold)

( )  R44  47k ohm   (yellow-violet-orange-gold)

( )  R45  47k ohm   (yellow-violet-orange-gold)

( )  R46  1k ohm    (brown-black-red-gold)

( )  R47  1k ohm    (brown-black-red-gold)

( )  R48  1k ohm    (brown-black-red-gold)

( )       Solder and clip leads (14 total)

(quadrant 2)

( )  R9   10k ohm   (brown-black-orange-gold)

( )  R10  10k ohm   (brown-black-orange-gold)

( )  R11  1k ohm    (brown-black-red-gold)

( )  R12  10k ohm   (brown-black-orange-gold)

( )  R13  47k ohm   (yellow-violet-orange-gold)

( )       Solder and clip leads (10 total)

( )  R29  10k ohm   (brown-black-orange-gold)

( )  R30  10k ohm   (brown-black-orange-gold)

( )  R31  100k ohm  (brown-black-yellow-gold)

( )  R32  4.7k ohm  (yellow-violet-red-gold)

( )  R33  1k ohm    (brown-black-red-gold)

( )  R34  470 ohm   (yellow-violet-brown-gold)

( )  R35  470 ohm   (yellow-violet-brown-gold)

( )       Solder and clip leads (14 total)

(quadrant 3)

( )  R55  10k ohm   (brown-black-orange-gold)

( )  R56  5.6k ohm  (green-blue-red-gold)

9

Tucson Amateur Packet Radio                 TNC 2 Assembly Manual

( )  R57  560 ohm   (green-blue-brown-gold)

( )  R58  1 Megohm  (brown-black-green-gold)

( )  R59  1 Megohm  (brown-black-green-gold)

( )       Solder and clip leads (10 total)

( )  R60  200 ohm   (red-black-brown-gold)

( )  R61  10k ohm   (brown-black-orange-gold)

( )  R62  47k ohm   (yellow-violet-orange-gold)

( )  R64  100k ohm  (brown-black-yellow-gold)

( )  R65  100k ohm  (brown-black-yellow-gold)

( )       Solder and clip leads (10 total)

( )  R67  10k ohm   (brown-black-orange-gold)

( )  R68  10k ohm   (brown-black-orange-gold)

( )  R69  10k ohm   (brown-black-orange-gold)

( )  R70  10k ohm   (brown-black-orange-gold)

( )       Solder and clip leads (8 total)

( )  R71  10k ohm   (brown-black-orange-gold)

( )  R72  100k ohm  (brown-black-yellow-gold)

( )  R73  470k ohm  (yellow-violet-yellow-gold)

( )  R74  10k ohm   (brown-black-orange-gold)

( )  R98  4.7k ohm  (yellow-violet-red-gold)

( )       Solder and clip leads (10 total)

(quadrant 4)

( )  R49  10k ohm   (brown-black-orange-gold)

( )  R50  10k ohm   (brown-black-orange-gold)

( )  R51  10k ohm   (brown-black-orange-gold)

( )  R96  10k ohm   (brown-black-orange-gold)

( )  R97  470 ohm   (yellow-violet-brown-gold)

( )  R52  10k ohm   (brown-black-orange-gold)

( )  R53  470 ohm   (yellow-violet-brown-gold)

( )  R54  470 ohm   (yellow-violet-brown-gold)

( )  R75  10k ohm   (brown-black-orange-gold)

( )       Solder and clip leads (18 total)

There  should be  two 5%  resistors remaining,  one 510k  (green-

brown-yellow-gold), and one 1 Megohm (brown-black-green- gold).

( )       Two 5% resistors remaining.

10

Tucson Amateur Packet Radio                 TNC 2 Assembly Manual

The following devices are  trimpots.  When you install  one, tack

solder the center lead to the PC  board.  Do not solder the other

leads, nor clip any leads,  until directed.  This enables you  to

line up  all the trimpots neatly before you commit any of them to

a "final" position.

NOTE:  Orient each  trimpot with the adjustment screw  toward the

left edge of the board.

(quadrant 3 only)

( )  R76  50k

( )  R77  10k

( )  R78  20k

( )  R79  10k

( )      Line up R76-79.

( )       Solder and clip leads (12 total)

Now check your work.   All leads  should be soldered and  clipped

close to  the  bottom of  the  PC board.    You should  have  all

thirteen  1% and two 5% resistors remaining.   There should be no

solder bridges nor cold solder connections.

( )       OK so far.

Diodes

WARNING:  Diodes are  polarized!  The marked (banded  or tapered)

end of the diodes  (cathode) should match  the banded end of  the

silk screen.  You may  need to use a magnifying glass to read the

diode part numbers.

(quadrant 1)

( )  CR22 1N4001    (cathode to the top of the board)

( )  CR1  1N4746    (cathode to the top of the board)

( )  CR2  1N4001    (cathode to the top of the board)

( )  CR3  1N4001    (cathode to the top of the board)

( )  CR4  1N4001    (cathode to the top of the board)

( )  CR5  1N4001    (cathode to the top of the board)

( )  CR6  1N754     (cathode to the bottom of the board)

( )       Solder and clip leads (14 total)

( )  CR9  1N4148    (cathode to the top of the board)

( )  CR13 1N4148    (cathode to the left of the board)

11

Tucson Amateur Packet Radio                 TNC 2 Assembly Manual

( )  CR10 1N4148    (cathode to the bottom of the board)

( )       Solder and clip leads (6 total)

( )   Save one of  the clipped 1N4148  leads for use  during

modem calibration.

(quadrant 2)

( )  CR7  1N4001    (cathode to the top of the board)

( )  CR8  1N754     (cathode to the top of the board)

( )  CR11 1N4148    (cathode to the top of the board)

( )  CR16 1N4148    (cathode to the top of the board)

( )       Solder and clip leads (8 total)

NOTE:  The LEDs will be installed later.

(quadrant 3)

( )  CR12 1N4752    (cathode to the top of the board)

( )  CR14 1N4148    (cathode to the top of the board)

( )  CR15 1N4148    (cathode to the top of the board)

( )       Solder and clip leads (6 total)

(quadrant 4 -- none)

Now check  your work.   You  should have 5  LEDs remaining.   All

leads  should be soldered and clipped close  to the bottom of the

PC board.   There  should be  no solder  bridges nor  cold solder

connections.

( )       OK so far.

Capacitors

WARNING:   The  electrolytic capacitors  are polarity  sensitive.

The  silk   screen  indicates  the  positive   lead,  while  most

electrolytic  capacitors  have  the  negative lead  marked.    Be

careful to ensure the parts are correctly installed or damage may

occur at power up!

NOTE:  All capacitors, especially electrolytic, should be mounted

as nearly  flush to  the surface  of the  PC  board as  practical

12

Tucson Amateur Packet Radio                 TNC 2 Assembly Manual

without overstressing the leads.

(quadrant 1)

( )  C1   0.1 uF    (104)

( )  C2   0.01 uF   (103)

( )  C3   10 uF 16v radial electrolytic

( )  C4   0.1 uF    (104)

( )  C5   0.001 uF  (102)

( )  C6   10 uF 16v radial electrolytic

( )  C7   10 uF 16v radial electrolytic

( )  C58  1 uF 16v radial electrolytic

( )  C10  10 uF 16v radial electrolytic

( )  C11  0.1 uF    (104)

( )       Solder and clip leads (20 total)

( )  C31  10 uF 16v radial electrolytic

( )  C56  0.1 uF    (104)

( )  C32  0.1 uF    (104)

( )  C21  0.1 uF    (104)

( )  C22  10 uF 16v radial electrolytic

( )  C23  0.1 uF    (104)

( )  C59  0.01 uF   (103)

( )  C24  0.01 uF   (103)

( )  C51  10 pF     (10 or 100)

( )       Solder and clip leads (18 total)

(quadrant 2)

( )  C8   10 uF 16v radial electrolytic

( )  C9   0.1 uF    (104)

( )  C60  220 pF    (221)

( )  C13  0.1 uF    (104)

( )  C14  0.1 uF    (104)

( )  C25  0.1 uF    (104)

( )  C26  0.1 uF    (104)

( )  C61  100 pF    (101)

( )       Solder and clip leads (16 total)

( )  C12  1000 uF25v axial electrolytic

( )       Solder and clip leads (2 total)

( )  C15  0.1 uF    (104)

( )  C17  0.1 uF    (104)

( )  C19  0.01 uF   (103)

13

Tucson Amateur Packet Radio                 TNC 2 Assembly Manual

( )  C20 1 uF radial TANTALUM (105) (+ to bottom of board)

( )  C18  10 uF 16v radial electrolytic

( )  C16  100 uF 16v radial electrolytic

( )       Solder and clip leads (12 total)

(quadrant 3)

( )  C33  1 uF 16v radial electrolytic

( )  C34  1 uF 16v radial electrolytic

( )  C35  0.01 uF   (103)

( )  C57  0.1 uF    (104)

( )  C36  0.0015 uF (152)

( )  C37  0.1 uF    (104)

( )  C38  0.1 uF    (104)

( )       Solder and clip leads (14 total)

( )  C39  0.01 uF   (103)

( )  C40  0.1 uF    (104)

( )  C41  0.1 uF    (104)

( )  C42  0.01 uF   (104)

( )  C43  0.1 uF    (104)

( )  C44  1 uF 16v radial electrolytic

( )  C45  0.15 uF   (154)

( )  C46  0.1 uF    (104)

( )       Solder and clip leads (16 total)

( )  C47  60pF trimmer capacitor

( )  C48  0.1 uF    (104)

( )      Solder and clip leads (2 total - none on trimmer)

(quadrant 4)

( )  C27  0.1 uF    (104)

( )  C28  0.1 uF    (104)

( )  C29  0.1 uF    (104)

( )  C30  0.1 uF    (104)

( )  C49  0.1 uF    (104)

( )  C50  0.1 uF    (104)

( )       Solder and clip leads (12 total)

You should have six (6) monolithic capacitors remaining: one each

of 0.0022 uF  (222), 0.0047 uF  (472), 0.01 uF  (103) and 0.1  uF

14

Tucson Amateur Packet Radio                 TNC 2 Assembly Manual

(104); and two each of 0.022 uF (223).

( )       6 capacitors remaining.

Now  check your work.   All leads should  be soldered and clipped

close to the  bottom of the PC board.  There  should be no solder

bridges nor cold solder connections.

( )       OK so far.

Transistors

NOTE:   Transistors have three  leads and are polarity sensitive!

Be  sure  to  properly  match the  transistor  outline  with  the

silkscreened  image on the PC board.  The transistors have a flat

side and the  directions will  call out the  orientation of  this

outline  as   an  additional  precaution.     When  installing  a

transistor, be careful not to overheat the device.

Before installing  a transistor,  carefully bend the  center lead

away  from  the flat  side of  the  transistor body  and slightly

spread the two outside leads apart.

(quadrant 1)

( )  Q8   2N3906 (Flat side to the left edge.)

( )  Q9   2N3904 (Flat side to the left edge.)

( )       Solder and clip leads (6 total)

(quadrant 2)

( )  Q1   2N3904  (Flat side to the lower edge.)

( )  Q2   2N3904  (Flat side to the left edge.)

( )       Solder and clip leads (6 total)

( )  Q4   2N3904  (Flat side to the lower edge.)

( )  Q5   2N3906  (Flat side to the lower edge.)

( )       Solder and clip leads (6 total)

( )  Q6   2N3904  (Flat side to the lower edge.)

( )  Q7   2N3904  (Flat side to the left edge.)

( )       Solder and clip leads (6 total)

( )  Q15  2N3904  (Flat side to the lower edge.)

15

Tucson Amateur Packet Radio                 TNC 2 Assembly Manual

( )       Solder and clip leads (3 total)

(quadrant 3)

( )  Q10  VN10KM/TX101  (Flat side to the left edge.)

( )  Q12  2N3906  (Flat side to the right edge.)

( )       Solder and clip leads (6 total)

( )  Q13  2N3904  (Flat side to the right edge.)

( )  Q14  2N3904  (Flat side to the right edge.)

( )       Solder and clip leads (6 total)

(quadrant 4 -- none)

You  should  have  no  transistors (but  one  voltage  regulator)

remaining.

( )       No transistors remaining.

Remaining Components

NOTE:  In the  following steps you will complete  soldering parts

on the TAPR TNC 2 printed  circuit board.  Solder each  component

as you install  it in this  assembly phase.   Be sure to  proceed

carefully and continue to exercise good workmanship!

(quadrant 1)

( )  SW1 Locate the DPDT Power switch.  Note that it has two

sets of contacts; one set consists of 6 pins while  the other set

consists of 6 lugs with holes in them.

( )  Carefully install SW1  on the PC  board with the  white

plastic  actuator extending over the  left edge of  the PC board.

Be sure the switch is  "square" with the PC board and  resting on

the small legs protruding from the switch bottom.

( ) Clip off the 6 (six) lugs on the top of SW1.

( )  Now firmly press  on the grey  cap on the  power switch

shaft.  The cap should be horizontal, that is, parallel to the PC

board.   Press  it on  until it  "snaps" in  place on  the switch

shaft.

( ) P1  Locate the 2.1mm Power  Connector.  This  device has

16

Tucson Amateur Packet Radio                 TNC 2 Assembly Manual

three lugs on the bottom surface and one lug on the top surface.

(  ) Install P1 on the PC board.  Be sure it rests firmly on

the PC board surface.

( ) Clip off the lug on the top of P1.

( )  SW2 Install  the  8 pole  DIP switch.    Note that  SW2

"rides" about 1/16" (1.6 mm) above the surface of the PC board.

(  )        BE SURE THAT ALL SWITCHES  OF SW2 ARE IN THE OFF

POSITION  (or you may cause damage to  integrated circuit U1 in a

later step)!

NOTE:   When installing jumper  strips, be sure  that the shorter

pins are soldered to the PC board and the longer pins stick up.

WARNING:  Don't grip  jumpers with your fingers while  soldering.

The pins quickly get very hot!

( )  JMP4 Install a 2-pin header.

( )  L2  10 uH choke (this is one of the smaller chokes)

(quadrant 2)

( ) L1  10 uH choke (this is the larger choke)

( )  JMP5 Install a 2-pin header.

( )  Q3 Place the 7805CT regulator at location Q3,  with the

metal tab side of the part away from the near (right) edge of the

PC board (do not solder yet).

( )  Using one  of the #4 black self-tapping screws,  secure

Q3  to the case or other heat sink.   Be sure it is vertical, and

that the leads are passing through the corresponding holes in the

PC board.

( ) Solder the three leads of Q3 in place.

( )  Now unscrew Q3 and  remove the PC board  from the case.

Set the case aside until called for later.

( )       Trim the leads on Q3.

17

Tucson Amateur Packet Radio                 TNC 2 Assembly Manual

(quadrant 3)

( )    J1   25-pin  female  DB-25 connector.    Attach  this

connector with two  4-40 x  3/8 screws, #4  lockwashers and  4-40

nuts.  The  screw heads should  be on the solder  side of the  PC

board.  Be careful not to damage the traces near the power switch

end  of  the connector.   Be  sure to  solder  all 25  pins after

fastening this connector!

( )  J2   Install the 5-pin DIN connector.

( ) JMP7 Install a 2-pin male header

( ) JMP8 Install a 2-pin male header

( ) JMP9 Install two 3-pin male headers

( ) JMP10 Install a 2-pin male header

( ) J3   Install a 5-pin male header (wall toward U20)

( ) Y1   Install the 4.9152 MHz crystal (Do not overheat!)

(quadrant 2 and quadrant 4)

NOTE:   When soldering the LEDs in place, use a pair of long-nose

pliers and carefully bend  the leads at right  angles so the  LED

will lie flat on the PC board, with the shorter of the  two leads

(cathode) towards the bottom edge of the board and the "lens" end

of the LED hanging over the right side of  the PC board.  The LED

leads should be bent  at the point where the  leads become narrow

after exiting the  plastic body of the  device.  Solder  and clip

both leads before proceeding to the next LED.

WARNING:  When installing LEDS, do not spread the leads to secure

the LED to the board.  Bending the leads can fracture the plastic

casing and cause the device to fail.  Instead, hold the LED while

soldering it.

NOTE:  Remember  that  all LEDs  are  oriented  with  the cathode

towards the bottom edge of the PC board.  The LED cathode lead is

the shorter of the two.

( )  CR17 LED  "PWR"

( )       Solder and clip leads (2 total)

( )  CR19 LED  "CON"

( )       Solder and clip leads (2 total)

( )  CR18 LED  "STA"

( )       Solder and clip leads (2 total)

( )  CR21 LED  "PTT"

( )       Solder and clip leads (2 total)

( )  CR20 LED  "DCD"

( )       Solder and clip leads (2 total)

18

Tucson Amateur Packet Radio                 TNC 2 Assembly Manual

Bottom of PC Board

There are two  bypass capacitors,  one rf choke  and a  shielded,

insulated jumper that  mount on  the underside of  the PC  board.

There are no silkscreened locations for these parts, so pay espe-

cially close attention to the schematic diagram and board layout.

( )  C63  0.1 uF    (104) (at SW1)

( )  C62  0.01 uF   (103) (at J2)

( )  L3   10 uH choke (this is a smaller choke) (at J1)

( )       Shielded jumper.  Solder one  end at the hole near

SW1.

( )       Shielded jumper.  Solder the other end at the hole

near Q3.

Now lay the PC board  aside and prepare to wire the  DIP headers.

These  are the  devices  that plug  into  an IC  socket but  have

notched  pins   above  the  plug  to   allow  soldering  discrete

components to the header.

NOTE:   DO NOT overheat  the headers  when soldering or  they may

deform!  To help prevent heat damage to the header, put the extra

16-pin DIP socket in a vise and insert the header  in the socket.

Then, solder the parts to the header.

U15

The following devices mount  on a 14-pin  DIP header, U15.   Form

the  leads on  the  parts  so the  components  lay in  the  slots

provided on  the top of the  header.  Solder the  two leads, then

clip them  flush with the sides  of the post.   Note the bevelled

corner or notch marking pin 1 of the header.

NOTE:  The resistor in the next step is 5%.

( )  R83  1 Meg 5% (brown-black-green-gold)

(pins 3 and 12)

( )  C55  0.022 uF monolithic (223)

(pins 5 and 10)

( )  R81  16.2k 1% (brown-blue-red-red-brown)

(pins 6 and 9)

( )  R82  30.1k 1% (orange-black-brown-red-brown)

(pins 7 and 8)

( ) Set the header aside until it is called for later.

19

Tucson Amateur Packet Radio                 TNC 2 Assembly Manual

U19

The  following devices mount  on a 16-pin DIP  header, U19.  Note

the bevelled corner or notch marking pin 1 of the header.

( )  C52  0.022 uF monolithic (223)

(pins 2 and 15)

( )  R92  22.1k 1% (red-red-brown-red-brown)

(pins 3 and 14)

( )  R93  47.5k 1% (yellow-violet-green-red-brown)

(pins 4 and 13)

( )  C53  0.0047 uF monolithic (472)

(pins 5 and 12)

( )  C54  0.0022 uF monolithic (222)

(pins 6 and 11)

( )  R94  100k 1% (brown-black-black-orange-brown)

(pins 7 and 10)

NOTE:  The resistor in the next step is 5%.

( )  R95  510k (green-brown-yellow-gold)

(pins 8 and 9)

( ) Set the header aside until it is called for later.

( ) Discard the extra 16-pin socket.

Initial Checkout of TNC 2

For initial  checkout  you  will  need  a  voltmeter  capable  of

measuring up to 30 volts dc.  If you encounter any problems, such

equipment  as an  ohmmeter,  oscilloscope, and  logic probe  will

prove very useful.

NOTE:  Be sure  to insulate the TNC 2 PC  board assembly from any

conductive work surface during testing!

( ) Locate the 2.1mm power connector/cable.

( ) Strip about 1/4" (0.6 cm) of insulation from each lead on the

free end of the cable.

NOTE:   The power  supply used  should be current  limited to  or

fused for 500 mA.

( ) Connect the lead  with a white stripe  to a source of +10  to

+15 volts dc.

( ) Connect the  other lead to the (-) return side  of the +10 to

20

Tucson Amateur Packet Radio                 TNC 2 Assembly Manual

+15 volts dc power source.

( ) Depress  the power switch  on your TNC  2 a couple of  times.

Note that it latches in two positions alternately.

( )  Cycle the  power switch  until it latches  in the  OFF (out)

position.

( ) Connect the 2.1mm power cable to P1.

In the following steps, power will be applied  to  your  TNC.  Be

ready  to  remove   power quickly!   If you smell anything   that

resembles a burning TNC, remove power immediately.

If you have problems at any point, disconnect power and determine

the cause of the problem before proceeding!

( ) With power switch SW1 still in the out, or OFF position,

apply 10 to 15 volts dc to the power cable.

(  ) Using  an  accurate dc  voltmeter, measure  the voltage

between ground (at - lead of C12 in quadrant 2) and either of the

lug  stubs on top of switch SW1  nearest the push button.  Record

the voltage in the space below.

(Measured value: ___________________________)

( ) If the voltage recorded in the step above is below +10v,

greater than +15v or is negative, take corrective action.  DO NOT

PROCEED UNTIL  THIS  MEASUREMENT  IS  CORRECT OR  YOU  MAY  CAUSE

SERIOUS DAMAGE TO YOUR TNC 2!

( ) Switch SW1 to the in, or ON  position.  The PWR, PTT and

DCD LEDs should glow.

( ) Measure  the voltage at the  input side of Q3.   This is

the lead closest to LED  CR17 (PWR). BE CAREFUL TO NOT  SHORT THE

INPUT LEAD TO ANYTHING WHILE MAKING THIS MEASUREMENT!

(Measured value: ___________________________)

( ) The voltage measured in the above step should be no more

than 2 volts below  the input voltage  measured earlier.  DO  NOT

PROCEED  UNTIL THIS  MEASUREMENT  IS  CORRECT  OR YOU  MAY  CAUSE

SERIOUS DAMAGE TO YOUR TNC 2!

( ) Measure the voltage at the  output side of Q3.  This  is

the  lead farthest  from LED CR17.  BE CAREFUL  TO NOT  SHORT THE

OUTPUT LEAD TO ANYTHING WHILE MAKING THIS MEASUREMENT!

21

Tucson Amateur Packet Radio                 TNC 2 Assembly Manual

(Measured value: ___________________________)

( ) The voltage measured in the above step should be between

+4.7 and +5.3 volts dc.  DO NOT PROCEED UNTIL THIS MEASUREMENT IS

CORRECT OR YOU MAY CAUSE SERIOUS DAMAGE TO YOUR TNC 2!

( ) Switch SW1 to the out, or OFF position.

At this point, your TNC 2 positive voltage supply  is checked and

verified.  If  the above tests were not  successful, refer to the

Troubleshooting chapter  of the TNC 2 System Manual for help.  In

the following steps you will verify the operation of the negative

supplies used in the modem and computer interface circuits.

First,  however, it  is necessary  to review  proper IC  handling

procedures.

IC Installation

The ICs should be shipped to you in an anti-static carrier.  Many

of  them are VERY sensitive to static discharge.  The probability

of them being defective as shipped is extremely low.  They should

be handled with extreme care.  If you live in a climate  with low

humidity,  or if  you  have carpeting  in  your work  area,  take

special precautions.  You might try spraying the carpet with some

sort of  fabric softener/anti-static  preparation (such as  a 50%

solution of "Downy" and water) to reduce static.

If you have  a metal working surface, ground it!  Place a hand on

the  working surface  before  you touch  the ICs  and be  sure to

frequently  ground yourself to the working surface.  If you don't

have such a surface, use  a large cookie sheet or  aluminum foil.

Handle ICs  only  while sitting  quietly, never  while moving  or

walking!

Avoid static discharge near the ICs!

( ) Anti-static precautions understood and implemented.

Install  the ICs using special  IC handling tools,  such as those

produced by OK  Machine Tool.  If you lack  these, you may gently

rock both sides of an IC  on your metal work surface to make  the

leads straight and parallel to  each other and install it  in its

socket with firm, even pressure applied over the length of the IC

body.

WARNING:   The  TNC  circuit  board may  flex  excessively  (when

installing ICs) if it is laid on a hard, flat surface, due to the

mounting screws on J1 and the components on the bottom  of the PC

22

Tucson Amateur Packet Radio                 TNC 2 Assembly Manual

board.  Do not allow the board to flex when inserting ICs.

If you  make a mistake, or have  to remove an IC  for any reason,

special  anti-static removal tools  are available.   If you don't

have  access to such tools, you may try using a small screwdriver

and gently pry the IC out of the socket, a little at a time, from

alternate ends  of the IC, but to use other than the proper tools

exposes you to risking damage to your IC.

NOTE: It is  very easy  to bend  a pin or  fold it  under the  IC

during installation. Be sure to  check each IC as you  install it

in  the steps below to be sure  that all pins are actually in the

socket and  not tucked under the  IC or hanging over  the socket!

It is easy to make this error!

Now you may begin.

( ) Locate the LM556 IC.

( ) Install this IC in socket U2.  Be careful to ensure that

no leads  are bent.   Pin 1  of this  IC goes to  the upper  left

corner of the socket (as do all pin 1s on all ICs).

( ) Switch SW1 to the in, or ON position.

( ) Measure the  voltage between ground (the   negative lead

of C12) and the top end of R14 in quadrant 1.  This is the end of

R14 immediately adjacent to socket IC1 pin 9.

(Measured value: ___________________________)

( ) The voltage  measured in the  above step should be  more

negative than -6 volts.  For  a +12 volt input to TNC 2,  it will

typically measure around -11  v (around -7.5 v  when all ICs  are

later  installed).   DO  NOT  PROCEED UNTIL  THIS  MEASUREMENT IS

CORRECT OR YOU MAY CAUSE SERIOUS DAMAGE TO YOUR TNC 2!

( ) Switch SW1 to the out, or OFF position.

( ) Locate the LM324 IC

( ) Install this IC in socket U3.  Be careful to ensure that

no leads  are bent.   Pin  1 of this  IC goes  to the  upper left

corner of the socket (as do all pin 1s on all ICs).

( ) Switch SW1 to the in, or ON position.

( ) Measure the voltage at the end of R5 nearest the edge of

the PC board in quadrant 1.

23

Tucson Amateur Packet Radio                 TNC 2 Assembly Manual

(Measured value: ___________________________)

( ) The voltage measured in the above step should be between

- 4.7v  and -5.3v.    DO NOT  PROCEED UNTIL  THIS MEASUREMENT  IS

CORRECT OR YOU MAY CAUSE SERIOUS DAMAGE TO YOUR TNC 2!

( ) Switch SW1 to the out, or OFF position.

You have now verified the operation of all the  power supplies of

your TNC  2.   Once again, check  for any  loose wire  clippings,

solder splashes or other possible problem areas on your TNC 2.

( ) OK so far.

In the next steps you will  be installing the digital ICs on your

TNC 2.  Exercise extreme care when handling the ICs!

Install the following ICs:

(quadrant 1)

( )  U1   CD4040

( )  U7   74HC14

( )  U8   LM358

( )  U9   74HC14

(quadrant 2)

( )  U4   74HCT393

( )  U5   74HCT374

( )  U6   2764 (or 27C64) "STATE MACHINE"

( )  U10  74HCT04

( )  U11  74HC107

(quadrant 3 - none)

(quadrant 4)

( )  U21  8440 (or 84C40)

( )  U22  84C00

( )  U12  74HCT139

( )  U13  74HC4066

( )  U14  74HC14

( )  U23  27C256

( )  U25  43256C-15L

Double check to be sure that all ICs are in their correct sockets

and that there are no bent or  tucked under leads.  Pin 1 of  all

ICs should be at the upper left corner.

24

Tucson Amateur Packet Radio                 TNC 2 Assembly Manual

( )       All ICs installed correctly.

You  should have two ICs remaining (XR2206, XR2211) and all three

headers.  These will be installed later.

( )       Three ICs and three headers remaining.

( ) Carefully inspect all ICs again to be sure  they are all

seated  in their  sockets  and there  are  no bent  or  otherwise

improperly installed  pins.  Be sure pin 1 of each IC matches pin

1 of its socket or you may destroy the IC.

( )       Attach a clip-on heat sink to Q3.

The next  step will be  to apply power to  your TNC 2.   Exercise

care  and be  ready  to remove  power  the instant  any  problems

appear.  Remember,  if you let all the smoke out of a part during

these tests, it may never work  properly again!  If it is working

properly,  you  will see  a  certain  sequence of  LED  activity.

Please  read the following five  steps before you  perform any of

them!

( ) Read the  following 4 steps through and  understand them

thoroughly before you perform any of them!

( ) Switch SW1 to the ON position.

( ) Observe that all LEDs except PTT glow.

( ) After  a period of about 2 seconds, the CON and STA LEDs

should extinguish (CR18 and CR19).

( ) Switch SW1 to the OFF position.

Your  TNC 2 has just exercised the  power-on routines.  The "CON"

and  "STA" LEDs turning off  after first being  on indicates that

all the digital circuitry is probably working.   If this test was

not successful,  please refer  to the Troubleshooting  chapter in

your TNC 2 System Manual for helpful information.

The next  thing  to do  is verify  that  your TNC  2 is  able  to

communicate with  your terminal,  or computer running  a terminal

program.   Please refer to the chapter on Computer Interfacing in

your  TNC 2 System Manual  for details on  fabricating an RS-232C

serial  cable.   Once your cable  is made, please  return to this

section and continue with the following steps.

( ) RS-232C serial cable properly made.

Now set  your terminal  to 300,  1200,  2400, 4800  or 9600  baud

(slower is better).  Set the word length to 7 and parity to even.

25

Tucson Amateur Packet Radio                 TNC 2 Assembly Manual

If the number of stop bits is settable, select 1.

( ) Terminal set to one  of the above baud rates with proper

word length and parity.

( ) Now, connect your terminal (or computer) to your RS-232C

cable.

( ) Connect  the other end of the RS-232C  cable to your TNC

2.

( ) Apply power to your terminal (or computer).

( ) If you are using a computer, load and  run your terminal

program.

( )  Set switches 1 through  5 of SW2 to  correspond to your

terminal (or computer) baud rate as shown below:

Baud Rate           SW2 Setting

1    2    3    4    5

300          ON   OFF  OFF  OFF  OFF

1200          OFF  ON   OFF  OFF  OFF

2400          OFF  OFF  ON   OFF  OFF

4800          OFF  OFF  OFF  ON   OFF

9600          OFF  OFF  OFF  OFF  ON

( )       In addition, set SW2 switch 7 on, and SW2 switches

6 and 8 off.

( ) Switch SW1 ON.

( )       Observe that all LEDs except PTT (CR21) glow.

( )  After a period of about 2 seconds, the STA and CON LEDs

(CR18 and CR19) should extinguish.

(  )  After the  two LEDs  turn  off, the  following message

should appear on your terminal (or computer) display:

Tucson Amateur Packet Radio TNC 2

AX.25 Level 2 Version 2.0

Release nn

Checksum $xx

cmd:

(see software  release notes for values  of nn and

xx)

( )       A message stating "bbRAM loaded with defaults" may

26

Tucson Amateur Packet Radio                 TNC 2 Assembly Manual

also appear.  Do not be alarmed by this message!

( ) Now  enter a  <carriage return> (may  be marked  RETURN,

ENTER, NEWLINE, etc. on your terminal or computer).

The display should now look like that below:

Tucson Amateur Packet Radio TNC 2

AX.25 Level 2 Version 2.0

Release nn

Checksum $xx

cmd:

cmd:

Note  that the  second "cmd:"  line appears  after you  enter the

carriage return.

( ) Switch SW1 OFF.

Congratulations!   Your  TNC 2's  digital circuitry  and computer

interface circuitry has now been verified.  If you didn't get the

expected results, recheck the Computer Interfacing chapter in the

TNC  2 System  Manual  and consult  the troubleshooting  section.

After you have the problem (if any) resolved, the following steps

will take you through verification of the digital portion of your

TNC  2  radio interface,  then  the  completion, calibration  and

verification of the modem section!

Digital Loopback Testing

You  will now  conduct  a fairly  thorough  test of  the  digital

portion of  your TNC 2 radio interface system.  Known as "digital

loopback,"  this procedure  will feed  your packet  transmit data

signal at digital levels to your packet receiving circuitry.

Once  these tests are successfully completed, you will be able to

proceed  with  the  final  modem construction  and  testing  with

confidence that the digital  portion of your TNC 2  is performing

properly.   This test facility  may also prove  useful for future

troubleshooting of TNC 2.

In  some of the following steps you will install push-on jumpers.

They  are  installed  by  simply  pressing  them  down  over  the

indicated pins.  DO NOT solder them in place!

(  ) Install a  push-on jumper at JMP10.   The jumper should

bridge both pins at JMP10.

( ) Place your terminal or computer in proper operating mode

27

Tucson Amateur Packet Radio                 TNC 2 Assembly Manual

as outlined in the previous test.

(  ) Ensure  that  the RS-232C  cable  is properly  attached

between your terminal or computer and TNC 2.

( ) Switch SW1 to the ON position.

( ) After the signon message and  the cmd: prompt, enter the

following:

MY W1QRM

followed by a carriage return (abbreviated

<cr> in all following steps).

( ) TNC 2 should respond with:

MYCALL   was NOCALL

cmd:

( )       Now enter:

MY<cr>

( )       TNC 2 should respond with:

MYCALL   W1QRM

cmd:

( ) Verify that SW2 switch  7 in the ON position, and ensure

that SW2 switches 6 and 8 are in the OFF position.

( ) Now enter the following command to your TNC 2:

FULL ON<cr>

( ) TNC 2 should respond with:

FULLDUP  was OFF

cmd:

( )       Enter:

C W1QRM<cr>

( ) You should observe the PTT LED (CR21) blink on then off.

The CON LED (CR19) will then come on shortly after  you enter the

<cr> and the message:

cmd:*** CONNECTED to W1QRM

28

Tucson Amateur Packet Radio                 TNC 2 Assembly Manual

with NO cmd: prompt on the next line.

( )       Now enter:

hello self<cr>

( ) The PTT and STA LEDs should briefly come on, the PTT LED

blink off then on, then both LEDs go off, after which the display

should look like:

hello self

hello self

( ) Now enter a  Control-C character.  This is usually  done

by  depressing the "CTRL" key on your terminal, then pressing and

releasing  the letter "C" key, then releasing the "CTRL" key.  If

your  terminal or  computer doesn't work  this way,  consult your

terminal or computer manual  for instructions on entering control

characters.

( ) Your TNC 2 should respond with the cmd: prompt.

( )       Now enter:

D<cr>

( )       The PTT LED will blink, then the CON LED will turn

off and your TNC 2 should respond with:

cmd:*** DISCONNECTED

followed by NO cmd: prompt.

If you are successful to this point, you have entered a call sign

in your TNC  2, instructed the TNC to ignore  the DCD LED (CR20),

then established a "connection"  with yourself.  You had  a brief

packet QSO, then ended the QSO!

The next test will verify the  operation of the DCD circuitry, at

least from the digital point of view.

( )       Enter:

FULL OFF<cr>

( ) TNC 2 should respond with:

FULLDUP  was ON

cmd:

29

Tucson Amateur Packet Radio                 TNC 2 Assembly Manual

( )       Now enter:

C W1QRM<cr>

(  ) The  cmd: prompt  should appear.   After ten  or twenty

seconds, enter:

D<cr>

( ) Your TNC 2 should respond with:

*** retry count exceeded

*** DISCONNECTED

cmd:

( ) Switch SW1 to the OFF (out) position.

( ) Using a VERY THIN piece of cut off lead (from the 1N4148

diode) place a short at socket U20 (quadrant 3) from pin 4 to pin

6.  Be very careful not to stress the socket contacts.

( ) Switch SW1 to the ON (in) position.

( )       The DCD LED (CR20) should remain OFF.

( )       After the sign-on, enter:

MY W1QRM<cr>

C W1QRM<cr>

( )         The PTT  LED should blink, TNC  2 should respond

with:

cmd:*** CONNECTED TO W1QRM

and the CON LED should be illuminated.

( )       Now enter a Control-C character, then:

D<cr>

( )        The PTT LED should blink, your display should now

read:

cmd:*** DISCONNECTED

and the CON LED should extinguish.

( )       Switch SW1 to the OFF (out) position.

30

Tucson Amateur Packet Radio                 TNC 2 Assembly Manual

( ) Remove the push-on jumper from JMP10.

( ) Remove the wire jumper from socket U20 pins 4 and 6.

If  you experienced  problems in  the above  steps, refer  to the

troubleshooting section of your TNC 2 manual for help.

In  the next section, you will complete construction of the modem

and calibrate it.

Keeping in  mind the precautions regarding  handling ICs, install

the following ICs (quadrant 3).

( )  U16  XR2206

( )  U20  XR2211

Now install the  headers as follows, being careful to align pin 1

of the header with pin 1 of the socket:

( ) U15  14-pin header with one (1) capacitor  and three (3)

resistors.

( ) U17  16-pin header with eight (8) resistors only.

( )  U19  16-pin header  with three (3) capacitors  and four

(4) resistors.

CALIBRATION

Effective  with TNC  2  firmware version  1.1.7, new  calibration

procedures have been established.  These are described in the TNC

2  System  Manual.   The  original  calibrations described  below

should  be  compared to  the  new instructions  to  determine the

differences.

In  the  following steps,  you will  calibrate  your TNC  2 modem

section.   Exercise care, as a poorly calibrated modem may result

in poor performance in  packet operation, especially when dealing

with weak signals.

If  you have  access to  an accurate  frequency counter,  you may

increase the calibration accuracy by using it as described in the

calibration procedure below.

When  adjusting the trimpots, you should hear a soft "click" when

the element  is adjusted to the  end of its travel.   Turning the

element past this point will not result in damage to the trimpot;

however, it  is  suggested that  you do  not make  a practice  of

turning the adjustment screw past this point!

Use a small-tipped screwdriver  or special trimpot adjusting tool

31

Tucson Amateur Packet Radio                 TNC 2 Assembly Manual

in the following steps  as you adjust the  four trimpots on  your

TNC 2.

( )   R76  Preset  20 turns counterclockwise (CCW)  or until

you hear the element "click" at full CCW rotation.

( )   Now turn  the adjustment  screw on R76  3-1/2 complete

turns clockwise (CW).

(  )  R77  Preset 20 turns CCW or until you hear the element

"click" at full CCW rotation.

NOTE:  R78 and R79 will be preset CLOCKWISE, not counterclockwise

as the previous trimpots.

(  )  R78  Preset 20 turns  clockwise (CW) or until you hear

the element "click" at full CW rotation.

( )   R79  Preset 20 turns clockwise (CW)  or until you hear

the element "click" at full CW rotation.

(  ) Verify that your computer or terminal is still attached

to your TNC 2 and the proper baud rate selected.

( ) Switch SW1 to the ON position.

(  ) Three LEDs should  illuminate (PWR, CON  and STA), then

CON and  STA should  extinguish.   The DCD  and  PTT LEDs  should

remain off during the power-on cycle.

( ) After  the sign-on  message and cmd:  prompt, enter  the

following to set the 1200 Hz tone:

CALSET 438<cr>

( ) Your TNC 2 should respond with:

CALSET   was nn

cmd:

where nn is a number from 0 to 65535

( )       Enter the command

CALIBRATE<cr>

( ) Push on a jumper at JMP9 pins  1 and 6 (the top two pins

of JMP9).

( ) If you are using a frequency counter, place the probe at

32

Tucson Amateur Packet Radio                 TNC 2 Assembly Manual

the push-on jumper on JMP9.

( )  Push on a jumper at JMP4.

NOTE:   The STA and CON  LEDs are used as  a software- controlled

frequency  counter in  the calibrate  mode.   STA means  that the

frequency is  too high, go  lower.   The CON LED  means that  the

frequency is too low, go higher.

( ) Verify that the CON and STA LEDs are extinguished.

(  ) Press  the "K"  key on  your computer  or  terminal (no

carriage return is  needed for this command).  Either  the STA or

CON LED should illuminate along with the PTT LED.

( ) Press the space bar on your keyboard and verify that the

other   LED  (CON  or  STA)  illuminates  in  place  of  the  one

illuminated in the preceding step.

(  )  Press  the  space  bar  again  until  the CON  LED  is

illuminated.

( ) Slowly rotate the adjustment screw on trimpot R78 in the

CCW direction  until both the STA  and CON LEDs illuminate.   The

LEDs  are a bit slow to respond,  so be very careful when the STA

LED  begins to flicker.  You will  probably note a portion of the

adjustment when both LEDs illuminate; set the adjustment screw to

the center  of this band.   If you go  too far, only  the STA LED

will be illuminated; in this case, turn the adjustment screw CW.

( ) If you are  using a frequency counter, adjust R78  for a

reading of 1200 Hz.

( )       When both LEDs are steadily illuminated, press the

"Q" key  (with no carriage return).   Both LEDs (and  PTT) should

extinguish and the cmd: prompt should appear on your display.

( )         Enter the  following command to set  the 2200 Hz

tone on your TNC 2:

CALSET 239<cr>

( )       TNC 2 should respond with:

CALSET   was 438

cmd:

( )       Type

CALIBRATE<cr>

33

Tucson Amateur Packet Radio                 TNC 2 Assembly Manual

( ) Now  press the "K" key.   Either the CON or  the STA LED

should illuminate as well as PTT.

( )       Tap the space bar until the STA LED glows.

( )       Slowly rotate the adjustment screw of  trimpot R77

CW,  following the  instructions  explained above  in association

with the setting of R78.  Continue  the adjustment until both the

STA and CON LEDs glow steadily.

( ) If you are  using a frequency counter, adjust  R77 until

the counter reads 2200 Hz.

( )       Remove the jumper at JMP4.

( ) Verify that the STA, CON and PTT LEDs extinguish after 5

to  40  seconds.   This verifies  the  operation of  the transmit

watchdog timer.

( ) Press the "K"  key twice.  At the second press, the LEDs

should again glow.

(  ) While the LEDs are illuminated, and before the transmit

watchdog  times  out,  press  the  "Q"  key.    The  LEDs  should

extinguish and the cmd: prompt should appear on your display.

( ) If you  are using a frequency counter,  remove the probe

from JMP9.

( ) Remove the jumper at JMP9 pins 1 and 6.

In the next several steps you will be calibrating the demodulator

portion of  the modem.  This  is the most  critical adjustment on

TNC 2's modem.  The first procedure uses the on-board calibration

facility.  If you have access to a frequency counter, it includes

instructions for using it.

( ) To  calibrate the  1685 Hz demodulator  tone, enter  the

following command to your TNC 2:

CALSET 157<cr>

( )       Your TNC 2 should respond with:

CALSET   was 239

cmd:

( )       Type

CALIBRATE<cr>

34

Tucson Amateur Packet Radio                 TNC 2 Assembly Manual

( ) Note that the STA and CON LEDs are extinguished.

(  ) Now place a jumper at JMP9 pins 2 and 5 (the center two

pins).

( ) If you are using a frequency counter, place the probe at

the push-on jumper on JMP9.

( )       The CON LED should illuminate.

( )  SLOWLY rotate the adjustment screw of R79 CCW until the

STA LED blinks.

( )        Carefully  rock the adjustment of R79  until both

the CON and STA  LEDs glow.  This adjustment is  very touchy.  It

should be  possible to get them to  both glow for a  second or so

before they begin alternately blinking.

( ) If you are  using a frequency counter, adjust R79  for a

reading of 3370 Hz.

( ) If you are  using a frequency counter, remove  the probe

from JMP9.

( )       Remove the jumper at JMP9.

( )       Verify that the STA and CON LEDs extinguish.

(  ) Type "Q" and note that  the cmd: prompt appears on your

display.

( ) Install a jumper at JMP8.

If you didn't have access to a frequency counter while performing

the  demodulator calibration above, but  you do have  access to a

triggered-sweep oscilloscope  (even if the  oscilloscope timebase

and/or vertical deflection system isn't well-calibrated), perform

the following 11 steps.

If you  used a  frequency counter to  calibrate the  demodulator,

skip the following 11 steps.

Be sure  you have  completed demodulator calibration  as outlined

above before proceeding.

( ) Place a push-on jumper at JMP4.

( ) Place a push-on jumper at JMP7.

( ) Verify that a push-on jumper is installed at JMP8.

35

Tucson Amateur Packet Radio                 TNC 2 Assembly Manual

( )       Type

CALIBRATE<cr>

on your terminal, then  press "K" followed by "D".

The PTT and DCD LEDs should glow.

(  )  Place the  oscilloscope probe  at the  top end  of R68

(nearest Q12).

( ) You should observe a  square-wave pattern, probably with

jittery edges.

( ) Adjust the timebase so one (1)  cycle of the square-wave

fills  the  screen-edge  horizontal calibrations  marks  exactly.

Thus, if your oscilloscope screen is calibrated 10 division wide,

one cycle of the observed waveform should be 10 divisions wide.

( ) Now,  carefully adjust  R79 so the  transition near  the

center of the oscilloscope screen occurs exactly at the center of

the screen.  Again,  the trace will  probably be jittery, so  use

the middle  of the jittery  area as  if it were  the edge  of the

waveform.

( )  Press the "Q" key  on your terminal.   The PTT  and DCD

LEDs should extinguish.

( ) Remove the jumper at JMP4.

( ) Remove the jumper at JMP7.

If you have  access to a calibrated oscilloscope  or a meter that

accurately reads low level audio, perform the following 7 steps.

( ) Enter the CALIBRATE command.

( ) Place a jumper at JMP4.

( ) Press the "K" key.  The PTT LED should glow.

(  )  Place the  instrument probe  at the  right end  of R56

(5.6k).  This resistor is located in quadrant 3 just below U15.

( ) Adjust  R76 for a  level of about  300 mV peak-  to-peak

(100 mV RMS) at this point.

( ) Remove the jumper from JMP4.

( )       Switch SW1 to the OFF position.

36

Tucson Amateur Packet Radio                 TNC 2 Assembly Manual

Congratulations!  Your TNC 2 modem is calibrated.

The  next steps will  guide you through  a test  known as "analog

loopback" which exercises your  modem as if you were  sending and

receiving signals on-the-air.  In addition to modem testing, this

is  a useful technique for operating your TNC 2 and familiarizing

yourself with its commands  and features without actually placing

a signal on the air.

( ) Verify that a push-on jumper is on JMP8.

( ) Install a push-on jumper at JMP7.

( ) Switch SW1 to the ON position.

(  ) Three LEDs should  illuminate (PWR, CON  and STA), then

CON and  STA should  extinguish.   The DCD  and  PTT LEDs  should

remain off.

( ) After  the sign-on  message and cmd:  prompt, enter  the

following:

MY W1QRM<cr>

( ) TNC 2 should respond with:

MYCALL   was NOCALL

cmd:

( )  Verify that SW2  switch 7  is in the  ON position,  and

ensure that SW2 switches 6 and 8 are in the OFF position.

( )       Enter:

C W1QRM<cr>

( )  You should observe the  PTT and DCD LEDs  blink on then

off, followed by the CON LED illuminating shortly after you enter

the <cr>.  Next, the PTT and DCD LEDs should again blink  on then

off, then the message:

cmd:*** CONNECTED to W1QRM

(with NO cmd: prompt on the next line) should appear.

( )       Now enter:

hello self<cr>

( ) The  PTT, DCD and STA LEDs should  briefly come on, then

the PTT  and DCD  LEDs will  turn  off, after  which the  display

should look like:

37

Tucson Amateur Packet Radio                 TNC 2 Assembly Manual

hello self

hello self

( )  Next, the PTT and DCD LEDs should again blink, then the

STA LED should extinguish.

( ) Now enter a Control-C character, then:

D<cr>

( )       TNC 2 should respond with:

cmd:*** DISCONNECTED

followed by NO cmd: prompt.

( ) The PTT and DCD LEDs should blink on, then  the PTT, DCD

and  CON LEDs should extinguish.  This action occurs very rapidly

and may be hard to see.

( ) Switch SW1 to the OFF position.

( ) Remove the push-on jumper from JMP7.

You have now verified the correct operation of the modem section.

The next  section  deals with  installing  and checking  out  the

battery-backed memory portion of TNC 2.

( )       Install a push-on jumper at JMP5.

( )       Apply power to TNC 2 and switch it on.

(  ) Measure the voltage at the battery (BATT1) terminals of

your TNC 2 PC board.

Measured value:______________________________

( ) The  measured value should  be well under 0.2  volts dc.

If it is more, diode CR16 is probably installed backwards.

NOTE:   If you do not have a microammeter for the following step,

use  a high-impedance (10 Megohms  or more) voltmeter and measure

the voltage  drop across a  100 k ohm  resistor used as  a shunt.

The conversion factor is 10 uA/volt.

( )  Using a microammeter,  measure the current  through the

battery mounting holes on your TNC 2.

Measured value:______________________________

( ) The  value measured  should be less  than 1  microampere

38

Tucson Amateur Packet Radio                 TNC 2 Assembly Manual

(less than 0.1 volt if using the voltmeter and shunt method).

( )       Turn TNC 2 off and remove power.

( )       Remove the jumper at JMP5.

NOTE:  The  Lithium battery used  at BATT1  must be handled  with

extreme care.   NEVER ALLOW  A SHORT CIRCUIT  ACROSS THE  BATTERY

TERMINALS OR THE BATTERY COULD EXPLODE AND CAUSE SERIOUS INJURY!

WARNING:  Once the Lithium battery is installed, NEVER allow your

TNC  to rest  on  a conductive  surface or  you  could short  the

Lithium battery with the attendant dangers!

( ) I understand the danger of the Lithium battery.

( ) Remove the Lithium battery from its envelope.

( ) Note  that there are  two terminals on  one side of  the

battery and  one terminal on  the other side.   At the  side with

only one terminal there  is a red-colored ring.   This red-banded

end is the NEGATIVE terminal of the battery.

(  ) Carefully install  the battery on your  TNC 2 PC board,

then solder it in place.   If you have to straighten the  pins on

BATT1, be careful to not short the battery terminals.

( )       Install a push-on jumper at JMP5.

(  )       Using a high-impedance (at  least 20k ohms/ volt)

voltmeter, measure the voltage across U25 pin 28 to ground.

Measured value:______________________________

( )       The voltage should be greater than 2.5 volts.

(  )         Using a  voltmeter, measure the  voltage across

resistor R33 (the 1k resistor just  below the battery and to  the

right of Q6).

Measured value:______________________________

( )       The voltage should be under 5 millivolts.

( )       Now apply power and switch TNC 2 on.

( )       Measure the voltage across R33 again.

Measured value:______________________________

39

Tucson Amateur Packet Radio                 TNC 2 Assembly Manual

( )       The voltage should be under 5 millivolts.

( )       At the cmd: prompt, enter

MY W1QRM<cr>

( )       Turn off power via SW1.

( )       After a few moments, reapply power.

( )       At the cmd: prompt, enter

MY<cr>

( )       TNC 2 should respond with

MYCALL   W1QRM

( )       Remove power from the TNC.

You have just completed  the battery-backed memory tests.   TNC 2

is now  fully assembled (except for the cabinet).  In the future,

you will only have to enter your call sign one time.   TNC 2 will

remember it for you until you decide to change it.  Likewise, all

other  values that  you set  by non-  immediate commands  will be

retained until you change them.

Now it is time to connect a radio to the TNC.

Please read through the chapter on Radio Interfacing in the TNC 2

System Manual before proceeding.

( )       Radio Interfacing chapter read.

The  TNC 2 radio port is designed  to work with almost all common

(and many not so common) transceivers or transmitter- receivers.

The  instructions  below assume  you are  using the  "Method One"

interface described in  Radio Interfacing.  This is a good way to

get on  the air quickly,  but is  not recommended as  a permanent

means of connecting a radio to TNC 2.

In constructing  the  radio interface  cable,  you will  need  to

obtain  a mating  connector(s)  for your  radio's microphone  and

headphone or external speaker jack(s).  We recommend you keep the

length of the radio interface cable to under about four feet (1.2

m).  Use shielded wire for all audio connections.

( )       Locate the 5-pin male DIN connector pieces.

40

Tucson Amateur Packet Radio                 TNC 2 Assembly Manual

(  )  Prepare  a length  of  shielded  wire.  This means  to

separate the braided shield  from the insulated center conductor,

then strip about 1/4" of insulation from the center conductor.

( ) Solder the center conductor to the audio output terminal

of your radio  headphone/ speaker connector.

( ) Solder the shield to the audio common (or negative power

common,  NOT  audio  common if  audio  common  is  at a  positive

potential with respect to your radio's power "ground").

( ) Slide the  backshell of your headphone/speaker connector

over the shielded wire and fasten to the connector.

( ) Pass the  shielded wire through the insulating  boot and

metal  ferrule  of the  DIN  connector and  carefully  solder the

center conductor to a pin for the DIN connector

( ) Insert this connector pin at pin 4 of the DIN connector.

( ) Prepare another length of shielded wire.

(  ) Solder  the center  conductor to  the microphone  audio

input contact of your radio microphone connector.

( ) Solder the  shield to the microphone audio  common (this

common  MUST be at the same dc potential as the headphone/speaker

audio common as well as dc power common).

( )  Slide  the backshell of your microphone  connector over

the shielded wire.

( ) Pass the  shielded wire through the insulating  boot and

metal  ferrule  of the  DIN  connector and  carefully  solder the

center conductor to a DIN connector pin.

( ) Insert this pin at pin 1 of the DIN connector.

( ) Prepare  a length of unshielded wire the  same length as

the microphone audio wire just completed.

( ) Solder one end of this wire to the PTT (transmitter key)

contact of your radio microphone connector.

(  )  Pass  this   wire  through  the  microphone  connector

backshell and secure the backshell to the microphone connector.

( ) Pass the  other end of this wire  through the insulating

boot and  metal ferrule of the  DIN connector and solder  it to a

41

Tucson Amateur Packet Radio                 TNC 2 Assembly Manual

DIN connector pin.

( ) Insert this pin at DIN connector pin 3.

( ) Solder both shields to a DIN connector pin.

( ) Insert this pin at DIN connector pin 2.

(  ) Inspect the DIN connector  for any stray wires or other

potential causes of shorts within the connector.

( )  Place  the two shield halves  around the DIN  connector

plug, then slide the metal ferrule over the shield halves.

(  ) Slide  the insulating  boot over  the DIN  connector to

secure the connector assembly.

(  ) With  your radio  and TNC  2 switched  off, attach  the

microphone and headphone/speaker connectors to your radio.

( ) Insert the DIN plug in the DIN receptacle on your TNC 2.

Now,  refer to the Radio Interfacing chapter  in the TNC 2 System

Manual and verify that the DCD LED flickers with audio input.

( ) Audio causes DCD LED to flicker.

Congratulations!   You are now finished with the assembly of your

TNC 2!   Refer to the Basic Operation chapter of the TNC 2 System

Manual  for  guidance through  the  procedures  of proper  packet

operation.  And thank you for joining us in...

The Packet Radio Revolution!

42