AutoWeather 2000

(C) 1994,95,96,97 N5KOA

VER 5.x  01/97

(docs updated 01/06/97)

Visit the AutoWx Home Page at:

http://www.apex2000.net/personal/lcb911/main.htm

If the file 2Kx.NEW exists, it contains last minute info...

INTRODUCTION

AutoWeather 2000 is a PC compatible packet program that allows

access to the Ultimeter 500 or Ultimeter 2000 home weather

stations.  Packet users can connect and retrieve almost all the

weather values the Ultimeter is capable of providing and many

functions of AutoWx can be controlled from a remote location via

packet.

AutoWx can even be used without a weather station attached to

collect, record and distribute APRS and AutoWx weather beacons

from other packet stations.

To take full advantage of AutoWx you should read ALL the

documentation. Read the version updates in the .NEW file on the

disk carefully - sometimes that is the only hint of new

features until the documentation is updated.

AutoWeather was written to run in DOS.  The idea was for

AutoWeather to use a minimal personal computer, including only

640K RAM and an XT-type processor.  This should allow more people

to leave their AutoWeather node on full-time without tying up

their "good" computer.  The program provides adequate keyboarding

capabilities.

There are three main functions of AutoWx.  First, it allows

remote users to access weather values.  Second, it supports Bob

Bruninga's Automatic Position Reporting System (APRS) with

automatic weather beacons that show up on the APRS display.

Last, AutoWx records weather values every five minutes in an

efficient binary log file.  The log file can be exported for use

with database and/or graphics display programs.  The log file can

also be read with the Peet Brothers Data Display software, which

graphically displays the values.

Registered users also receive two companion programs, WxRead and

KWeather.  WxRead will convert the binary log file to plain text

files.  KWeather allows access to the Ultimeter while running

paKet 6.1 or the pk/kaGOLD terminal programs.

Versions of AutoWx and KWeather are also available for the older

Ultimeter II.

2

Features of AutoWeather 2000:

Split-screen display shows weather data, and/or APRS

weather beacons from other stations, for the past half

hour, or so, on bottom - packet on top.

Creates binary log files compatible with the Ultimeter

Data Display software.

Continuously displays Hi/Lo Temperatures and Hi Wind with

the time of day.

Records and displays highest wind speed for each 5 minute

interval.

Displays Hi/Lo Barometer for the day

Allows packet stations to connect and request current

weather conditions, including rain, and highs and lows

for the date and longer terms.

Indicates rainfall for hourly, daily, monthly and annual

periods.

Sends weather beacons in APRS format.

Logs weather beacons from other AutoWx or APRS stations.

Creates ASCII text file showing highs and lows for the

date.

Serves as a simple packet terminal program.

Runs on a minimal computer system

and, Many Other Functions...

AutoWx assumes that your computer has COM1 and COM2 serial ports

available.  One must be used for the tnc and the other for the

Ultimeter.

3

INSTALLATION

This section describes the setup necessary to begin using

AutoWeather.  For simplicity, copy only the AutoWx executable

file (.EXE) to it's own subdirectory from the distribution

diskette.  After you get everything running, you can refer to the

Advanced Features section to add optional files/features.

First you must assure that the weather station and tnc are

properly connected and working.  Use any terminal communications

program (such as ProComm, QModem, PacPro, etc.) to verify

connections, baud rates and port assignments.  Write down each of

these parameters for reference during the configuration of

AutoWx.  Note that the weather station is set at 2400 baud.

Neither the weather station nor AutoWx can be adjusted for the

weather station baud rate.

The Ultimeter MUST be in COMPLETE RECORD LOGGING mode prior to

starting the AutoWeather program.  Press and hold the CLEAR +

RAIN keys for three seconds.  From a terminal program (or

TESTPORT if you have the Peet Bros. software), you should see a

steady stream of characters.  (A version of AutoWx included on

the distribution disk uses the DATA LOGGING MODE.  This version

does not provide as much weather information but does allow you

to run AutoWx and APRS at the same time on two different

computers using a common data source.)

The tnc must be in terminal mode - not KISS or HOST mode. ECHO

should be ON, AUTOLF OFF, CSTAMP ON and MAXUSERS 1.  When using a

terminal program to verify the mode, you should see the cmd:

prompt each time you press the ENTER key.  If you do not see the

cmd: prompt, refer to your tnc documentation and correct this

before continuing with AutoWx configuration.

After you verify that the tnc and weather station are working

properly, run the AutoWx program.  (If you do not have a wx

station attached, use the /RCV startup switch; i.e. AW2000 /RCV)

It will detect that there is no prior configuration and will

prompt you for the necessary information.  Each configuration

item is described below, in the order they will appear.

The COM port of the TNC must be identified as (COM) "1" or "2".

The other COM port will automatically be assigned to the

Ultimeter 2000.  AutoWeather does not support COM3 or COM4 at

this time.

Supported baud rates for the TNC port are: 1200,2400,4800 & 9600

If you need a different baud rate supported, contact me for a

custom version.  Remember that this baud rate is the rate the

computer and tnc communicate - not the rate the tnc and radio

communicate over the air.

Your station's latitude and longitude are necessary to beacon in

4

a format recognized by the Automatic Position Reporting System

(APRS by WB4APR).  If you accept the default values of 0000.00N

and 00000.00W, your beacons will not appear on local APRS

screens. There is very little error validation in the entry of

the coordinates so be very careful that they are entered in the

format shown at the config prompt.  Notice that the values are

entered in Degrees, Minutes, and Tenths of Minutes (not Seconds).

The entry for 31d 52m 40s North would be 3152.66N The "N" and "W"

MUST be included.

A temperature adjustment value is included because the Ultimeter

2000 temperature sensor can, in rare cases, be affected if

additional lengths of cable are attached to it.  The default

value for the temperature adjustment is 0 which will be correct

if the display on the U-2000 is correct. [ AutoWx does not

support metric measures at this time ] When prompted for the Log

File Name, your entry should NOT include an extension.  The

extension .LOG is automatically added.  A common Log File Name is

ULTIM.  If you have previously created a log file using the Peet

Bros. TSR software, it is recommended that you do not use the

same filename.  Let AutoWx create a new file the first time you

run it.  The old file can be appended later using the DOS copy /b

command.

You have the option of specifying a default "beacon text" that

will be appended to the end of the wx beacon.  You can enter

whatever you want up to 30 characters or just press ENTER.

The next option is a string of characters that can be used to

remotely turn the WxNode function ON or OFF.  Enter a short code

(make sure it's not something that would normally be seen on the

air like common names) or just press ENTER.

Finally enter your callsign exactly as it appears in the tnc

MYCALL.  This entry is optional but is required it you plan to

send/receive APRS type messages or bulletins.

After the configuration information is entered, it is written to

the ASCII file 2KWX.CFG.  The .CFG file is an ASCII text file and

is pretty self-explanatory.  You can view or edit it to make

changes as long as you adhere to proper syntax.  If, after you

edit the 2KWX.CFG file, things appear to work incorrectly (or not

at all!), delete the 2KWX.CFG file and re-configure AutoWx.  If

it works, don't fix it.

5

You should also include information in the tnc CTEXT to instruct

users how to access the weather information.  For example,

include in your CTEXT something to the effect of,

Type //WX for weather information.   //Bye to disconnect...

Don't forget to turn CMSG ON so that users see this CTEXT when

they connect.  Note that if another AutoWx user connects to you

with this CTEXT, he will be immediately disconnected if he forgot

to toggle his WxNode OFF.  (See details of the ALT-X function.)

One more thing... check and/or set your computer's time clock

with the DOS Date and Time commands.  AutoWx will use both the

DATE and TIME and the Ultimeter clock should be set at least 30

seconds slow or your computer clock should be set at least 30

seconds fast. Otherwise, AutoWx may read the peak windspeed

immediately after the Ultimeter resets it to zero each five

minutes.  *** IMPORTANT *** It is VERY important to keep the two

clocks set correctly with respect to each other. If the computer

clock is behind the Ultimeter clock, the daily Hi's and Lo's will

be inaccurate.

At this point, AutoWx should be ready and running.  Other users

can connect for weather information, the bottom half of the

display will begin showing weather values every five minutes and

values will be logged to the binary log file.  If that's not the

case, see the Trouble Shooting section near the end of this

documentation.  If you've made an error in the configuration

entries, it may be easier to delete the file 2KWX.CFG and start

over than to try and figure out how to edit the file to make

corrections.

TNC PARAMETERS

ECHO ON                 AUTOLF OFF              CSTAMP ON

CTEXT (as needed)       CSMG ON                 HEADRLIN OFF

MCOM OFF                MSTAMP OFF

6

USING AUTOWX  (with Basic Features)

Remote Commands

All remote commands must be preceded by two slashes and most can

be upper or lower case.  These commands will only be recognized

by AutoWx if another station is connected.  AutoWx looks for the

string "*** CONNECTED " to determine if a connection has been

made.  If your tnc does not display this EXACT string (followed

by the station's callsign and SSID) you may have to start AutoWx

with the /NC switch to make the // commands work.

A connected packet station can send any of the valid commands:

//WX  for current weather values

The response from AutoWeather will be:

03-18-1996            15:53

TEMP  62.9 F

HI  64.3      15:19

LO  39.6      06:46

WIND  NW  12.9 MPH

HI NNE  27.7 MPH  10:20

BAROMETER  30.07s

RAIN TODAY   0.00

YEAR   0.30

AutoWx Ver 2K2.7

If you have the optional outdoor humidity sensor

attached, the response will also include humidity and

dewpoint.

If the ambient temperature is 50 or less and windspeed is

at least 4 mph, you will also see the windchill

temperature.

If you are using the Ultimeter 500, which has no

barometer, the BAROMETER line will be omitted.

If you have not created the YTD.DAT file, the Year-to-

date rainfall will be omitted.  (See the Advanced

Features section of the documentation.)

7

//yes   for yesterday's weather values.

The AutoWeather response will be:

Yesterday's weather...

HI Temp   73.9 F        13:55

LO Temp   45.7 F        07:29

Hi Wind ENE  17.8 MPH  13:21

RAIN       0.0

AutoWx Ver 2K2.3

//all   Transmits the last 6 wx beacons the station has heard.

(Same as the information seen with ALT+A but limited to

the last 6 beacons.)

//bar   for the barometer values

The AutoWeather response will be:

3 hour Barometer change is -0.08 inHg.

Hi is 30.18 inHg at 10:50

Lo is 29.84 inHg at 00:00

The Hi and Lo values are for today.  See the Ultimeter

documentation for details on the 3-hour change.

//long  for the long-term weather values

The AutoWeather response will be:

Long Term Hi's/Lo's...

HI Temp   91.8 F      14:10   02/21/1996

LO Temp   14.1 F      06:30   02/03/1996

Hi Wind  W   43.7 MPH    16:23   02/18/1996

Hi Barom  30.70 inHg  01:07   03/08/1996

Lo Barom  29.49 inHg  17:49   03/17/1996

Hi Rain       0.3                03/21/1996

AutoWx Ver 2K2.7c

(these were actual values in West Texas)

See the Ultimeter documentation for details on how to

clear and/or set the long term values.  Also, see the

Advanced Features section for more information about Long

Term Values.

//help  to send the contents of the optional HELP.AWX file (See

the Advanced Features section.)  Actually only //H is

necessary as of version 3.8b

If you have the optional HELP.AWX file present in the

AutoWeather directory the connected user can send //HELP

to see the contents of that file.  (If the file is not

8

present, the user will see the message:  "No help

available..."

The HELP.AWX should be kept as short as possible,

especially considering that a user may be connected via a

network or several digis.  The sample HELP.AWX file on

the distribution disk should be edited to reflect your

own callsign and any other information specific to your

station if you plan to use it.

//ITEMP displays the Inside Temperature

//MSG   New at version 3.0

This command allows the user to send a message which will

be saved to the MSG.AWX file and can be viewed from the

console with ALT+M.  The format for sending a one-line

message to AutoWx is:

//msg This is the text of my message.  It's all on one line.

Obviously, //MSG is the one command that is not sent by

itself on a line.  The msg text must be on the same line.

*NOTE* APRS messages directed to your station are logged

in the MSG.AWX file.  Also, all APRS BULLETINS are logged

there.

//BYE    to disconnect

While a station is connected, the weather Beacon is suspended.

Also, if UPDATES are OFF (ALT-U) a connected user will see the

message, "Weather data temporarily not available...".

Remote SYSOP commands

//BEACON Turns the Beacon function ON/OFF

//BTx %optional text%

Sets the Beacon time interval and, optionally, the beacon

text.  x indicates the number of 5-minute "clicks". //BT4,

for example, means set the beacon interval at 20 minutes

(4 x 5 = 20).

Any text included after //BTx and beginning and ending

with the percent sign (%) will become the beacon text

appended to your weather beacon.  Make sure there is one

space between the x value and the first %.

//CON  Shows the last ten stations that connected

9

//DIGI Allows you to change the UNPROTO path.  For example,

//DIGI <UNP WX VIA digi1, digi2, digi3 would change

the UNPROTO just as if you had typed UNPROTO WX VIA

digi1, digi2, digi3 at the cmd: prompt.  (The

command could actually be used to change ANY tnc

parameter.  Be careful!)  The tnc will be left in

COMMAND mode (at the cmd: prompt) after this command

is issued so you would normally want to disconnect

immediately.  Note that the // commands that can

actually change parameters MUST be sent in capital

letters.

//STATUS Sends information about the current status of AutoWx,

including whether the automatic BEACON is ON or OFF and

what time interval if ON.  It will also show the date and

time the program last started so you can see if power has

been interrupted at a remote site.  (Don't forget to build

an AUTOEXEC.BAT file that will start AutoWx automatically

upon rebooting.)

//TIME hhmmss

As of version 4.3, you can set the DOS clock from a remote

location using this command.  Note that this command MUST

be in CAPITAL LETTERS and the time must be in the exact

format hhmmss.  For hours less than 10, be sure to include

the leading zero.  Example:

//TIME  080423

Sets the DOS clock to 08:04:23 (am).  If AutoWx does not

see the proper syntax, the program will respond with

"Invalid Entry" and the callsign of the remote station

will be indicated with the error message.  If the command

is proper, AutoWx will respond with the new time (and

Date).

//TIME ? will return the DOS time and the Ultimeter time.

10

DISPLAYS and FUNCTIONS

The hi's and lo's for the current date are displayed in the

status line at the bottom of the screen.

ALT+A   Look at (A)ll the other wx beacons heard.  This will list

the last 20 or so beacons.  When the optional file

LOC.AWX is present, the location of each station can also

be indicated.

As of version 4.3, the last 20 wx beacons heard are saved

to a temporary file to be re-loaded when you start AutoWx

again.  If you don't want them to re-load, delete

TBEACON.AWX before running AutoWx.

ALT+B   toggles the APRS beacons ON/OFF.  When toggled ON the

user is prompted for the time interval and any message to

be appended on the end of the beacon.  The status lines

at the bottom of the screen will indicate whether the

beacon is OFF or ON and, if ON, will show the interval.

If you enter "H+L" (without the quotes of course) the

day's High and Low temperatures will be included as the

"beacon text" with the wx beacon.

Beacons can also be turned off/on and interval selected

remotely with the //BEACON and //BT commands documented

under REMOTE COMMANDS for the Sysop.

ALT+C   will copy the WX.HST, xxxx.LOG, and WX.ERR files to

floppy disk drive A: (or drive B: if you started AutoWx

with the /B: switch).  There will be no WX.ERR file

unless an error is generated from within AutoWx.

ALT+D   will "shell" to DOS.  This function allows you to perform

some DOS commands.  Type EXIT at the DOS command prompt

to return to AutoWeather. It will take a few seconds for

the Ultimeter data stream to be read and parsed so be patient.

Shelling to DOS may cause "bad things" to happen, Man.

Don't make a habit of using it.  Just quit AutoWeather

for your DOS tasks unless they are very brief and require

very little memory.

A minor bug sometimes causes the Highs and Lows in the

lower status line to be messed up when you return from

the DOS Shell.  Just hit ALT+Y or ALT+L and the status

line will be returned to normal.

ALT+E   History of your updatEs.  Shows the last 20 or so updates

so you can see what the wx has done over the past hour-

and-a-half.

11

ALT+F   Sends the contents of INIT.TNC.   INIT.TNC is a plain

text file.  Example:  If INIT.TNC consisted of

MONITOR ON

AUTOLF OFF

CSTAMP ON

Then these parameters would be set in the tnc when you

press ALT+F

ALT+G   allows you to enter a one-line Greeting message.  This

message will be sent immediately after any CTEXT message.

Remember that Greetings will be echoed back from the tnc

(unlike CTEXT) so don't put any valid // commands in it.

ALT+H   will display a Help Window

ALT+I   Sets the tnc with computer date/time (Kantronics format)

ALT+J   Deletes the WX.ERR file if it exists.  Look at this file

from time to time (with ALT+V) and delete it if errors

are occasional.  If errors are frequent, we need to try

to determine the source of the errors and this file can

be very helpful to troubleshooting.

ALT+L   will display long term values

The values held in the Ultimeter memory will be displayed

in the top box - the values of the LONG.AWX file (if

present) will be displayed in the bottom box.

NOTE - the syntax of the LONG.AWX *must* be correct or an

error will occur when you use ALT+L.  Use the sample

LONG.AWX file included on the distribution disk.

ALT+M   This command lists the MSG.AWX file (if present).  The

MSG.AWX file is created (or added to) when a connected

user sends message lines with the //MSG command.  An

entry is also made when an APRS message or bulletin is

received.

ALT+O   Send an APRS formatted Message or Bulletin.  Consecutive

messages or bulletins should be consecutively numbered

unless you want to replace a previous bulletin, in which

case you would number the replacement bulletin the same

as the original line to be replaced.  Messages and

Bulletins are transmitted only once - no automatic decay

process as in APRS.  AutoWx will recognize messages

addressed to you from APRS users (if you entered your

callsign correctly in the configuration process) and will

ACK the messages.  Messages will be saved in the MSG.AWX

file (just like a //MSG) and will be indicated with the

MSG flag on the middle status line on the screen.

12

ALT+R   Displays a page showing how many times each station has

been heard since midnite, the time it was first heard and

the time it was last heard.

These counts are for *every* packet heard - direct,

digipeated, whatever.  If the station sends APRS

formatted wx beacons, the number of beacons heard is

displayed in parentheses. (The Wx Beacon Count counts

only once each beacon - regardless of how many times it's

digipeated.)

The Traffic Count keeps up with 100 different stations

but only the 44 with the highest traffic count are displayed.

When you quit AutoWx, the current stats are saved to a

plain text file (TRAFFIC.AWX) and reloaded when you

restart AutoWx.  If you restart AutoWx on another date,

the old TRAFFIC.AWX file is ignored.

ALT+S   clears the scroll back buffer

ALT+T   causes an APRS beacon to be sent immediately.  If beacon

is ON, the interval timer will be reset.

ALT+U   Toggles UPDATES ON/OFF.  I forgot why I put this in but

it will stop logging to the screen and to the binary log

file.  As a matter of fact it's probably useless except

to cause problems somewhere.

ALT+V   Enters the Viewer routine and prompts for a filename.

The file MUST be a plain text (ASCII) file.  Only the

first 450 lines of the file are available in the viewer

and if the filename entered is not found in the current

directory, the Viewer will abort.

The Viewer page (and all other pages) will abort if it

comes time to perform an update/log entry.

ALT+W   turns on Weather beacon logging.  That means that any

APRS or AutoWeather weather beacons seen and logged in

the bottom half of the display will be written to a plain

text file named BEACON.LOG

ALT+X   will toggle the WxNode function OFF/ON as indicated at

the bottom of the screen.  Use this BEFORE you connect to

another AutoWx station that has any valid // commands in

his Connect Text and BEFORE you try to send any //

commands.  Otherwise, your copy of AutoWx will see the

//commands and try to perform them.

ALT+Y   will display yesterday's information in a pop-up window.

ALT+Z   will allow you to change the time on your computer.  It

was originally a programmer tool I used to test with and

13

it was (and still is) pretty primitive.  I did find it

handy though so I decided to leave it in.

You MUST enter the time in the format indicated (no

commas, colons, etc.) and if you enter a time prior to

the current time, it might cause the AutoWx update to

wait for 24 hours.  (If in doubt, quit and then restart

AutoWeather.)

A null response to the prompt leaves the time unchanged.

PgDn    will show the last ten stations that were connected with

you if you have CSTAMP turned on.

PgUp    will display the last few pages of monitored packets.

Press the ESCape key to abort the display.  While in this

"scrollback" screen, press "*" to write the contents of

the scrollback buffer to a file named TMP.AWX which you

can view with the Viewer function (ALT+V)

DEL     The DELETE key will clear all text from the main screen

(like CLS).  On most computers, CTRL+L will clear only

the upper half of the main screen.

All screens called or popped up for functions will return to the

main AutoWeather screen after a period of time or you can press

any key to return to the main screen.

14

LOGGING

AutoWeather maintains a binary log of weather conditions for

every five minutes.  This feature was necessary because the

Terminate-Stay-Resident software that is available with the

Ultimeter (ULTIMTSR) does not share the serial port.  In other

words, if you are running the TSR and then try to read the serial

port with another program, the TSR quits logging. Besides, if you

want to keep the log up-to-date, you have to leave your computer

on all the time.  Running a dedicated computer for the weather

and sharing it with other packet users makes more sense.

The log file will be named whatever you specified during the

configuration of AutoWx.  It can be read, exported or graphically

displayed with the Data Display software available from Peet

Bros.  (You can also read log files with the WxRead program if

you have registered AutoWx.

At midnite on the last day of each month AutoWx will create a new

log file.  The old log file will be named MMYY.LOG where MM =

month and YY = year.  Log files can be appended by using the DOS

Copy command as it applies to binary files.  For example, to

combine the monthly files 0196.LOG and 0296.LOG into a new file

named 2MOS.LOG, use the DOS command:

COPY /b 0196.LOG+0296.LOG 2MOS.LOG

The original files are not altered.  After March, you could add

the 0396.LOG file to create 3MOS.LOG using:

Copy /b 2MOS.LOG+0396.LOG 3MOS.LOG

See your DOS manual for more details on combining binary files.

The U-2000 Data Display program assumes that all log entries are

at five minute intervals.  Therefore, if you suspend logging for

more than a couple of minutes, the Data Display program will be

reading data that was logged after the time displayed in the Data

Display program.  For example:  If you suspended the logger with

ALT-U at 10:28 and did not resume logging until 10:58, the

Display program will show the data logged at 11:00 as if it were

logged at 10:30 and there will be thirty minutes of no data at

the end of the day.

If you plan to NEVER use the log files you can delete it/them as

often as you like.  Just remember... NEVER is a long time; you

might change your mind someday.  Besides,  you will probably get

a year's data on a 360K floppy so the log files don't take up

much disk space.

NOTE:  The logging of data to the log file is suspended if you

toggle Updates OFF (ALT-U).

15

APRS Beacons

When an AutoWx user beacons in APRS format all local APRS users

will see the AutoWx station on their maps just as if the AutoWx

user was running APRS.  You MUST include APRS (or WX) in your tnc

UNPROTO parameter. (i.e.  UNPROTO APRS VIA DIGI1) The beacon will

be digi'd through any valid path specified in the UNPROTO tnc

parameter. Again, UNPROTO should be set to APRS or to APRS VIA

xxxx in order to be identified by stations running APRS.

The APRS beacon format is:

/Txxx = current temperature

Rxxx = Rain since top of the hour (in 1/100ths)

pxxx = Current (today's) rainfall total as indicated

by the Ultimeter 2000

bxxxx = Barometer (tenths of millibars)

hxx  = Humidity (or, "..." if no sensor attached)

gxxx = Gust windspeed (for each 5 minute period)

Automatic beacons can be toggled ON/OFF with ALT-B. When Beacon

is toggled ON, the user is prompted for the interval (in minutes;

default = 30 minutes) and for any text to be tagged on the end of

the weather data.  The Beacon time will be in the nearest 5-

minute increment regardless of the value entered;  for example,

if you enter 9 minutes, the actual beacon time will be 10

minutes.  Beacons are transmitted only when the log file is

updated.

The beacon will automatically include the Longitude and Latitude

from the config file.   If you enter text to be appended to the

beacon (Btext), it will be limited to 30 characters.

The Beacon can be forced to xmit immediately by typing ALT-T.

After using ALT-T, the beacon timer (if Beacon is ON), restarts.

The Beacon can be turned off/on remotely with //BEACON.  Make

sure that you have the correct path specified in UNPROTO prior to

turning the beacon on.  The default time for beacons turned on

REMOTELY is 20 minutes.  You can change this by starting

AutoWeather 2000 with the /RB switch.  You will be prompted for

the new interval.  The new time will remain in effect until the

program is stopped.

If you enable Weather Beacon Logging with ALT-W the program will

capture any monitored APRS-type weather beacons and save them to

an ASCII text file named BEACON.LOG.  While Weather Beacon

Logging is enabled, an asterisk (*) will appear to the left of

the UPDATE status at the bottom of the screen.  Weather Beacon

Logging may not be compatible with slower 8088 class computers

and/or you may have to use a slower baud rate to the tnc.  (See

Tips)

16

AutoWeather also looks for the APRS command string ?WX? when you

are monitoring other packets.  If this weather query is detected

AutoWeather will send an APRS beacon automatically.  This can be

disabled by starting AutoWeather with the /NOWX switch if you do

not want AutoWeather to respond to APRS weather queries.

AUTOWEATHER USERS MUST INCLUDE "APRS" IN THEIR UNPROTO COMMAND

FOR THEIR BEACONS TO BE LOGGED BY APRS AND/OR AUTOWEATHER.

APRS is a map-based packet program by WB4APR, Bob Bruninga,

Annapolis, MD

17

Advanced Features

All the files below are optional.

FKEYS.AWX  If this file is created, the contents will be used

to define F-Keys, F1 thru F10.  The syntax for each

line is:

x, contents

where x is the F-key number and contents is whatever

you want the F-Key to contain.  For example if your

FKEYS.AWX files consists of:

1,MONITOR ON

2,MONITOR OFF

then F1 will turn the MONITOR ON in your tnc and F2

will turn it OFF

If you added the line:

3, UNP APRS VIA DIGI1,DIGI2,DIGI3

F3 would then change the digipath of your tnc

accordingly.

You can use the Viewer (ALT+V) to view the FKEYS.AWX

file if you forget what you programmed into the Fkeys.

YTD.DAT    This file can be created to keep Month-To-Date and

Year-To-Date rainfall amounts regardless of the

amounts shown by the Ultimeter display. The file

should contain two lines.  The first line is the MTD

rainfall amount.  The second line is the YTD rainfall

amount.  For example:

31

250

as the only two lines of the file indicate there has

been 31/100ths inches of rain for the month and 2 and

a half inches for the year.  If YTD.DAT is present,

AutoWx will update it at the end of the month

automatically.

LONG.AWX   This file contains the longterm highs and lows for

whatever period you want.  It must be in the correct

format.

If this file is present, the values will be considered

in the //LONG longterm response. A sample file is

included on your disk.

LOC.AWX    This file contains a lookup table for callsigns/

18

locations for captured weather beacons.  When present,

beacons shown with ALT+A or //ALL can show a location.

For example, if the LOC.AWX file contains:

N5KOA, East Odessa

N5KOA-1, Pleasant Farms

the beacons from those two stations will have East

Odessa or Pleasant Farms at the right margin when you

view the beacons with ALT+A.  Stations receiving a

response to //ALL will also see those locations.  EACH

LINE OF THE LOC.AWX FILE MUST CONTAIN A CALLSIGN A

COMMA AND A LOCATION.  ONLY ONE COMMA PER LINE!  The

file must also be pure ascii text.

INIT.TNC   If this file is present, the contents will be sent to

the tnc with ALT+F.  Make sure the tnc is in "command"

mode and the cmd: prompt is displayed before pressing

ALT+F.  If you create INIT.TNC, place one command on

each line just as the command would be typed at the

cmd: prompt.  Example:

MONITOR ON

AUTOLF OFF

If the INIT.TNC file consisted of just these two

lines, the tnc parameter MONITOR would be turned on

and AUTOLF would be turned off.

If you only change a couple of parameters when you run

AutoWx, you might consider putting them in FKeys

instead of creating INIT.TNC.

If  you want to verify the correct operation of // commands,

connect to your own mailbox and issue the // command while

connected there.  You will get error messages back from the PBBS

but you can see what information is being sent by AutoWx in

response to the // command.  This works if your tnc allows you to

connect to you PBBS internally rather than by rf.  The MAXUSERS

parameter should be set to 1 so it is not recommended to connect

to yourself via rf to check // commands.

Another option for testing is to type:   *** CON at the cmd:

prompt.  This makes AutoWx think it has connected to another

station.  You can then type the // commands and see the AutoWx

response.  Again, the response will include error messages from

the tnc because the AutoWx response is being sent to the tnc

which is expecting a command.

19

AutoWeather is released two ways; a freeware version is one-

generation-old but fully functional.  It can be found at the TAPR

and ARRL ftp sites on the Internet.  The registered version is

the latest version and is accompanied on the diskette by the

KWeather and WxRead programs, all sample files and a few other

related programs.  Registration is $20

AutoWeather may NOT be distributed with any commercial

merchandise without written permission from the author.

Comments and questions can be directed to N5KOA at N5RKN.#WTX

(Odessa, TX) or lcb911@apex2000.net.  Please drop me a line if

you try the program. Disk copies of the freeware version are

available only when accompanied with $5 to pay for the postage,

disk, etc. Do not send diskettes.

Ultimeter home weather stations are products of Peet Bros., Inc.

Peet Bros. is not affiliated with the author of AutoWeather and

does not warranty or promote the use of the program.  (In other

words, don't blame Peet Bros. if AutoWeather does not work like

you think it should.  They have been very supportive of my

project and of the ham population in general.  They have offered

help and encouragement for amateur applications beyond that of

any other home weather station manufacturer.)

Errors/Bugs and other annoyances...

If you start AutoWeather WITH A FLOPPY DRIVE, make sure the

startup floppy is in it's drive all the time.  If the diskette is

not in it's drive when the write buffer is flushed the program

will endlessly loop.   Also, do not attempt to use the ALT-C copy

feature when running from a floppy; the log file is already on

the diskette and a file cannot be copied onto itself! Be sure to

properly Quit AutoWx before removing the diskette to copy or look

at the binary log file.  Unless you Quit AutoWx, the log file

will still be open and could become corrupted.

HF transmissions can interfere with the Ultimeter and bad data

can be passed to the AutoWeather program.  The use of an rf

filter is highly recommended.  (A 30 db filter is available from

PEET BROS.)  AutoWeather attempts to detect and discard invalid

data before it is displayed or logged however, the long term

values in your Ultimeter can be wiped out by rf.

The National Weather Service calculates wind speed using a 3-

minute average of sustained wind.  AutoWeather records the

highest wind speed during a 5-minute period.  The HI WindSpeed

indicated by AutoWeather is closer to a "gust" windspeed if you

want to be technical.  The beacon contains both the current

windspeed and a "gust" speed as of version 4.x

20

TIPS

Create a custom "help" file and look at it on-line using the

Viewer (ALT+V).  Include the FKey assignments if you tend to

forget how you programmed them.

Turn off your computer monitor and leave it off when not sitting

at the keyboard, or look at the SS3.ZIP file for a screen blanker

program. READ THE SS3 DOCS FIRST TO AVOID DISPLAY DAMAGE.

Import the WX.HST file to a database program and create custom

statistical reports and graphs.  The WX.HST file is easy to

import as a "comma-delimited ASCII file" into most popular

database programs and many spreadsheet programs.

If a BBS in your town supports modem connections, consider

leaving your weather node on the BBS frequency (or a gateway

port) so the node can be accessed by telephone, too.

An ASCII error log file (WX.ERR) is created if an error is

generated by AutoWeather 2000. If this file appears in your

directory you can look at and/or delete it. Just make sure it

doesn't get big enough to eat up disk space.

Use BUDCALLS and BUDLIST to minimize traffic on the screen. This

is highly recommended if you are running an 8088 computer and

want to leave MONITOR ON.  Even callsigns that are not included

in the BUDCALLS can still connect to you (at least on the

Kantronics tncs - not necessarily on others).  For example, if

you want to monitor only the local packet stations that are

sending APRS weather beacons, include them in the BUDCALLS and

set BUDCALLS to FROM or BOTH.  You'll see their beacons and

anybody else can connect to AutoWx and get weather info. Using

BUDLIST also maximizes the use of the "scrollback/review" buffer.

Connected users can leave msgs or notes for you while they are

connected and you can scrollback through to read them.

21

Log File Notes

The binary log file is much like the one created by the Ultimeter

Data Logger and is very efficient.  You can probably get a whole

year's data on a 720K diskette.  The AutoWX program must run for

at least ten minutes before any useable data is logged.  Also,

starting and stopping the AutoWX program before any useable data

is logged can cause errors in the display.  The Peet Data Display

program reads a file header in the log file to determine the

starting date and time but all data entries between headers are

assumed to be in five minute intervals.  The AutoWX program

writes a new date/time header each time it starts but if logging

is suspended by the ALT-U toggle, the data may miss the 5 minute

interval and therefore be incorrectly displayed by the U-II Data

Display program. If the logger is suspended for more than 10

minutes, quit the AutoWX program and restart it to avoid the

problem.  Although data may be missing for the suspended period,

the data logged will be displayed with the correct time if you

follow this procedure.

If you want to build up same data fast but don't care if it's

accurate, start AutoWx and then change the computer time after

every update.  For example: start AutoWx at 13:03, then after an

update, change the time to 13:09.  At 1310 another update will

occur and be logged.  Then bump the time up 4 minutes again, etc.

Be sure to set the clock back to the correct time when you finish

experimenting.  You can use ALT-Z to change the time.

22

AutoWeather 2000 Quick Guide

DEL   Clear Screen

PgDn   Show last 10 connected stations

PgUp   Scroll back buffer for packet screen  (* saves it)

ALT - A   Show the last 20 wx beacons heard

ALT - B   BEACON on/off

ALT - C   Copy data files to Floppy

ALT - D   Shell to DOS

ALT - E   Show the last 20 updates

ALT - F   Send contents of INIT.TNC to the tnc

ALT - G   Enter GREETING message for additional "connect text"

ALT - H   Help Window

ALT - I   Set TNC date/time

ALT - J   Deletes WX.ERR file

ALT - L   Show Longterm values

ALT - M   Read messages left with //msg command

ALT - O   Opt to send APRS Messages/Bulletins

ALT - Q   Quit

ALT - R   Traffic Count (like MHeard list)

ALT - S   Clear scrollback buffer

ALT - T   Xmit Beacon

ALT - U   UPDATES off/on

ALT - V   Viewer (for text files only)

ALT - W   Wx Beacon logging  (shows * near UPDATE when on)

ALT - X   Wx Node off/on

ALT - Y   Show Yesterday's values

ALT - Z   Check/Set TIME clock

Startup Options:

/ABx  Automatic Beacon ON at interval of x * 5 minutes

/B:   Copy data files to DRIVE B: instead of DRIVE A: when

using ALT-C

/NOWX Inhibit the AutoWeather response to ?WX? from APRS users

/RCV  Runs AutoWx in a "monitor mode".  For use without a

weather station attached.

/NC   AutoWx responds to //commands even if not "connected"

to another station

/NOCLOCK  Inhibits the on-screen realtime clock

23

Files:

2KWX.CFG  AutoWeather 2000 configuration data.

WX.HST  Data file showing Hi's/Lo's for each date

WX.ERR  Data file created only if program generates an

error. (This file can be deleted at any time.)

Error Codes that may appear in WX.ERR:

#5  - Illegal function call      #64 - Bad file name

#11 - Division by zero           #68 - Device unavailable

#24 - Device timeout             #69 - Comm-buffer overflow

#55 - File already open          #71 - Disk not ready

#57 - Device I/O error           #72 - Disk media error

#61 - Disk full                  #75 - Path/File access error

#62 - Input past end of file

24

HELP

PROBLEM:  When I start AutoWx, it says it can't see my Ultimeter

on the COM port.

SOLUTION: Use a terminal program such as ProComm, Qmodem,

PacTerm, etc. to verify your equipment before running

AutoWx again.

PROBLEM:  My station does nothing when someone sends //WX

SOLUTION: The tnc MUST indicate a connect with "*** CONNECTED

to...." before AutoWx recognizes that you are

connected.  You MUST be connected to another station

before //commands are valid. Try starting AutoWx with

the /NC switch if you can't get it to respond any other

way.

PROBLEM:  When I use PgDn to see the last ten connected stations,

there is no Date or Time indicated.

SOLUTION: Make sure the CSTAMP (or equivalent) parameter is ON.

If there is no Date and Time with the ***CONNECTED msg,

there will be none in the PgDn list.  Also, check the

HeaderLine parameter of your tnc.

PROBLEM:  When I start AutoWx I see the weather data but I can't

seem to control my tnc.

SOLUTION: The tnc MUST be in terminal mode - not KISS or HOST

mode.  Run a terminal program such as ProComm or QModem

to verify correct operation of the tnc.  You should see

the cmd: prompt if the tnc is in terminal mode.  Make

sure the baud rates of AutoWx and the tnc match.

PROBLEM:  I am sending the weather beacon but APRS users are not

seeing me on their map.

SOLUTION: Make sure your tnc UNPROTO parameter begins with

"APRS". (No quotes, of course.)  Example: APRS V DIGI

This will cause the beacon to digipeat thru DIGI with

the required syntax to be recognized by APRS stations.

Also, if you added text to the end of your beacon and

it is too long, APRS will ignore the beacon.

As of version 7.7, APRS uses a new beacon format and

this version of AutoWx uses it also.  Older versions of

APRS and AutoWx may not recognize the new format

25

PROBLEM:  I don't have COM1 and COM2 available

SOLTION:  You will have to reconfigure something else if you want

to use AutoWx.  My compiler supports only COM1 and COM2

and the add-on libraries I've been able to locate don't

handle multiple open ports well at all.  Some software

allows you to use COM3 and COM4 by fooling the hardware

with software pokes but you can't change the port

addresses effectively that way.

PROBLEM:  Other wx station beacons are not displayed properly on

my screen.

SOLUTION: Make sure your tnc parameters are set correctly.

Especially MCOM OFF - AutoWx does not want to see the

frame ID's such as <UI> in the header.  HEADERLN OFF -

The header and text of the packet should be all on one

line.  CSTAMP ON - AutoWx uses the date and time stamp

when a user connects.  You should see the connected

station's callsign followed by date and time in

brackets. i.e. [08/01/96 16:34:43]

PROBLEM:  The program seems to stop logging to the binary log

file from time to time.  (And the program may crash)

SOLUTION: Rename or move the old .log file and let AutoWx create

a new one.  It's possible that the old .log file got

corrupted.

I've enjoyed hearing from lots of AutoWx users and their ideas

keep new features in the works.  I would like to hear any

comments, complaints and suggestions.

Les Blalock, N5KOA    lcb911@apex2000.net

1711 Pagewood

Odessa, TX  79761

(915) 332-7112 day

332-1942 fax

367-0408 eve  after 6 pm CST

26

INDEX

Advanced Features             18   MSG.AWX file                  12

ALT functions                 11   Optional files                18

APRS Bulletin                 12   rain (YTD or MTD)             18

APRS Message                  12   RCV                            4

APRS wx beacons               16   registration                  20

baud                           4   Remote Commands                7

beacon format                 16   remote control code            5

beacon interval               16   Remote SYSOP Commands          9

beacon text                    5   Scrollback buffer             13

beacon text                    9   scrollback buffer (capture)   14

beacon text                   16   Startup Options               23

BEACON.LOG file               13   Switches (startup)            23

BEACON.LOG file               16   TBEACON.AWX file              11

binary log file               15   temperature adjustment         5

binary log files (copying)    15   testing // commands           19

callsign                       5   time                           6

clear screen                  14   time                          10

clock                          6   time                          14

clock                         10   TIPS                          21

COM PORT                       3   TIPS                          25

CTEXT entry                    6   TNC                            4

digipath                       9   TNC PARAMETERS                 6

digipaths                     16   tnc time                      12

FKEYS.AWX file                18   Traffic Count                 13

H+L                           11   TRAFFIC.AWX file              13

HELP.AWX file                  8   Trouble Shooting              25

highs/lows                    11   Ultimeter mode                 4

INIT.TNC                      11   unproto                       16

INIT.TNC file                 19   Viewer                        13

initialize tnc                11   Viewer                        18

KWeather program              20   wx query                      16

Latitude                       4   WX.ERR file                   12

latitude                      16   WxRead program                20

LOC.AWX file                  11   YTD.DAT                        7

LOC.AWX file                  19   YTD.DAT file                  18

log file (name)                5   /NC switch                     7

logging file                  15   //commands                     7

LONG.AWX file                 12

LONG.AWX file                 18

Longitude                      4

longitude                     16

metric                         5

Monitor Mode                   4

MSG.AWX file                   9

27